3GPP TSG-RAN WG2 Meeting #129bis                                                   R2-2501902

Wuhan, China, 7th –11th April, 2025

Source:	CATT

Title:	Consideration on on-demand SIB1 issues

Agenda Item:	8.5.3

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the details on remaining issues on OD-SIB1case 2. The contribution is organized as follows.

In section 2, we discuss the following aspects,

- How to handle RRC_CONNECTED UE

- One case for cell selection

- Cell barring for NES OD-SIB1 Cell

- SIB1 transmission for emergency case

- Paging issue

- Clarification on Cell status

- Whether SIBxx is essential SIB

Our proposals are summarized in Section 3.

Discussion

In this section, regarding the achieved agreements in [1], the further issues associated to RAN2 are listed and discussed for OD-SIB1.

  - How to handle RRC_CONNECTED UE

In RAN2#129 meeting, the following agreement has been achieved for NES UE in RRC_CONNECTED state in RLF case:

RAN2#129 agreements: Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running: - When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid. - The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition. - The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

Besides, the following agreement was reached on the SIB1 acquisition for UE in RRC_CONNECTED state:

RAN2#129 agreements: For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

However, there is one open issue for the case that the searchSpaceSIB1 is configured and SIB1 is not broadcasting. In our understanding, the agreement for the case that searchSpaceSIB1 is not configured can be extended to cover the case that the searchSpaceSIB1 is configured. Considering that the NES Cell is in power saving mode, it can be predicted that the number of UE in RRC_CONNECTED stat in NES Cell is not large, so the signaling overhead to send dedicatedSIB1-Delivery message to UE is acceptable.

Proposal 1: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting, rely on the NW implementation to deliver SIB1.

  - Normal cell selection

Besides performing cell selection in RLF case, R19 NES UE may power on and perform cell selection at a NES Cell which is transmitting the OD-SIB1 triggered by other UEs or broadcasting periodic SIB1 message, the R19 NES UE behavior should be defined if it could camp on NES Cell. In our view, even it is a corner case and the UE cannot trigger the WUS, but the NES UE could read out the OD-SIB1 triggered by other UEs, so the UE is able to camp on the NES Cell if allowed.

Proposal 2: In normal cell selection, R19 NES UE without WUS configuration can camp on NES Cell if it can acquire SIB1 (e.g., as requested by other R19 NES UEs) from the cell.

  - Cell barring for NES OD-SIB1 Cell

In legacy R18 NES as follows, network introduced a specific barring indication cellBarred in SIB1 for NES UEs, the NES UE will read the cellBarredNES to see whether the UE (with NES DTX/DRX feature) will be barred or not if cellBarred in the acquired MIB is set to barred. Based on this indication, the NES Cell working on CellDTX-DRX feature could allow only NES UEs supporting cell DTX/DRX capability to camp on the NES Cell by broadcasting cellBarredNES.

TS 38.331 5.2.2.4 Actions upon receipt of System Information <skip> 1> if the UE supports nes-CellDTX-DRX and it is in RRC_IDLE or in RRC_INACTIVE, or if the UE supporting nes-CellDTX-DRX is in RRC_CONNECTED while T311 is running: 2> if cellBarred in the acquired MIB is set to barred: 3> if cellBarredNES is absent in the acquired SIB1: 4> consider the cell as barred in accordance with TS 38.304 [20]; 4> perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20], upon which the procedure ends; <skip> – SIB1 SIB1-v1800-IEs ::= SEQUENCE { … cellBarredNES-r18 ENUMERATED {notBarred} OPTIONAL, -- Need R … } … cellBarredNES This field indicates the cell barring status for UEs supporting nes-CellDTX-DRX as described in 5.2.2.4.2..

Observation 1: A barring indication in SIB1 is used for R18 NES UE with Cell DTX/DRX feature.

Agreements on OD-SIB1 There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

For R19 NES Cell, it has been agreed that the legacy UE and R19 UE not supporting OD-SIB capability will be barred by the NES Cell, and k_ssb signaling “no SIB1” indication in MIB will be used. It seems that all the UE not supporting OD-SIB1 capability will be barred by the k_ssb at NES Cell. There is also no good reason for NES Cell to bar the NES OD-SIB1 UE, otherwise, no UE could camp on OD-SIB1 Cell. So it is suggested that RAN2 to clarify the barring mechanism is used or not at NES Cell for NES OD-SIB1 UE. For non OD-SIB1 Cell, the legacy mechanism could be used to bar all the UE including NES OD-SIB1 UE, the UE supporting OD-SIB1 capability could follow the legacy procedure to determine whether the non OD-SIB1 Cell is barred or not.

Proposal 3: RAN2 clarifies whether there is a scenario where the access of NES OD-SIB1 UEs is restricted on NES OD-SIB1 Cell.

  - SIB1 transmission for emergency case

According to legacy procedure, if UE is ETWS capable or CMAS capable and receives the etwsAndCmasIndication bit in the Short Message, it will re-acquire the SIB1 immediately.

Observation 2: In legacy, UE needs to acquire the SIB1 immediately when receiving the etwsAndCmasIndication in Short Message, which is beneficial for latency.

In this case, the network will change to normal mode other than NES mode, or the network will broadcast the SIB periodically as agreed in last meeting, the NES UE could receive the SIB1 immediately without any delay. Then, there is no issue is explored, it could be considered as network implementation.

Proposal 4a: In emergency cases, it is up to network implementation to switch to normal mode or broadcast the SIB periodically.

Proposal 4b: UE acquires the SIB1 immediately on OD-SIB1 enabled NES cell without sending WUS request when receiving the etwsAndCmasIndication in Short Message.

  - Paging issue

Paging reachability

When NES UE triggers one OD-SIB1 procedure to NES Cell from Cell A for cell reselection, it needs to monitor the RAR and OD-SIB1 from NES cell before camping on the NES Cell. Meanwhile, the UE may need to monitor paging message on its camping Cell A. To avoid missing paging message, the UE will try to select the suitable occasions for PRACH transmitting and RAR/OD-SIB1 monitoring, which are not overlapped with paging occasion on Cell A. Even if the paging is missed during the OD-SIB1 acquisition procedure, UE could also receive the paging message transmitted several times by network after camping on NES Cell. So no big issue on paging reachability needs to be further discussed, it is UE implementation to avoid the failed paging.

Proposal 5: It is up to UE implementation to avoid the overlapped occasions for monitoring paging message and preamble/RAR/OD-SIB1 in OD-SIB1 requiring procedure.

Unnecessary Paging for legacy UE

RAN2#127bis meeting agreement: 9. Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced. RAN2#128 meeting agreement: Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

RAN2 has agreed several methods as above to make legacy UE not camp on OD-SIB1 Cell, then the paging for legacy UE seems not necessary at NES Cell. It is also network power saving if the paging for legacy UE is not transmitted at NES Cell. Hence, considering the legacy UE is not allowed to camp on NES Cell, also from the view of network power saving, it can be concluded that the NES Cell will not transmit the legacy UE paging message.

Observation 3: The paging message for legacy UE is not necessary to be transmitted at OD-SIB1 Cell.

In RAN2#129 meeting, the following was agreed for R19 paging adaptation:

5. A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

Similarly, in order to avoid OD-SIB1 Cell paging legacy UEs, a new UE capability for support of OD-SIB1 can be introduced in UE-RadioPagingInfo. As UE-RadioPagingInfo is included in the CN paging and RAN paging NW nodes, OD-SIB1 Cell can decide whether to initiate paging procedure for a UE based on  the new UE capability when CN paging or RAN paging is triggered. For example: When OD-SIB1 Cell receives a PAGING message from AMF and the new UE capability for support of OD-SIB1 is also included, OD-SIB1 Cell initiates paging procedure for the UE via Uu interface. Otherwise, OD-SIB1 Cell doesn’t initiate paging procedure via Uu interface as it identifies the PAGING message is for a UE that doesn’t support OD-SIB1 feature.

Proposal 6: A new UE capability for support of OD-SIB1 is introduced in UE-RadioPagingInfo.

  - Clarification on Cell status

RAN2#127bis agreements 7. A cell for which SIB1 request configuration is available, can periodically broadcast SIB1. 8. If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

Based on RAN2#127 agreements, the cell could either transmit SIB1 by OD-SIB1 request procedure or periodically broadcast SIB1. For UE, it will check whether SIB1 is broadcasted or provided before requesting SIB1 of that cell. RAN1 also discussed how the UE check SIB1. Based on current progress, it can be seen that the Cell with on-going periodically broadcast SIB1 in above agreement is NES Cell, as the agreement said that the SIB1 request configuration is still available to this Cell. So, when UE performs above check procedure, it is possible that it is NES Cell, i.e., a Cell with SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2. The UE could try to read the SIB1 based on its implementation, no extra explicit or implicit indication is needed to indicate the SIB1 status, such as whether SIB1 is currently being broadcasted or provided on demand.

Proposal 7： The NES Cell on which SIB1 is currently being broadcasted regularly could still be NES cell, i.e., a cell where K_SSB>=24 for FR1 or K_SSB>=13 for FR2.

Proposal 8：It is up to UE implementation to determine if SIB1 is currently being broadcasted or not. i.e., No indication is required to indicate whether SIB1 is currently being broadcasted or provided on demand.

  - Whether SIBxx is essential

5.2.2.5 Essential system information missing The UE shall: 1> if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running: 2> if the UE is unable to acquire the MIB or SIB1: 3> consider the cell as barred in accordance with TS 38.304 [20]; 3> perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20]. NOTE 1: The SIB19 is essential for NTN access. If UE is unable to acquire the SIB19 for NTN access, the action is up to UE implementation (e.g., cell re-selection to other cells). NOTE 2: The SIB22 is essential for ATG access. If UE is unable to acquire the SIB22 for ATG access, the action is up to UE implementation (e.g., cell re-selection to other cells). Editors notes: FFS if SIBxx is essential system information

Based on above RAN2#129 Post Email [102], one FFS issue is whether SIBxx is essential system information or not.

For NTN and ATG, if the UE is unable to acquire the corresponding system message, it means that the UE could not obtain the essential parameters for the ATG or NTN features. Besides, the NTN/ATG Cell will always broadcast the SIB19/SIB22 for NTN/ATG UE accessing. Thus, it is reasonable to set the SIB19/SIB22 as essential system information.

For Cell A, obviously, the SIBxx is not necessary to be essential system information, the UE could access to Cell A even if UE could not acquire the SIBxx in Cell A. For NES UE with OD-SIB1 feature, if it camps or handover to a normal Cell not with SIBxx, it could fallback to legacy UE mode, so, the SIBxx is also not to be essential system. For OD-SBI1 NES Cell, SIBxx may only include UL WUS configurations for serving cell. A UE reselected to the OD-SIB1 NES Cell has acquired UL WUS configurations from Cell A and has acquired SIB1 of the OD-SIB1 NES cell successfully. If the UE doesn’t need to acquire SIB1 again during the period it camped on the OD-SIB1 NES cell, it doesn’t matter if it is unable to acquire SIBxx. Hence, it is suggested not to consider the SIBxx as essential system information.

Proposal 9: SIBxx is not essential system information.

Conclusion

According to the analysis in section 2, our observations and proposals are summarized as follows:

Observation 1: A barring indication in SIB1 is used for R18 NES UE with Cell DTX/DRX feature.

Observation 2: In legacy, UE needs to acquire the SIB1 immediately when receiving the etwsAndCmasIndication in Short Message, which is beneficial for latency.

Observation 3: The paging message for legacy UE is not necessary to be transmitted at OD-SIB1 Cell.

Proposal 1: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting, rely on the NW implementation to deliver SIB1.

Proposal 2: In normal cell selection, R19 NES UE without WUS configuration can camp on NES Cell if it can acquire SIB1 (e.g., as requested by other R19 NES UEs) from the cell.

Proposal 3: RAN2 clarifies whether there is a scenario where the access of NES OD-SIB1 UEs is restricted on NES OD-SIB1 Cell.

Proposal 4a: In emergency cases, it is up to network implementation to switch to normal mode or broadcast the SIB periodically.

Proposal 4b: UE acquires the SIB1 immediately on OD-SIB1 enabled NES cell without sending WUS request when receiving the etwsAndCmasIndication in Short Message.

Proposal 5: It is up to UE implementation to avoid the overlapped occasions for monitoring paging message and preamble/RAR/OD-SIB1 in OD-SIB1 requiring procedure.

Proposal 6: A new UE capability for support of OD-SIB1 is introduced in UE-RadioPagingInfo.

Proposal 7： The NES Cell on which SIB1 is currently being broadcasted regularly could still be NES cell, i.e., a cell where K_SSB>=24 for FR1 or K_SSB>=13 for FR2.

Proposal 8：It is up to UE implementation to determine if SIB1 is currently being broadcasted or not. i.e., No indication is required to indicate whether SIB1 is currently being broadcasted or provided on demand.

Proposal 9: SIBxx is not essential system information.

Reference

[1] Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES, Samsung, RAN2#129 meeting.