3GPP TSG-RAN WG2 Meeting #129bis                                                              R2-2501903

Wuhan, China, 7th - 11th April 2025

Source:	CATT

Title:	Adaptation of Common signal channel transmissions

Agenda Item:	8.5.4

Document for:	Discussion and Decision

Introduction

In RAN2#129 meeting, some agreements on paging adaptation, SSB adaptation and PRACH adaptation were achieved.

In this contribution, remaining issues on paging adaptation and SSB adaptation are discussed.

Discussion

Paging adaptation

- Extension of firstPDCCH-MonitoringOccasionOfPO

The following is the legacy PMO (i.e. firstPDCCH-MonitoringOccasionOfPO) configuration [1].

[[ firstPDCCH-MonitoringOccasionOfPO CHOICE { sCS15KHZoneT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139), sCS30KHZoneT-SCS15KHZhalfT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279), sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559), sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119), sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239), sCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479), sCS120KHZoneEighthT-SCS60KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959), sCS120KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919) } OPTIONAL -- Cond OtherBWP ]], //skip firstPDCCH-MonitoringOccasionOfPO-v1710 CHOICE { sCS480KHZoneEighthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839), sCS480KHZoneSixteenthT SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679) } OPTIONAL, -- Need R

In legacy paging configurations, the maximum value of Ns is 4 and the maximum number of bits for each PMO is 17, where the maximum offset of the first PDCCH monitoring occasion for paging of each PO of the PF is 16 radio frames. The maximum signaling overhead for PMO configuration is maximum value of Ns*bits for each PMO (4*17=68 bits).

In RAN2#129 meeting, the following agreements were achieved on N and Ns.

1. For N, values smaller than T/32 are not supported. 2. The maximum possible value for Ns is 8.

firstPDCCH-MonitoringOccasionOfPO is configured to point out the first PDCCH monitoring occasion for paging of each PO of the PF. When the maximum number of Ns is extended to 8, the maximum PO per PF (i.e. maxPO-perPF) needs to be extended to 8 correspondingly. Although POs are expected to be clustering together, the possible values of the first PDCCH monitoring occasion for R19 paging of each PO of PF need to be extended correspondingly as well as they may be clustered in any time domain within 32 radio frames. The maximum number of bits for each PMO needs to be extended to 18. With these extensions, the maximum signaling overhead for firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation is 144 bits (8*18),   which is acceptable to the limited massage size of SIB1.

Proposal 1: Introduce a separate firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation:

- The maximum PO per PF is extended to 8 ;

- The maximum offset value is extended to 32 radio frame at symbol level.

- Extension of firstPDCCH-MonitoringOccasionOfPEI-O

In RAN2#129 meeting, it was agreed to introduce a separate PEI configuration. And it is FFS whether to introduce a separate pei-ConfigBWP for paging adaptation.

As shown below, within pei-ConfigBWP, there are two fields, including pei-SearchSpace and firstPDCCH-MonitoringOccasionOfPEI-O.

//skip pei-ConfigBWP-r17 SEQUENCE { pei-SearchSpace-r17 SearchSpaceId, firstPDCCH-MonitoringOccasionOfPEI-O-r17 CHOICE { sCS15KHZoneT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139), sCS30KHZoneT-SCS15KHZhalfT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279), sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559), sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..1119), sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239), sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479), sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959), sCS480KHZquarterT-SCS120KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919), sCS480KHZoneEighthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..35839), sCS480KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..71679) } } //skip maxPEI-perPF-r17 INTEGER ::= 4 -- Maximum number of PEI occasion per paging frame

The same search space for paging is applied to both legacy paging monitoring and R19 paging monitoring. It can allow legacy and R19 UEs to co-exist in the same PF/PO. Similarly, the same search space can be applied to both legacy PEI and R19 PEI with paging adaptation. Then legacy and R19 UEs can co-exist in the same PEI.

Proposal 2: No need to introduce a separate pei-SearchSpace for R19 paging adaptation.

firstPDCCH-MonitoringOccasionOfPEI-O is used to indicate the offset from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O with symbol level. Similar as the extension of firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation, a new parameter to extend firstPDCCH-MonitoringOccasionOfPEI-O for R19 paging adaptation needs to be introduced. The maximum number of PEI occasion per paging frame is extended to 8, while the maximum value of the offset, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O, is extended to 32 radio frame at symbol level.

Proposal 3: Introduce a separate firstPDCCH-MonitoringOccasionOfPEI-O for R19 paging adaptation:

- The maximum number of PEI occasion per paging frame is extended to 8

- The maximum offset value is extended to 32 radio frame at symbol level.

- Configuration restrictions of pagingAdaptationNAndPagingFrameOffset

The following is the ASN.1 for pagingAdaptationNAndPagingFrameOffset. And it is FFS the field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions.

pagingAdaptationNAndPagingFrameOffset CHOICE { oneT NULL, halfT INTEGER (0..1), quarterT INTEGER (0..3), oneEighthT INTEGER (0..7), oneSixteenthT INTEGER (0..15), oneThirtySecondT INTEGER (0..31) }

Compared with legacy paging, R19 paging adaptation must increase interval between PFs. Therefore, we prefer to add the restriction in the field description that the number of total paging frames in T for paging adaptation should be less than that for legacy paging (i.e. the configuration within nAndPagingFrameOffset). Furthermore, with the restriction, “oneT” is not possible to be configured for pagingAdaptationNAndPagingFrameOffset. It can be removed from pagingAdaptationNAndPagingFrameOffset.

Proposal 4: Add a restriction in the field description of pagingAdaptationNAndPagingFrameOffset, i.e., that the number of total paging frames in T with should be less than that with nAndPagingFrameOffset.

Proposal 5: “oneT” is removed from pagingAdaptationNAndPagingFrameOffset.

SSB adaptation

In RAN2#129 meeting, the following agreements were made:

Agreements on SSB adaptation 1. RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell. 2. RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. 3. For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

In RAN1#120 meeting, the following agreements on SSB adaptation were made:

Agreement For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

According to RAN1’s agreement, UE is configured with X additional SSB burst periodicity for an SCell for SSB adaptation. Then the network may use DCI format 2_9 to switch the SSB burst periodicity for SSB adaptation for the SCell. In order to support the agreement that RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation, Y (Y<=X) additional SMTC configuration(s) for L3 RRM measurement on SCell with SSB adaptation need to be configured to the UE together with the mapping relationship between X additional SSB burst periodicity with Y (Y<=X) additional SMTC configuration(s) via RRC. Then upon reception of DCI format 2_9 for switching one of X additional SSB burst periodicity for the SCell, SMTC configuration for L3 RRM measurement on the SCell is updated accordingly.

Proposal 6: For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

Proposal 7: Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

Conclusion

According to the analysis in section 2, we have the following observations and proposals:

Paging Adaptation

Proposal 1: Introduce a separate firstPDCCH-MonitoringOccasionOfPO for R19 paging adaptation:

- The maximum PO per PF is extended to 8 ;

- The maximum offset value is extended to 32 radio frame at symbol level.

Proposal 2: No need to introduce a separate pei-SearchSpace for R19 paging adaptation.

Proposal 3: Introduce a separate firstPDCCH-MonitoringOccasionOfPEI-O for R19 paging adaptation:

- The maximum number of PEI occasion per paging frame is extended to 8

- The maximum offset value is extended to 32 radio frame at symbol level.

Proposal 4: Add a restriction in the field description of pagingAdaptationNAndPagingFrameOffset, i.e., that the number of total paging frames in T with should be less than that with nAndPagingFrameOffset.

Proposal 5: “oneT” is removed from pagingAdaptationNAndPagingFrameOffset.

SSB Adaptation

Proposal 6: For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

Proposal 7: Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

Reference

- TS 38.331 v18.4.0