3GPP TSG-RAN WG2 Meeting #129bis                                                R2-2501917

Wuhan, China, Apr. 7th – 11th, 2025

Source:	CATT, Vivo, Lenovo, Huawei, HiSilicon, CMCC

Title:	New SHR Trigger for CHO with Candidate SCGs

Agenda Item:	8.10.2.2

Document for:	Discussion and Decision

1. Introduction

In RAN2#129 meeting, MRO enhancements for CHO with candidate SCGs were discussed and some progress has been achieved [1]. But regarding whether to introduce a new SHR trigger for CHO with candidate SCGs [2], there was still no conclusion.

In this contribution, we continue the discussion on the benefit and necessity of new SHR trigger for CHO with candidate SCGs.

2. Discussion

In case of CHO with candidate SCGs, both CHO and CPAC are configured with execution conditions, and only when the CHO and CPAC execution conditions are fulfilled at the same time, the UE triggers the CHO with candidate SCGs execution. If one execution condition (e.g., the execution condition of PSCell change) is not fulfilled for a long time while the other execution condition (e.g., the execution condition of PCell chagne) is fulfilled, the UE will not execute the CHO with candidate SCGs. This may cause an unexpected situation, i.e., the CPAC blocks the CHO execution, or vice versa. The ideal situation is the execution conditions of CHO and CPAC can be fulfilled as closely as possible in terms of time.

Observation 1: The ideal situation for CHO with candidate SCGs is the execution conditions of CHO and CPAC are fulfilled as closely as possible in terms of time.

In previous meetings, it was agreed that UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO) in RLF report, SHR report and SCGFailureInformation. For RLF report and SCGFailureInformation, since the failure already occurred, there is no need to introduce a separate trigger. But for a SHR report, this assistance information can be missed due to the lack of specific trigger condition of SHR, so there is no opportunity for network to avoid this potential failure in the future.

Agreements - UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS. Agreements - RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements.

Without this further enhancement of specific trigger, SHR logging still follows existing specification, i.e., the elapsed time of a running T304/T310/T312timer exceeds the T304/T310/T312 threshold. Therefore, the UE will only record the time gap between first met condition (CHO or CPAC) and the second met condition (CPAC or CHO) in SHR when the T310, or T312, or T304 trigger condition is met. In other words, if no T310, T312, or T304 trigger condition has been met, the UE will not log the SHR, let alone the time gap information.

Observation 2: The time gap information between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO) will not be recorded and reported if the existing trigger condition of SHR is not met (e.g., T310, T312, or T304 threshold).

In our understanding, the intention to support logging the time gap information is that this information is important to optimize the corresponding configurations of CHO/CPAC execution conditions. If the time gap is already large due to the improper configurations, this information should be recorded in SHR. Therefore, we suggest introducing a specific SHR trigger condition for successful CHO with candidate SCGs execution, i.e., the time gap between fulfilled CHO execution condition and fulfilled CPAC execution condition exceeds a configured threshold. In this way, network will not miss the time gap information when it’s unexpectedly large.

Proposal 1: In order for the network not to miss abnormal time gap information, introduce a specific SHR trigger condition, i.e., the time gap between fulfilled CHO execution condition and fulfilled CPAC execution condition exceeds a configured threshold.

3. Conclusion

According to the discussion in section 2, we have the following observations and proposal:

Observation 1: The ideal situation for CHO with candidate SCGs is the execution conditions of CHO and CPAC are fulfilled as closely as possible in terms of time.

Observation 2: The time gap information between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO) will not be recorded and reported if the existing trigger condition of SHR is not met (e.g., T310, T312, or T304 threshold).

Proposal 1: In order for the network not to miss abnormal time gap information, introduce a specific SHR trigger condition, i.e., the time gap between fulfilled CHO execution condition and fulfilled CPAC execution condition exceeds a configured threshold.

4. Reference

- R2-2501702, RAN2#129 Meeting Report, MCC

- R2-2500235, MRO Enhancements for CHO with Candidate SCGs, CATT