3GPP TSG-RAN WG2 #129bis	R2-2501929

Wuhan, China, 7th– 11th April 2025

Agenda Item:	8.6.2

Source:	MediaTek Inc.

Title:	Security discussion and other remaining issues of inter-CU LTM

Document for:	Discussion, Decision

Introduction

This contribution further discusses the security handling for inter-CU LTM (non-DC) and other remaining issues for inter-CU LTM.

Discussion

LS on the security handling for inter-CU LTM (non-DC)

In the coming LS from SA3 (S3-251124), SA3 confirm that the NCC can be transmitted to UE via MAC-CE message, even though there may be some risks of attack and potential signal loss.

S3-251124 Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future.

Therefore, the options of using MAC CE to deliver NCC value for security update can be further discussed.

According to the original LS (R2-2404037) to SA3, the related options and descriptions are as follows:

R2-2404037 Option 1: Use a new information in MAC CE to deliver the security information. Whether the UE uses horizontal or vertical key derivation is derived from this new information in MAC CE (which is currently, neither integrity protected nor ciphered). Option 1A: the NCC value to be used at inter-CU LTM execution is included in the LTM cell switch command MAC CE. Option 1B: the UE is preconfigured with a list of NCC values in a ciphered and integrity protected RRC message and the index of an NCC value in the list is included in the LTM cell switch command MAC CE.

In the LS from SA3, they highlight that the NCC can be transmitted to the UE in plaintext via MAC CE message. In our understanding, this excludes the original Option 1B, which preconfigures an NCC list in RRC and uses an NCC index in MAC CE. Instead, the NCC value should be directly included in the MAC CE.

Proposal 1: Based on the SA3 LS, the NCC value is included directly in the MAC CE. (i.e., no pre-configured NCC list and NCC index are introduced)

There may be some options to indicate the NCC value in the MAC CE:

- Option 1: Reuse and extend the R18 LTM cell switch command MAC CE to be compatible with R19 inter-CU LTM

- Option 2: Introduce R19 LTM cell switch command MAC CE for inter-CU LTM.

- Option 3: Introduce R19 NCC MAC CE for inter-CU LTM. When inter-CU LTM cell switch is triggered, the R18 LTM cell switch command MAC CE + R19 NCC MAC CE is sent to UE.

If Option 1 is feasible, it is naturally better than Options 2 or 3. Let's first discuss the feasibility of Option 1.

Feasibility to extend R18 LTM CSC MAC CE

The R18 LTM Cell switch command MAC CE has variable size with the following fields.

In the fields, there are reserved bits on Oct 3, Oct 4 and Oct 7. However, the octet 4 and 7 are optionally present, depending on whether there are separate TCI state, or CFRA configuration.

| Oct 4 | The octet containing this field (i.e. this field and the two reserved bits in the same octet) is included if the value of unifiedTCI-StateType in the ltm-TCI-Info of the configuration indicated by Target Configuration ID field is separate. |
| --- | --- |
| Oct 5-7 | If the value of C field is set to 1, the following fields are present: Random Access Preamble index field, S/U field, SS/PBCH index field, PRACH Mask index field, Repetition number field and the reserved bits in the same octet. If the value of this field is set to 0, these fields are absent. |

Therefore, only the reserved bit in Oct 3 is always present and applicable.

Observation 1: The reserved bits in Octet 4 and 7 in LTM CSC MAC CE is optionally present, depends on separate TCI states or CFRA configuration. The reserved bit in Octet 3 is always present.

The NCC value range is 0-7(3 bits), so it can be represented by three bits plus one bit to indicate the presence of NCC value. In case that there is only one always-on reserved bit, if that bit is used to indicate the presence of the NCC field, there will be no possibility to further extend the MAC CE in the future. Therefore, it is better to use the reserved bit to indicate the presence of the new Oct 8, then add one bit plus 3 NCC bits in Oct 8 to indicate the presence and the NCC value. The details are as follows:

| E | TCI state ID | Oct 3 |
| --- | --- | --- |

……

| R | R | R | R | R | S/U | Repetition Number | Oct 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R | R | R | R | N | NCC |  | Oct 8 |

C: This field indicates the presence of the contention-free Random Access Resources fields. If the value of this field is set to 1, the following fields are present: Random Access Preamble index field, S/U field, SS/PBCH index field, PRACH Mask index field, Repetition number field and the reserved bits in the same octet. If the value of this field is set to 0, these fields are absent.

……

E: This field indicates the presence of the octet containing the associated N field and the NCC field. If the value of this field is set to 1, the following fields are present: N field, NCC field and the reserved bits in the same octet. If the value of this field is set to 0, these fields are absent.

N: This field indicates the presence of NCC. If the value of this field is set to 1, the NCC field is present. If the value of the field is set to 0, the NCC field is absent.

NCC: This field indicates the NCC value for the security update of the LTM cell switch procedure. The length of the field is 3 bits.

It should be noted that in this R19 extension, the E and N fields should have the same value. However, this may change in future releases based on potential extensions.

Since extending the R18 LTM cell switch command MAC CE is feasible, RAN2 can agree the following proposal 2 and further discuss proposal 3.

Proposal 2: The R18 LTM cell switch command MAC CE is extended to indicate the NCC value for R19 inter-CU LTM cell switch.

Proposal 3: The reserved bit in Octet 3 is used to indicate the presence of Octet 8. In Octet 8, one bit is used to indicate the presence of the NCC field, and three bits are used to represent the NCC field.

LTM fast recovery

According to the LS from SA3, it is most likely that the NCC value is indicated to UE via MAC CE in each LTM cell switch. Therefore, it is nearly impossible to support LTM fast recovery after inter-CU LTM cell switch, as the UE cannot receive MAC CE when an LTM failure happens. Consequently, the UE will have no opportunity to obtain new security keys of any other candidate cells.

Observation 2: The NCC value is indicate via MAC-CE. The UE cannot obtain the security key of other candidate cells when an LTM failure occurs.

The support of fast recovery is beneficial to the interruption reduction when the failure occurs. We see one aspect that the LTM candidate list also includes intra-CU candidate cells, as shown in Figure 1. For these intra-CU candidate cells, there is no security update issue for fast recovery, and the legacy rules of intra-CU LTM fast recovery still apply.

Figure 1 The CUs of candidate cells of inter-CU LTM configuration

The only thing is that the UE needs to know which cell belongs to which CU. In the last meeting, RAN2 agreed to introduce a Rel-19 No-reset-ID in RRC to indicate whether to perform PDCP re-establishment. If the ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including a security key update. Otherwise, it implies that PDCP re-establishment and security update are not performed.

Observation 3: In inter-CU LTM configuration, for the candidate cells within the same CU as the source cell, PDCP re-establishment and security update may not needed. The Rel-19 No-reset-ID can be used to identify these candidate cells.

Therefore, fast recovery can be supported at least among those candidate cells within the same CU as the source cell. Although the concept of "the same CU" may not be equal to "No PDCP re-establishment and security update" (the network may indicate PDCP re-establishment and security update for cell change even within the same CU), it is still feasible to use the Rel-19 No-reset-ID to determine the candidate cells that do not require PDCP re-establishment and security update.

However, it should be noted that the following scenarios cannot be supported:

- L3 HO failure: Based on Rel-18 LTM discussion, the LTM fast recovery can only be performed after LTM cell switch failure or RLF, but not after L3 HO failure. This is because the UE cannot revert the PDCP re-establishment performed in the L3 HO attempt. For Rel-19, the same rule should apply.

- Inter-CU LTM cell switch failure: Even if an intra-CU candidate exists, the LTM fast recovery cannot be performed after an inter-CU LTM failure. The reason is the same as above; the PDCP re-establishment is already performed during the inter-CU LTM cell switch.

Based on the above discussion, we propose the following proposals for a unified description for R18 and R19 LTM.

Proposal 4: Fast LTM recovery is not supported after L3 HO failure or inter-CU LTM cell switch failure.

Proposal 5: For intra-CU LTM cell switch failure or RLF, an intra-CU LTM candidate can be used for LTM fast recovery.

Proposal 6: For intra-CU LTM cell switch failure or RLF, an inter-CU LTM candidate cannot be used for LTM fast recovery.

Co-existence with MIMO 2TA

In Rel-18 LTM, the LTM can co-existence with MIMO 2TA, but cannot be configured together with UE-based TA measurement:

- RAN2#125 - For LTM with MIMO two TA, - - Use post-email discussion to discuss the TP with outcome of endurable TP for next meeting, aiming to reuse the MIMO design as much as possible; - - To use option 2, not signal additional info but use the mapping from TCI state to TAG ID to understand the applicable TAG, in the TP. - RAN2#126 UE-based TA measurement should not be configured if candidate cell is configured with Rel-18 MIMO 2TA

In RAN2#125 meeting, the coexistence of LTM and MIMO 2TA features were discussed. For the case that 2TA is configured, UE needs to know which TAG the TA value belongs to. RAN2 agreed that no additional signalling is needed, since UE can determine the TAG ID based on TCI state. This is also captured in TS 38.321 (6.1.3.75, field description of Timing Advance Command) as follows:

“…If tag-Id-ptr is configured for the TCI state indicated by the UL TCI state ID field, if present, or by the TCI state ID field otherwise, in the LTM target cell and tag-Id-ptr is set to value n1, this field indicates the TA for the TAG indicated by the tag2-Id of the LTM target cell; otherwise, this field indicates the TA for the TAG indicated by the tag-id of the LTM target cell….”

However, if UE obtains TA value via UE-based TA measurement but not from the LTM cell switch command, and once 2TAGs configured, UE has no idea which TAG the measured SSB is associated with.

Therefore, RAN2 introduced a rule that UE-based TA measurement is not be configured together with MIMO 2TA. This rule should be also applicable for Rel-19 inter-CU mobility.

Proposal 7: For Rel-19 inter-CU LTM, at least UE-based TA measurement is not configured if candidate cell is configured with MIMO 2TA.

Conclusion

In this contribution, the following observations and proposals are made:

LS on the security handling for inter-CU LTM (non-DC)

Proposal 1: Based on the SA3 LS, the NCC value is included directly in the MAC CE. (i.e., no pre-configured NCC list and NCC index are introduced)

Observation 1: The reserved bits in Octet 4 and 7 in LTM CSC MAC CE is optionally present, depends on separate TCI states or CFRA configuration. The reserved bit in Octet 3 is always present.

Proposal 2: The R18 LTM cell switch command MAC CE is extended to indicate the NCC value for R19 inter-CU LTM cell switch.

Proposal 3: The reserved bit in Octet 3 is used to indicate the presence of Octet 8. In Octet 8, one bit is used to indicate the presence of the NCC field, and three bits are used to represent the NCC field.

LTM fast recovery

Observation 2: The NCC value is indicate via MAC-CE. The UE cannot obtain the security key of other candidate cells when an LTM failure occurs.

Observation 3: In inter-CU LTM configuration, for the candidate cells within the same CU as the source cell, PDCP re-establishment and security update may not needed. The Rel-19 No-reset-ID can be used to identify these candidate cells.

Proposal 4: Fast LTM recovery is not supported after L3 HO failure or inter-CU LTM cell switch failure.

Proposal 5: For intra-CU LTM cell switch failure or RLF, an intra-CU LTM candidate can be used for LTM fast recovery.

Proposal 6: For intra-CU LTM cell switch failure or RLF, an inter-CU LTM candidate cannot be used for LTM fast recovery.

Co-existence with MIMO 2TA

Proposal 7: For Rel-19 inter-CU LTM, at least UE-based TA measurement is not configured if candidate cell is configured with MIMO 2TA.

References

- S3-251124      Reply LS on security handling for inter-CU LTM in non-DC cases

- R2_129_R17R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM