3GPP TSG-RAN WG2 Meeting #129bis	R2- 2501930

Wuhan, China, 7th– 11th April 2025

Agenda Item:	8.6.3

Source:	MediaTek Inc.

Title:	Remaining issues on event triggered L1 MR

Document for:	Discussion, Decision

Introduction

In this contribution, the remaining details for event triggered L1 measurement report are discussed.

Discussion

Explicit indication for satisfying the entering/leaving condition

In the last meeting, RAN2 discussed the indication of which beams satisfy the event condition/leaving condition and made the following agreement:

Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling. Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

However, the detailed solutions are still FFS. Based on the offline/online discussion, the options can be summarized as follow:

- Option 1: The number of beams that satisfy the condition is indicated. Satisfied beams are listed first, followed by non-satisfied beams.

- Option 2: Introduce per-beam flags to indicate whether each beam has met the event condition or not.

The legacy RSRP value ranges from 0 to 127, which is represented in 8 bits. RAN2 agreed that differential RSRP can be used to further reduce overhead, which is represented in 4 bits. Considering that the entering condition should be higher than the leaving condition, Beam satisfies the leaving condition and the entering condition are mutually exclusive. There should be three types of report beams:

- Type1: Beam satisfies the entering condition (including satisfying TTT).

- Type2: Beam satisfies the leaving condition (including satisfying TTT).

- Type3: Beam does not satisfy the entering or leaving condition.

In Rel-18, up to L candidate cells and M beams can be supported in one L1 measurement report. The sum of M×L is 16 (supportedMaxReportCellsPlusBeamsReports-r18 ENUMERATED {n1,n2,n3,n4,n6,n8,n9,n12, n16}), which is subject to UE capability. Let's use the legacy value in event-triggered L1 MR MAC CE and assume X beams are reported (X is no more than 16), and differential RSRP is used:

- With Option 1, an extra 4 bits + 4 bits are used to indicate the number of beams that satisfy the entering/leaving condition (maximum 16 beams for each type). The first beam is 8 bits, and the rest of the beams are 4 bits for each, so the total bits are 4 + 4 + 8 + 4*(X-1).

- With Option 2, 2 bits for each beam are added to indicate whether the beam satisfies the entering/leaving condition. The first beam is 8+2 bits, and the rest of the beams are 4+2 bits for each, so the total bits are 10 + 6*(X-1).

The total number of bits is calculated as shown in the following table:

Table1: Total bits used with option1 and option2 for explicit indication in event triggered L1 MR MAC CE

| X (beams) | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Option1 | 16 | 20 | 24 | 28 | 32 | 36 | 40 | 44 | 48 | 52 | 56 | 60 | 64 | 68 | 72 | 76 |
| Option2 | 10 | 16 | 22 | 28 | 34 | 40 | 46 | 52 | 58 | 64 | 70 | 76 | 82 | 88 | 94 | 100 |

Observation 1: Option 1 has lower overhead if the number of reported beams is less than 4, while Option 2 has lower overhead if the number of reported beams is more than 4.

Considering real scenarios, it is less likely that a large number of beams will be triggered simultaneously. The total number of beams reported in a single L1 measurement report is not necessarily limited to N (the maximum number) if the number of triggered/satisfied beams is much less than N.

It should also be noted that the maximum number of reportable beams is subject to UE capability and also based on the network indication. I.e., if only one beam meets the event, UE can just report one beam.

Observation 2: The reported beam may not necessarily be limited to the maximum number if the triggered/satisfy beam is much less than that. The number to be reported is also subject to UE capability and network indication.

Based on the above analysis, a lower number of beam reports is more likely to be used in real cases. Therefore, we prefer Option 2 for an efficient and clear report.

Proposal 1: Introduce per-beam flags to indicate whether each beam has met the event/leaving condition or not (1 bit for each).

Unsatisfied beam selection

In the last meeting, it was agreed that the selection of beams not satisfying the event condition should be based on measured quality.

Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s))

However, the detailed behavior for selecting unsatisfied beams remains unclear and requires further discussion.

For example, assume LTM3 event is set to 2dB with a TTT of 80 ms. At one moment, UE has four measurement results:

- Beam A (triggered beam): offset = 3dB with TTT = 80 ms

- Beam B (unsatisfied beam): offset = 1dB for 300 ms (low quality but stable)

- Beam C (unsatisfied beam): offset = 2.5dB but only one sample (high quality but unstable)

- Beam D (unsatisfied beam): offset = 1.5dB for 50 ms (medium quality and medium stability)

If the maximum reported beam N=2, it is hard to select one unsatisfied beam among B, C, and D in addition to the triggered beam A. If we simply assume that channel quality overweight stability, it may become not that correct to choose a beam with offset=2dB with only one sample over a beam with offset=1.8 dB with 1000 ms of stability. Real L1 measurement results are more complex than these examples.

Observation 3: It is hard to define specific rules for unsatisfied beam selection, considering the measured quality and stability. Otherwise it will be overcomplicated.

Based on the above observation, we suggest to simply add a note in the spec as a guidance for UE to select unsatisfied beams (i.e., based on the measurement quality), while leaving the detailed rules unspecified and up to UE implementation. This proposal does not intend to revise the agreement from the last meeting.

Proposal 2: Add a note in the spec as a guidance for UE to select unsatisfied beams based on measurement quality. RAN2 does not specify detailed rules for this and leave it to UE implementation.

Considering different UE capabilities, the reporting of unsatisfied beams should also be subject to UE capability.

Proposal 3: Introduce a UE capability to indicate the support for reporting unsatisfied beams in event-triggered L1 MR MAC CE.

Single/multiple result for one beam

Considering event L1 MR is send after TTT expires, UE may perform several L1 measurement during TTT. In that case, there are two options on how to report the beam quantity:

- Option 1a: UE reports each measured result during the event evaluation period (e.g., TTT) of the RS.

- Option 1b: UE only reports one latest value for the measured result for the target RS.

In our understanding, the triggering of LTM depends on the latest measured beam quality and the event. It should be enough for network to make decision based on the reported latest measured beam quality and the event type. Reporting each measured results during TTT seems unnecessary and cause more signalling overhead.

Proposal 4: If multiple samples are measured for the target RS during the event evaluation period, UE only reports the latest measured result.

TTT for leaving condition

In the last meetings, the agreements on leaving condition of event triggered L1 MR were made:

- MR can be sent when the leaving condition is met, based on NW configuration. - TTT is applied to the leaving condition

In L3 measurement reports, the leaving condition is used to terminate periodic measurement reporting. Additionally, it can be used to send another measurement report if reportOnLeave is set to True. For event-triggered L1 MR, the usage of the leaving condition and the TTT should be similar to L3 MR.

In the latest MAC running CRs, the TTT for the entry condition and the TTT for the leaving condition are both configured by RRC using the same timeToTrigger value..

The MAC entity shall for the serving cell configured for LTM event evaluation procedure: [1> for each ltm-CSI-ReportConfigId of the serving cell included in the LTM-CSI-ReportConfig:] …… 2>if the corresponding ltm-ReportConfigType is set to eventTriggered: …… 2> if the entry condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams, i.e. reference signalling associated with LTM-CSI-SSB-ResourceConfigId or LTM-CSI-RS-ResourceConfigId in the LTM-CSI-ReportConfig, for the measurement from lower layer after layer 1 filtering taken during timeToTrigger defined for this event: …… 2> else if the leaving condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams for the measurement from lower layer after layer 1 filtering taken during timeToTrigger defined for this event: 3> remove the concerned beam(s) in the MR_LIST for this ltm-CSI-ReportConfigId; 3> if reportOnLeave is set to true for this ltm-CSI-ReportConfigId: 4> initiate the measurement reporting procedure, as specified in 5.x.3; ……

However, considering the different use cases for these two TTTs, we should further discuss whether a single TTT value or two separate TTT values should be used for the entry and leaving conditions.

Proposal 5: RAN2 to discuss whether the TTT for the leaving condition should be always equal to the TTT for the entry condition, i.e., whether the same pre-configured timeToTrigger should be used for both.

reportInterval of L1 measurement report

In the summary of [POST129][108][MOB] RRC running CR, an discussion is raised on the additional candidate values for report interval for event-triggered periodic report for LTM. An proposal is raised based on companies’ comments: Add additional candidate values {20ms, 40ms, 60ms} for report interval for event-triggered periodic report for LTM.

However, it should be noted that the legacy ReportInterval already has 14 values and only two bits can be added without further IE extension.

ReportInterval ::= ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1,min6, min12, min30 }

In that case, two candidate values among {20ms, 40ms, 60ms} should be selected to avoid unnecessary IE extension.

Proposal 6: For the ReportInterval for event-triggered periodic report, RAN2 to discuss and select two candidate values among {20ms, 40ms, 60ms}

The support of multiple triggering event

In the latest running CR, the eventTriggered-r19 is structured under the R18 LTM-CSI-ReportConfig, which is part of the report configuration list ltm-CSI-ReportConfigToAddModList. With this structure, the UE may be configured with multiple LTM report configurations, subject to the maximum number of LTM-CSI-ReportConfigurations-r18. We understand that this is beneficial for the R18 L1 MR when parallel periodic/aperiodic/semi-persistent reports are configured by the network.

However, for event-triggered L1 MR, it has not yet been agreed to configure multiple trigger events simultaneously. Therefore, we suggest discussing this and to see whether to add such a limitation in the specification.

Proposal 7: RAN2 to discuss whether to support configuring multiple triggering event simultaneously.

Conclusion

In this contribution, the following observations and proposals are made:

Explicit indication for satisfying the entering/leaving condition

Observation 1: Option 1 has lower overhead if the number of reported beams is less than 4, while Option 2 has lower overhead if the number of reported beams is more than 4.

Observation 2: The reported beam may not necessarily be limited to the maximum number if the triggered/satisfy beam is much less than that. The number to be reported is also subject to UE capability and network indication.

Proposal 1: Introduce per-beam flags to indicate whether each beam has met the event/leaving condition or not (1 bit for each).

Unsatisfied beam selection

Observation 3: It is hard to define specific rules for unsatisfied beam selection, considering the measured quality and stability. Otherwise it will be overcomplicated.

Proposal 2: Add a note in the spec as a guidance for UE to select unsatisfied beams based on measurement quality. RAN2 does not specify detailed rules for this and leave it to UE implementation.

Proposal 3: Introduce a UE capability to indicate the support for reporting unsatisfied beams in event-triggered L1 MR MAC CE.

Single/multiple result for one beam

Proposal 4: If multiple samples are measured for the target RS during the event evaluation period, UE only reports the latest measured result.

TTT for leaving condition

Proposal 5: RAN2 to discuss whether the TTT for the leaving condition should be always equal to the TTT for the entry condition, i.e., whether the same pre-configured timeToTrigger should be used for both.

reportInterval of L1 measurement report

Proposal 6: For the ReportInterval for event-triggered periodic report, RAN2 to discuss and select two candidate values among {20ms, 40ms, 60ms}

The support of multiple triggering event

Proposal 7: RAN2 to discuss whether to support configuring multiple triggering event simultaneously.

References

- R2_129_R17R18 SL_R18R19 MOB_R19 NES_Session Notes_EoM

- R2_128_R18 SL_MOB_R19 NES_MOB_Session Notes_EoM

- RP-240299-Revised-WID-R19-Mobility-Enhancements