3GPP TSG-RAN WG2 Meeting #129bis											R2-2501948

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.7.5

Source: 	Sharp

Title:  	Discussion on RLC Enhancements in XR

Document for: 	Discussion

# Introduction

In RAN2#129 meeting, RAN2 made the following agreements on RLC enhancements:

Agreements on RLC enhancements Autonomous retransmissions and polling enhancements - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - Only a single autonomous retransmission will be triggered per RLC SDU. - There is no dynamic activation/deactivation of the autonomous retransmission mechanism. - We have separate thresholds for autonomous reTx and for polling Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly - Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

This document discusses remaining open issues.

# Discussion

## Autonomous Retransmission

In RAN2#129 meeting, RAN2 agreed to use remaining time for triggering autonomous retransmission and polling. An FFS is whether this remaining time is based on PDCP discard timer or not. In our view, it would be better to use PDCP discard timer.

Firstly, reuse of PDCP remaining timer is already used by other XR features. In Rel-18 and Rel-19, XR WIs introduced multiple features which use SDU remaining time, e.g., PDU set discard, DSR, LCP enhancement, etc. All those XR features use remaining time based on PDCP discard timer. Thus, it is preferred to use exiting concept to reduce the complexity of the specification and minimize the development efforts, unless any critical issue is found.

Secondly, the autonomous retransmission is introduced to support timely retransmission which means the transmission before the packet becomes obsolete from latency point of view. The obsoleteness and latency are related to how long the packet is stored at the UE modem. Remaining time based on PDCP discard timer is the most relevant timer.

In Autonomous Retransmission, remaining time is determined based on discardTimer at PDCP.

The second issue is whether the autonomous retransmission should increase RETX_COUNT. In RAN2#129, RAN2 agreed to allow at most one autonomous retransmission for an SDU. Usually, the maximum number of RLC retransmissions is set to a very high number, e.g. 16 or 32, since the maximum retransmission triggers a radio link failure (RLF) which should be declared only in an extreme radio condition. We see that one additional count does not have any side effect. In this context, we can rely on the existing retransmission procedure and do not see a big necessity of the exception case.

Autonomous retransmission may increase RETX_COUNT, same as legacy.

In the current running CR, the autonomous retransmission is performed whenever the remaining time falls within the threshold. If the ACK is received, the packet does not need to be retransmitted anymore. As it can be considered as a detailed procedure, RAN2 is requested to agree.

Autonomous transmission is triggered only if ACK has not been received.

Terminology “autonomous retransmission” is already being used as a standardized terminology of a Rel-16 NR-U CG transmission feature, as follows [1]:

### 16.1.7 URLLC in Unlicensed Controlled Environment URLLC services can be supported in shared spectrum where LBT failures are assumed to be not frequent. In this case, a channel access procedure for semi-static channel occupancy can be initiated by the gNB or the UE, or the gNB operates in dynamic channel access mode, as described in TS 37.213 [37]. To handle potential LBT failures on configured grants (CG), the CG retransmission timer can be optionally configured to enable autonomous retransmissions, and it may be configured simultaneously with enhanced intra-UE overlapping resource prioritization mechanisms. When the CG retransmission timer is configured, the UE shall select the HARQ process for each CG resource by itself. If the enhanced intra-UE overlapping resource prioritization mechanisms is also configured, the UE may be further configured to select the HARQ process for a CG resource based on logical channel priority.

In our understanding, “autonomous retransmission” for Rel-19 XR is a working terminology. Different features having the same name may bring a confusion to readers of the specification. For the Rel-19 XR feature of autonomous retransmission, RAN2 may need to define a new name which can replace the autonomous retransmission. Any nice descriptive name would be considered, e.g. timer-based retransmission.

RAN2 to consider renaming the autonomous retransmission as a descriptive spec terminology, e.g. timer-based retransmission.

## Autonomous Polling

In RLC running CR, there is an editor’s note on excessive polling, as follows [2]:

Editor’s Note: It is still open on how to avoid excessive polling for the polling enhancement, e.g. only one polling or multiple.

The current procedure captured in the running CR is that a poll is triggered whenever an SDU has remaining timer below the remaining time threshold. It may trigger an excessive polling. However, even if multiple triggered polls, only one poll per RLC entity is transmitted by a MAC PDU. There might not be a critical problem if only one autonomous polling is triggered per SDU.

If RAN2 needs to consider an enhancement to avoid excessive polling, we may consider using a different set of polling parameters to be used when the remaining time is below the threshold. The new set with smaller pollPDU and pollByte can accelerate poll triggering and avoid the excessive polling.

RAN2 to consider one of the following options:

- Option 1: Only a single autonomous retransmission will be triggered per RLC SDU. (No other enhancement)

- Option 2: Another set of polling parameters is used when the remaining time is below the threshold.

## Unnecessary Retransmission Avoidance

In unnecessary retransmission avoidance, RAN2 agreed to introduce a combined TX-RX approach where both TX and RX sides stop transmission and reception of an obsolete SDU, respectively. An open issue is whether the detection of the obsolete SDU at the RX side needs to trigger an RLC status report. In RAN2#127, RAN2 agreed to use existing SR, as follows:

- Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified

There might be two interpretations on the agreement above:

- 1) Reusing existing SR format with a new triggering

- 2) Reusing existing SR format and relying on existing triggering.

In our view, even without this new trigger, existing status report triggering conditions can inform the detection of obsolete SDU. In the agreed combined approach, TX side can decide to abandon the outdated SDU transmission, so the status report itself is not so urgent or even unnecessary. Also, an RLC status report may include NACK which may trigger early retransmission which may be too early due to ongoing HARQ retransmission.

Status reporting relies on the legacy triggering events. t-RxDiscard expiry does not trigger a status report.

# Conclusions

Based on the discussion given above, RAN2 is requested to discuss and agree the following proposals:

- In Autonomous Retransmission, remaining time is determined based on discardTimer at PDCP.

- Autonomous retransmission may increase RETX_COUNT, same as legacy.

- Autonomous transmission is triggered only if ACK has not been received.

- RAN2 to consider renaming the autonomous retransmission as a descriptive spec terminology, e.g. timer-based retransmission.

- RAN2 to consider one of the following options:

- Option 1: Only a single autonomous retransmission will be triggered per RLC SDU. (No other enhancement)

- Option 2: Another set of polling parameters is used when the remaining time is below the threshold.

- Status reporting relies on the legacy triggering events. t-RxDiscard expiry does not trigger a status report.

# References

[1] TS 38.300 	NR and NG-RAN Overall Description; Stage 2, v18.3.0

[2] R2-2502162	RLC running CR for XR, vivo