3GPP TSG-RAN WG2 Meeting #129bis	R2-2501964

Wuhan, China, April 7th –11th , 2025

Agenda Item:	8.2.4

Source: 	Xiaomi

Title:  	Protocol design principle and considerations on data transmission

Document for:	Discussion and decision

# Introduction

In last RAN2 meeting, RAN2 discussed data transmission and the MAC PDU design. However following issues still need further discussion:

Issue 1: AS ID related issues (covered in postemail discussion 036)

Issue 2: Segmentation, FFS on 1 bit, Msg size, retransmission, Msg 3?

Issue 3: Message design, multiplexing for other R2D message? New messages?

In this contribution, we continue the discussion on issue 2 and 3.

# Discussion

## 2.1 Issue 2 Segmentation related issues

### 2.1.1 Msg 3 segmentation

In last meeting, we discussed segmentation and achieved the following agreements.

Agreements on segmentation

- To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size.   If not possible, FFS if the 1 bit is sufficient.

- Segment retransmission is supported.

- For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

- R2D segmentation is not supported for R19 A-IoT.

One remaining issue is whether segmentation/segment retransmission applies to Msg3. In our understanding, Msg3 contains device ID or temporary device ID, of which the message size is fixed and limited, e.g., 128 or 96 bits designed/specified in SA2. With the information of the message size of Msg3, reader should allocate a proper resource to include the whole (temporary) device ID. From the device perspective, since energy saving is not considered in this release, it should be assumed as long as it participates in a paging round, the device should be able to complete the inventory/inventory+command procedure. Therefore, there is no motivation to consider the support of Msg3 segmentation. However, considering we have detailed design on segment/segment retransmission of data, it is still possible to segment a Msg3 if needed without additional impact on the specification. Therefore, whether to segment a Msg 3 can be up to the reader implementation, e.g., Msg3 is segmented if the allocated resource is not sufficient, otherwise the entire Msg3 is transmitted.

Proposal 1: RAN2 confirms there is no additional effort to support Msg3 segmentation/segmentation retransmission. Whether to segment a Msg3 is up to the reader implementation.

### 2.1.2 D2R retransmission

Another issue is whether to use the same method to trigger the retransmission of the unsegmented packet, e.g., reader indicates the received Msg size/offset as 0. However, we think there are two cases when the reader sends the R2D command but no D2R response.

- Case 1: the device fails to receive the R2D command

- Case 2: the reader fails to receive the D2R response

For case 1, if the device fails to receive the R2D command, only indicating the offset as 0 is not sufficient. However, for case 2, this method works. Considering the reader is not able to distinguish whether the reception failure of D2R response is due to case 1 or case 2, the reader should include the whole R2D command for safety in case the reader fails to receive the D2R response. Besides, since the whole command is indicated, there is no need to indicate the offset as 0.

Proposal 2: Reader resends the R2D command without an offset to trigger the retransmission of the entire D2R response/first D2R segment.

While for the retransmission of non-first segment, besides offset, the R2D command should be included. This is because the motivation to rely on the reader to indicate the offset explicitly is to relax the memory of the device, e.g., device does not need to store the information of the starting/ending point of the segment/entire D2R response, thus reduces the complexity. However, if only offset is indicated without the R2D command, from the device’s perspective, at least the starting/ending point of the entire D2R response still needs to be remembered, then the benefit of this method is not significant, i.e., only saving the memory of an offset. Therefore, considering the motivation is to reduce the device complexity when designing the retransmission scheme, we think the R2D command should be included together with the offset for the retransmission of each non-first segment.

Proposal 3: Reader resends the R2D command with an offset to trigger the retransmission of the non-first segment.

In last meeting, we agreed to support NACK feedback to trigger re-access at least for Msg3.

NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback

As discussed above, reader is able to trigger a retransmission of the entire Msg5/Msg5 segment. However, after several times of retransmission failure, it should be possible for the reader to end the retransmission and trigger the device to re-access, e.g., via NACK feedback as in Msg3. From the device’s perspective, whenever it receives the NACK feedback, it considers the existing procedure ends and re-access is triggered, no matter the NACK feedback is for Msg3 or Msg5.

Proposal 4: NACK feedback is supported for Msg5/Msg5 segment.

### 2.1.3 Msg size indication

Msg size information is useful to the reader to perform resource scheduling. During the SI phase, we have sent LS to SA2 to ask if this information can be provided by CN. Based on their response, it is observed that this information cannot be always provided, e.g., only if provided by AF. Therefore, the reader needs to get the message size information from the device when it fails to obtain this information from CN.

SA2 answer: If provided by AF, AIOTF provides the reader with the approximate D2R message size based on AF request. Whether the CN can determine the approximate D2R message size if not provided by AF is under discussion in SA2.

Proposal 5: Reader should get D2R Msg size information from the device if not available from CN.

The next issue is whether/how reader to control the D2R Msg size indication. In our understanding, at least in the following scenarios, there is no need for the device to report the D2R Msg size to the reader.

- The D2R Msg size information is provided by CN

- Device is able to complete the D2R response without segmentation

- The D2R transmission is not the first segment of the whole D2R response

Considering the calculation of the Msg size will increase the device complexity/power consumption, it is preferred to reduce the calculation/reporting even this function is supported. Therefore, we think the reporting of D2R Msg size can be controlled by the reader, e.g., when this information is not available from CN, reader can blindly schedule a relatively small resource and indicate the device to report the remaining D2R Msg size.

Proposal 6: Reader indicates the device to report the remaining D2R Msg size via the R2D message, e.g., Msg4 in CBRA.

In addition, considering the number of bits of each received D2R/D2R segment is visible to the reader, there is no need to indicate the remaining Msg size in each segment, i.e., the first segment to report is sufficient. Base on this information and the bits of each received D2R segment, the reader is able to derive the remaining Msg size after the reception of each D2R segment.

Proposal 7: If the D2R transmission is segmented, device only includes the remaining D2R Msg size in the first segment message.

In last meeting, we agreed to use one-bit indication to indicate whether there is more data or not. However, if the Msg size report from the device is supported, there is no need to include the one-bit indication. As mentioned above, based on the remaining Msg size and the bits of each received D2R segment, the reader can derive whether there is more data or not by itself.

Proposal 8: One-bit indication on whether the packet is segmented/last segment is not needed if the remaining Msg size is reported.

## 2.2.	Issue 3 MAC PDU design

### 2.2.1 General principle

As defined in [2], a MAC PDU consists of one or more MAC subPDUs, and each MAC subPDU consists of one of the following:

- A MAC subheader only (including padding); - A MAC subheader and a MAC SDU; - A MAC subheader and a MAC CE; - A MAC subheader and padding. Figure 6.1.2-4: Example of a DL MAC PDU

To our understanding, it is to support the multiplexing of the data from different channels, and MAC CE for different purpose. However, for a single device, there is no multiplexing requirement and no MAC CE requirement for AIoT. To simplify the AIoT PDU design, for a single device, an AIoT MAC PDU shall only consist of an AIoT MAC header and an MAC SDU (for the data from AIoT protocol above MAC). All relevant information introduced in RAN2 shall be defined as part of AIoT MAC header.

Proposal 9: For a single device, an A-IoT MAC PDU shall only consist of an A-IoT MAC header (including relevant information introduced in RAN2) and an MAC SDU (includes upper layer data e.g., command, read/write/disable).

As described in the WID [1], the intention of the WI is to define a new 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. To reduce the complexity of the implementation for such very low end IoT device, we do not see the possibility to support multiple options for such device for the same feature based on different capabilities. To simplify the implementation for both device and Reader, all messages, features defined in AIoT MAC shall be mandatory support.

Proposal 10: Do not introduce capability exchange procedure in A-IoT MAC. All messages, features defined in AIoT MAC shall be mandatory support;

There were some discussions on whether to combine messages, e.g. same message for paging and command. To combine different purpose/functions in the same message, means lots of choice bit shall be added which will increase the overhead, and also increase the complexity of the implementation.

Proposal 11: To simplify the device implementation, RAN2 should avoid mixing multiple service functions in the same message based on choice, optional mechanism. Separate messages are introduced for different service, e.g. Inventory request and command.

To distinguish different function, service, message type shall be introduced for messages from the same direction (R2D or D2D) but for different services..

For the new extension/changes introduced in future release, new messages shall be introduced instead of extending the existing message. Reserve bit defined in existing MAC is not needed for AIoT MAC. There are two options:

Option 1: introduce version number in the AIoT MAC header. The message type can be reused if the purpose is same.

For instance, 2 bits version number, and 00 for the messages introduced in Rel-19, and message type 0011 is used for Command Write. Then for the Command message introduced in Rel-20 with additional changes, version number shall be 01.

Option 2: distinguish messages introduced in different release based on the message type

Option 2 may lead less overhead compare to option 1. From readness perspective, the option 1 is cleaner considering the same message type can be reused.

Proposal 12: Message type is introduced to distinguish messages for different function, service from the same direction for R2D message. New extension/changes introduced in future release shall be supported based on new message with different version number. Reserve bit is not introduced in AIoT MAC.

### 2.1.2 R2D MAC PDU design

In last meeting, we discussed the detailed design of R2D/D2R MAC PDU and achieved the following agreements.

Agreements on MAC PDU format design - Aim to design simple MAC PDU format design - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages - At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). - FFS whether for D2R we need message type field, any length and need for padding - Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements.

RAN2 has agreed to support the multiplexing of Msg2. While FFS others for multicast messages. In last RAN2 meeting, RAN2 has agreed to support NACK based feedback to trigger re-access at least for Msg3.

NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback

Considering RAN1 agreed to support FDMA of D2R transmissions for Msg3 from multiple devices, to respond the failure of multiple devices, it makes sense to support the multiplexing of NACK feedback, similar as multiplexing of Msg2. In addition, to support the multiplexing of NACK feedback, a new R2D MAC PDU (NACK feedback) should be defined, similar as R2D MAC PDU (Msg2)

Support FDMA of D2R transmissions for Msg3 from multiple devices in response to a PRDCH for Msg2 transmission corresponding to multiple Msg1 during access procedure.

Proposal 13: Multiplexing of NACK feedback is supported and a new R2D MAC PDU (NACK feedback) is defined. Send LS to RAN1 to inform them of this conclusion.

### 2.1.3 D2R MAC PDU design

Based on the above agreement, currently there are two types of D2R MAC PDU, i.e., D2R MAC PDU (MSG1) and D2R MAC PDU (MSG3 and data). Regarding whether to support MAC header for Msg1 MAC PDU, we think there is no need to add a MAC header for Msg1 MAC PDU due to

- The message size of Msg 1 is fixed, i.e., 16 bits, length field is not needed.

- The Msg1 is transmitted via common resources configured by the reader dedicated for RACH, the reader is able to distinguish whether it is a Msg1 MAC PDU according to the resource where the MAC PDU is received, message type is not needed.

Therefore, for D2R MAC PDU (Msg1), the MAC header is not needed.

Proposal 14: RAN2 to confirm MAC header is not needed for D2R MAC PDU (Msg1).

In last meeting, we have an FFS on whether we need message type field, any length and need for padding. As we analysed above, the resource for the transmission of D2R MAC PDU (MSG1) is a common resource for RACH while the resource for the transmission of D2R MAC PDU (MSG3 and data) is a dedicated resource. Since both the common resource and dedicated resource are under the reader’s control, the reader should be able to separate them, then based on the resource where a D2R MAC PDU is received, the reader is able to distinguish whether it is a Msg 1 or Msg3/data. From this perspective, there is no need to introduce the message type for D2R MAC PDU.

Proposal 15: RAN2 to confirm message type is not introduced for D2R MAC PDU.

The next issue is whether we need length/padding/byte alignment. From the device complexity perspective, the more things to add, the more complexity/power consumption to have. Therefore, for such low-cost device, there should be no requirement on byte alignment. Regarding padding or length field, we can discuss case by case. For the D2R MAC PDU (Msg1), considering it has fixed size, no need to support length field or padding. For the D2R MAC PDU (Msg3), considering Msg3 is used to contain the device ID or temporary device ID, of which the message size is fixed and limited, e.g., 128 or 96 bits designed/specified in SA2, there is no need to support length field or padding. For the D2R MAC PDU (data), if the reader obtains the Msg size from CN or the device; the reader should be able to allocate a proper resource with suitable TBS for each segment, and the reader is able to derive the length of the D2R based on the allocated resource/TBS, which means length field or padding is not needed. However it is also related to RAN1 design on granularity of TBS. We can further check with RAN1 whether they have any concern or not.

Proposal 16: For D2R MAC PDU, RAN2 to confirm the following and check with RAN1 if they have any concern

- Byte alignment is not needed

- Padding is not needed

- Length field is not needed

# Conclusions

Based on the above discussion, we propose following proposals:

Segmentation

Proposal 1: RAN2 confirms there is no additional effort to support Msg3 segmentation/segmentation retransmission. Whether to segment a Msg3 is up to the reader implementation.

Proposal 2: Reader resends the R2D command without an offset to trigger the retransmission of the entire D2R response/first D2R segment.

Proposal 3: Reader resends the R2D command with an offset to trigger the retransmission of the non-first segment.

Proposal 4: NACK feedback is supported for Msg5/Msg5 segment.

.

Msg size indication

Proposal 5: Reader should get D2R Msg size information from the device if not available from CN.

Proposal 6: Reader indicates the device to report the remaining D2R Msg size via the R2D message, e.g., Msg4 in CBRA.

Proposal 7: If the D2R transmission is segmented, device only includes the remaining D2R Msg size in the first segment message.

Proposal 8: One-bit indication on whether the packet is segmented/last segment is not needed if the remaining Msg size is reported.

MAC PDU design general principle

Proposal 9: For a single device, an A-IoT MAC PDU shall only consist of an A-IoT MAC header (including relevant information introduced in RAN2) and an MAC SDU (includes upper layer data e.g., command, read/write/disable).

Proposal 10: Do not introduce capability exchange procedure in A-IoT MAC. All messages, features defined in AIoT MAC shall be mandatory support;

Proposal 11: To simplify the device implementation, RAN2 should avoid mixing multiple service functions in the same message based on choice, optional mechanism. Separate messages are introduced for different service, e.g. Inventory request and command.

Proposal 12: Message type is introduced to distinguish messages for different function, service from the same direction for R2D message. New extension/changes introduced in future release shall be supported based on new message with different version number. Reserve bit is not introduced in AIoT MAC.

R2D MAC PDU design

Proposal 13: Multiplexing of NACK feedback is supported and a new R2D MAC PDU (NACK feedback) is defined. Send LS to RAN1 to inform them of this conclusion.

D2R MAC PDU design

Proposal 14: RAN2 to confirm MAC header is not needed for D2R MAC PDU (Msg1).

Proposal 15: RAN2 to confirm message type is not introduced for D2R MAC PDU.

Proposal 16: For D2R MAC PDU, RAN2 to confirm the following and check with RAN1 if any concern

- Byte alignment is not needed

- Padding is not needed

- Length field is not needed

# Reference

[1] RAN2#129 meeting agreement 
[2] TS 38.321 MAC specification