3GPP TSG-RAN WG2 Meeting #129bis									R2-2502014

Wuhan, China, April 7th– 11th, 2025

Agenda item		: 8.4.2

Title	: IDLE/INACTIVE procedures for LP-WUS

Source				: Tejas Networks Limited

Document for		: Discussion and Decision

# Introduction

This contribution further addressed the procedure and configuration of LP-WUS for IDLE/INACTIVE modes.

# Discussion

## 2.1 UEID definition for LP-WUS Subgroup calculation:

From R2#129 meeting, For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - UE_ID is related to 5G-S-TMSI, - N is the number of total paging frames in one DRX cycle, - Ns is the number of the PO for a PF, - Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

RAN2 agreed on the formula to calculate subgroupID for LP-WUS but the details on how the UEID is derived from 5G-TMSI is not concluded. Since the UEID should take into account the case of N, Ns, Np and subgroupsNumForUEID having the maximum value as N = 256, Ns = 4 and Np = 8, subgroupsNumForUEID = 32 (31 subgroups + a common code point used for cases such as SI change notification), the UEID should be derived by taking 5G-TMSI mod (max_N* max_Ns* max_Np * max_subgroupsNumForUEID) which is 5G_S_TMSI mod 262144.

Proposal 1: RAN2 to agree that UE_ID is calculated as 5G-S-TMSI mod 262144

## 2.2 Entry/Exit criteria for using LR:

The main advantage of using LR to monitor wake up indication is that MR can move to ultra-deep sleep/ deep sleep state to save power. The currently agreed entry and exit conditions only consider the signal qualities of LP-SS/SSB measured by LR/MR. Considering just the thresholds might lead to ping-pong effect of moving between use of LR and MR resulting in UE not having sufficient time to move to between sleep states. This can even result in adverse effect as there is power penalty associated with transition between sleep states. In order to overcome this, we propose to reuse the similar principle used in Rel16 relaxed measurement criteria where there is a time period configured for which the evaluation criteria need to be satisfied to consider the condition as fulfilled.

Proposal 2: RAN2 to consider evaluation time period for which the entry/exit conditions needs to be satisfied to switch between LR/MR.

## 2.3 LO and PO association

RAN1 in R1#119 agreed to discuss the following options for determining the association of LO with a PO:

For the offset value(s) between an LO and a reference PO/PF, adopt Option 2B-1. - gNB can configure 1 or 2 offset values. - FFS whether gNB can configure 3 offset values - If multiple offset values are configured and if the gap between the LO associated with the largest offset value and the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the smallest offset value that has a gap between the LO and the PO no less than the wake-up delay. - Note: if a single offset value is configured, UE behaviour is according to Option 1-1. - All the UEs supporting LP-WUS for idle/inactive mode supports the configuration of 2 offset values (FFS: 3 values).

In previous RAN1 meeting, it was agreed to support 1 or 2 offset value for LO monitoring. The UE monitors an LO by selecting the lowest offset value which is at least greater than or equal to the wake delay reported by the UE.

Proposal 3: RAN2 to consider NW broadcasting multiple LO to PO offset values.

Proposal 4: The definition of valid LO offset is the lest offset value broadcasted by the network which is greater than the maximum MR wake up delay reported by the UE.

## 2.4 Clarification on support of LP-WUS along with MBS group paging:

Since LP-WUS in IDLE/INACTIVE mode follows the PEI as baseline, in rel17, it was agreed that UE is not required to monitor PEI is UE is expecting to receive a group paging. We should extend the same principle for LP-WUS and agree explicitly that if UE is waiting for a group paging message, UE does not use LP-WUS.

Proposal 5: RAN2 to agree UE is not expected to use LP-WUS (if supported and configured) , if UE is expecting a group paging notification.

# Conclusion

UEID definition for LP-WUS subgroup:

Proposal 1: RAN2 to agree that UE_ID is calculated as 5G-S-TMSI mod 262144

Entry/Exit criteria for using LR

Proposal 2: RAN2 to consider evaluation time period for which the entry/exit conditions needs to be satisfied to switch between LR/MR.

LO and PO association

Proposal 3: RAN2 to consider NW broadcasting multiple LO to PO offset values.

Proposal 4: The definition of valid LO offset is the lest offset value broadcasted by the network which is greater than the maximum MR wake up delay reported by the UE.

LP-WUS for Group paging:

Proposal 5: RAN2 to agree UE is not expected to use LP-WUS (if supported and configured) , if UE is expecting a group paging notification.