3GPP TSG-RAN WG2 Meeting #129bis	R2-2502015

# Wuhan, China, 7 – 11 April 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0432 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on usage of q-RxLevMinSUL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated (Rapporteur) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The UE behavior is not clear if a cell is configured with SUL carrier but the NW doees not signal SUL related parameters in SIBs. For q-RxLevMinSUL, the related procedural text has the if-branch as "If the UE supports SUL frequency for this cell “ but without a condition in the if-branch for the behavior of such a UE when the q-RxLevMinSUL is not present. In other words, the text has a condition “if present” for the IE without a branch when it is not present A similar problem exists also for p-max, albeit the optionality of the parameter in the form of “if present” was not captured for p-max in the procedural text. In other words, it is already assumed in the specification that the NW should always signal SUL related p-max if the cell is configured with SUL carrier. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that if a cell is conifgured with SUL carrier and the UE supports the SUL frequency for this cell, the UE always uses q-RxLevMinSUL and SUL associated p-max. This implies that the NW should always signal these parameters for that cell. Impact analysis Impacted 5G architecture options: NR SA, NR-DC, NE-DC Impacted functionality: Inter-operability: If the network is implemented according to this CR while the UE is not, there are no inter-operability issues. If the UE is implemented according to this CR while the network is not, the UE behavior is unspecified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior when SUL parameters are not signaled will not be clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

#### 5.2.3.2	Cell Selection Criterion

The cell selection criterion S is fulfilled when:

Srxlev > 0 AND Squal > 0

where:

Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset )– Pcompensation - Qoffsettemp Squal = Qqualmeas – (Qqualmin + Qqualminoffset) - Qoffsettemp

where:

| Srxlev | Cell selection RX level value (dB) |
| --- | --- |
| Squal | Cell selection quality value (dB) |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 38.331 [3] (dB) |
| Qrxlevmeas | Measured cell RX level value (RSRP) |
| Qqualmeas | Measured cell quality value (RSRQ) |
| Qrxlevmin | Minimum required RX level in the cell (dBm). If the UE supports SUL frequency for this cell and the cell is configured with SUL carrier, Qrxlevmin is obtained from q-RxLevMinSULin SIB1, SIB2 and SIB4, additionally, if QrxlevminoffsetcellSUL is present in SIB3 and SIB4 for the concerned cell, this cell specific offset is added to the corresponding Qrxlevmin to achieve the required minimum RX level in the concerned cell; else Qrxlevmin is obtained from q-RxLevMin in SIB1, SIB2 and SIB4, additionally, if Qrxlevminoffsetcell is present in SIB3 and SIB4 for the concerned cell, this cell specific offset is added to the corresponding Qrxlevmin to achieve the required minimum RX level in the concerned cell. |
| Qqualmin | Minimum required quality level in the cell (dB). Additionally, if Qqualminoffsetcell is signalled for the concerned cell, this cell specific offset is added to achieve the required minimum quality level in the concerned cell. |
| Qrxlevminoffset | Offset to the signalled Qrxlevmin taken into account in the Srxlev evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN, as specified in TS 23.122 [9]. |
| Qqualminoffset | Offset to the signalled Qqualmin taken into account in the Squal evaluation as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN, as specified in TS 23.122 [9]. |
| Pcompensation | For FR1, if the UE supports the additionalPmax in the NR-NS-PmaxList, if present, in SIB1, SIB2 and SIB4: max(PEMAX1 –PPowerClass, 0) – (min(PEMAX2, PPowerClass) – min(PEMAX1, PPowerClass)) (dB); else: max(PEMAX1 –PPowerClass, 0) (dB) For FR2, Pcompensation is set to 0. For IAB-MT, Pcompensation is set to 0. |
| PEMAX1, PEMAX2 | Maximum TX power level of a UE may use when transmitting on the uplink in the cell (dBm) defined as PEMAX in TS 38.101 [15]. If UE supports SUL frequency for this cell and the cell is configured with SUL carrier, PEMAX1 and PEMAX2 are obtained from the p-Max for SUL in SIB1 and NR-NS-PmaxList for SUL respectively in SIB1, SIB2 and SIB4 as specified in TS 38.331 [3], else PEMAX1 and PEMAX2 are obtained from the p-Max and NR-NS-PmaxList respectively in SIB1, SIB2 and SIB4 for normal UL as specified in TS 38.331 [3]. |
| PPowerClass | Maximum RF output power of the UE (dBm) according to the UE power class as defined in TS 38.101-1 [15]. |

The signalled values Qrxlevminoffset and Qqualminoffset are only applied when a cell is evaluated for cell selection as a result of a periodic search for a higher priority PLMN while camped normally in a VPLMN (TS 23.122 [9]). During this periodic search for higher priority PLMN, the UE may check the S criteria of a cell using parameter values stored from a different cell of this higher priority PLMN.

End of Change