3GPP TSG-RAN WG2 Meeting #129                                     R2-2502041

Wuhan, China, Apr. 7th – 11th, 2025

Source: 			OPPO

Title: 	Discussion on providing MBS service area in NTN network

Agenda item:		8.8.4

Document for: 	Discussion and Decision

- Introduction

Agreements:

- In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list

- It shall be possible to signal multiple area IDs to one MBS service (we insert a list of service area IDs in MCCH)

- Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signaling detailes for “warning area coordinates” (SIB6 is not segmented)

In the last RAN2#129 meeting, agreements on the support of broadcast service in the NTN network have been achieved as follows:

In this paper, we would like to further present our opinions on the topic.

- Discussion

- 2.1 how to link the service area for each MBS session

As indicated in the introduction section, in the last RAN2#129 meeting, we have made an agreement that it shall be possible to signal multiple area IDs to one MBS service (we insert a list of service area IDs in MCCH).Note that also in the last RAN2#128 meeting, we also made an agreement that a signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbour cell(s), which means that in the MCCH, not only the intended service area of the current serving cell, but also those of neighbour cells are needed to be included. In our opinion, there are two possible operations. The first one is to isolate the intended service area of the current cell from the service areas of neighbour cells belonging to each MBS session and attach different IDs for these service areas. Subsequently, only the ID of the service area of the current cell is included in the mbs-SessionInfoList-r17 IE included in the MBSBroadcastConfiguration message, and the IDs of the services areas of neighbour cells belonging to the same MBS session are included in other IEs, e.g., MBS-NeighbourCellList. Another choice is to capture the intended service area for which a part is in current cell and another part is in the neighbour cell of one particular MBS session as a whole in the mbs-SessionInfoList-r17and attach the ID of the whole area to the mbs-SessionInfo IE. In addition, IDs of other unjointed areas, if any, will be included in the mbs-SessionInfoList-r17 or MBS-NeighbourCellList IE, depending on if the area belong to current cell and neighbour cell.

Generally, the first option may result in overwhelmingly large signalling overhead for updating the service area range periodically in the new SIB in case of earth-moving cell, since the co-ordinates of the range of the service area of the both of serving cell and neighbour cell are always changing. Hence, we suggest RAN2 to agree on the second option

Observation 1: for one particular MBS session, isolating the intended service area restricted in current cell range from the ones restricted in other neighbour cells range may result in overwhelmingly large signalling overhead for updating the service area range periodically in the new SIB in case of earth-moving cell, since the service area of the both of serving cell and neighbour cell are always changing.

Proposal 1: RAN2 to agree to capture the intended service area for which some parts are in current cell and other parts are in the neighbour cell as a whole in the mbs-SessionInfoList-r17and link the ID of the area to the corresponding MBS session in the mbs-SessionInfo IE. In addition, IDs of Other unjointed areas, if any, will be included in the mbs-SessionInfoList-r17 or MBS-NeighbourCellList IE, depending if the area belong to current cell and neighbour cells.

- 2.2 MBS interest indication

According to TS 38.331, the purpose of this procedure is to inform the network that the UE in RRC_Connected is receiving or is interested in receiving MBS broadcast service(s) and/or to inform the network of the priority of MBS broadcast versus unicast and multicast MRB reception. Even if the UE is not entering any intended service area, the UE could still perform the MBS interest indication procedure towards the network. Subsequently, if the network would like to provide the MBS service to the UE, no matter where the UE is, the network could just send the MBS content towards the UE via unicast. As a result, we think the legacy mechanism of MBS interest indication could be followed by the NTN cell, and no other additional enhancement such as UE indicating the MBS interest indication only when the UE enters one intended service area is needed.

Observation 2: after the UE sends the MBS interest indication, if the network would like to provide the MBS service towards the UE no matter where the UE is, the network could just send the MBS content towards the UE via unicast.

Proposal 2: does not introduce any additional restriction on when to initialize the MBS interest indication, on the top of the legacy UE behaviour.

- 2.3 is MBS service continuity necessary in NTN?

According to the legacy MBS mechanism, the UE firstly retrieves the mapping relationship between MBS sessions and FSAI from the USD information, and secondly retrieves the mapping relationship between FSAIs and frequency information from the SIB21. Combining such two pieces of information, the UE can derive the mapping relationship between MBS sessions and frequencies. Based on such information, the UE can know whether the frequency of each candidate target cell can provide the interested MBS session(s). As a result, the UE can choose not to read the MCCH to check if the service is provided after camping on the target cell, if the interested MBS session(s) is not served by the frequency at all; also, the UE can choose to prioritize performing reselection to a neighbour cell on which the interested MBS session(s) is provided.

Observation 3: mapping relationship between MBS sessions and frequencies from SIB21 and USD fulfils the following two purposes

- the UE can choose not to read the MCCH to check if the service is provided after camping on the target cell, if the interested MBS session(s) is not served by the frequency at all.

- prioritize performing reselection to a neighbour cell on which the interested MBS session(s) is provided

However, the reason why intended service area shall be included in the SIB21 is not clear. In the current MBS procedure, to the best of our knowledge, the FSAI geographic range is not specified anywhere and is unknown to the UE. So from SIB21, the only thing can be known by the UE is whether or not each neighbour cell frequency can support the interested MBS session service. Such principle shall be followed, i.e., not including the absolute geographic range in the SIB21.

Observation 4: the geographic range of each FSAI included in the SIB21 is not specified anywhere and is unknown to the UE, which implies that currently no absolute applicable geographic range for each MBS frequency is indicated.

Proposal 3: not pursue to including intended service area related information in the SIB21.

- Conclusion and proposals

In this paper, following observations and proposals have been made by us:

Observation 1: for one particular MBS session, isolating the intended service area restricted in current cell range from the ones restricted in other neighbour cells range may result in overwhelmingly large signalling overhead for updating the service area range periodically in the new SIB in case of earth-moving cell, since the service area of the both of serving cell and neighbour cell are always changing.

Proposal 1: RAN2 to agree to capture the intended service area for which some parts are in current cell and other parts are in the neighbour cell as a whole in the mbs-SessionInfoList-r17and link the ID of the area to the corresponding MBS session in the mbs-SessionInfo IE. In addition, IDs of Other unjointed areas, if any, will be included in the mbs-SessionInfoList-r17 or MBS-NeighbourCellList IE, depending if the area belong to current cell and neighbour cells.

Observation 2: after the UE sends the MBS interest indication, if the network would like to provide the MBS service towards the UE no matter where the UE is, the network could just send the MBS content towards the UE via unicast.

Proposal 2: does not introduce any additional restriction on when to initialize the MBS interest indication, on the top of the legacy UE behaviour.

Observation 3: mapping relationship between MBS sessions and frequencies from SIB21 and USD fulfils the following two purposes

- the UE can choose not to read the MCCH to check if the service is provided after camping on the target cell, if the interested MBS session(s) is not served by the frequency at all.

- prioritize performing reselection to a neighbour cell on which the interested MBS session(s) is provided

Observation 4: the geographic range of each FSAI included in the SIB21 is not specified anywhere and is unknown to the UE, which implies that currently no absolute applicable geographic range for each MBS frequency is indicated.

Proposal 3: not pursue to including intended service area related information in the SIB21.