3GPP TSG-RAN WG2 Meeting #129bis	R2-2502061

Wuhan, China, April 7th – April 11th, 2025

Agenda Item	: 7.6 (NR_SL_enh2-Core)

Source	: LG Electronics Inc.

Title	: Discussion on resource re-selection in case of LBT failure on MCSt

Document for	: Discussion and Decision

1.	Introduction

This contribution discusses the resource re-selection in case of LBT failure on MCSt.

2.	Discussion

At the #129 meeting [1], RAN2 discussed as follows regarding resource re-selection in case of LBT failure on MCSt.

R2-2500875 SL Resource reselection in case of LBT failure Ericsson discussion Rel-18 NR_SL_enh2-Core Proposal 1: In the current MAC procedure specified in clause 5.22.1.2c (which is specified for non MCSt case), “the selected number of HARQ retransmissions” need to be deleted in the procedure texts. Proposal 2: add a note in clause 5.22.1.2c stating that the procedure specified in clause 5.22.1.2c is only applicable for non MCSt. FFS if more changes are needed for the case of multiple TB MCSt using single slot candidate resources. Proposal 3: add “as specified in clause 5.22.1.1” in the procedure texts in clause 5.22.1.2 [Huawei, Lenovo, Apple, Ericsson]: For proposal 2, it needs to be specified as normative text. [Vivo]: With normative text, “FFS if more changes are needed for the case of multiple TB MCSt using single slot candidate resources” may need to be discussed. It is good to have complete texts all together. [Session chair]: Let’s postpone it to the next meeting. We’ll capture proposal 2 as normative text. CR will be revisited next meeting (also with the consideration of FFS).

In this contribution, we introduce the impact on the R16 resource selection procedure when performing UE resource reselection in case of LBT failure in MCSt scenarios, and also how the RAN2 agreement below can be applied in each MCSt scenario.

Agreement on MCSt: RAN2 confirms that only Approach 1 and 2 are supported for MCSt in Rel-18 SL-U. Working assumption: Trigger resource (re)selection if all initial transmission and retransmission within MCSt fail due to LBT failure. It should provide minimum specification change.

First of all, the most important thing to consider is whether the above RAN2 agreement is applied to each MCSt scenario, and whether the following prerequisites of the R16 resource (re-)selection operation are violated.

R16 resource (re-)selection procedure: - In the R16 resource selection procedure, when a UE is triggered for resource (re-)selection, it is not allowed to generate multiple grants for multiple TBs.

In the table below, we checked whether the premise for resource selection is violated for each MCSt scenario. In addition, in the table below, we described whether the resource reselection in case of LBT failure in each MCSt scenario is supported in the current MAC specification and the behavior that the UE can take in case of LBT failure.

|  | Multi-slot candidate resources based single TB MCSt | Multi-slot candidate resources based multi-TBs MCSt | Single slot candidate resources based single TB MCSt | Single slot candidate resources based multi-TBs MCSt |  |
| --- | --- | --- | --- | --- | --- |
| Example scenario | e.g., - n_MCSt = 2, - TB1-TB1 (two consecutive slots MCSt) | e.g., - n_MCSt = 2, - TB1-TB1-TB2-TB2 (four consecutive slots MCSt) | e.g., - n_MCSt = 1 or MAC does not pass n_MCSt, -TB1-TB1-TB1-TB1 (four consecutive slots MCSt) | e.g., - n_MCSt = 1 or MAC does not pass n_MCSt, - TB1-TB1-TB2-TB2 (four consecutive slots MCSt) | e.g., - n_MCSt = 1 or MAC does not pass n_MCSt, - TB1-TB2-TB1-TB2 (four consecutive slots MCSt) |
| Support in MAC specification | O | X | X | X | X |
| When applying RAN2 agreement, whether there is a violation of the logic of R16 resource (re-)selection | Not violating - two consecutive slots resource reselection for single grant for TB1 | Not violating - Even if resource reselection is triggered after the last slot of TB2, the UE performs resource reselection independently for individually generated grants for TB1/TB2. | Not violating - four consecutive slots resource reselection for single grant for TB1 | Not violating - Even if resource reselection is triggered after the last slot of TB2, the UE performs resource reselection independently for individually generated grants for TB1/TB2. | Not violating - Even if resource reselection is triggered after the last slot of TB2, the UE performs resource reselection independently for individually generated grants for TB1/TB2. |
| Possible UE behavior for resource re-selection | When LBT failure occurs for all MCSt resources, the UE triggers resource reselection to reselect all MCSt resources. | When LBT failure occurs for all MCSt resources, the UE triggers resource reselection and reselects all MCSt resources independently for each grant. | When LBT failure occurs for all MCSt resources, the UE triggers resource reselection to reselect all MCSt resources. | When LBT failure occurs for all MCSt resources, the UE triggers resource reselection and reselects all MCSt resources independently for each grant. | When LBT failure occurs for all MCSt resources, the UE triggers resource reselection and reselects all MCSt resources independently for each grant. |

As shown in the table above, according to our observation, when applying a common solution based on RAN2 agreement (“Trigger resource (re)selection if all initial transmission and retransmission within MCSt fail due to LBT failure.”) to all MCSt scenarios, there is no behavior that violates the R16 resource selection procedure. And it is desirable to apply the RAN2 consensus-based solution in the same way to multi-TB MCSt scenarios, both in terms of reducing Type 1 LBT overhead and also in terms of not triggering frequent resource re-selection. When it comes to the resourece re-selection, the performing resource re-selection every time there is an LBT failure would result in frequent resource conflicts with other UEs.

Observation 1. According to our observation, when applying a common solution based on RAN2 agreement (“Trigger resource (re)selection if all initial transmission and retransmission within MCSt fail due to LBT failure.”) to all MCSt scenarios, there is no behavior that violates the R16 resource selection procedure.

Observation 2. It is desirable to apply the RAN2 agreement-based solution in the same way to multi-TB MCSt scenarios, both in terms of reducing Type 1 LBT overhead and also in terms of not triggering frequent resource re-selection. When it comes to the resourece re-selection, the performing resource re-selection every time there is an LBT failure would result in frequent resource conflicts with other UEs.

Also, previously RAN2 reached an agreement on resource re-selection on LBT failure only for “multi-slots candidate resource based single TB MCSt” case. The UE behavior related to this is captured in MAC as below. In fact, the sentence captured below is an UE behavior that does not specify individual MCSt scenarios (e.g., single TB MCSt or multi-TBs MCSt). Since RAN2 did not discuss other MCSt scenarios (e.g., multi-TBs MCSt) when reaching the agreement, it is correct to interpret the sentence below as text that applies only to single TB MCSt case (n_MCSt > 1).

#### 5.22.1.2 TX resource (re-)selection check If the TX resource (re-)selection check procedure is triggered on the selected pool of resources for a Sidelink process according to clause 5.22.1.1, the MAC entity shall for the Sidelink process: 1> if PSCCH duration(s) and 2nd stage SCI on PSSCH for all transmissions of a MAC PDU of any selected sidelink grant(s) are not in SL DRX Active time as specified in clause 5.28.3 of the destination that has data to be sent; or 1> if SL_RESOURCE_RESELECTION_COUNTER = 0 and when SL_RESOURCE_RESELECTION_COUNTER was equal to 1 the MAC entity randomly selected, with equal probability, a value in the interval [0, 1] which is above the probability configured by RRC in sl-ProbResourceKeep; or 1> if the pool of resources is configured or reconfigured by RRC; or 1> if there is no selected sidelink grant on the selected pool of resources; or 1> if neither transmission nor retransmission has been performed by the MAC entity on any resource indicated in the selected sidelink grant during the last second; or 1> if sl-ReselectAfter is configured and the number of consecutive unused transmission opportunities on resources indicated in the selected sidelink grant, which is incremented by 1 when none of the resources of the selected sidelink grant within a resource reservation interval is used, is equal to sl-ReselectAfter; or 1> if the selected sidelink grant cannot accommodate a RLC SDU by using the maximum allowed MCS configured by RRC in sl-MaxMCS-PSSCH associated with the selected MCS table and the UE selects not to segment the RLC SDU; or NOTE 1: If the selected sidelink grant cannot accommodate the RLC SDU, it is left for UE implementation whether to perform segmentation or sidelink resource reselection. 1> if transmission(s) with the selected sidelink grant cannot fulfil the remaining PDB of the data in a logical or the remaining SL-PRS delay budget for SL-PRS transmission, if available channel, and the MAC entity selects not to perform transmission(s) corresponding to a single MAC PDU or SL-PRS transmission; or 1> if Sidelink consistent LBT failure is detected as specified in clause 5.31.2 in some RB set(s) of the selected resource pool that spans multiple RB sets for the logical channel, if single carrier frequency is configured; or 1> if a MAC PDU is not transmitted (i.e. initial transmission or retransmission) in any of the resources for this MAC PDU that are associated with the sidelink process for Multi-consecutive slots transmission due to the Sidelink LBT failure: NOTE 2: If the remaining PDB is not met, it is left for UE implementation whether to perform transmission(s) corresponding to single MAC PDU or sidelink resource reselection. NOTE 3: It is left for UE implementation whether to trigger the TX resource (re-)selection due to the latency requirement of the MAC CEs triggered according to clause 5.22.1.7 and clause 5.22.1.10.1. 2> if multiple carrier frequencies are configured: 3> trigger the TX carrier (re-)selection procedure as specified in clause 5.22.1.11. NOTE 3a: It is up to UE implementation to avoid triggering the TX carrier (re-)selection procedure again if it has just performed TX carrier (re-)selection procedure. 2> clear the selected sidelink grant associated to the Sidelink process, if available; 2> trigger the TX resource (re-)selection.

RAN2 discussed the resource selection in case of LBT failure in the case of “multi-TBs MCSt using single slot candidate resources” in the last meeting, but failed to reach an agreement. There are two possible UE behaviors:

Option 1. Trigger resource re-selection when LBT failure occurs in one of the MCSt resources.

Option 2. Trigger resource re-selection when LBT failure occurs in all MCSt resources (same as single TB MCSt solution).

RAN2 is also discussing the possibility of applying option 1 in some scenarios (e.g., TB1-TB2-TB1-TB2) and option 2 in other scenarios (e.g., TB1-TB1-TB2-TB2) for multi-TB MCSt scenarios. If different solutions are to be applied for different scenarios (e.g., single TB MCSt, multi-TB MCSt using single slot candidate resources, multi-TB MCSt using multi-slot candidate resources), the current MAC specification sentences are not sufficient. New scenario-specific behaviors need to be added in the section 5.22.1.2. If the same solution as option 2 is to be applied for all MCSt cases, the current MAC specification sentences can be used as common text, with a simple clarification text added in section 5.22.1.2c as follows:

#### 5.22.1.2c Resource re-selection from SL LBT Failure indication If the MAC entity has been configured with Sidelink resource allocation mode 2 to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] based on sensing or random selection the MAC entity shall for each Sidelink process: 1> if SL LBT failure indication is received from lower layers: 2> for the resource(s) where SL LBT failure is detected from the selected sidelink grant associated to the Sidelink process: 3> if transmission based on random selection is configured by upper layers: 4> randomly select the time and frequency resources for one transmission opportunity from the resource pool excluding all RB sets for which Sidelink consistent LBT failures were detected and not cancelled, if configured according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources. 3> else: 4> randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources. The MAC entity shall only apply the above procedure for non-Multi-consecutive slots transmission case. If a MAC PDU is not transmitted (i.e. initial transmission or retransmission) in any of the resources for this MAC PDU that are associated with the sidelink process for Multi-consecutive slots transmission due to the Sidelink LBT failure, the MAC entity shall reselect the entire Multi-consecutive slots transmission resources according to clause 5.22.1.1.

Observation 3. If different solutions are to be applied for different scenarios (e.g., single TB MCSt, multi-TB MCSt using single slot candidate resources, multi-TB MCSt using multi-slot candidate resources), the current MAC specification sentences are not sufficient. New scenario-specific behaviors need to be added in the section 5.22.1.2. If the same solution as option 2 is to be applied for all MCSt cases, the current MAC specification sentences can be used as common text, with a simple clarification text added in section 5.22.1.2c. That is, from a technical perspective as well as from the perspective of describing UE behavior in the specification, applying a common solution to all scenarios may be a desirable solution because it avoids the complexity of defining UE behavior in the specification.

Proposal 1. RAN2 agrees that the RAN2 agreement (“Trigger resource (re)selection if all initial transmission and retransmission within MCSt fail due to LBT failure.”) for single TB MCSt based on multi-slot candidate resources can be applied to multi-TB MCSt (i.e., Single slot candidate resources based multi-TBs MCSt, Multi-slot candidate resources based multi-TBs MCSt) as well.

Proposal 2. Adopt the corresponding TP captured in Appendix.

3.	Conclusion

Observation 1. According to our observation, when applying a common solution based on RAN2 agreement (“Trigger resource (re)selection if all initial transmission and retransmission within MCSt fail due to LBT failure.”) to all MCSt scenarios, there is no behavior that violates the R16 resource selection procedure.

Observation 2. It is desirable to apply the RAN2 agreement-based solution in the same way to multi-TB MCSt scenarios, both in terms of reducing Type 1 LBT overhead and also in terms of not triggering frequent resource re-selection. When it comes to the resourece re-selection, the performing resource re-selection every time there is an LBT failure would result in frequent resource conflicts with other UEs.

Observation 3. If different solutions are to be applied for different scenarios (e.g., single TB MCSt, multi-TB MCSt using single slot candidate resources, multi-TB MCSt using multi-slot candidate resources), the current MAC specification sentences are not sufficient. New scenario-specific behaviors need to be added in the section 5.22.1.2. If the same solution as option 2 is to be applied for all MCSt cases, the current MAC specification sentences can be used as common text, with a simple clarification text added in section 5.22.1.2c. That is, from a technical perspective as well as from the perspective of describing UE behavior in the specification, applying a common solution to all scenarios may be a desirable solution because it avoids the complexity of defining UE behavior in the specification.

Proposal 1. RAN2 agrees that the RAN2 agreement (“Trigger resource (re)selection if all initial transmission and retransmission within MCSt fail due to LBT failure.”) for single TB MCSt based on multi-slot candidate resources can be applied to multi-TB MCSt (i.e., Single slot candidate resources based multi-TBs MCSt, Multi-slot candidate resources based multi-TBs MCSt) as well.

Proposal 2. Adopt the corresponding TP captured in Appendix.

4.	Reference

[1] Session chair’s note for R18 SL-Evo of #129 meeting (Samsung)

5.	Appendix

Text proposal

START OF CHANGE

5.22.1.2	TX resource (re-)selection check

If the TX resource (re-)selection check procedure is triggered on the selected pool of resources for a Sidelink process according to clause 5.22.1.1, the MAC entity shall for the Sidelink process:

1>	if PSCCH duration(s) and 2nd stage SCI on PSSCH for all transmissions of a MAC PDU of any selected sidelink grant(s) are not in SL DRX Active time as specified in clause 5.28.3 of the destination that has data to be sent; or

1>	if SL_RESOURCE_RESELECTION_COUNTER = 0 and when SL_RESOURCE_RESELECTION_COUNTER was equal to 1 the MAC entity randomly selected, with equal probability, a value in the interval [0, 1] which is above the probability configured by RRC in sl-ProbResourceKeep; or

1>	if the pool of resources is configured or reconfigured by RRC; or

1>	if there is no selected sidelink grant on the selected pool of resources; or

1>	if neither transmission nor retransmission has been performed by the MAC entity on any resource indicated in the selected sidelink grant during the last second; or

1>	if sl-ReselectAfter is configured and the number of consecutive unused transmission opportunities on resources indicated in the selected sidelink grant, which is incremented by 1 when none of the resources of the selected sidelink grant within a resource reservation interval is used, is equal to sl-ReselectAfter; or

1>	if the selected sidelink grant cannot accommodate a RLC SDU by using the maximum allowed MCS configured by RRC in sl-MaxMCS-PSSCH associated with the selected MCS table and the UE selects not to segment the RLC SDU; or

NOTE 1:	If the selected sidelink grant cannot accommodate the RLC SDU, it is left for UE implementation whether to perform segmentation or sidelink resource reselection.

1>	if transmission(s) with the selected sidelink grant cannot fulfil the remaining PDB of the data in a logical or the remaining SL-PRS delay budget for SL-PRS transmission, if available channel, and the MAC entity selects not to perform transmission(s) corresponding to a single MAC PDU or SL-PRS transmission; or

1>	if Sidelink consistent LBT failure is detected as specified in clause 5.31.2 in some RB set(s) of the selected resource pool that spans multiple RB sets for the logical channel, if single carrier frequency is configured; or

1>	if a MAC PDU is not transmitted (i.e. initial transmission or retransmission) in any of the resources for this MAC PDU that are associated with the sidelink process for Multi-consecutive slots transmission due to the Sidelink LBT failure:

NOTE 2:	If the remaining PDB is not met, it is left for UE implementation whether to perform transmission(s) corresponding to single MAC PDU or sidelink resource reselection.

NOTE 3:	It is left for UE implementation whether to trigger the TX resource (re-)selection due to the latency requirement of the MAC CEs triggered according to clause 5.22.1.7 and clause 5.22.1.10.1.

2>	if multiple carrier frequencies are configured:

3>	trigger the TX carrier (re-)selection procedure as specified in clause 5.22.1.11.

NOTE 3a:	It is up to UE implementation to avoid triggering the TX carrier (re-)selection procedure again if it has just performed TX carrier (re-)selection procedure.

2>	clear the selected sidelink grant associated to the Sidelink process, if available;

2>	trigger the TX resource (re-)selection as specified in clause 5.22.1.1.

NOTE 4:	Void.

NOTE 5:	Void.

NEXT CHANGE

5.22.1.2c	Resource re-selection from SL LBT Failure indication

If the MAC entity has been configured with Sidelink resource allocation mode 2 to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] based on sensing or random selection the MAC entity shall for each Sidelink process:

1>	if SL LBT failure indication is received from lower layers:

2>	for the resource(s) where SL LBT failure is detected from the selected sidelink grant associated to the Sidelink process:

3>	if transmission based on random selection is configured by upper layers:

4>	randomly select the time and frequency resources for one transmission opportunity from the resource pool excluding all RB sets for which Sidelink consistent LBT failures were detected and not cancelled, if configured according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources.

3>	else:

4>	randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources.

The MAC entity shall only apply the above procedure for non-Multi-consecutive slots transmission case. If a MAC PDU is not transmitted (i.e. initial transmission or retransmission) in any of the resources for this MAC PDU that are associated with the sidelink process for Multi-consecutive slots transmission due to the Sidelink LBT failure, the MAC entity shall reselect the entire Multi-consecutive slots transmission resources according to clause 5.22.1.1.

END of CHANGE