# 3GPP TSG-RAN WG2 Meeting #129bis														R2-2502078

# Wuhan, P. R. China, Apr. 7th – 11st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2058 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UTW for positioning SRS frequency hopping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-04-07 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are currently some confusions on the wording “MAC instructs the PHY for SRS for positioning Tx frequency hopping” in the MAC spec within the UTW window. This does not mean that the UE does not transmit SRS outside of the UTW window. Some improvements in the wording have been proposed to make matters clear. On the confusion for MAC modelling, MAC spec has control over the transmission of SRS already as of today. One example can be seen in the DRX operations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise the spec that within the UTW, the UE procedure for SRS transmission for positioning Tx frequency hopping is defined in the PHY spec. Inter-Operability analysis: Impacted functionality: SRS for Positioning Tx Frequency Hopping Impacted 5G architecture options: NR SA Inter-operability: If the UE is implemented according to this CR while the NW is not or then the NW is implemented according to this CR while the UE is not, there is no inter-operability issue |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec continues to be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.26, 5.32 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

============================CHANGE BEGINS===========================================

5.26	Positioning SRS transmission in RRC_INACTIVE

5.26.1	General

Periodic and semi-persistent Positioning SRS with or without positioning SRS bandwidth aggregation can be configured for Positioning SRS transmission in RRC_INACTIVE.

SRS for positioning Tx frequency hopping as in clause 5.32 can also be configured for Positioning SRS transmission in RRC_INACTIVE.

The MAC entity shall,

1>	if the TA of the configured Positioning SRS is valid according to clause 5.26.2, and the conditions for positioning SRS transmission in clause 7.3.1 of TS 38.213 [6] and clause 6.2.1.4 of TS 38.214 [7] are satisfied:

2>	instruct to the lower layer according to TS 38.214 [7] to transmit Periodic or Semi-Persistent Positioning SRS that is activated according to clause 5.18.17 or clause 5.18.37.

===========================NEXT CHANGE===========================================

5.32	SRS for positioning Tx frequency hopping

SRS for positioning Tx frequency hopping can be supported for UE in RRC_CONNECTED and RRC_INACTIVE. The UE can be configured with UTW during which the UE is allowed to transmit SRS for positioning Tx frequency hopping.

RRC configures the following parameters for the UTW for SRS for positioning Tx frequency hopping:

-	utw-SlotPeriodicity: Periodicity of the UTW in slot;

-	utw-SlotOffset: Offset of the beginning of the UTW with respect to the beginning of the UTW periodicity in time domain in slot;

-	duration: Time duration of the UTW for SRS transmission for positioning Tx frequency hopping.

When UTW is configured, the  UTW starts in the first symbol of the slot for which the following condition is satisfied

CURRENT_slot modulo (utw-SlotPeriodicity) = utw-SlotOffset

where CURRENT_slot = SFN × numberOfSlotsPerFrame + slot number in the frame and numberOfSlotsPerFrame refers to the number of consecutive slots per frame as specified in TS 38.211 [8] and continues for time duration defined by duration.

When UTW is configured and within the UTW, the UE's procedure for SRS for positioning Tx frequency hopping is specified in TS 38.214 [7].

========================= CHANGE ENDS==============================================