3GPP TSG-RAN WG2 Meeting #129-bis                                      R2-2502084

Wuhan, China, Apr 7th – 11th, 2025

Source:        ZTE Corporation

Title:           Discussion on LCM for positioning use case

Agenda item:   8.1.2.3

Document for:  Discussion and Decision

- Introduction

The revised WID [1] of Rel-19 NR_AIML_air related to AI/ML positioning is given as follows:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

This contribution provides our view on LCM for AI/ML positioning use case, focusing on use case 1.

- Discussion

The inference procedure of 3 AI/ML positioning use cases described in the WID are briefly depicted as follows:

Figure 1. Use case 1, model inference is at UE side (1st priority)

Figure 2. Use case 3a, model inference is at gNB side (1st priority)

Figure 3. Use case 3b, model inference is at LMF side (1st priority)

Fallback from new AI method to legacy non-AI method

Legacy positioning only requires UE to support legacy/normal UE processing capability. In AI positioning, the UE’s real-time applicable functionality reporting is supported. That means UE has to confirm AI availability all the time since the AI functionality may become unavailable at any time. Therefore, even if AI positioning has better performance than legacy positioning, AI positioning may fail/be unavailable more frequently than the legacy positioning methods.

Observation 1: AI positioning may fail/be unavailable more frequently than the legacy positioning methods.

When LMF thinks it is ok to activate AI positioning and activates AI, UE may report Error when the AI functionality becomes non-applicable. Then LMF has to activate another positioning method, and UE has to return the positioning measurement/positioning estimate of the scheduled another positioning method. Such procedure requires two round of LPP message transfer which takes a lot of time, and the QoS of timing requirement may be hard to achieve. However, the LCS client does not care whether UE uses AI positioning or legacy positioning, the LCS client only wants to get UE’s location within the QoS requirement.

Observation 2: If AI positioning fails and then UE is scheduled with another positioning method, such procedure requires two round of LPP message transfer which takes a lot of time, and the QoS of timing requirement may be hard to achieve.

To solve the issue, LMF can schedule AI positioning together with other fallback positioning method(s) to UE, UE will take AI positioning as first priority. If it fails, UE can use the fallback positioning method to perform positioning and report positioning measurements/estimates on time. For usecase 1, UE needs to send AI-estimated location to LMF, so the fallback method can be DL-TDOA or DL-AoD which are UE-based positioning methods.

For the concern that LMF may transmit ProvideAssistanceData / RequestLocationInformation twice, since the ProvideAssistanceData / RequestLocationInformation of different positioning method can be embedded in one LPP message type via SEQUENCE structure, only one LPP message is enough. So the latency can also be reduced.

Figure 4. (a) LMF does not configure fallback method vs. (b) LMF configures fallback method

If multiple positioning methods are configured to UE, in legacy positioning procedure, UE choose the method based on its implementation. However, due to AI positioning is more power consuming and requires more computing capability, if LMF configures AI and non-AI methods together, UE may likely to choose non-AI positioning method. So LMF can indicate the first priority of using AI positioning method in some cases when LMF wants UE to choose AI positioning first.

Proposal 1: LMF can schedule AI positioning method together with DL-TDOA or DL-AoD positioning method to UE, and LMF indicates the first priority of using AI positioning method.

Proposal 2: If AI positioning fails or currently non-applicable, LMF can allow UE to use DL-TDOA or DL-AoD as fallback, in order to report positioning measurements/estimates on time.

Functionality/AI model activation

Based on the discussion so far, a comprehensive understanding of functionality and AI model is, the functionality can be recognized per use case, different functionality can be distinguished by different UE capability and different UE applicable functionality. Under each functionality, there can be multiple AI models, different models can be distinguished by different NW/UE additional conditions.

Figure 5. An example relationship between functionality and AI model

RAN2#125-bis [2] has made the following agreement for UE side model:

For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19.

Based on the above understanding of functionality and AI model, there can have following approaches to control the LCM:

- Approach 1: LMF activates/deactivates the functionality, and LMF activates/deactivates the AI model of the functionality.

  - Approach 1 means whether UE activates/deactivates use case 1 is determined and indicated by LMF. For use case 1 where the AI model is at UE side, LMF should further determine which AI model under the indicated use case should be activated/deactivated. That is to say, LMF needs to get UE’s applicable functionality to know which functionalities is available/unavailable to UE, and LMF should also know the relationship between UE side additional condition and AI model, so that LMF knows which AI model is available/unavailable to UE.

- Approach 2: LMF activates/deactivates the functionality, and LMF allows UE to activate/deactivate the AI model of the functionality

- Approach 2 means whether UE activates/deactivates use case 1 is determined and indicated by LMF. For use case 1 where the AI model is at UE side, LMF can allow UE to activate/deactivate the AI model of the indicated use case according to UE’s own additional condition and already provided NW side additional conditions. That is to say, LMF needs to get UE’s applicable functionality to know which functionalities is available/unavailable to UE.

- Approach 3: LMF allows UE to activate/deactivate the functionality, and LMF allows UE to activate/deactivate the AI model of the functionality, and UE reports the activated/deactivated functionality to LMF

- Approach 3 means LMF can allow UE to activate/deactivate use case1 according to UE’s applicable functionalities. That is to say, LMF can allow UE to report AI UE location. Then UE should let LMF know which functionalities is activated/deactivated (e.g., using different measurement report). Further, LMF can allow UE to activate/deactivate the AI model of the indicated use case according to UE’s own additional condition and already provided NW side additional conditions.

- Or Approach 3 can be utilized when LMF configures some fallback functionalities to UE. That is to say, LMF configures more than one positioning methods to UE, wherein the AI positioning case 1 is the highest priority. But, LMF also allows UE to activate other configured positioning methods (e.g., non-AI positioning method) if AI positioning method is not available right now -- to acquire timely and accurate measurement result.

As we analysed above, NW side additional condition is already provided to UE. It is not necessary and not efficient to let UE report UE additional condition to LMF, then LMF decides the AI model and feedback to UE, i.e., approach 1 is not preferred in AI/ML positioning.

Also as mentioned above, the relationship between additional condition and AI model is hard to specify, so LMF is hard to know such information. Since UE knows the UE side additional condition and NW side additional condition, it is more efficient to let UE decide the AI model under each functionality, i.e., go with approach 2 or approach 3.

RAN2#129 has achieved the following agreement:

FFS UE autonomous switching between AI/ML and non-AI/ML methods is not allowed. FFS if this is unconditional or linked to condition of multiple positioning method are not configured in RequestLocationInformation.

Our view is that, if LMF allows UE to perform autonomous switching between AI/ML and non-AI/ML methods (such as configure fallback mechanism), UE can perform autonomous switching. If LMF does not allow or confirm any fallback mechanism, UE cannot perform autonomous switching. In summary, the UE autonomous switching between AI/ML and non-AI/ML methods is only applicable under LMF’s allowance, i.e., only applicable when LMF configures multiple positioning methods and allows UE to perform such switching.

Observation 3: Since NW side additional condition is already provided to UE, it is not necessary and not efficient to let UE report UE additional condition to LMF, then LMF decides the AI model and feedback to UE.

Proposal 3: For AI/ML positioning use case 1, RAN2 supports both following approaches for functionality-LCM:

- LMF activates/deactivates the functionality

- LMF allows UE to activate/deactivate the functionality by configuring fallback positioning method(s)

Proposal 4: For AI/ML positioning use case 1, LMF allows UE to activate/deactivate the AI model of the activated functionality.

LCM procedure using the new AI positioning method

In 37.355 logic, one positioning method contains the following 5 basic procedures:

Figure 6. LPP signaling in a positioning method in current spec

The signaling contained in the new AI/ML positioning method should be designed to support all of training, inference and performance monitoring.

Proposal 5: The signaling of the new AI/ML positioning method should support all of LCM procedures including training, inference and performance monitoring.

Training data collection request/response

For usecase 1, UE needs to get training data set for training. The training data set can be UE’s own channel PRS measurement (with the label of UE’s own location), or can be other PRU’s PRS channel measurement (with the label of other PRU’s location/intermediate feature).

To acquire the UE’s own PRS channel measurement and the UE’s own location, UE needs to get PRS configuration and/or TRP information in advance. So, the LMF should be able to provide PRS configuration and/or TRP information to a UE (i.e., green arrows in the figure 8).

LMF should also be able to provide other UE/PRU’s PRS channel measurement with PRU/UE location/intermediate feature to a UE, for AI/ML training or monitoring (see green arrows in the figure 8).

Figure 7. LMF provides PRS configuration, TRP information and other UE/PRU’s PRS channel measurement with PRU/UE location to target UE for AI training

The provision of such assistance data for AI/ML training can be embedded in ProvideAssistanceData message. And the request of such assistance data for AI/ML training can be embedded in RequestAssistanceData message.

Proposal 6: For usecase 1, UE can request assistance data for AI/ML training using RequestAssistanceData message, and LMF can provide assistance data for AI/ML training using ProvideAssistanceData message.

Different training data set associated with different TRP information will have different impact on UE AI model’s performance, for example, the model trained using DL-PRS measurement results from 20 TRPs will be more accurate than using DL-PRS measurement results from 3 TRPs. To suit the current UE’s condition or given QoS requirement for AI/ML positioning, UE can request some specific assistance data that will have impact on model training, e.g.:

- UE requests the number of TRPs that the UE wants to receive DL-PRS from;

- UE requests the area of TRPs that the UE wants to receive DL-PRS from;

LMF may also provide other PRU’s measurement and PRU label to target UE for training. So UE is better to request the number of PRUs that the UE wants to receive the corresponding measurement and labels.

Proposal 7: UE can request some specific assistance data for training, as following:

- UE requests the number of TRPs that the UE wants to receive DL-PRS from;

- UE requests the area of TRPs that the UE wants to receive DL-PRS from;

- UE requests the number of PRUs that the UE wants to receive the corresponding measurement and labels from.

Performance monitoring results transfer

RAN1#116-bis [5] has the following agreement on performance monitoring:

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Based on the agreement, there are 4 kinds of performance monitoring control procedure under RAN1’s decision:

- Option 1: UE generates monitoring metric, and UE makes monitoring decision;

- Option 2: LMF generates monitoring metric, and LMF makes monitoring decision;

- Option 3: UE generates monitoring metric, and LMF makes monitoring decision;

- Option 4: LMF generates monitoring metric, and UE makes monitoring decision.

Option 1 and option 2 appear to have no LPP specification on performance monitoring.

If option 3 is adopted, LMF needs to request UE to perform monitoring, and UE needs to report monitoring metric to LMF.

If option 4 is adopted, LMF needs to indicate monitoring metric to UE.

RAN1#119 further agrees the following to support the above option 1 or option 3 which requires spec impact:

Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B

For the above potential spec impact, RAN2 needs to discuss whether a new set of signaling/procedure dedicated for performance monitoring is necessary, or to embed it in other existing procedures (e.g. Location Information transfer procedure).

Proposal 8: RAN2 needs to discuss whether to introduce a new signaling procedure in LPP for performance monitoring metric transfer, or to include the performance monitoring metric transfer in the Request/ProvideLocationInformation message.

Abort and Error procedure in the new positioning method

In current LPP specification, the Abort message is to allow the UE or LMF to abort an ongoing procedure due to some unexpected event (e.g., cancellation of a location request by an LCS client). It can also be used to stop an ongoing procedure (e.g., periodic location reporting from the UE).

Therefore, the abort procedure can be reused in this new AI positioning method for LMF to deactivate a functionality of UE. For example, if LMF makes the monitoring decision and decides the current AI/ML positioning result is bad, LMF can send the Abort message to abort the on-going PRS measurement/report of the AI positioning method.

Proposal 9: The Abort message should be used in the AI/ML positioning method to deactivate a functionality of UE.

In current LPP specification, the Error message is to indicate an error occurs at LMF or UE side during an on-going positioning method. For this new AI/ML positioning method, the Error message can be reused with some new error cause indication dedicated for AI/ML positioning use cases. RAN2#128 has already agreed to use Error message when the AIML based positioning method becomes non-applicable.

The error message can indicate error occurred during training, e.g., UE does not get enough model training data to train a model, or UE cannot download or acquire a AI model, etc.; The error message can also indicate error occurred during inference, e.g., UE cannot generate a AI UE location using a AI model, or UE cannot generate a AI intermediate feature using a AI model, etc.

Proposal 10: New error causes should be dedicated for specific LCM procedures (e.g. training or inference or monitoring).

Capability transfer procedure

RAN2#126 [4] has agreed to reuse Capability transfer procedure to indicate supported AI/ML positioning capabilities, and agreed to reuse Location Information transfer procedure to provide the results of the UE sided model inference:

RAN2#126 agreement The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality. At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation. FFS further details on signaling enhancements.

So the capability transfer procedure should be common for training, inference and monitoring.

RAN2#127-bis has agreed an example procedure for inference:

The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: - Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. - Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. - Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). - Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. - Step 5: The LMF requests the inferred location information using the LPP request location information message. - Step 6: UE reports the inferred location using LPP provide location information message. Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details.

RAN2 agreed that dynamic UE capability and static UE capability should be both contained in LPP Provide Capabilities message, wherein the dynamic UE capability can use unsolicited report, static UE capability can use solicited report.

RAN2#129 has the following FFS:

Agreement UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality. FFS if any additional LMF control is needed.

In legacy mechanism, the unsolicited report is not controlled by LMF, i.e., UE is allowed to report dynamic UE capability at any time when the UE feels the need. In AI/ML positioning, LMF also do not need to control UE sending unsolicited LPP provide capabilities, similar like legacy mechanism. In addition, LMF does not know when/how the UE’s dynamic capability will change, and LMF does not know the frequency of change. It is hard for LMF to control such unsolicited report.

Proposal 11: Do NOT support LMF to control the UE sending unsolicited LPP provide capabilities using explicit signaling.

Consistency between training and inference

There is a discussion on how to ensure the consistency between training and inference. The basic assumption is that, each AI model that UE trains should be under some certain condition, and the trained AI model should also be used for inference under the same or similar condition to achieve higher accuracy. From specification perspective, this means the set of attribute/parameter/condition of the training data input of an AI model should be same or similar as the set of attribute/parameter/condition of the inference data input.

For AI/ML positioning usecase 1, UE receives and measures TRP’s PRS for training and inference, so the set of attribute/parameter/condition of the training data input or inference data input can include NW additional conditions such as TRP location, TRP synchronization error, TRP number, PRS configuration, etc. It can be seen that for AI/ML positioning, the aforementioned NW additional conditions can already be indicated to the UE in current LPP spec since Rel-16.

Since detailed NW side additional condition is already provided to UE without any privacy concern, there is no need to use associated ID to represent a specific kind of NW side additional conditions. In addition, it is hard to set up explicit relationship between NW additional conditions and AI models, for example, an AI model can have good generalization and it can work well under several different additional conditions.

Observation 4: For AI/ML positioning, the NW additional conditions (e.g., TRP location, TRP synchronization error, TRP number, PRS configuration) can already be indicated to the UE in current LPP spec.

Proposal 12: For AI/ML positioning usecase 1, ensuring the consistency between training and inference does not need an associated ID.

LMF is the node to provide inference configuration to UE, UE should strictly following NW’s configuration to perform inference. But LMF does not know which kind of inference configuration best suits the training configuration using which UE trained its AI model, e.g., how many number of TRPs used during training, which area the used TRPs locate on, the synchronization error range that the used TRP belongs to, etc. Therefore, UE should be able to request some specific TRP attributes/NW side additional conditions (for inference) that matches UE’s current well-trained AI model to ensure the training-inference consistency. The specific request can include the wanted number of TRPs, the wanted area of TRPs, the wanted synchronization error between TRPs for inference.

Since different AI model has different generalization and it is only known by UE, so only the UE knows how much difference the training and inference owns that will not have big impact to the inference performance, and to ensure the QoS requirement. The UE is able to ensure the requested value of TRP attribute to be consistent with the value used during training, and the UE ensures such difference can be covered by the good generalization performance.

The above request can reuse the current on-demand PRS request procedure.

Observation 5: When providing inference configuration to UE, LMF may not know which kind of inference configuration best suits the training configuration that UE used to train an AI model.

Proposal 13: In order to ensure consistency between training and inference, UE should be able to request some specific assistance data for inference which suits the UE’s current trained AI model, for example:

- UE can request the wanted number of TRPs/cells for inference;

- UE can request the wanted area of TRPs/cells for inference;

- UE can request the wanted synchronization error between TRPs for inference.

- Conclusion

In this contribution, we propose the following observation and proposals:

Fallback from new AI method to legacy non-AI method

Observation 1: AI positioning may fail/be unavailable more frequently than the legacy positioning methods.

Observation 2: If AI positioning fails and then UE is scheduled with another positioning method, such procedure requires two round of LPP message transfer which takes a lot of time, and the QoS of timing requirement may be hard to achieve.

Proposal 1: LMF can schedule AI positioning method together with DL-TDOA or DL-AoD positioning method to UE, and LMF indicates the first priority of using AI positioning method.

Proposal 2: If AI positioning fails or currently non-applicable, LMF can allow UE to use DL-TDOA or DL-AoD as fallback, in order to report positioning measurements/estimates on time.

Functionality/AI model activation

Observation 3: Since NW side additional condition is already provided to UE, it is not necessary and not efficient to let UE report UE additional condition to LMF, then LMF decides the AI model and feedback to UE.

Proposal 3: For AI/ML positioning use case 1, RAN2 supports both following approaches for functionality-LCM:

- LMF activates/deactivates the functionality

- LMF allows UE to activate/deactivate the functionality by configuring fallback positioning method(s)

Proposal 4: For AI/ML positioning use case 1, LMF allows UE to activate/deactivate the AI model of the activated functionality.

LCM procedure using the new AI positioning method

Proposal 5: The signaling of the new AI/ML positioning method should support all of LCM procedures including training, inference and performance monitoring.

Proposal 6: For usecase 1, UE can request assistance data for AI/ML training using RequestAssistanceData message, and LMF can provide assistance data for AI/ML training using ProvideAssistanceData message.

Proposal 7: UE can request some specific assistance data for training, as following:

- UE requests the number of TRPs that the UE wants to receive DL-PRS from;

- UE requests the area of TRPs that the UE wants to receive DL-PRS from;

- UE requests the number of PRUs that the UE wants to receive the corresponding measurement and labels from.

Proposal 8: RAN2 needs to discuss whether to introduce a new signaling procedure in LPP for performance monitoring metric transfer, or to include the performance monitoring metric transfer in the Request/ProvideLocationInformation message.

Abort and Error procedure

Proposal 9: The Abort message should be used in the AI/ML positioning method to deactivate a functionality of UE.

Proposal 10: New error causes should be dedicated for specific LCM procedures (e.g. training or inference or monitoring).

Capability transfer procedure

Proposal 11: Do NOT support LMF to control the UE sending unsolicited LPP provide capabilities using explicit signaling.

Consistency between training and inference

Observation 4: For AI/ML positioning, the NW additional conditions (e.g., TRP location, TRP synchronization error, TRP number, PRS configuration) can already be indicated to the UE in current LPP spec.

Observation 5: When providing inference configuration to UE, LMF may not know which kind of inference configuration best suits the training configuration that UE used to train an AI model.

Proposal 12: For AI/ML positioning usecase 1, ensuring the consistency between training and inference does not need an associated ID.

Proposal 13: In order to ensure consistency between training and inference, UE should be able to request some specific assistance data for inference which suits the UE’s current trained AI model, for example:

- UE can request the wanted number of TRPs/cells for inference;

- UE can request the wanted area of TRPs/cells for inference;

- UE can request the wanted synchronization error between TRPs for inference.

- Reference

- RP-250792, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 2025-03

- Report of 3GPP TSG RAN WG2 meeting #125-bis, Changsha, China, 2024-04

- TS 38.305-i10, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN, 2024-03

- Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan, 2024-05

- R1-2403741, Summary of specification support for positioning accuracy enhancement, Changsha, China, 2024-04