3GPP TSG-RAN WG2 Meeting #129bis	R2-2502098

Wuhan, China, April 07th – 11th, 2025

Agenda item:	8.4.1

Source:	Huawei, HiSilicon

Title:	Discussion on UE capability for LP-WUS

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss UE capability for LP-WUS.

# Discussion

## Subgrouping capability

CN-assigned subgrouping

For PEI, the support of CN-assigned subgrouping for PEI is reported to CN from UE in the Registration Request message, and CN decides whether to use it and assigns the subgrouping ID to the UE if it is used. For LP-WUS, the same procedure as PEI is reused for CN-assigned subgrouping. The UE capability reporting of CN-assigned subgrouping for LP-WUS has been captured in SA2 specification [1].

#### 5.4.12a.2 Core Network Assistance for LP-WUSPS To support the LP-WUSPS, LP-WUS Paging Subgrouping Support Indication and the LP-WUSPS Assistance Information is used by the AMF and NG-RAN to help determine whether LP-WUSPS applies to the UE and which paging subgroup is used when paging the UE (see TS 38.300 [27]). In the Registration Request message, the UE includes the LP-WUS Paging Subgrouping Support Indication if the UE supports LP-WUS Paging Subgrouping. If the UE includes LP-WUS Paging Subgrouping Support Indication, the UE may also include the paging probability information to assist the AMF. The AMF supporting LP-WUSPS stores the UE provided LP-WUS Support Indication in the UE context and the AMF determines AMF LP-WUSPS Assistance Information based on this indication. The AMF may further consider the local configuration, the UE's paging probability information if provided, information provided by the RAN (e.g. any of the Information On Recommended Cells And RAN nodes For Paging), and/or previous statistical information for the UE to determines AMF LP-WUSPS Assistance Information. The AMF LP-WUSPS Assistance Information includes the Paging Subgroup ID.

Observation 1: The support of CN-assigned subgrouping for PEI is reported to CN from UE by NAS signaling. The support of CN-assigned subgrouping for LP-WUS is reported to the CN reusing the same procedure as PEI, which has been captured in SA2 specification.

UE ID-based subgrouping

For PEI, UE ID-based subgrouping capability is coupled with PEI capability. That is if a UE supports PEI reception, it shall also support UEID based subgrouping.

pei-SubgroupingSupportBandList-r17 Indicates whether the UE supports receiving paging early indication in DCI format 2_7 as specified in TS 38.304 [21] for a list of frequency band. The UE shall support UEID based subgrouping for a frequency band if it indicates supporting of paging early indication reception for the frequency band. The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the requirement for paging PDCCH in Type 2 CSS for IDLE and INACTIVE mode UEs.

Observation 2: If a UE supports PEI reception, it shall also support UE ID based subgrouping.

As LP-WUS is also used to wake-up one or several groups of UEs, the principle of PEI can be applied to LP-WUS. The reception of LP-WUS is supported by either OOK-based LR or OFDM-based LR which are defined as separate UE features in RAN1. Hence, support of LP-WUS reception in IDLE/INACTIVE by any type of LR indicates supporting UE ID-based subgrouping. The latest RAN1 UE features is provided in Appendix 3.

Proposal 1: Support of LP-WUS in IDLE/INACTIVE (either OOK-based or OFDM-based) indicates support of UE ID-based subgrouping by default.

In last RAN2 meeting, we had the following agreement for the UE radio paging capability issue raised by SA2. It was agreed that UE ID-based subgrouping capability should be included in UE-RadioPagingInfo-r17 container.

- Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.

If the Proposal 1 above can be agreed, the capability of LP-WUS in IDLE/INACTIVE defined in RAN1 is coupled with UE ID-based subgrouping capability as one combined capability, thus, the capability of LP-WUS in IDLE/INACTIVE (coupled with UE ID-based subgrouping capability) should be included in UE-RadioPagingInfo-r17 in UECapabilityInformation message. Note that PEI capability is also included in UE-RadioPagingInfo-r17. In this case, the capability of LP-WUS in IDLE/INACTIVE can be forwarded from gNB to CN by UERadioPagingInformation message, and included in the Paging message from CN to gNB naturally. The granularity (such as per band) and other details for capability of LP-WUS in IDLE/INACTIVE depends on RAN1 features list.

Proposal 2: The capability of LP-WUS in IDLE/INACTIVE (coupled with UE ID-based subgrouping capability) is included in UE-RadioPagingInfo-r17 in RRC UECapabilityInformation message. The granularity (such as per band) and other details for capability of LP-WUS in IDLE/INACTIVE depends on RAN1 features list.

## RRM measurement capability

RRM measurement relaxation capabilities discussed in Rel-16 and Rel-17 are defined as RAN2 capabilities. Thus, capability of RRM measurement fully offloading and RRM measurement relaxation for LP-WUS can also be considered as RAN2 capabilities.

Observation 3: RRM measurement relaxation capabilities discussed in Rel-16 and Rel-17 are defined as RAN2 capabilities.

Proposal 3: RAN2 capability defines RRM measurement fully offloading and RRM measurement relaxation for LP-WUS.

In RAN2#127 meeting, we had the following agreement. Only the LP-WUS capable UE will support and perform RRM measurement fully offloading and relaxation. Thus, UE supporting of RRM measurement fully offloading and relaxation should also support LP-WUS.

- RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

Observation 4: UE supporting RRM measurement fully offloading and relaxation shall also support LP-WUS.

If the UE is monitoring LP-WUS without RRM measurement fully offloading or RRM measurement relaxation, there is almost no power saving gain since UE still needs to turn on the MR frequently to perform the normal RRM measurement on either serving cell or neighbour cell, as stated in TR 38.869 [2]. Thus, if the UE supports LP-WUS, the UE shall also support RRM measurement fully offloading and relaxation.

Observation 5: UE supporting LP-WUS shall also support RRM measurement fully offloading and relaxation, otherwise there is almost no power saving gain.

We don’t see the necessity of only supporting RRM measurement fully offloading or only RRM measurement relaxation, both can be supported without much additional complexity but with more power saving benefit. Thus, RRM measurement fully offloading and relaxation can be one combined capability.

Proposal 4: Define RRM measurement capability as “Conditionally mandatory features without UE radio access capability parameters”. It is mandatory to support RRM measurement fully offloading and RRM measurement relaxation in RRC_IDLE/RRC_INACTIVE for UEs which support LP-WUS.

# Conclusion

In this paper, we discuss UE capability for LP-WUS, and have the following proposals.

Subgrouping capability

Proposal 1: Support of LP-WUS in IDLE/INACTIVE (either OOK-based or OFDM-based) indicates support of UE ID-based subgrouping by default.

Proposal 2: The capability of LP-WUS in IDLE/INACTIVE (coupled with UE ID-based subgrouping capability) is included in UE-RadioPagingInfo-r17 in RRC UECapabilityInformation message. The granularity (such as per band) and other details for capability of LP-WUS in IDLE/INACTIVE depends on RAN1 features list.

RRM measurement capability

Proposal 3: RAN2 capability defines RRM measurement fully offloading and RRM measurement relaxation for LP-WUS.

Proposal 4: Define RRM measurement capability as “Conditionally mandatory features without UE radio access capability parameters”. It is mandatory to support RRM measurement fully offloading and RRM measurement relaxation in RRC_IDLE/RRC_INACTIVE for UEs which support LP-WUS.

# References

3GPP TS 23.501, System architecture for the 5G System (5GS); Stage 2 (Release 19).

TR 38.869, Study on low-power wake up signal and receiver for NR.

# Appendix 1 – 38.306

6    Conditionally mandatory features without UE radio access capability parameters

| Features | Condition |
| --- | --- |
| Acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList | It is mandatory to support acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList for UEs which support the acquisition of the posSIB types in posSchedulingInfoList as specified in TS 38.331 [9]. |
| Acquisition of SI messages with explicit SI window positions | It is mandatory to support acquisition of SI messages with explicit SI window positions for UEs which support the SIB types in schedulingInfoList2 as specified in TS 38.331 [9]. |
| AS layer memory size for QoE paused measurement reports | It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. |
| AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE | It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports" |
| ATG specific P-max | It is mandatory to support the ATG specific P-max configured by network for UEs supporting airToGroundNetwork-r18. |
| Downlink SDAP header | Either NAS reflective QoS or as-ReflectiveQoS is supported. |
| Extended values for drx-HARQ-RTT-TimerDL/UL | It is mandatory for UEs which support FR2-2 bands with SCS 480kHz and/or 960kHz. |
| IMS emergency call | It is mandatory to support IMS emergency call over PLMN for UEs which are IMS voice capable in NR. It is mandatory to support IMS emergency call over SNPN for UEs that are SNPN capable and IMS voice capable over SNPNs. |
| Logged measurements suspension due to IDC interference | It is mandatory to support Logged measurements suspension due to IDC interference for UEs which are supporting logged measurements in RRC_IDLE and RRC_INACTIVE upon request from the network and in-device coexistence indication as specified in TS 38.331 [9]. |
| MAC subheaders with LX field | It is mandatory to support MAC subheaders with LX field for UEs supporting MAC SDU(s) using the LCID value(s) as specified in Table 6.2.1-2c in TS 38.321 [8]. |
| MAC subheaders with one-octet eLCID field | It is mandatory to support MAC subheaders with one-octet eLCID field for UEs/IAB-MTs supporting MAC CEs using extended LCID values as specified in TS 38.321 [8]. |
| Paging cause in RAN paging message | It is mandatory for a UE to support paging cause in RAN paging if UE supports paging cause in CN paging. |
| Receiving PSCCH/PSSCH from 2nd starting symbol in a slot | It is mandatory for a UE supporting NR sidelink in shared spectrum and when shared spectrum channel access must be used to support receiving PSCCH/PSSCH transmitted from 2nd starting symbol in a slot in addition to the first starting symbol and monitor a total up to the number reported in pscch-RxSidelink-r16 of PSCCHs in a slot in the 1st and 2nd starting symbols. A UE supporting this feature shall indicate support of sl-Reception-r16. |
| Receiving UE to UE COT sharing information | It is mandatory for a UE supporting NR SL in shared spectrum where shared spectrum channel access must be used to support monitoring SCI to read COT sharing information and transmitting NR SL based on COT sharing information subject to COT sharing conditions. A UE supporting this feature shall indicate support of sl-DynamicChannelAccess-r18. |
| RRM measurement fully offloading and RRM measurement relaxation for LP-WUS | It is mandatory to support RRM measurement fully offloading and RRM measurement relaxation in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [21] for UEs which support [LP-WUS]. |
| SON report in PNI-NPN | It is mandatory for a UE to support a SON report in PNI-NPN if UE supports PNI-NPN and supports the SON report in PLMN. |
| Skipping UL configured grant if no data to transmit, as specified in release-15 version of TS 38.321 [8]. | Either configuredUL-GrantType1 or configuredUL-GrantType1-v1650 or configuredUL-GrantType2 or configuredUL-GrantType2-v1650 is supported. |
| TA reporting during initial access | It is mandatory to support TA reporting during initial access for UEs supporting uplink-TA-Reporting-r17 or uplinkTA-ReportingATG-r18 as specified in TS 38.321 [8]. |

# Appendix 2 – 38.822

## x.y	Layer-2 and Layer-3

### x.y.z	NR_LPWUS-Core

Table x.y.z-1: Layer-2 and Layer-3 feature list for NR_LPWUS-Core

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x. NR_LPWUS-Core | x-1 | RRM measurement fully offloading and relaxation | It is mandatory to support RRM measurement fully offloading and RRM measurement relaxation in RRC_IDLE/RRC_INACTIVE as specified in TS 38.304 [21] for UEs which support [LP-WUS]. | X-1 or X-1a | N/A | N/A | N/A | N/A |  | Conditional mandatory without capability signalling |

# Appendix 3 – RAN1 UE features

1.1 UE features for LP-WUS/WUR for NR

R1-2501394	Session Notes of AI 9.15.5	Ad-Hoc Chair (NTT DOCOMO, INC.)

Session notes are endorsed.

Agreement:

Support Alt-2 below:

- Alt-2: Introduce the following FG(s), where the supported LP-WUS/WUR waveform and operation are reported in a FG, with FFSs on whether/which to define as a basic FG for LP-WUS/WUR (and potential prerequisite relationship in between), Type and further coupling/decoupling of the FG(s)

| X-1 | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OOK [2. The support of LP-SS based RRM measurement and/or SSB-based measurement, corresponding to component 1] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OOK signal is not supported | [Per Band] | n/a | n/a | n/a | Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-1a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM [overlaid] sequence | 1. LP-WUS operation in IDLE/INACTIVE mode to trigger paging monitoring based on OFDM [2. FFS: The support of LP-SS based RRM measurement and/or [X. FFS: The support of SSB-based RRM measurement] [3. The support of further RRM relaxation of UE MR] [4. The support of serving RRM measurement offloading] [5. The supported wake-up delay] FFS: M values FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in IDLE/INACTIVE mode based on OFDM is not supported | [Per Band] | n/a | n/a | n/a | Component 1 candidate values: {OOK-based, OFDM-based, both} Component 5 candidate values: FFS [For UE supporting both X-1 and X-2, UE is not allowed to report different candidate values for component 1] | Optional with capability signalling |

Agreement:

Introduce the following FG(s), with FFSs highlighted yellow:

| X-2 | LP-WUS operation in CONNECTED mode based on OOK signal | 1. LP-WUS operation in CONNECTED mode based on OOK signal 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OOK signal is not supported | [Per Band] | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X-2a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence | 1. LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence(s) 2. The supported procedure(s) 3. Minimum time gap between LP-WUS reception and UE to start PDCCH monitoring in CONNECTED mode FFS: M values FFS: SCS FFS: Other components and/or further FG separation |  | YES | n/a | LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence is not supported | [Per Band] | n/a | n/a | n/a | Component 2 candidate values: FFS Component 3 candidate values: FFS | Optional with capability signalling |