3GPP TSG-RAN WG2 Meeting #129bis	R2-2502099

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.9.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Further consideration on UL capacity enhancement for IoT NTN

WID/SID:	IoT_NTN_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

As one target of WID[1], support of capacity enhancement for UL needs to be studied and specified. One of the motivations is to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction. More specifically, it includes enhancement to transmit Msg3 without Msg1/Msg2 and enhancement to efficiently deliver Msg4.

- Support of Capacity enhancements for uplink - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In RAN2-127bis meeting, RAN2 agreed to continue working on a Contention-Based (CB) msg3 EDT mechanism as well as the Diversity Slotted-Aloha (DSA).

RAN2-127bis meeting Agreements: - RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19. - RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed. - At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details. - CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA) - RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA) - At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission - The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA) - CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

In RAN2-128 meeting, RAN2 progressed well on how to support CB-Msg3 in both SA and DSA cases but many FFS remain.

RAN2-128 meeting Agreements: - Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B) - Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels - The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels) - There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC) - The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific - There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts) - For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time - For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)IThe UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.nRAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

In RAN2-129 meeting, RAN2 agreed the working assumption on the CB-Msg3 transmission window and Msg4 enhancement.

RAN2-129 meeting Agreements: - It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used - RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC - RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects) - Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN) - Frequency domain resources for (N)PUSCH occasions - repetition number - (N)PDCCH resource - MCS - For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels. - The UE shall at most have one ongoing CB EDT procedure at any time. - The CB EDT Config has one minimum RSRP threshold (as agreed in RAN2#128) to use CB EDT. - The CB EDT Config has two RSRP thresholds for NB-IoT for the three CE levels. - CB EDT Config has three RSRP thresholds for eMTC for the four CE levels. - As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config. - RAN2 consider a new CBEDT-ConfigSIB-NB IE for configuring the CB EDT feature Working assumption: 1. One CB-MSG4 can target multiple UEs simultaneously (FFS on the details) 2. For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today The UE first selects the next DSA transmission window and then randomly select K replicas inside the window. RAN2 assumes that a pointer solution is not needed in Rel-19

In this contribution, we provide further view on the remaining issues for CB-msg3 EDT, DSA as well as the enhancement for Msg4.

# 2	Discussion

## 2.1	CB-Msg4 monitoring window for DSA

For CB-Msg3 transmission in DSA, RAN2 agreed the working assumption to introduce an Msg3 transmission window. The UE shall select the next transmission window and then randomly select K replicas inside the window for CB-Msg3 transmissions. Based on this WA, in the post-meeting email discussion [2], companies showed divergent views on the number of CB-Msg4 monitoring window as well as how to start the window(s).

One of the key agreement reached in last meeting is that RAN2 assumes that a pointer solution is not needed in Rel-19 which means the NW is not aware of the Msg3 transmission occasions within the Msg3 transmission window. For half-duplex UE, it brings challenge to the UE and NW on how to avoid the UL(Msg3) and DL(Msg4) collision if the NW can schedule the Msg4 within the Msg3 transmission window. Though there are some solutions to address the UL and DL collision such as prioritizing the DL recpetion or UE delays the UL transmission in the collision slots, it seems to bring complicated impact to the IoT device. Therefore, for half-duplex UE, a single Msg4 monitoring window could be started at least after the end of Msg3 transmission window (plus UE-eNB RTT) hence the NW schedules Msg4 within the Msg4 monitoring window to avoid the UL/DL collision. However, the drawback of a single Msg4 monitoring window is that, the NW has no means to respond Msg4 to UE as early as possible even if NW has decoded a Msg3 successfully. This is not only a waste of UE’s power consumption but also a waste of NW ressource, because the UE may transmit additional Msg3 replicas (which will increase the Msg3 collision rate as well).

Observation 1: For half-duplex UE, a single CB-Msg4 window starting after the end of the Msg3 transmission window can avoid potential UL/DL collisions during Msg3 transmission and Msg4 reception.

Observation 2: A single CB-Msg4 window starting after the end of the Msg3 transmission window results in increased UE power consumption and inefficient use of network resources, as it is not possible for the network to respond with Msg4 to the UE as early as possible, even if the network has successfully decoded Msg3.

For half-duplex UE, we tend to agree single CB-Msg4 recption window is a simple solution, especially for the low-cost NB-IoT device.

Proposal 1: For half-duplex UE, UE starts a single CB-Msg4 window after the end of the Msg3 transmission window plus UE-eNB RTT.

For full-duplex UEs, such as LTE UEs operating in enhanced coverage mode, the UE is capable enough to transmit and receive packets simultaneously. There is no UL/Msg3 and DL/Msg4 collision issue at all. Therefore, once the eNB successfully decodes one of the multiple Msg3 replicas, it may respond as early as possible (i.e., without waiting for the reception of the remaining replica(s)). The UE could stop transmitting the remaining replicas if it has received a CB-Msg4 containing a matching Contention Resolution Identity. In RAN2-128 meeting, above principle has been agreed.

RAN2-128 agreements:

RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

The early termination of Msg3 replica transmissions would benefit both the UE and the network by avoiding not only increased UE power consumption but also the inefficient use of network resources. In this case, a Msg4 reception window should be started after the first Msg3 replica (plus UE-eNB RTT) to receive the possible Msg4 response as early as possible.

Observation 3: For full-duplex UE, the UE can early terminate CB-Msg3 transmission by stopping the transmission of the remaining Msg3 replicas if it has received a CB-Msg4 containing a matching Contention Resolution Identity.

Observation 4: CB-Msg3 early termination can benefit both the UE and the network by improving the UE's power efficiency and the system's resource efficiency.

Furthermore, PUSCH repetition early termination is a legacy feature which has been supported in legacy eMTC Full Duplex system (see mpdcch-UL-HARQ-ACK-FeedbackConfig). We don’t see any reason why this legacy function should be disabled for CB-Msg3 transmission.

Proposal 2: For full-duplex UE, early termination of CB-Msg3 (PUSCH) transmission shall be supported.

There may have two options to (re)start the Msg4 reception window in order to enable early termination of CB-msg3 transmission. In one way (Option#1), the UE may apply the Msg4 reception window with quite long duration to cover the whole period to receive possible Msg4 reception responding to the first or subsequent Msg3 replicas. However, this may cause UE and NW misalignment because NW may miss the first Msg3 replica reception hence UE and NW have different understanding on when the window is started and the exact window length. To avoid the NW and UE misalignment, the Msg4 reception window should be restarted after each Msg3 replica. In the other way (Option#2), the UE may start an individual Msg4 reception window for each of the replica. (i.e., multiple Msg4 reception windows). From specification implementation point of view, Option#2 seems easy for window modelling, and it anyway needs to be specified for SA with single replica.

Proposal 3: For full-duplex UEs, the UE starts multiple CB-Msg4 windows. The UE initiates an individual Msg4 reception window for each replica in the subframe containing the last PUSCH repetition of the replica, plus the UE-eNB RTT.

## 2.2	RNTI determination for CB-Msg4 monitoring

In the post-meeting email discussion [2], the other controversial aspect is that: for UEs transmitting replicas within a Msg3 transmission window, how many RNTIs should the UE monitor for CB-Msg4 reception? The discussion focuses on below two options:

- Option 1: Multiple RNTIs for a Msg3 transmission window

- Option 2: Single RNTI for a Msg3 transmission window

For Option1, the UE may need to monitor up to 4 RNTIs just for CB-Msg4 reception since RAN2 agreed the number of replicas within a Msg3 transmission window can be: 1 (SA), 2, 3, 4. This will add extra complexity to IoT NTN UEs. Furthermore, as RAN2 now agreed to address multiple UEs with one Msg4 to save the PDCCH for Msg4 scheduling, enabling multiple RNTIs for a Msg3 transmission window may impact the multiplexing gain of Msg4. In contrast, Option2 is simplier from UE perspective, having to monitor only a single RNTI for Msg4, and also give more possibility to multiplex Msg4 of different UEs. Some companies have the concern on false alarm of decoding Msg4 PDSCH when a single RNTI is used for a Msg3 transmission window. But it seems a tradeoff between the UE simplicity, the PDCCH saving gain got from Msg4 multiplexing and the UE power consumption for Msg4 PDSCH decoding. From system point of view, we think Option2 is simple for IoT device and good for efficient Msg4 delivery. Therefore, we prefer to have option 2.

Proposal 4: For a Msg3 transmission window, only one RNTI is derived for CB-Msg4 reception.

Proposal 4 only proposes to have a single RNTI corresponding to one Msg3 transmission window. The  different RNTIs may still be needed for different Msg3 transmission windows if Msg4 monitoring windows corresponding to the different Msg3 transmission window are partially overlapping. For instance as illustrated in Figure 1, only different RNTI is needed for monitoring Msg4 corresponding to Msgs Tx window#0 and Msg3 Tx window#1 as they are partially overlapping, but corresponding to different Msg3 Tx windows. The RNTI for monitoring Msg4 in Msg4 monitoring window #0 and #2 can be reused.

Proposal 5: The RNTI for CB-Msg4 reception can be reused by different Msg3 transmission windows if the Msg4 monitoring windows corresponding to the different Msg3 transmission windows are not overlapping.

Figure 1 Illustration of the Msg3 transmission window and Msg4 monitoring window

So the RNTI may be derived by taking into account the Msg4 monitoring window size and Msg3 transmission window periodicity. For example, the RNTI for Msg4 reception may be derived using formular:

RNTI=X + Msg3_W_index mod (ceil (Msg4_WS/Msg3_WP)) + ceil (Msg4_WS/Msg3_WP)*carrier_id,

wherein:

- X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=1 or any other value,

- Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19.

- Msg4_WS is the window size of Msg4,

- Msg3_WP: is the window periodicity of Msg3.

Proposal 6: The above formula is used for deriving RNTI for Msg4 monitoring if single RNTI per Msg3 transmission window is agreed.

## 2.3	UE capability on SA and DSA

In the post-meeting email discussions, another contentious point is whether UEs capable of CB-Msg3 shall support DSA by default. In our view, the requirements for supporting DSA are different from those for SA therefore it is better to differenate SA and DSA UE capabilities.

For SA, it is simple on how to derive the RNTI for Msg4 recpetion and how to start the CB-Msg4 reception window since there is only one Msg3 transmission replica. RAN2 already reached below agreements in previous meetings on these aspects.

- For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

- The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

For DSA, RAN2 introduces the CB-Msg3 transmission window and, accordingly, it is controversial on how to decide the RNTI for CB-Msg4 recpetion as well as the Msg4 recpetion windows. To support DSA, it will complicate not only UE but also NW implementation. Therefore, we think DSA is an enhancement for CB-Msg3 (i.e., DSA is an optional sub-feature for CB-Msg3).

Proposal 7: DSA is an optional sub-feature for CB-Msg3.

If both SA and DSA capable UEs are supported in the same eNB, another issue is whether the NW-configured CB-Msg3 resource can be shared between the SA and DSA UEs. In our understanding, to avoid the resourse fragmentation, the resource sharing should be supported.

Proposal 8: CB-Msg3 resources can be shared between SA-capable UEs and DSA-capable UEs that support CB-Msg3.

Regarding how to capture UE behavior for SA in the specification, one view is that SA can be treated as a special case of DSA, where the DSA Msg3 transmission window is reduced to a single time unit (the length of one replica transmission, including repetitions), and only one replica is configured. However, if the eNB must support both SA and DSA UEs, it will need to configure a long DSA Msg3 transmission window to accommodate multiple replica transmission opportunities. With such a long DSA transmission window, it becomes unclear for SA UEs how they should start the Msg4 reception window or determine the RNTI for CB-Msg4 monitoring. In this sense, we prefer to specify the UE behavior for SA and DSA UEs separately.

Proposal 9: RAN2 to capture the UE behavior for CB-Msg3 SA and CB-Msg3 DSA UEs separately.

## 2.4	Maximum TBS for CB-Msg3 triggering

It was agreed in RAN2-128 meeting that the UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS. But it is open whether the maximum TBS threshold is the same or different for different CE levels. The maximum TBS threshold for CB-Msg3 is similar to the threshold used to decide whether UE can trigger the legacy EDT transmission. As defined in TS36.331 section 5.3.3.1b, the UE can initiate legacy CP or UP EDT when the size of the resulting MAC PDU including the total UL data is expected to be smaller than or equal to the TBS signalled in edt-TBS. The edt-TBS is defined as per-CE level, which is given in RACH-CE-LevelInfo-r13. Since the legacy EDT and CB-Msg3 EDT are similar from the Msg3 data transmission point of view, we believe the same principle used for legacy EDT can be applied to CB-Msg3 EDT. The per-CE level TBS threshold enables the flexibility to configure different thresholds considering the repetition request and resource cost for different CE levels.

edt-TBS Largest TBS for Msg3 for a CE level applicable to a UE performing EDT. Value in bits. Value b328 corresponds to 328 bits, b408 corresponds to 408 bits and so on. Additionally, value b1000or456 corresponds to 1000 bits for CE levels 0 and 1, and 456 bits for CE levels 2 and 3. See TS 36.213 [23].

Observation 5: The TBS threshold for legacy EDT is defined as per Coverage Enhancement level.

Proposal 10: The maximum TBS for CB-Msg3 can be defined differently for different Coverage Enhancement (CE) levels.

In legacy EDT, there is some optimization to enable UE to select smaller TBS than edt-TBS for Msg3.  The UEs can adapt their TBS selection based on their specific coverage conditions and pending data size, improving reliability and efficiency (e.g., to avoid padding bits to be transmitted in vain). If the UE selects a smaller TBS, the number of UL transmission repetitions should be adapted as well.

edt-SmallTBS-Enabled Value TRUE indicates UE performing EDT is allowed to select TBS smaller than edt-TBS for Msg3 in the corresponding CE level, as specified in TS 36.213 [23]. Excerpt from 36.213 section 16.3.3: … - if the UE is configured with higher layer parameter edt-SmallTBS-Enabled set to 'false', - the TBS is given by higher layer parameter edt-TBS - otherwise, - the UE selects a TBS from the allowed TBS values according to Table 16.3.3-2 - the repetition number for Msg3 is the smallest integer multiple of L value that is equal to or larger than where is the selected TBS for Msg3, and is given by higher layer parameter edt-TBS

For CB-Msg3, it worth to discuss whether the similar mechanism should be introduced. Or alternatively, NW may configure multiple TBS for a CE level hence UE can select the TBS based on its pending data size to avoid unnecessary bits padding.

Proposal 11: RAN2 to discuss whether to introduce a flexible TBS selection for CB-Msg3 based on UE’s pending data size.

## 2.5	CB-Msg3 EDT Termination and further communication

After a CB-Msg3 transmission, the contention resolution mechanism should be supported because multiple UEs may send Msg3 to network on the same/shared PUSCH. In legacy 4-step RACH, the RACH collisions are handled using the contention resolution scheme, which relies on uniquely identifying the UEs in Msg3 (i.e., first 48 bits of CCCH SDU) and NW acknowledgement in Msg4. For CB-Msg3 transmission in shared resource, the most straightforward way is to reuse the contention resolution scheme in 4-step RACH.

Proposal 12: Reuse the contention resolution scheme in legacy 4-step RACH for CB-Msg3 EDT transmission.

In legacy EDT, the network may terminate the EDT and move the UE to RRC IDLE if there is no more data to be transmitted. Alternatively, the network may trigger RRC connection establishment (i.e. fallback to RRC Connection setup) for further commincation. For example, if the MO data triggers further downlink information from application, then NW may move the UE to RRC CONNECTED mode. For CB-Msg3 EDT, we think both of the same mechanisms can also be applied.

Proposal 13: The network may terminate the CB-Msg3 EDT procedure and move the UE to RRC IDLE, or fallback the CB-Msg3 EDT procedure to legacy RRC connection establishment procedure if NW decides to move the UE in RRC_CONNECTED mode.

Regarding how network terminates the EDT procedure and moves the UE to RRC IDLE, we see below possible cases:

- Case 1: The UE goes to RRC IDLE when the CB-Msg4 includes the Contention Resolution Identity MAC CE only (i.e., without RRC message for connection release).

- Case 2: The UE goes to RRC IDLE when the CB-Msg4 includes both the Contention Resolution Identity MAC CE and an RRC message (e.g., RRCEarlyDataComplete, RRCConnectionReject, RRCConnectionRelease) to release the UE to RRC idle.

- Case 3: The UE goes to RRC IDLE when the CB-Msg4 includes the Contention Resolution Identity MAC CE in CB-Msg4 and there is a following separate RRC message (e.g., RRCEarlyDataComplete, RRCConnectionReject, RRCConnectionRelease) to release the UE to RRC idle.

Regarding how network fallback the CB-Msg3 EDT procedure to legacy RRC connection establishment procedure and move the UE to RRC CONNECTED, we see below possible cases:

- Case 4: The UE goes to RRC CONNECTED when the CB-Msg4 includes both the Contention Resolution Identity MAC CE, and the RRC message (e.g., RRCConnectionSetup, RRCConnectionResume) for RRC Connection Setup/Resume.

- Case 5: The UE goes to RRC CONNECTED when the CB-Msg4 includes the Contention Resolution Identity MAC CE in CB-Msg4 and there is a following separate RRC message (e.g., RRCConnectionSetup, RRCConnectionResume) for RRC Connection Setup/Resume.

In our understanding, the Contention Resolution Identity MAC CE shall always be included in the CB-Msg4 to enable UE to address the contention resolution. Therefore, the L1-signalling based EDT termination procedure should not be considered.

Proposal 14: The Contention Resolution Identity MAC CE shall always be included in the CB-Msg4 to enable the UE to perform the contention resolution.

Proposal 15: The L1-signaling-based CB-Msg3 EDT termination is not needed.

For the Case1 to Case5 listed above, following current EDT procedure principle, we think the Case2/3 and Case4/5 should be supported. i.e., whether the RRC message is sent in CB-Msg4 or in a separate following RRC message is up to network implementation.

Observation 6: It is up to network implementation to include both the Contention Resolution Identity MAC CE and the RRC message in one CB-Msg4 or to include only the Contention Resolution Identity MAC CE in the CB-Msg4, followed by a separate RRC message.

Specifically, for Case3 where the UE needs to receive the separate following RRC message to release the UE to RRC idle, the network shall assign a C-RNTI to the UE because the RNTI for CB-Msg3 transmission or Msg4 reception is from a common pool resource which may be used by other UEs later and therefore a dedicated C-RNTI is needed for the UE to receive the separate RRC message. Similarly, for Case4 and Case5, the network shall assign a C-RNTI to the UE as well. The C-RNTI MAC CE shall be included in the CB-Msg4 for further data transmission.

Proposal 16: The C-RNTI MAC CE shall be included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4.

For Case1 above where CB-Msg4 includes the Contention Resolution Identity MAC CE only, we understand it can save some RRC message to move the UE to RRC idle while it may add more interactions between RRC layer and MAC layer. RAN2 can further discuss the details on the impact.

Proposal 17: The CB-Msg3 procedure can be terminated by including the Contention Resolution Identity MAC CE only in CB-Msg4.

Based on all possible information (e.g., Contention Resolution Identity MAC CE, C-RNTI MAC CE, and MAC SDU for RRC messages) agreed to be included in CB-Msg4, RAN2 can further discuss the MAC format for CB-Msg4.

## 2.6	CB-Msg3 EDT backoff, fallback and CE level increase

The CB-Msg3 procedure may fail due to, e.g., bad RF conditions or CB-Msg3 resource collision. If UE cannot receive the CB-Msg4 response within the contention resolution window, the UE can try another CB-Msg3 attempt. Similar to legacy 4-step RACH and 2-step RACH in which the Backoff Indicator field is included in the RAR/MsgB message, the backoff mechanism can be introduced for CB-Msg3 to avoid the same UEs transmit the subsequent CB-Msg3 at the same time. The UE shall delay the subsequent CB-Msg3 transmission for another CB-Msg3 re-attempt by the backoff time. The UE decides the backoff based on the BI field indicated from NW as legacy.

Proposal 18: Backoff mechanism is supported for CB-Msg3 EDT. The network includes the Backoff Indicator in CB-Msg4, allowing the UEs to derive the backoff time for the subsequent CB-Msg3 re-attempts.

The UE may suffer consecutive failed CB-Msg3 procedure after multiple re-attempts. If the number of contention resolution failures for CB-Msg3 EDT reaches a threshold, the UE may perform below options:

- Option1: Increase the CE level and perform the CB-Msg3 re-attempts in new CE level

- Option2: Fallback to 4-step RACH or EDT

If the contention resolution failures are caused by bad RF conditions, it is reasonable to have a higher CE level which is typically configured with higher number of repetitions to support more robust transmission scheme. However, if the transmission failures are caused by resource collision due to the limited CB-Msg3 resource, it is not good to move to the next CE level since it will consume more UE energy to transmit additional repetitions and it will waste more system resource. Instead, we think it is better to fallback to 4-step RACH/EDT and select proper CE level for subsequent preamble transmissions. Considering the amount of PUSCH resources which can be allocated for a CB-Msg3 occasion is less than the preamble resources (e.g., 64) which can be allocated for a RO, the CB-Msg3 PUSCH resource collision may be worse than the legacy RACH procedure. To control the unnecessary CE level increase due to CB-Msg3 PUSCH resource collision instead of bad RF conditions, we think the UE should evaluate its latest measured RSRP to decide whether it should increase the CE level or fallback to 4-step RACH/EDT.

Proposal 19: If the number of contention resolution failures for CB-Msg3 EDT reaches a threshold, the UE should evaluate its latest measured RSRP to decide whether it should increase the CE level or fallback to 4-step RACH/EDT.

Proposal 20: If the measured RSRP is significantly better than the threshold to select the next robust CE level, the UE should not increase the CE level for CB-Msg3 re-attempts. Instead, it should fallback to 4-step RACH or EDT procedure.

Therefore, on top of the RSRP threshold for initial CE level selection, RAN2 can introduce an offset to decide whether to increase the CB-Msg3 CE level. If the latest measured RSRP is better than the threshold for next CE level plus the offset, the UE should not increase the CE level, but can instead fallback to perform 4-step RACH.

Proposal 21: On top of the RSRP threshold for initial CE level selection, introduce an offset to decide whether to increase the CB-Msg3 CE level.

In addition, from network point of view, if the network detects high overload condition in the cell (e.g., the CB-Msg3 resource is in congestion), the network may offload the UEs from re-attempting over the CB-Msg3 resource and ask UEs fallback to legacy 4-step RACH or EDT.  In this case, the network can issue a fallback indicator in the CB-Msg4 to immediately order fallback to legacy RACH procedure. The UEs who decodes the CB-Msg4 but without matching contention resolution identity should fallback to legacy 4-step RACH and EDT procedure. If many UEs fall back to 4-step RACH or EDT procedures, it may shift the congestion issue to the preamble domain. Similar to the backoff for preamble re-attempt, the UEs performing fallback from CB-Msg3 to preamble transmission shall apply a backoff time before transmitting the preamble.

Proposal 22: Network-indicated fallback mechanism is supported for CB-Msg3 EDT. The UEs with failed contention resolution shall fallback to legacy 4-step RACH and EDT procedure if it receives a fallback indicator in CB-Msg4.

## 2.7	CB-Msg3 Power Control

For PUSCH transmission, UE shall determine its UL transmission power as specified in TS36.213. In legacy PUR, the eNB configures the UL power control parameter for the PUSCH transmission in the PUR-Config via RRC. Then the UE determines the PUSCH transmission power based on the configured parameter (e.g., p0-UE-NPUSCH and alpha), as specified in TS36.213 section 16.2.1.

For NPUSCH (re)transmissions corresponding to the random access response grant if enhanced random access power control is not applied, and for all other NPUSCH transmissions except for NPUSCH (re)transmission corresponding to preconfigured uplink resource, when the number of repetitions of the allocated NPUSCH RUs is greater than 2: [dBm] otherwise PPUSCH,c(i)=min&PCMAX,c(i),&10log10(MNPUSCH,c(i))+PO_NPUSCH,c(j)+αc(j)⋅PLc+ΔTF,c(i)) [dBm]

For the PUSCH transmission for CB-Msg3 EDT, the UE needs to determine UL transmission power for direct PUSCH transmission as well.  Similar to PUR, we think the dedicated parameter (e.g., p0-UE-NPUSCH and alpha) for CB-Msg3 EDT transmission should be defined to faciliate the PUSCH transmission power calculation.

Proposal 23: Define dedicated parameters (e.g., p0-UE-NPUSCH and alpha) for CB-Msg3 EDT transmission.

Furthermore, in legacy Msg3 UL transmission power determination, there is a mechanism to dynamically adjust the target power at the receiver side (P0_NPUSCH). i.e., based on the number of preamble transmissions attempted before the UE receives the current UL grant (RAR) to transmit message 3 (e.g., PREAMBLE_TRANSMISSION_COUNTER_CE as defined in TS36.321). The UE increases its UL transmission power after multiple attempts, and according to the power ramping step, which is beneficial to increase the successful PUSCH decoding rate and avoid certain UEs being starved.

- the MSG3_RECEIVED_TARGET_POWER is set to preambleInitialReceivedTargetPower + (PREAMBLE_TRANSMISSION_COUNTER_CE – 1) * powerRampingStep;

For CB-Msg3 transmissions, as there is no preamble attempts in Msg1 (since no Msg1 is transmitted), the UE may fail multiple times for Msg3 transmissions attempts due to RF conditions or contention-based resource collisions. In this case, it may be also beneficial to consider the number of attempts to decide the UL transmission power and potential transmit power ramping.

Proposal 24: CB-Msg3 UL transmission power ramping is supported for multiple CB-Msg3 re-attempts.

## 2.8	OCC for CB-Msg3

Based on the LS response from RAN1 [3], the detailed design for the OCC is still ongoing in RAN1 and whether the OCC can be applied to Msg3-EDT depends on the power imbalance between UEs. In the RAN2-129 meeting, whether OCC can be applied to CB-Msg3 was discussed with below agreements.

RAN2-129 agreements: 1. It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used 2. RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC

At RAN1 #120, the following working assumption was made (R1-2500665):

Working assumption For 3.75kHz SCS OCC for NPUSCH format 1, support TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa, where the DMRS REs are as in legacy NB-IoT and the guard period within the slot is as in legacy NB-IoT. Send LS to RAN4 asking whether there would be any issue (e.g. phase continuity) for supporting such TDM DMRS for IoT NTN.

The working assumption defines the DMRS are time-division multiplexed over 4 slots, such that a first UE will use the first 2 slots and blank the next 2 slots, and a second UE will blank the first 2 slots and use the next 2 slots for DMRS.

This can work for release 19 UEs using OCC, but for the legacy UEs not using OCC for CB-Msg3 it means they will not follow the TDM DMRS pattern, because it is different from the legacy pattern.

Observation 7: UEs using OCC for CB-Msg3 will use a TDM DMRS pattern, which is different from UEs using CB-Msg3 without OCC.

Since the UEs will not be using the same DMRS pattern it may result in issues if the UEs share the same CB-Msg3 resources. In our view, RAN2 can therefore deprioritize the work on OCC for CB-Msg3, because it will increase the resource demand instead of reducing it.

Proposal 25: RAN2 to deprioritize the work on OCC for CB-Msg3.

## 2.9	Efficient Msg4 delivery

On the objective of efficient delivery of msg4, according to the WID [1], the intention is to identify methods to de-couple the UL from the DL as much as possible hence unlock the additional UL capacity potential. We understand it is to avoid the PDCCH/PDSCH congestion which may impact the UL scheduling as well as the UL capacity. Therefore, how to reduce the Msg4 overhead should be studied.

In legacy, a dedicated Msg4 with both PDCCH and PDSCH is always needed for the corresponding Msg3 for contention resolution. In RAN2-129 meeting, as one working assumption, it was agreed that one CB-Msg4 can target multiple UEs simultaneously. It saves the PDCCH usage since one PDCCH scheduling (with one scheduled Msg4 PDSCH) can target for multiple UEs.

As discussed in section 2.5, for CB-Msg3, the Contention Resolution Identity MAC CE shall always be included in the CB-Msg4 to enable the UE to perform the contention resolution. Since the UE Contention Resolution Identity MAC CE shall be included the associated subheader may not be needed. If multiple UEs are included in one CB-Msg4, multiple Contention Resolution Identity MAC CE should be included in the CB-Msg4.

Proposal 26: A single CB-Msg4 can include multiple Contention-Resolution Identity MAC CEs, each targeting a distinct UE.

Furthermore, as discussed in section 2.5, the C-RNTI MAC CE shall be included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4. If multiple UEs are included in one CB-Msg4, multiple C-RNTI MAC CEs should be included in the CB-Msg4.

Proposal 27: A single CB-Msg4 can include multiple C-RNTI MAC CEs, when the UEs are expected to receive additional RRC messages or data from the network after CB-Msg4.

Additionally, if one of the multiple UEs scheduled in one CB-Msg4 decodes the CB-Msg4 successfully, the UE shall send HARQ Acknowledge to network. Similar to MsgB (successRAR) defined in 2-step RACH, to enable multiple UEs send HARQ feedback to network, multiple HARQ feedback resource should be indicated in the CB-Msg4. Therefore, additional the HARQ feedback Timing Indicator should be included in the CB-Msg4

Proposal 28: HARQ-Ack resource indicator could be included in the CB-Msg4 to support multiple UEs feedback.

Another aspect is whether one or multiple RRC messages can be included in one CB-Msg4. In 2-step RACH, the MsgB MAC PDU has two formats, i.e., without MAC SDU(s) and with MAC SDU(s) which can accommodate the RRC messages. For CB-Msg4, in our understanding, one or more MAC SDUs can be included in CB-Msg4 to include multiple RRC messages.

Proposal 29: RAN2 to discuss whether multiple RRC message(MAC SDU(s)) can be included into one CB-Msg4.

Based on the type of agreed MAC CE and MAC SDU to be included in the CB-Msg4, RAN2 can further discuss the CB-Msg4 MAC PDU format. The MsgB MAC PDU defined for 2-step RACH can be used as baseline.

Figure 1: Example of a MSGB MAC PDU with MAC SDU(s)

Figure 2: Example of a MSGB MAC PDU without MAC SDU(s)

Proposal 30: RAN2 to discuss the Msg4 MAC PDU format to support multiple UEs scheduled in one CB-Msg4, taking the MSGB MAC PDU format as baseline.

# 3	Conclusion

This document has made the following observations and proposals:

CB-Msg4 monitoring window for DSA

Observation 1: For half-duplex UE, a single CB-Msg4 window starting after the end of the Msg3 transmission window can avoid potential UL/DL collisions during Msg3 transmission and Msg4 reception.

Observation 2: A single CB-Msg4 window starting after the end of the Msg3 transmission window results in increased UE power consumption and inefficient use of network resources, as it is not possible for the network to respond with Msg4 to the UE as early as possible, even if the network has successfully decoded Msg3.

Proposal 1: For half-duplex UE, UE starts a single CB-Msg4 window after the end of the Msg3 transmission window plus UE-eNB RTT.

Observation 3: For full-duplex UE, the UE can early terminate CB-Msg3 transmission by stopping the transmission of the remaining Msg3 replicas if it has received a CB-Msg4 containing a matching Contention Resolution Identity.

Observation 4: CB-Msg3 early termination can benefit both the UE and the network by improving the UE's power efficiency and the system's resource efficiency.

Proposal 2: For full-duplex UE, early termination of CB-Msg3 (PUSCH) transmission shall be supported.

Proposal 3: For full-duplex UEs, the UE starts multiple CB-Msg4 windows. The UE initiates an individual Msg4 reception window for each replica in the subframe containing the last PUSCH repetition of the replica, plus the UE-eNB RTT.

RNTI determination for CB-Msg4 monitoring

Proposal 4: For a Msg3 transmission window, only one RNTI is derived for CB-Msg4 reception.

Proposal 5: The RNTI for CB-Msg4 reception can be reused by different Msg3 transmission windows if the Msg4 monitoring windows corresponding to the different Msg3 transmission windows are not overlapping.

Proposal 6: The below formula is used for deriving RNTI for Msg4 monitoring if single RNTI per Msg3 transmission window is agreed.

RNTI=X + Msg3_W_index mod (ceil (Msg4_WS/Msg3_WP)) + ceil (Msg4_WS/Msg3_WP)*carrier_id,

wherein:

- X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=1 or any other value,

- Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19.

- Msg4_WS is the window size of Msg4,

- Msg3_WP: is the window periodicity of Msg3.

UE capability on SA and DSA

Proposal 7: DSA is an optional sub-feature for CB-Msg3.

Proposal 8: CB-Msg3 resources can be shared between SA-capable UEs and DSA-capable UEs that support CB-Msg3.

Proposal 9: RAN2 to capture the UE behavior for CB-Msg3 SA and CB-Msg3 DSA UEs separately.

Maximum TBS for CB-Msg3 triggering

Observation 5: The TBS threshold for legacy EDT is defined as per Coverage Enhancement level.

Proposal 10: The maximum TBS for CB-Msg3 can be defined differently for different Coverage Enhancement (CE) levels.

Proposal 11: RAN2 to discuss whether to introduce a flexible TBS selection for CB-Msg3 based on UE’s pending data size.

CB-Msg3 EDT Termination and further communication

Proposal 12: Reuse the contention resolution scheme in legacy 4-step RACH for CB-Msg3 EDT transmission.

Proposal 13: The network may terminate the CB-Msg3 EDT procedure and move the UE to RRC IDLE, or fallback the CB-Msg3 EDT procedure to legacy RRC connection establishment procedure if NW decides to move the UE in RRC_CONNECTED mode.

Proposal 14: The Contention Resolution Identity MAC CE shall always be included in the CB-Msg4 to enable the UE to perform the contention resolution.

Proposal 15: The L1-signaling-based CB-Msg3 EDT termination is not needed.

Observation 6: It is up to network implementation to include both the Contention Resolution Identity MAC CE and the RRC message in one CB-Msg4 or to include only the Contention Resolution Identity MAC CE in the CB-Msg4, followed by a separate RRC message.

Proposal 16: The C-RNTI MAC CE shall be included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4.

Proposal 17: The CB-Msg3 procedure can be terminated by including the Contention Resolution Identity MAC CE only in CB-Msg4.

CB-Msg3 EDT backoff, fallback and CE level increase

Proposal 18: Backoff mechanism is supported for CB-Msg3 EDT. The network includes the Backoff Indicator in CB-Msg4, allowing the UEs to derive the backoff time for the subsequent CB-Msg3 re-attempts.

Proposal 19: If the number of contention resolution failures for CB-Msg3 EDT reaches a threshold, the UE should evaluate its latest measured RSRP to decide whether it should increase the CE level or fallback to 4-step RACH/EDT.

Proposal 20: If the measured RSRP is significantly better than the threshold to select the next robust CE level, the UE should not increase the CE level for CB-Msg3 re-attempts. Instead, it should fallback to 4-step RACH or EDT procedure.

Proposal 21: On top of the RSRP threshold for initial CE level selection, introduce an offset to decide whether to increase the CB-Msg3 CE level.

Proposal 22: Network-indicated fallback mechanism is supported for CB-Msg3 EDT. The UEs with failed contention resolution shall fallback to legacy 4-step RACH and EDT procedure if it receives a fallback indicator in CB-Msg4.

CB-Msg3 Power Control

Proposal 23: Define dedicated parameters (e.g., p0-UE-NPUSCH and alpha) for CB-Msg3 EDT transmission.

Proposal 24: CB-Msg3 UL transmission power ramping is supported for multiple CB-Msg3 re-attempts.

OCC for CB-Msg3

Observation 7: UEs using OCC for CB-Msg3 will use a TDM DMRS pattern, which is different from UEs using CB-Msg3 without OCC.

Proposal 25: RAN2 to deprioritize the work on OCC for CB-Msg3.

Efficient Msg4 delivery

Proposal 26: A single CB-Msg4 can include multiple Contention-Resolution Identity MAC CEs, each targeting a distinct UE.

Proposal 27: A single CB-Msg4 can include multiple C-RNTI MAC CEs, when the UEs are expected to receive additional RRC messages or data from the network after CB-Msg4.

Proposal 28: HARQ-Ack resource indicator could be included in the CB-Msg4 to support multiple UEs feedback.

Proposal 29: RAN2 to discuss whether multiple RRC message(MAC SDU(s)) can be included into one CB-Msg4.

Proposal 30: RAN2 to discuss the Msg4 MAC PDU format to support multiple UEs scheduled in one CB-Msg4, taking the MSGB MAC PDU format as baseline.

# Reference

[1] RP-250472	Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2] R2-250xxxx	Report of [Post129][307][R19 IoT NTN] CB-msg3/CB-msg4 (Mediatek)

[3] R1-2410895	Reply LS on OCC for CB-msg3 NPUSCH