3GPP TSG-RAN WG2 Meeting #129bis	R2-2502107

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.3.1

Source:	MediaTek Inc.

Title:	Discussion on the Simulation results illustration for AI mobility

Document for:	Discussion, Decision

Introduction

In the previous #129 meeting, RAN2 made the following agreements on key parameters/setting in AI mobility scenarios and the form of the graphic illustration.

Agreements: - Categorize the parameters/settings into two groups: one group containing key parameters and the other group containing other parameters. Provide graphical illustrations for the combinations of parameters/settings in the Key Parameters group, with the simulation results for all other parameters combined. - The key parameters for scenario 2 for graphical illustration are MRRT and UE speed. - Restrict the number of combinations of the key parameters for capturing the simulation results in a graphic illustration in the TR. - For scenario 2 graphic illustration, the UE speed is set to 30 km/h and 90 km/h; MRRT is set to 50%, 66%, 80% and 90%. - For scenario 4 graphic illustration, the key parameters are [OW:PW] or PW, and UE speed. - For scenario 4 graphic illustration, the UE speed is set to 60 km/h and 120 km/h. Two options will be considered: OW:PW ratio is set to 1:1, 2:1 and 1:2. The setting of PW and OW will be considered if needed. Only consider PW value. - For scenario 3, use the cluster setting for graphic illustration. Channel correlation coefficient range associated with the results will be mentioned with the results. - For Scenario 6, the key parameters for graphic illustration are MRRS and UE speed. If necessary, graphic illustrations including clustering can be added later. - For Scenario 6, rely on the rapporteur to illustrate the findings based on the available simulation results in a best-effort manner. We can later discuss whether to capture those findings in the TR. - A CDF-like curve will be used to present the statistical results from companies for each key parameter setting per scenario. We may consider selecting some representative settings with a percentile threshold in the CDF curve or illustrating them using a boxchart later. - Select a few representative settings from the agreed settings for each scenario to illustrate the trends for generalization performance. - As a starting point, the same settings agreed upon for scenarios 2 and 4, the baseline setting, and the baseline case will be used to capture the trends and observations for measurement event prediction in case B and case A. - The figures for different settings and scenarios will first be stored in a new sheet within the corresponding spreadsheet. Then, some representative figures will be selected to be included in the TR. Agreements: The CDF curves to reflect the simulation results from different companies: - Average L3 cell RSRP difference and last predicted point L3 cell RSRP difference of measurement results within PW is captured in TR - For both temporal domain case A and case B, simulation result with different filtering approach is listed separately. - Take the optimal result, e.g., the one with the lowest RSRP difference, from each company. Agreements 1 Two sets of parameters (ISD, BS antenna height, BS Tx power) are used for the generalization across cell configurations study. 2 FR1 is the primary focus, companies can also submit results for FR2 (however, each set of generalization results covers either FR1 or FR2). 3 Agree on the two sets of configurations as in tables 2 and 3 (for FR1 and FR2). Parameter; Configuration #1; Configuration #2. Deployment scenario; UMi; UMa. ISD; 200m; 500m. BS antenna height; 10m; 25m. BS Tx power; 40dBm; 44dBm. Table 2: generalization parameters for FR1 Parameter; Configuration #1; Configuration #2. Deployment scenario; UMi; UMa. ISD; 200m; 500m. BS antenna height; 10m; 25m. BS Tx power; 40dBm; 44dBm. Table 3: generalization parameters for FR2

In this paper, we provide preliminary graphs for scenario 2/3/4 and further discuss the graphical illustration issue for generalization and event prediction cases.

Discussion

Merge sub-use cases

Given that sub-use case is a significant setting in the evaluation of AI mobility, initially, we attempted to plot separate CDF graphs for each sub-case in scenarios 2, 3, and 4, as shown in Figure 1. However, we found that for sub-use cases 1 and 3, due to the limited quantity of results, there are numerous group settings for which only one company has provided data. Consequently, this precludes the generation of CDF curves for these settings.

Observation 1: For sub-use case 1 and 3, due to the limited quantity of results, there are numerous group settings for which only one company has provided data, which precludes the generation of CDF curves for these settings.

Figure 1(a). CDF of Optimal Average L3 cell RSRP difference for sub case 1 and 3 in Scenario 2

Figure 1(b). CDF of Optimal Average L3 cell RSRP difference for sub case 1 and 3 in Scenario 4

Furthermore, our understanding is that the choice of sub-use case is implementation choice for both the network-side model and the UE-side model, which will not have significant specification impact, we propose consolidating the results from all sub-cases to generate comprehensive CDF curves in the graphic illustration.

Proposal 1: Merge the results from all sub-use cases 1, 2, and 3 to illustrate the CDF graph for different RRM prediction Scenarios.

Scenario 2

RAN2 has agreed the key parameters/settings for scenario 2 are MRRT and UE speed and the UE speed is set to 30 km/h and 90 km/h and MRRT is set to 50%, 66%, 80% and 90%. Meanwhile, considering the filtering options with sliding and non-sliding are listed separately. We illustrate two sets of CDFs of key parameters/settings in two figures Figure 2(a) and Figure 2(b) corresponding to the sliding and non-sliding filtering respectively. The optimal average L3 cell RSRP difference from each company is selected.

Figure 2(a). CDF of Average L3 cell RSRP difference for Scenario 2 with Sliding filtering

Figure 2(b). CDF of Average L3 cell RSRP difference for Scenario 2 with Non-sliding filtering

From figure 2(a) and 2(b), we can observe that the overall average L3 cell RSRP difference of L1/L3 filtering with sliding is below 1.2dB, while the overall average L3 cell RSRP difference of the L1/L3 filtering with non-sliding is larger than that of sliding, with the maximum RSRP difference even reaching 3.5dB.

Observation 2: The overall average L3 cell RSRP difference of L1/L3 filtering option with “Non-sliding” is larger than that filtering option of “Sliding”.

Proposal 2: To plot CDF curves in one graph by combining all key parameters/settings, which provides an overview of the simulation results for all settings from all companies.

However, due to combining all key parameters/settings in one graph, it is difficult to discern the impact of specific parameters (e.g., Speed or MRRT) on the results from the numerous CDF curves. Therefore, in addition to plotting Figure 1 with figures combining all key parameters/settings, we tried to plot CDF curves in one figure that combine all other parameters except for a specific key parameter. For example, Figure 3(a) and Figure 3(b) consider the impact of speed with sliding and non-sliding, and Figure 4(a) and Figure 4(b) consider the impact of MRRT with sliding and non-sliding.

Figure 3(a) Sliding						Figure 3(b) Non-sliding

Figure 3. CDF of Optimal Average L3 cell RSRP difference for Speeds

Figure 4(a) Sliding							Figure 4(b) Non-sliding

Figure 4. CDF of Optimal Average L3 cell RSRP difference for MRRTs

From the Figures 3 and 4, we can clearly see that the impact trend of specific parameters/setting on the average L3 RSRP difference. As speed increases, with the non-sliding filtering, the RSRP difference trend is increasing. Although this trend does not consistently increase with speed at 60 km/h and 90 km/h in the sliding setting case due to simulation variations among companies, the RSRP difference statistics for all companies at 30km/h show a consistently lower trend compared to those at 60km/h and 90km/h. A similar observation can be made for MRRT.

Observation 3: By combining all other parameters except for a specific key parameter (e.g., Speed or MRRT), the impact trend of specific parameters/setting on the results average L3 cell RSRP difference is clear.

Observation 4: The observed CDF trends of speed align with the current agreement that “For intra-frequency temporal domain, higher UE speeds result in larger prediction errors”.

Observation 5: The observed CDF trends of MRRT align with the current agreement that “The prediction accuracy for intra-frequency temporal domain case B reduces as MRRT increases”.

Proposal 3: To plot separate CDF graph for a specific key parameter with all other parameters combined, making it more clearer to observe the impact of the specific key parameter on the results.

Besides, in the agreement for scenario 2, “For scenario 2 graphic illustration, the UE speed is set to 30 km/h and 90 km/h; MRRT is set to 50%, 66%, 80% and 90%.” There is only one company that has provided results for MRRT=90%. Consequently, it is not feasible to discern a trend from the graph based on this single data point. To establish a meaningful trend, unless additional data from other companies becomes available, we need to adjust the agreement to “For scenario 2 graphic illustration, the UE speed is set to 30 km/h and 90 km/h; MRRT is set to 50%, 66%, 80%.”

Proposal 4: Unless additional results from other companies are available, the CDF curves are plotted with MRRT is set to 50%, 66%, 80% for scenario 2 graphic illustration.

Scenario 3

For scenario 3, RAN2 agrees to use the cluster setting for graphic illustration. We plot a comparison between clusters and cells extracted from the results of all companies in Figure 5. We can observe that the cluster-based method exhibits lower RSRP difference than the cell-based method.

Figure 5. CDF of Optimal Average L3 cell RSRP difference for cluster vs cell

Observation 6: The observed CDF of cluster vs. cell trends align with the current agreement that “For inter-frequency, cluster as input (i.e. measurements from different cells as inputs) can improve the prediction accuracy than single cell as input.”

Scenario 4

RAN2 has agreed that for scenario 4 graphic illustration, the key parameters are [OW:PW] or PW, and UE speed. We try both illustration with [OW:PW] and PW, as showed in Figure 6 and 7.

Figure 6(a) Sliding							Figure 6(b) Non-sliding

Figure 6. CDF of Optimal Average L3 cell RSRP difference for Scenario 4 with [OW:PW]

Figure 7(a) Sliding							Figure 7(b) Non-sliding

Figure 7. CDF of Optimal Average L3 cell RSRP difference for Scenario 4 with PW

Similar to scenario 2, the graphs (Figure 6 and 7) with all key parameters combined make it difficult to discern the trends of key parameters’ impact on the results. Therefore, for Scenario 4, we separately plot the key parameters [OW:PW] (Figure 8), PW (Figure 9) and UE speed (Figure 10) with all other parameters/settings combined in the following figures.

Figure 8(a) Sliding Filtering							Figure 8(b) Non-sliding Filtering

Figure 7. CDF of Optimal Average L3 cell RSRP difference for [OW:PW]

Figure 9(a) Sliding Filtering						Figure 9(b) Non-sliding Filtering

Figure 8. CDF of Optimal Average L3 cell RSRP difference for PWs

Compared the two plotting methods: [OW:PW] and PW. We found that in the [OW:PW] method, the value of [OW:PW] does not correspond to a unique pair of OW and PW values. For example, both OW=PW=40ms and OW=PW=1600ms would be counted as [OW:PW] =1. This leads to an exceptionally wide range of results for [OW:PW] =1 in the final analysis, resulting in poor comparability and making it difficult to observe the impact trend of OW:PW on the results. Therefore, we propose that RAN2 should consider only PW in the illustration of Scenario 4, rather than [OW:PW].

Proposal 5: RAN2 consider the key parameter PW in the graphic illustration for scenario 4.

Figure 10(a)							Figure 10(b)

Figure 10. CDF of Optimal Average L3 cell RSRP difference for Speeds

From Figure 10, we can observe that the basic trend of speed's impact on the results is that as speed increases, the overall RSRP difference values for all companies also increase.

It is important to note that the result from various companies has distinct channel models and implementation. Consequently, for same settings, there may be significant variations in results. This diversity can lead to key parameter CDF curves that do not strictly adhere to the trends outlined in the agreement. Nevertheless, the overall trends generally align with the current agreements.

Observation 7: Due to variations in channel models and algorithms among different companies, the aggregated simulation results, as represented in the CDF curves, do not strictly adhere to the agreement. However, the overall trends remain consistent with the agreement.

Considering that our graph illustrations are based on average L3 cell RSRP difference, we encourage companies to provide, at least, their average results. This will enable us to incorporate the work from all companies in our statistical analysis.

Proposal 6: Companies are encouraged to at least provide “Average L3 cell RSRP difference” in simulation results.

Generalization and event prediction cases

RAN2 has agreed for generalization study, i.e., the generalization performance is evaluated with the following cases,

•	Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B.

•	Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B.

•	Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B.

The generalization study cases with Configuration #A and Configuration #B including studies over UE speeds, and two sets of parameters (ISD, BS antenna height, BS Tx power) for cell configuration, represented by Configuration#1 and Configuration#2. When illustrating the graphs, it is essential to take these as key parameters (or set of parameters) into consideration.

Porposal 7: For generalization evaluation of UE speed in scenario 2, CDF curves of the baseline, GC1, and GC2 for different UE speeds with fixed MRRT(e.g., MRRT=50%) will be plotted.

Porposal 8: For generalization evaluation of cell configuration(i.e., Configuration #1 and Configuration #2) in scenario 2, CDF curves of the baseline, GC1, and GC2 for with fixed UE speed(e.g., 30km/h) and MRRT(e.g., MRRT=50%) will be plotted.

Proposal 9: For generalization evaluation of UE speed Scenario 4, CDF curves of the baseline, GC1, and GC2 for different UE speeds with fixed PW length will be plot. FFS on the specific PW length value.

Porposal 10: For generalization evaluation of cell configuration(i.e., Configuration #1 and Configuration #2) in scenario 4, CDF curves of the baseline, GC1, and GC2 with fixed UE speed(e.g., 60km/h) and PW will be plotted. FFS on the specific PW length value.

Proposal 11: For generalization evaluation of frequency prediction direction(e.g., 2GHz=>4GHz or 4GHz => 2GHz) Scenario 3, CDF curves of the baseline, GC1, and GC2 for with fixed UE speed(e.g., 30km/h) will be plotted.

Porposal 12: For generalization evaluation of cell configuration(i.e., Configuration #1 and Configuration #2) in scenario 3, CDF curves of the baseline, GC1, and GC2 with fixed UE speed(e.g., 30km/h) will be plotted.

For measurement event prediction for case A and case B with indirect and direct methods with diverse intermediate KPIs results. It is important to note that system-level results are also influenced by handover (HO) modeling. Therefore, further consideration is needed to determine whether handover (HO) modeling should be included as a key parameter.

Proposal 13: For event prediction cases A and B, the key parameters corresponding to Scenarios 2 and 4 can be reused.

Proposal 14: Handover (HO) modelling can be one key parameter/setting of graphic illustration for event prediction cases A and B.

In the performance evaluation of event prediction, both intermediate KPIs and system-level results are considered. To demonstrate the impact of AI on system performance, we propose incorporating CDF curves of system-level metrics in the graph illustration.

Proposal 15: For event prediction cases A and B, to capture system-level performance metric for graphic illustration. At least the representative system-level performance metric “HOF rate” and “total number of HOF per UE per second” can be considered.

Conclusion

Observation

Merge sub-use cases

Observation 1: For sub-use case 1 and 3, due to the limited quantity of results, there are numerous group settings for which only one company has provided data, which precludes the generation of CDF curves for these settings.

Scenario 2/3/4

Observation 2: The overall average L3 cell RSRP difference of L1/L3 filtering option with “Non-sliding” is larger than that filtering option of “Sliding”.

Observation 3: By combining all other parameters except for a specific key parameter (e.g., Speed or MRRT), the impact trend of specific parameters/setting on the results average L3 cell RSRP difference is clear.

Observation 4: The observed CDF trends of speed align with the current agreement that “For intra-frequency temporal domain, higher UE speeds result in larger prediction errors”.

Observation 5: The observed CDF trends of MRRT align with the current agreement that “The prediction accuracy for intra-frequency temporal domain case B reduces as MRRT increases”.

Observation 6: The observed CDF of cluster vs. cell trends align with the current agreement that “For inter-frequency, cluster as input (i.e. measurements from different cells as inputs) can improve the prediction accuracy than single cell as input.”

Observation 7: Due to variations in channel models and algorithms among different companies, the aggregated simulation results, as represented in the CDF curves, do not strictly adhere to the agreement. However, the overall trends remain consistent with the agreement.

Proposal

Merge sub-use cases

Proposal 1: Merge the results from all sub-use cases 1, 2, and 3 to illustrate the CDF graph for different RRM prediction Scenarios.

Scenario 2/3/4

Proposal 2: To plot CDF curves in one graph by combining all key parameters/settings, which provides an overview of the simulation results for all settings from all companies.

Proposal 3: To plot separate CDF graph for a specific key parameter with all other parameters combined, making it more clearer to observe the impact of the specific key parameter on the results.

Proposal 4: Unless additional results from other companies are available, the CDF curves are plotted with MRRT is set to 50%, 66%, 80% for scenario 2 graphic illustration.

Proposal 5: RAN2 consider the key parameter PW in the graphic illustration for scenario 4.

Proposal 6: Companies are encouraged to at least provide “Average L3 cell RSRP difference” in simulation results.

Generalization and event prediction cases

Porposal 7: For generalization evaluation of UE speed in scenario 2, CDF curves of the baseline, GC1, and GC2 for different UE speeds with fixed MRRT(e.g., MRRT=50%) will be plotted.

Porposal 8: For generalization evaluation of cell configuration(i.e., Configuration #1 and Configuration #2) in scenario 2, CDF curves of the baseline, GC1, and GC2 for with fixed UE speed(e.g., 30km/h) and MRRT(e.g., MRRT=50%) will be plotted.

Proposal 9: For generalization evaluation of UE speed Scenario 4, CDF curves of the baseline, GC1, and GC2 for different UE speeds with fixed PW length will be plot. FFS on the specific PW length value.

Porposal 10: For generalization evaluation of cell configuration(i.e., Configuration #1 and Configuration #2) in scenario 4, CDF curves of the baseline, GC1, and GC2 with fixed UE speed(e.g., 60km/h) and PW will be plotted. FFS on the specific PW length value.

Proposal 11: For generalization evaluation of frequency prediction direction(e.g., 2GHz=>4GHz or 4GHz => 2GHz) Scenario 3, CDF curves of the baseline, GC1, and GC2 for with fixed UE speed(e.g., 30km/h) will be plotted.

Porposal 12: For generalization evaluation of cell configuration(i.e., Configuration #1 and Configuration #2) in scenario 3, CDF curves of the baseline, GC1, and GC2 with fixed UE speed(e.g., 30km/h) will be plotted.

Proposal 13: For event prediction cases A and B, the key parameters corresponding to Scenarios 2 and 4 can be reused.

Proposal 14: Handover (HO) modelling can be one key parameter/setting of graphic illustration for event prediction cases A and B.

Proposal 15: For event prediction cases A and B, to capture system-level performance metric for graphic illustration. At least the representative system-level performance metric “HOF rate” and “total number of HOF per UE per second” can be considered.

Reference

[1] R2_129_ChairNotes_25-02-22_21-00_eom

[2] R2_128_ChairNotes_24-11-22_17-00_eom

[3] R2_127bis_ChairNotes_24-10-18_15-45_eom_clean