3GPP TSG-RAN WG2 Meeting #129bis	R2-2502114

Wuhan, China, Apr. 7th – 11st, 2025

Agenda item:	8.1.3

Source:	Samsung

Title:	Discussion on NW side data collection

Document for:	Discussion & Decision

Introduction

This document discusses on NW side data collection for AI/ML air interface WI.

Discussion

Memory capability

The following agreement was made in RAN2#127, in order for UE to log training data for NW side data collection:

<Agreements (RAN2#127)> - UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases.

In legacy, there are existing memory requirements for UE to log measurement results. Firstly, for logged MDT, the fixed size (i.e., 64KB) of minimum memory requirement is defined. This is commonly mandatory for UEs who support logged MDT, and no separate capability is used for this memory requirement.

<TS 38.306> loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE state. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The minimum memory size of MDT logged measurements is 64KB. For eRedCap UE supporting this feature, the minimum memory size of MDT logged measurements is 16KB.; UE; No; No; No.

Meanwhile, there are two separate memories defined for QoE measurement.

- Memory 1. This is the memory for UEs to log QoE reports in RRC_CONNECTED while QoE reporting is temporarily paused by NW. The mandatory minimum requirement for this memory is 64KB as highlighted below.

- Memory 2. This is the memory for UE to log QoE reports in RRC_IDLE and RRC_INACTIVE for MBS broadcast service. The mandatory minimum requirement for this memory is 64KB as highlighted below. Besides, UE optionally may support more than 64KB for this memory. In this case, UE indicates the size (i.e., larger than 64KB) it supports to NW via an explicit capability indication.

<TS 38.306> # 6 Conditionally mandatory features without UE radio access capability parameters Features; Condition. Acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList; It is mandatory to support acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList for UEs which support the acquisition of the posSIB types in posSchedulingInfoList as specified in TS 38.331 [9].. Acquisition of SI messages with explicit SI window positions; It is mandatory to support acquisition of SI messages with explicit SI window positions for UEs which support the SIB types in schedulingInfoList2 as specified in TS 38.331 [9].. AS layer memory size for QoE paused measurement reports; It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17.. AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE; It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports". <SKIP> ## 4.2 UE Capability Parameters <SKIP> qoe-AdditionalMemoryMeasReport-r18 Indicates the minimum AS layer memory size the UE supports for QoE measurement in RRC_IDLE and RRC_INACTIVE in addition to the "AS layer memory size for QoE paused measurement reports". Value kB128 means the UE supports at least 128 kilobytes for this purpose, and so on. A UE supporting this feature shall also support qoe-IdleInactiveMeasReport-r18.; UE; No; No; No. <TS 38.331> #### – AppLayerMeasParameters The IE AppLayerMeasParameters is used to convey the capabilities supported by the UE for application layer measurements. AppLayerMeasParameters information element -- ASN1START -- TAG-APPLAYERMEASPARAMETERS-START AppLayerMeasParameters-r17 ::= SEQUENCE { qoe-Streaming-MeasReport-r17 ENUMERATED {supported} OPTIONAL, qoe-MTSI-MeasReport-r17 ENUMERATED {supported} OPTIONAL, qoe-VR-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ran-VisibleQoE-Streaming-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ran-VisibleQoE-VR-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ul-MeasurementReportAppLayer-Seg-r17 ENUMERATED {supported} OPTIONAL, ..., [[ qoe-IdleInactiveMeasReport-r18 ENUMERATED {supported} OPTIONAL, qoe-NRDC-MeasReport-r18 ENUMERATED {supported} OPTIONAL, qoe-AdditionalMemoryMeasReport-r18 ENUMERATED {kB128, kB256, kB512, kB1024} OPTIONAL, qoe-PriorityBasedDiscarding-r18 ENUMERATED {supported} OPTIONAL, srb5-r18 ENUMERATED {supported} OPTIONAL ]] } -- TAG-APPLAYERMEASPARAMETERS-STOP -- ASN1STOP

If UE does not indicate this optional capability in UECapabilityInformation message, it means the UE only supports the mandatory 64 KB memory for QoE measurement in RRC_IDLE and RRC_INACTIVE. The motivation for this optional capability is UEs may have different memory capabilities and NW may want to select the UE who has big memory capability for more and consistent data collection.

We assume the same benefit can be expected, if we have similar capability for NW side data collection. i.e., UE mandatorily supports the fixed size of minimum AS layer memory without UE capability signalling, and optionally reports via UE capability a larger minimum AS layer memory size. i.e., Taking different sizes of buffer capability from UEs into account, NW can select/configure UEs to perform NW-side data collection. The exact numbers (e.g., 64KB) could be discussed after RAN1/2’s progress on data content for NW-side data collection.

Proposal 1. UE mandatorily supports the fixed size of minimum AS layer memory without UE capability signalling, and optionally reports via UE capability a larger minimum AS layer memory size. FFS for the exact numbers.

Availability for logging

From last meetings, RAN2 agreed periodic logging and event-triggered logging for NW-side data collection:

<Agreements (RAN2#127bis)> - Periodic logging is supported for training data collection procedure in R19 - Event-triggered data logging will be supported. At least radio condition based event triggered logging will be supported. FFS the details of radio condition based event. FFS if other events are supported.

In addition, the availability indicator by UE was agreed to trigger UEInformationRequest message from NW.

<Agreements (RAN2#127bis)> - The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest. FFS trigger/definition of availability indication. and FFS how data availability indication is sent to the network.

Taking all aforementioned agreements into account, we assume the following procedure for NW side data collection:

- 1) NW provides measurement configuration for NW side data collection to UE (e.g., via RRCReconfiguration message).

- 2) UE logs the measurement data based on periodic logging and/or event-triggered logging.

- 3) UE indicates the availability for logged data to NW.

- 4) NW sends UEInformationRequest message to UE to request transmission of logged data.

- 5) UE reports the logged data via UEInformationResponse message.

Based on this procedure, we think there is no need for UE’s autonomous reporting (e.g., AS buffer event based reporting), which was left as FFS in the RAN2#127:

<Agreement (RAN2#127)> - FFS whether AS buffer event based reporting is supported. FFS if we send availability indication or full report if it is supported

Given multiple UEs may be configured in a cell to collect sufficient data for completion of NW side model training, UE’s autonomous reporting may cause severe network congestion. It is safer for UE to send reports only based on NW’s request.

Proposal 2. UE’s autonomous reporting (including AS buffer event based reporting) is NOT supported.

In Step 3 above, we assume the availability indication can be transferred via UAI. We do not see the need to design a new separate message for this indication. Another FFS issue for availability indication is when UE triggers transmission of the availability indication.

<Agreement (RAN2#129)> - Data availability indication is supported. FFS when this would be triggered

Assume NW configures CSI-RS resource to measure for NW-side data collection. Then, (assuming periodic logging) UE immediately starts to measure and log the measurement results. If UE transmits the availability indication as soon as any measurement result is obtained, it may not be the useful information to NW, because UE would report the availability indication immediately after NW configuration. Therefore, we think there should be a threshold for logged data size to trigger the transmission of the availability indication, and it should be configurable by NW depending on its needs. For example, NW can decide a threshold considering UE’s memory capability (as discussed in Section 2.2). Also, NW can set a certain threshold to retrieve data, before UE stops measurement and data collection (i.e., UE’s buffer becomes full).

Proposal 3. UE sends the availability indication via UAI, when the size of logged data exceeds a threshold which is configured by NW.

Full buffer

In last meetings, RAN2 agreed UE autonomously stops measurement for NW-side data collection in case of full buffer and indicates it to NW. It is still FFS when exactly UE indicates it (i.e., “upon” full buffer and/or “before” full buffer).

<Agreement (RAN2#127)> - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. <Agreement (RAN2#128)> - The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after).

The reason why this full buffer indication is needed is to inform NW of paused data collection by UE. Therefore, it is obvious this indication should be sent “upon” full buffer (i.e., as soon as UE stops measurement). It could be done via UAI. Someone may think it is also beneficial if UE indicates to NW “before” the buffer is full. However, we think it is more like an availability indication. i.e., As we discussed/proposed in Proposal 3, NW can configure a data size threshold to trigger availability indication to retrieve data before UE’s buffer becomes full.

Proposal 4. Before full buffer, no separate indication is needed other than availability indication. Upon full buffer, UE indicates it via UAI.

Besides, RAN2 also needs to discuss on when UE and NW resume suspended measurement due to full buffer. Simply, UE can autonomously resume measurement when any memory becomes available (i.e., when any data report is sent to NW after full buffer). NW can also know UE’s resumption if any UE’s report is received after the full buffer indication.

Proposal 5. UE autonomously resumes measurement/logging paused due to full buffer, when any memory becomes available (i.e., when any data report is sent to NW).

To summarize what we have discussed/proposed in this section, an example of procedure regarding full buffer is like:

- 1) NW provides measurement configuration for NW side data collection to UE (e.g., via RRCReconfiguration message).

- 2) UE logs the measurement data based on periodic logging and/or event-triggered logging.

- 3) UE sends the availability indication for logged data to NW via UAI.

- 4) When the buffer becomes full, UE stops measurement and indicates it to NW via UAI.

- 5) NW sends UEInformationRequest message to UE to request transmission of logged data.

- 6) UE reports some logged data via UEInformationResponse message.

- 7) Now that UE has some free space in the memory, UE resumes the measurement for NW-side data collection.

Mobility

In RAN2#129, RAN2 agreed to retain logged data during handover:

<Agreement (RAN2#129)> - UE retains logged data during handover (HO). FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed - UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

Target node may want UE to retain the logged data measured in source node, if it is willing to forward the data to the right training entity (e.g., source gNB in case of gNB centric data collection or OAM in case of OAM centric data collection). It could be beneficial by avoiding the waste of Uu resources and UE/NW energy used to collect the data. On the other hand, target may not want to forward the logged data measured in source node to training entity. In this case, UE does not need to retain the logged data. If the training entity (e.g., source gNB in case of gNB centric data collection or OAM in case of OAM centric data collection) needs more data for training, it may choose other UE instead for data collection.

Observation 1. If target gNB can forward the logged data measured in source to training entity, it would be beneficial for UE to retrain the data. Otherwise (i.e., if target gNB cannot forward the logged data measured in source to training entity), UE does not need to retain the data.

Having this in mind, it should be the target node who decides whether UE should retain or discard data during handover. Target node can indicate it to source node, and source can indicate to UE via handover command (i.e., RRCReconfiguration message). Meanwhile, we expect RAN3 impact on procedure details, so it would be good to check with RAN3.

Proposal 6. During handover, target node indicates to source node whether UE should retain or discard logged data. Then, source node indicates it to UE via handover command (i.e., RRCReconfiguration message). UE retains or discards the logged data measured in source according to the indication.

Proposal 7. If Proposal 6 is agreeable, send an LS to RAN3 to initiate discussion on detailed procedure and signalling for BM use case.

RAN2 also discussed RRC_IDLE and RRC_INACTIVE and RLF scenarios.

<Agreement (RAN2#129)> - FFS how to handle idle/inactive and RLF cases and whether we have a unified.

Handover is time critical operation, so there is no sufficient time for UE to send data to source. However, in case of RRC_IDLE and RRC_INACTIVE, in most cases, source node has enough time to retrieve data before sending RRCRelease message. Therefore, if UE retains data in RRC_IDLE and RRC_INACTIVE, we expect marginal benefit just requiring much RAN2 spec impact. So, we do not want to spend much time for this optimization.

Proposal 8. Upon going to RRC_IDLE or RRC_INACTIVE, UE discards any logged data.

Meanwhile, RLF is also time urgent procedure, so it would be good to design the way to retain the logged data in case of RLF. Similar to handover, the new gNB could indicate UE whether UE should retain or discard logged data via RRCReestablishment message. If UE is indicated to retain and has any logged data, UE can send availability via RRCReestablishmentComplete message

Proposal 9. In case of RLF, the new gNB indicates UE whether UE should retain or discard logged data via RRCReestablishment message. If UE is indicated to retain and has any logged data, UE can send availability via RRCReestablishmentComplete message.

Timestamp

In the last meeting, there were a few of proposals from companies to introduce timestamp in reports to indicate when the data is measured/logged. It is obvious any kind time information is needed for NW to train model for temporal prediction. However, we do not think the explicit timestamp is needed. i.e., NW can understand the time information implicitly, as discussed specifically below.

In case of periodic logging, for each CSI-RS or SSB resource, UE measures L1-RSRP periodically. Given NW configures the periodicity of the resource, it is enough for UE to contain/report the L1-RSRP in chronological order in a list. Then, NW can understand the differential time between two consecutive entries in the list is same as the configured periodicity of the resource.

On the other hand, in case of event-based logging, UE’s measurement may not be uniformly periodic, as there could be the period during which UE does not measure the configure resources. For example,

- 1) When the event is fulfilled (i.e., 1st event instance), UE logs L1-RSRPs periodically.

- 2) When the event is not fulfilled any longer, UE stops measurement/logging.

- 3) When the event is fulfilled again (i.e., 2nd event instance), UE resumes measurement and logs L1-RSRPs periodically again.

If UE contains L1-RSRPs for a common list sequentially, the differential time between two consecutive entries (i.e., the last L1-RSRP entry for the 1st event instance and the first L1-RSRP entry of the 2nd event instance) may not be same as the configured periodicity of the resource. i.e., it would be same as the period in which the event is not fulfilled. However, NW cannot know about it, so it may bring out degradation of model training performance. To address this issue, someone could think timestamp is needed to differentiate each event instance. However, we think, without timestamp, it is sufficient to contain the L1-RSRPs in a separate list for each event instance. For example,

- 1) When the event is fulfilled (i.e., 1st event instance), UE logs L1-RSRPs in a list (i.e., list 1).

- 2) When the event is not fulfilled any longer, UE stops measurement/logging.

- 3) When the event is fulfilled again (i.e., 2nd event instance), UE resumes measurement and logs L1-RSRPs in the different list (i.e., list 2).

Proposal 10. No timestamp is reported for logged data.

- For periodic logging, UE uses a common list, in order to log multiple L1-RSRPs per each beam.

- For event-based logging, UE uses different lists to differentiate each event instance, in order to log multiple L1-RSRPs per each beam.

Conclusion

RAN2 is requested to discuss and agree the following proposals:

Proposal 1. UE mandatorily supports the fixed size of minimum AS layer memory without UE capability signalling, and optionally reports via UE capability a larger minimum AS layer memory size. FFS for the exact numbers.

Proposal 2. UE’s autonomous reporting (including AS buffer event based reporting) is NOT supported.

Proposal 3. UE sends the availability indication via UAI, when the size of logged data exceeds a threshold which is configured by NW.

Proposal 4. Before full buffer, no separate indication is needed other than availability indication. Upon full buffer, UE indicates it via UAI.

Proposal 5. UE autonomously resumes measurement/logging paused due to full buffer, when any memory becomes available (i.e., when any data report is sent to NW).

Proposal 6. During handover, target node indicates to source node whether UE should retain or discard logged data. Then, source node indicates it to UE via handover command (i.e., RRCReconfiguration message). UE retains or discards the logged data measured in source according to the indication.

Proposal 7. If Proposal 6 is agreeable, send an LS to RAN3 to initiate discussion on detailed procedure and signalling for BM use case.

Proposal 8. Upon going to RRC_IDLE or RRC_INACTIVE, UE discards any logged data.

Proposal 9. In case of RLF, the new gNB indicates UE whether UE should retain or discard logged data via RRCReestablishment message. If UE is indicated to retain and has any logged data, UE can send availability via RRCReestablishmentComplete message.

Proposal 10. No timestamp is reported for logged data.

- For periodic logging, UE uses a common list, in order to log multiple L1-RSRPs per each beam.

- For event-based logging, UE uses different lists to differentiate each event instance, in order to log multiple L1-RSRPs per each beam.

References