# 3GPP TSG RAN WG2 Meeting #129b      	                           R2-2502124
Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.1.2.2

Source:	Apple

Title:	Remaining issues on LCM procedure of UE-sided model for AI/ML based beam management

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The revised Rel-19 WID of AI/ML for air interface (WID RP-243244) was agreed in RAN#106 [1]. The WI objective on general LCM framework, including applicable functionalities, is led by RAN2 which is copied below:

By RAN2#129 [2], most of essential issues on LCM framework for UE side model have been closed. However, there are some leftover issues. In this contribution, we discuss the following remaining issues on LCM procedure:

- Remaining issues on data collection configuration

- Remaining issues on option A

  - Processing requirement of initial report in RRCReconfigurationComplete

  - UE behavior for non-applicable CSI-ReportConfig(s)

  - Details on UAI for applicability update report

  - Whether associated ID is mandatory or optional

- Remaining issues on option B

  - Details on initial applicability report in RRCReconfigurationComplete

  - Co-existence of option A and option B

- Remaining issues common to option A and option B

  - Whether report explicit cause for non-applicability

# 2 Discussion

## 2.1 Remaining issues on data collection configuration

RAN2#129 extended the agreements on data collection in AI/ML based beam management to general case [2]:

- Agreements - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

It is straight forward that the above agreements are also applied to AI/ML based beam management (e.g. the start/stop indicaiton). We provide our view on above FFS in our companion contribution on general UE-side data collection [3]. Because the intention is similar, we think similar proposals in [3] are also applicable to AI/ML based beam management. Thus, we propose:

Proposal 1: The UE request signaling for data collection of AI/ML based beam management may include:

- The indication on start/stop of data collection

- Preferred configuration from a list of candidate configurations provided by NW, where each candidate configuration includes at least CSI resource set A/B and associated ID. Its details are up to RAN1.

Proposal 2: Introduce UAI message for UE request of data collection configuration. And it is up to UE implementation when to send the request.

Proposal 3: As the UE may send data collection request via UAI based on its implementation, no need to introduce a separate indication signaling when UE can’t perform data collection based on received configuration.

## 2.2 Remaining issues on option A

In RAN2#129 [2], it was agreed to support both Option A and Option B on applicability reporting configuration. And some agreements on how Option A works were also made:

Agreements

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

We discuss the remaining issues on Option A in this section.

### Processing requirement of report via RRCReconfigurationComplete

According to Section 12 of TS 38.331 [4], there are two processing latency requirements (10ms vs 16ms) between reception of RRCReconfiguration and reporting RRCReconfigurationComplete, depending on whether it is related to CA/DC operation.

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | 10 |  |
| RRC reconfiguration (scell addition/release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration (LTE/NR SCG establishment/ modification/ release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration (Intra-NR mobility with LTE/NR SCG establishment/ modification/ release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration | DLDedicatedMessageSegment | RRCReconfigurationComplete | 16+( Nseg -1)*10 | Nseg is number of RRC segments |

For Option A, it was agreed to report initial applicability report in RRCReconfigurationComplete. However, it is not clear what is its processing latency requirement because the applicability reporting is a new reporting different from CA/DC configuration.

As Rel-19 is the first release of AI/ML, we expect new / unpredicted challenges for the UE to handle AI/ML operation. Thus, we prefer to use relaxed RRC processing latency requirements (i.e. 16ms) which is a safer option.

Proposal 4: As Rel-19 is the first release to introduce AI/ML, relaxed RRC processing timing requirement is applied for reporting applicability in RRCReconfigurationComplete (i.e. within 16ms after reception of Step 3).

### UE behaviour for non-applicable CSI-ReportConfig(s)

In RAN2#129 [2], it was agreed that UE autonomously activates the applicable periodic CSI-ReportConfig upon reporting applicable functionalities via RRCReconfigurationComplete.

- Agreements applicability reporting and management If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework: - Semi-persistent reporting, activated by MAC CE/DCI - Aperiodic CSI reporting, activated by DCI

However, it is not clear how the UE handles non-applicable periodic CSI-ReportConfig(s) after reporting RRCReconfigurationComplete. It is different from legacy behavior because the UE receives RRC configuration but can’t apply them. Thus, the UE behavior needs to be specified.

Observation 1: It is not clear how the UE handles non-applicable periodic CSI-ReportConfig(s) after reporting RRCReconfigurationComplete. It is different from legacy behavior because the UE receives RRC configuration but can’t apply them.

There are the following two alternatives to resolve the issue:

- Alt-1: the UE keeps RRC configuration but suspends its execution until it becomes applicable (similar to suspension in SCell/SCG resumption).

- Alt-2: the UE releases the RRC configuration after reporting RRCReconfigurationComplete.

We prefer Alt-1 because the inference configuration may become applicable quickly (e.g. complete model downloading from its server) and RRC already has specified similar suspension behavior. Thus, we propose:

Proposal 5: For the non-applicable periodic CSI-ReportConfig(s) after reporting RRCReconfigurationComplete, the UE keeps RRC configuration but suspends its execution until it becomes applicable (similar to suspension operation in SCell/SCG resumption).

### Details on UAI for applicability update report

In RAN2#129 [2], it was agreed that UAI can be sent to update applicability.

Agreements

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

In legacy RRC, the UE reports UAI only if the NW required information are configured in OtherConfig of RRCReconfiguraiton. However, as Option A is configured in CSI-ReportConfig which is outside of OtherConfig of RRCReconfiguraiton, it is not clear how to specify the new UAI reporting behavior for applicability update.

Observation 2: In legacy RRC, the UE reports UAI only if the NW required information are configured in OtherConfig of RRCReconfiguraiton. However, as Option A is configured in CSI-ReportConfig which is outside of OtherConfig, it is not clear how to specify the new UAI reporting behavior for applicability update.

There are the following two alternatives to resolve the issue:

- Alt-1: Specify the UE to implicitly associate all the CSI-ReportConfig with beam prediction configuration (e.g. Set A/B configuration) with UAI for applicability update reporting.

- Alt-2: Introduce a new list of CSI-ReportConfigId in OtherConfig to explicitly configure the UE to report applicability update with UAI.

We think both alternatives can work, but Alt-2 is unnecessarily making things complex because it needs to introduce both ASN.1 change and the new UE behavior in Section 5.7.4 of TS 38.331. Thus, from specification perspective, Alt-2 has no benefit over Alt-1, and we propose:

Proposal 6: For all the CSI-ReportConfig(s) with beam prediction configuration (e.g. Set A/B configuration) in RRCReconfiguration, the UE implicitly associates them with UAI for applicability update reporting.

### 2.2.4 Whether associated ID is mandatory or optional

This issue was discussed in RAN1, but RAN1 can’t achieve consensus and will further discuss this issue:

Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition?

Answer to Q4-1: There is no consensus yet on whether it is mandatory or optional. There is no conclusion yet on whether it is feasible or not for UE to decide the applicability without NW-side additional condition, and RAN 1 is discussing the related issues.

We share our view from RAN2 perspective. We prefer the associated ID is mandatory provided in Step 3 because absence of associated ID may cause model ambiguity between UE and NW. As we know, one AI/ML functionality may include multiple NW-side additional conditions corresponding to multiple AI/ML models. Then, the UE may not know which model should be used for inference if the NW doesn’t indicate the current NW-side additional condition (i.e. associated ID) to the UE. We provide an example in Figure. 1, where the UE has model 1 (corresponding to associated ID 1) and model 2 (corresponding to associated ID 2) available for the same functionality A while the current NW condition is associated ID = 2. Then, after UE reports that the functionality is applicable under either associated ID=1 or ID=2 in step 4, the NW activates the functionality A in step 5, but the UE doesn’t know whether it should use model 1 or model 2 for inference. If the UE chooses to use model 1, it will lead to non-consistency with current NW condition.

Observation 3: Because applicability reporting is per inference configuration (CSI-ReportConfig) in Option A but the UE may train multiple models for one inference configuration based on its implementation, absence of associated ID may cause model ambiguity between UE and NW.

Thus, we propose that associated ID should be mandatory provided in Step 3 from RAN2 perspective. Otherwise, it may cause model ambiguity between UE and NW

Proposal 7: RAN2 assume that associated ID is mandatory provided in Step 3. Otherwise, it may cause model ambiguity issue between UE and NW.

Figure.1: Issue illustration of model ambiguity between UE and NW if associated ID is absent in Step 3

## 2.3 Remaining issues on option B

In RAN2#129 [2], although it was agreed to support both Option A and Option B, no agreement on Option B can be made because companies had different views on how Option B works.

From co-source contribution [5]:

Proposal 1: Option B works like the following way:

Procedure 1: UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig.

Procedure 2: For initial reporting, UE sends RRCReconfigurationComplete message including applicability and/or inapplicability, e.g. by indicating whether one set or multiple sets of inference related parameters are applicable or inapplicable.

Procedure 3: For subsequent reporting upon change: UE sends UEAssistanceInformation message including applicability and/or inapplicability, e.g. by indicating whether one set or multiple sets of inference related parameters are applicable or inapplicable.

Proposal 2: For Option B, after reporting applicability, the UE waits NW to send RRCReconfiguration message with full inference configuration in CSI-ReportConfig before starting inference.

Proposal 3: For Option B for BM Case 1/2, one set or multiple sets of inference related parameters can be configured in OtherConfig, where each set in OtherConfig contains the following parameters indicated in RAN1 reply LS (R1-2410898) as baseline:

- One or more associated ID(s).

- Set A related information.

- Set B related information.

- Time instances related information for prediction (For BM Case 2 only).

FFS the parameter details for Set A/Set B.

FFS whether associated ID is mandatory or optional.

We co-sourced the contribution [5] to provide our view how to support Option B, including the following 3 proposals.

On top of them, we discuss two further important aspects.

### 2.3.1 Details on initial applicability report in RRCReconfigurationComplete

As proposed in Proposal 1 of [5], option B is configured in OtherConfig of RRCReconfiguration, but RRCReconfigurationComplete is used for its initial applicability reporting. In legacy RRC, RRCReconfigurationComplete is not used to report the feedback for configuration in OtherConfig. Thus, it is not clear how to fill this gap in RRC spec.

Observation 4: Option B is configured in OtherConfig of RRCReconfiguration, but RRCReconfigurationComplete is used for its initial applicability reporting. As RRCReconfigurationComplete is not used to report the feedback for configuration in OtherConfig in legacy RRC. Thus, it is not clear how to fill this gap in RRC spec.

There are the following two alternatives to resolve the issue:

- Alt-1: Specify the UE to implicitly associate all the sets of inference related parameters configured in OtherConfig of RRCReconfiguration with RRCReconfigurationComplete for initial applicability reporting.

- Alt-2: Introduce a new list of set indices of inference related parameters in RRCReconfiguration to explicitly configure the UE to report initial applicability report in RRCReconfigurationComplete.

Similar to the discussion on using UAI for applicability update report in Option A, we prefer Alt-1 because Alt-2 unnecessarily makes things complex to have both ASN.1 change and the new UE behavior in TS 38.331. Thus, from specification perspective, Alt-2 has no benefit over Alt-1. Thus, we propose

Proposal 8: For all the sets of inference related parameters configured in OtherConfig of RRCReconfiguration, the UE implicitly associates them with RRCReconfigurationComplete for initial applicability reporting.

Meanwhile, one followed issue of Alt-1 is how the NW can differentiate whether the reported applicability/non-applicability is for full interference configuration (Option A) or for set of interference related parameter (Option B), which requires non-overlapping ID space to identify Option A and Option B.

Observation 5: The NW needs to differentiate whether the reported applicability/non-applicability is for full interference configuration (Option A) or for set of interference related parameter (Option B). Thus. non-overlapping ID space is required to identify Option A and Option B.

Please note that the current running RRC CR introduces two separate lists of CSI-ReportConfigId to indicate applicable CSI-ReportConfig(s) and non-applicable CSI-ReportConfig(s), respectively. Thus, to avoid ambiguity of applicability reporting, we think the simplest way is to introduce CSI-ReportConfigId to identify one set of inference related parameters. Thus, we propose:

Proposal 9: To avoid ambiguity of applicability reporting between Option A and B, introduce CSI-ReportConfigId under the set of inference related parameters as identifier of the set.

### 2.3.2 Co-existence of option A and option B

With Proposal 1 in contribution [5] and Proposal 8/9 in this contribution, we think that option A and option B have the unified applicability reporting procedure. Therefore, option A can co-exist with option B. Their RRC structure can be illustrated as follows:

Figure.2: Illustration of co-existence of Option A and Option B

Meanwhile, we understand that some company may have concern that simultaneous configuration of option A and option B in one RRCReconfiguration message may bring extra UE complexity. Thus, to allow more flexibility, we propose to introduce separate UE capability for option B.

Proposal 10: RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure. And a separate UE capability is introduced for option B to allow more flexibility.

## 2.4 Remaining issues common to option A and option B

### 2.4.1 Whether report explicit cause for non-applicability

In RAN2#129 [2], it was agreed to report both applicability and non-applicability. But it is FFS whether UE can explicitly report cause for non-applicability:

- Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause

We think it is useful to explicitly provide the NW the cause value on why one functionality becomes non-applicable because the potential NW reaction may depend on different cause values:

- “Non-applicable due to model non-availability”: this cause value means that the UE may complete model downloading shortly. Thus, the gNB can just wait the UE to complete the model downloading before reconfiguration.

- “Non-applicable due to NW-side additional condition”: this cause value means that the gNB can resolve the issue by reconfiguration of its inference configuration.

- “Non-applicable due to UE-side additional condition”: this cause value means that the UE may have battery or memory issue due to heavy inference operation. Correspondingly, the gNB may resolve the issue by reconfiguration of light inference operation (e.g., set A and/or set B with smaller size).

Observation 6: It is useful to provide the NW the cause value on why one functionality becomes non-applicable:

- “Due to model non-availability”: gNB can just wait the UE to complete the model downloading.

- “Due to NW-side additional condition”: gNB may reconfigure its inference configuration accordingly.

- “Due to UE-side additional condition”: gNB may reconfigure a light inference to reduce UE burden (e.g., set A and/or set B with smaller size).

Thus, to facilitate NW taking suitable reaction, we propose that the UE may include cause value in the reporting.

Proposal 11: Together with reporting of non-applicability, the UE can optionally report the following cause value of non-applicability:

- “Due to model non-availability”: gNB can just wait the UE to complete the model downloading.

- “Due to NW-side additional condition”: gNB may reconfigure its inference configuration accordingly.

- “Due to UE-side additional condition”: gNB may reconfigure a light inference to reduce UE burden (e.g., set A and/or set B with smaller size).

# 3 Conclusion

In this contribution, we discuss key remaining issues on LCM procedure of UE-sided model for AI/ML based beam management. Our observations are:

Observation 1: It is not clear how the UE handles non-applicable periodic CSI-ReportConfig(s) after reporting RRCReconfigurationComplete. It is different from legacy behavior because the UE receives RRC configuration but can’t apply them.

Observation 2: In legacy RRC, the UE reports UAI only if the NW required information are configured in OtherConfig of RRCReconfiguraiton. However, as Option A is configured in CSI-ReportConfig which is outside of OtherConfig, it is not clear how to specify the new UAI reporting behavior for applicability update.

Observation 3: Because applicability reporting is per inference configuration (CSI-ReportConfig) in Option A but the UE may train multiple models for one inference configuration based on its implementation, absence of associated ID may cause model ambiguity between UE and NW.

Observation 4: Option B is configured in OtherConfig of RRCReconfiguration, but RRCReconfigurationComplete is used for its initial applicability reporting. As RRCReconfigurationComplete is not used to report the feedback for configuration in OtherConfig in legacy RRC. Thus, it is not clear how to fill this gap in RRC spec.

Observation 5: The NW needs to differentiate whether the reported applicability/non-applicability is for full interference configuration (Option A) or for set of interference related parameter (Option B). Thus. non-overlapping ID space is required to identify Option A and Option B.

Observation 6: It is useful to provide the NW the cause value on why one functionality becomes non-applicable:

- “Due to model non-availability”: gNB can just wait the UE to complete the model downloading.

- “Due to NW-side additional condition”: gNB may reconfigure its inference configuration accordingly.

- “Due to UE-side additional condition”: gNB may reconfigure a light inference to reduce UE burden (e.g., set A and/or set B with smaller size).

Based on observations, our proposals can be found below:

Remaining issue of data collection configuration

Proposal 1: The UE request signaling for data collection of AI/ML based beam management may include:

- The indication on start/stop of data collection

- Preferred configuration from a list of candidate configurations provided by NW, where each candidate configuration includes at least CSI resource set A/B and associated ID. Its details are up to RAN1.

Proposal 2: Introduce UAI message for UE request of data collection configuration. And it is up to UE implementation when to send the request.

Proposal 3: As the UE may send data collection request via UAI based on its implementation, no need to introduce a separate indication signaling when UE can’t perform data collection based on received configuration.

Remaining issues on Option A

Proposal 4: As Rel-19 is the first release to introduce AI/ML, relaxed RRC processing timing requirement is applied for reporting applicability in RRCReconfigurationComplete (i.e. within 16ms after reception of Step 3).

Proposal 5: For the non-applicable periodic CSI-ReportConfig(s) after reporting RRCReconfigurationComplete, the UE keeps RRC configuration but suspends its execution until it becomes applicable (similar to suspension operation in SCell/SCG resumption).

Proposal 6: For all the CSI-ReportConfig(s) with beam prediction configuration (e.g. Set A/B configuration) in RRCReconfiguration, the UE implicitly associates them with UAI for applicability update reporting.

Proposal 7: RAN2 assume that associated ID is mandatory provided in Step 3. Otherwise, it may cause model ambiguity issue between UE and NW.

Remaining issues on Option B

Proposal 8: For all the sets of inference related parameters configured in OtherConfig of RRCReconfiguration, the UE implicitly associates them with RRCReconfigurationComplete for initial applicability reporting.

Proposal 9: To avoid ambiguity of applicability reporting between Option A and B, introduce CSI-ReportConfigId under the set of inference related parameters as identifier of the set.

Proposal 10: RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure. And a separate UE capability is introduced for option B to allow more flexibility.

Whether report explicit cause for non-applicability

Proposal 11: Together with reporting of non-applicability, the UE can optionally report the following cause value of non-applicability:

- “Due to model non-availability”: gNB can just wait the UE to complete the model downloading.

- “Due to NW-side additional condition”: gNB may reconfigure its inference configuration accordingly.

- “Due to UE-side additional condition”: gNB may reconfigure a light inference to reduce UE burden (e.g., set A and/or set B with smaller size).

# 4 References

[1] RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#129, Chair Notes.

[3] R2-2502127, Further discussion on UE-side data collection, Apple.

[4] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[5] R2-2501783, Discussion on Applicable Functionality Reporting Option B for BM, OPPO, Lenovo, ZTE Corporation, Apple, Huawei, HiSilicon, CATT, vivo, CMCC, NTT DOCOMO, Samsung, LG Electronics, Xiaomi, InterDigital.