# 3GPP TSG RAN WG2 Meeting #129b      	                           R2-2502132
Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.5.4

Source:	Apple

Title:	Remaining issues on common signal transmission adaptation

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

  - Adaptation of paging occasions including confining the paging occasions in the time domain

    - Note: there shall be no paging latency increase

  - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

The Rel-19 WID of Network Energy Savings (WID RP- RP-250343) was updated in RAN#107 [1]. The WI objective of adaptation of common signal transmission includes paging adaptation, PRACH adaptation and SSB adaptation, as copied below:

In RAN2#129 [2], good progress of common signal adaptation was made. However, there are still some leftover issues. In this contribution, we further discuss remaining issue on adaptation of common signal transmission.

# 2 Discussion

## 2.1 Adaptation of paging occasions

In RAN2#129 [2], the following agreements on paging adaptation were made:

Agreements on paging adaptation

- For N, values smaller than T/32 are not supported.

- The maximum possible value for Ns is 8.

- Introduce a separate PEI configuration.

- Paging adaptations are configured semi-statically and updated via system information update notification.

- A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

We think the main remaining issues include the below aspects:

- Further details on new paging configuration

- UE behavior when new PEI configuration is provided

- Further details of new PEI configuration

### 2.1.1 Further details on new paging configuration

PCCH-Config ::=             SEQUENCE {

defaultPagingCycle                  PagingCycle, OPTIONAL  -- Need R

…omit…

]],

[[

pagingAdaptation-NS-r19           ENUMERATED {eight,four, two, one}                 OPTIONAL,  -- Need R

pagingAdaptationNAndPagingFrameOffset               CHOICE {

oneT                                NULL,

halfT                               INTEGER (0..1),

quarterT                            INTEGER (0..3),

oneEighthT                          INTEGER (0..7),

oneSixteenthT                       INTEGER (0..15),

oneThirtySecondT                     INTEGER (0..31)

}

]]

}

pagingAdaptation-NS Number of paging occasions per paging frame for paging adaptation

pagingAdaptationNAndPagingFrameOffset Used to derive the number of total paging frames in T (corresponding to parameter N in TS 38.304 [20]) and paging frame offset (corresponding to parameter PF_offset in TS 38.304 [20]). A value of oneSixteenthT corresponds to T / 16, a value of oneEighthT corresponds to T / 8, and so on.

Editor’s note: FFS field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions. FFS: firstPDCCH-MonitoringOccasionOfPO for paging adaptations.

In RRC running CR [3], the new paging configuration is captured as below. We discuss the two FFSs captured in EN.

FFS#1: field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions

In previous RAN2 meetings, the following restriction on paging configuration were proposed:

- Only same PF_offset as legacy paging configuration can be configured by NW.

- UE specific DRX cycle provided in NAS signaling is not considered, i.e. only defaultPagingCycle is used for Rel-19 paging.

We don’t think these restrictions are needed:

- For 1), we think the flexibility of new PF_offset is useful if NW don’t want to allow Rel-19 POs to overlap with legacy POs

- For 2), we think its main intention is to resolve the issue that number of PF<1 if N takes larger values like 64 or 256. However, since RAN2 agreed to only extend N to 32, we don’t have this issue. Then, it doesn’t make sense to ignore UE dedicated paging cycle provided in NAS signaling.

Thus, we propose:

Proposal 1: No need to have any configuration restriction on Rel-19 paging configuration (i.e. pagingAdaptationNAndPagingFrameOffset, pagingAdaptation-NS).

Proposal 2: The UE derives Rel-19 paging cycle as legacy, i.e. min(default paging cycle, UE specific DRX cycle).

FFS#2: firstPDCCH-MonitoringOccasionOfPO for paging adaptations.

In previous RAN2 meetings, the following enhancement on firstPDCCH-MonitoringOccasionOfPO were proposed:

- firstPDCCH-MonitoringOccasionOfPO is not needed for Rel-19 paging.

- Introduce slot level offset rather than existing symbol level offset in firstPDCCH-MonitoringOccasionOfPO.

We don’t think these restrictions are necessary:

- For 1), although it may be beneficial in NES gain, the existing IE firstPDCCH-MonitoringOccasionOfPO is optional, and thereby we can rely on NW implementation.

- For 2), we think its main intention is to reduce RRC signaling overhead. However, as it was agreed to only extend N to 32 which is just double from legacy, the RRC overhead is not an issue anymore.

Thus, we propose:

Proposal 3: Reuse existing optional IE firstPDCCH-MonitoringOccasionOfPO for Rel-19 Paging configuration.

### 2.1.2 UE behavior when new PEI configuration is provided

In RAN2#129, it was agreed to introduce a separate PEI configuration.

Agreements on paging adaptation

- Introduce a separate PEI configuration.

However, it is not clear what is the Rel-19 UE behavior if the Rel-19 PEI configuration is provided in SIB. Therefore, running 38.304 CR captured below FFS [4]:

For a UE supporting paging adaptation and PEI, if pagingAdaptationPEI-Config is signaled in system information, its behavior is FFS.

Editor’s note: FFS the UE behavior if pagingAdaptationPEI-Config is configured.

During post email discussion [5], the following 2 options were proposed by proponents:

- Option 1) R19 NES UEs only monitor R19 PEI

- Option 2) R19 NES UEs monitor legacy PEI for legacy POs and R19 PEI for R19 POs

We support Option 1) because Option 2 doesn’t make sense, and it is conflicted with the following 2 RAN2 agreements:

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- Rel-19 UEs only monitor the PO(s)xx according to Rel-19 paging configuration.

Main concern of Option 1) is that NW may transmit two sets of PEIs which are bad for NES. However, it is not correct:

- NW can still only send one legacy PEI to cover both legacy POs and Rel-19 POs, as illustrated in Alt-1 of Figure.1. In this case, NW only send legacy PEI configuration in SIB.

- Only when NW can’t use one legacy PEI to cover both legacy POs and Rel-19 POs, it configures the Rel-19 PEI in SIB, which is illustrated in Alt-2 of Figure.1.

Figure.1 Illustration of two solutions of supporting PEI in paging adaptation

Based on above analysis, we propose the following UE behavior:

Proposal 4: For Rel-19 UE supporting PEI and paging adaptation:

- When only legacy PEI configuration is provided in SIB, it only monitors legacy PEI occasions derived from it.

- When new PEI configuration (pagingAdaptationPEI-Config) is provided in SIB, it only monitors Rel-19 PEI occasions derived from it.

### 2.1.3 Whether need a separate pei-ConfigBWP

In RRC running CR [3], the new PEI configuration is captured as below. We discuss the highlighted FFS in EN.

DownlinkConfigCommonSIB ::=     SEQUENCE {

[[

pei-Config-r17                  PEI-Config-r17                         OPTIONAL,     -- Need R

initialDownlinkBWP-RedCap-r17   BWP-DownlinkCommon                     OPTIONAL      -- Need R

]],

[[

frequencyInfoDL-v1800           FrequencyInfoDL-SIB-v1800              OPTIONAL      -- Need R

]],

[[

pagingAdaptationPEI-Config-r19          PEI-Config-r17                         OPTIONAL     -- Need R

]]

}

pagingAdaptationPEI-Config The PEI related configuration for paging adaptation

Editor’s note: FFS field description for pagingAdaptationNAndPagingFrameOffset with respect to possible configuration restrictions. FFS: firstPDCCH-MonitoringOccasionOfPO for paging adaptations. FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

We think the current pei-ConfigBWP can’t well support the case of Ns=8 in the new PEI-O configuration. As highlighted below, the existing symbol level offset IE (i.e. firstPDCCH-MonitoringOccasionOfPEI-O-r17) is configured per maxPEI-perPF-r17. maxPEI-perPF-r17 is fixed to 4 which means the maximum PO number in one PF associated with one PEI.

PEI-O configuration

pei-ConfigBWP-r17      SEQUENCE {

pei-SearchSpace-r17                 SearchSpaceId,

firstPDCCH-MonitoringOccasionOfPEI-O-r17  CHOICE {

sCS15KHZoneT                                                     SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                       SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT  SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919),

sCS480KHZoneEighthT                                           SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                                        SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..71679)

}

maxPEI-perPF-r17                        INTEGER ::= 4       -- Maximum number of PEI occasion per paging frame

As Ns is increased to 8, the above legacy symbol level offset IE may not well support the new PEI-O for Rel-19 UEs. For example, when po-NumPerPEI = 1 and Ns=8, only the first 4 POs (i.e. i_s =0/1/2/3) can be associated with PEI and the last 4 POs (i.e. i_s =4/5/6/7) can’t be associated with any PEI. The root cause is due to hard coding of ASN.1 with maxPEI-perPF-r17=4.

Observation 1: The legacy pei-ConfigBWP can’t well support the case of Ns = 8 in Rel-19 PEI. For example, when po-NumPerPEI = 1 and Ns=8, only the first 4 POs (i.e. i_s =0/1/2/3) can be associated with PEI and the last 4 POs (i.e. i_s =4/5/6/7) can’t be associated with any PEI.

We think the simplest way to resolve this issue is to introduce a separate pei-ConfigBWP which increases the value maxPEI-perPF to 8 in Rel-19 PEI-O configuration. As we propose to introduce a new PEI-O configuration, there is no legacy issue. Meanwhile, we also don’t see strong concern on increasing RRC payload size because it is only increased from 21bit to 42bit. Thus, we propose:

Proposal 5: Introduce a separate pei-ConfigBWP which increases maxPEI-perPF to 8 in symbol level offset IE firstPDCCH-MonitoringOccasionOfPEI-O (i.e. allow one PEI to associate with 8 POs in one PF when Ns=8).

## 2.2 Adaptation of SSB in time domain

SSB adaptation was discussed in RAN2#129 [2], and below agreement was made:

Agreements on SSB adaptation

- RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell.

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

R2-2500199	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: R2 adopts a unified MAC CE design for both OD-SSB (i.e., NES O1) and SSB adaptation (i.e., NES O3).

R2-2500258	Discussion on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: On SSB adaptation, RAN2 wait RAN1 conclusion on whether to use MAC-CE to indicate SSB adaptation (i.e. RAN2 doesn’t discuss whether the unified signaling is used in OD-SSB and SSB adaptation).

[Apple]: It is under RAN1 discussion. Want to wait for more RAN1 progress. In addition, implicit deactivation is also discussed in RAN1, which may make difference. [LG, IDC]: Agree with Apple. [OPPO]: Can we make a conclusion, if RAN1 decides MAC CE based option, RAN2 will aim the unified design? [Xiaomi]: No. [Nokia]: Assume we still start RAN2 job for OD-SSB MAC CE. [Apple]: Confirms Nokia’s comment.

Wait for more RAN1 progress.

Meanwhile, signalling for SSB adaptation was discussed but no conclusion was made.

We noticed that RAN1#120 has agreed to introduce UE group common DCI for SSB adaptation [6], but they left one note on MAC-CE:

Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

Since RAN1 has agreed DCI, we don’t think RAN2 need to introduce additional MAC-CE for the same purpose. Thus, we propose:

Proposal 6: As RAN1 agreed to extend DCI 2-9 as signaling of SSB adaptation, no need to introduce MAC-CE for SSB adaptation.

## 2.3 Adaptation of PRACH in time domain

### 2.3.1 Clarification on RAN1 agreement on PDCCH order RACH

RACH adaptation was discussed in RAN2#129 [2], and below agreement was made:

Agreements on RACH adaptation

- Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.

2a.	RAN2 starts CBRA for RACH adaptation.

2b. 	RAN2 starts 4-step RACH adaptation.

3. 	From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request.

- From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.

5a. 	From R2 perspective, RACH adaptation is not modelled as RA feature(s).

5b.	From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

- Will follow legacy mechanism regarding how to select RACH resource.

One issue is that RAN2 agreed to start from CBRA. However, RAN1#120 has agreed to use DCI 1-0 with C-RNTI to trigger adaptation of CFRA (PDCCH order RACH) [6].

Agreement For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH. - FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s) - FFS: Details on how to reuse existing bit for the 1-bit indication

However, it is not clear whether “PDCCH order” in the agreement means PDCCH order RACH in serving cell (e.g., for time alignment) or PDCCH order RACH in candidate cell (i.e. PDCCH order for early sync in LTM).

Observation 2: RAN1#120 has agreed to use DCI 1-0 with C-RNTI to trigger adaptation PDCCH order RACH. However, it is not clear whether “PDCCH order” in the agreement means PDCCH order RACH in serving cell (e.g., for time alignment) or PDCCH order RACH in candidate cell (i.e. PDCCH order for early sync in LTM).

We think RAN2 need to clarify the above ambiguity. Our view is:

- PDCCH order RACH in serving cell (e.g., for time alignment): it makes more sense because CFRA resource pool is configured in RRC to share across different UEs. RACH congestion may happen if DL data traffic arrives for many UEs during a short duration. In such case, similar mechanism of RACH adaptation can be reused for CFRA to achieve NES gain.

- PDCCH order RACH in candidate cell (i.e. PDCCH order for early sync in LTM): We prefer not to support it. Firstly, it requires candidate cell to reserve two PRACH resource sets and timely coordination between source cell and target cell on when to trigger RACH adaptation. Secondly, it requires further specification impacts on RRC to introduce additional RACH resource configuration in IE LTM-Candidate of LTM-Config.

Observation 3: If supporting PDCCH order RACH adaptation for early sync in LTM, it requires candidate cell to reserve two PRACH resource sets and timely coordination between source cell and target cell. Furthermore, it requires RRC specification change to add additional RACH resource configuration in IE LTM-Candidate.

Thus, we propose:

Proposal 7: On RAN1#120 agreement to use DCI 1-0 with C-RNTI to trigger adaptation of PDCCH order RACH, RAN2 clarify that only PDCCH order RACH in serving cell (e.g. for time alignment) supports it.

### 2.3.2 RA-RNTI ambiguity issue

In RAN1#120 meeting [6], there were agreements on configuring separately a Msg1-FDM for the additional PRACH resources.

Agreement - Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs) - FFS: When there is no configuration of Msg1-FDM - Separate configuration of number of SSB per RO is supported

Considering that the legacy ROs and additional ROs could be overlapping in time domain, then for the ROs which are FDMed with the same f_id, their RA-RNTI may be the same. In this case, although a legacy UE and a new UE are sending Msg 1 on different ROs, they use the same RA-RNTI to monitor RAR which increase the collision probability and also increase UE power consumption for detecting RAR not intended for itself.

Observation 4: As legacy ROs and additional ROs could be overlapping in time domain, for the ROs which are FDMed with the same f_id, their RA-RNTI are the same, which may lead to increasing RAR collision probability and UE power consumption for detecting RAR not intended for itself.

To solve the RA-RNTI ambiguity issue, the following alternatives can be considered:

- Alt 1: For RA-RNTI calculation, introduce a fixed k_offset for the additional PRACH resources, i.e.

RA-RNTI = 1 + s_id + 14 x t_id + 14 x 80 x f_id + 14 x 80 x8 x ul_carrier_id + 14 x 80x 8 x 2 x k_offset, where the k_offset >1.

- Alt 2: For RA-RNTI calculation, f_id for the additional PRACH resources overlapped in time domain with the legacy resources is calculated based on the number of FDMed legacy ROs, i.e. f_id_A is the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs.

As shown in Figure. 2, this can be applied to all the additional PRACH resources regardless whether they overlap with legacy ones in time domain or not, or can be applied only to the additional PRACH resources that are overlapped in time domain (Alt 2-2).

Figure.2 Illustration of Alt-2

Proposal 8: RAN2 discuss RA-RNTI ambiguity issue, and consider the following alternatives:

- Alt 1: Introduce a fixed k_offset for the additional PRACH resources, i.e. RA-RNTI = 1 + s_id + 14 x t_id + 14 x 80 x f_id + 14 x 80 x8 x ul_carrier_id + 14 x 80x 8 x 2 x k_offset (k_offset>1)

- Alt 2: f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs.

# 3 Conclusion

In this contribution, we further discuss RAN2 work on common signal transmission adaptation. Our observations are:

Observation 1: The legacy pei-ConfigBWP can’t well support the case of Ns = 8 in Rel-19 PEI. For example, when po-NumPerPEI = 1 and Ns=8, only the first 4 POs (i.e. i_s =0/1/2/3) can be associated with PEI and the last 4 POs (i.e. i_s =4/5/6/7) can’t be associated with any PEI.

Observation 2: RAN1#120 has agreed to use DCI 1-0 with C-RNTI to trigger adaptation PDCCH order RACH. However, it is not clear whether “PDCCH order” in the agreement means PDCCH order RACH in serving cell (e.g., for time alignment) or PDCCH order RACH in candidate cell (i.e. PDCCH order for early sync in LTM).

Observation 3: If supporting PDCCH order RACH adaptation for early sync in LTM, it requires candidate cell to reserve two PRACH resource sets and timely coordination between source cell and target cell. Furthermore, it requires RRC specification change to add additional RACH resource configuration in IE LTM-Candidate.

Observation 4: As legacy ROs and additional ROs could be overlapping in time domain, for the ROs which are FDMed with the same f_id, their RA-RNTI are the same, which may lead to increasing RAR collision probability and UE power consumption for detecting RAR not intended for itself.

Based on observations, our proposals are:

Adaptation of paging occasions

Proposal 1: No need to have any configuration restriction on Rel-19 paging configuration (i.e. pagingAdaptationNAndPagingFrameOffset, pagingAdaptation-NS).

Proposal 2: The UE derives Rel-19 paging cycle as legacy, i.e. min(default paging cycle, UE specific DRX cycle).

Proposal 3: Reuse existing optional IE firstPDCCH-MonitoringOccasionOfPO for Rel-19 Paging configuration.

Proposal 4: For Rel-19 UE supporting PEI and paging adaptation:

- When only legacy PEI configuration is provided in SIB, it only monitors legacy PEI occasions derived from it.

- When new PEI configuration (pagingAdaptationPEI-Config) is provided in SIB, it only monitors Rel-19 PEI occasions derived from it.

Proposal 5: Introduce a separate pei-ConfigBWP which increases maxPEI-perPF to 8 in symbol level offset IE firstPDCCH-MonitoringOccasionOfPEI-O (i.e. allow one PEI to associate with 8 POs in one PF when Ns=8).

Adaptation of SSB in time domain

Proposal 6: As RAN1 agreed to extend DCI 2-9 as signaling of SSB adaptation, no need to introduce MAC-CE for SSB adaptation.

RACH adaptation

Proposal 7: On RAN1#120 agreement to use DCI 1-0 with C-RNTI to trigger adaptation of PDCCH order RACH, RAN2 clarify that only PDCCH order RACH in serving cell (e.g. for time alignment) supports it.

Proposal 8: RAN2 discuss RA-RNTI ambiguity issue, and consider the following alternatives:

- Alt 1: Introduce a fixed k_offset for the additional PRACH resources, i.e. RA-RNTI = 1 + s_id + 14 x t_id + 14 x 80 x f_id + 14 x 80 x8 x ul_carrier_id + 14 x 80x 8 x 2 x k_offset (k_offset>1)

- Alt 2: f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs.

# 4 References

[1] RP-250343, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN2#129, Chair Notes.

[3] R2-2501462, Running RRC CR for enhancements for network energy efficiency, Ericsson.

[4] R2-2502323, Running 38.304 CR for network energy saving, Apple.

[5] R2-2502129, Summary report of [POST129][104][NES] 38.304 running CR and stage 3 open issues, Apple.

[6] RAN1#120, Chair Notes.