3GPP TSG-RAN WG2 Meeting #129bis                                                               R2-2502140

Wuhan, China, 7th - 11th April 2025

Source:	CATT

Title:	Discussion on UE Radio Capability for Paging Information

Agenda Item:	6.1.3.1

Document for:	Discussion and Decision

- Introduction

In RAN2#129 meeting, a reply LS from SA2 was received [1]. In the LS, SA2 think UE Radio Capability for Paging Information may be lost in some cases and there can be paging problems.

2) Initial Registration may be performed on a pre-Rel 19 gNB (or a later gNB that does not support LP-WUS) that analyses the UE’s UE Radio Access Capabilities to create the UE Radio Capability for Paging Information and send it to the AMF, see clause 5.4.4.3 of TS23.501. It will be important that LP-WUS related aspects of UE Radio Capability for Paging Information are not lost when Initial Registration is performed on a pre-Rel-19 gNB. If the LP-WUS aspects are lost, then there can be paging problems, when for example, paging is performed by a gNB that supports LP-WUS. SA2 invites RAN2 and RAN3 to investigate this and provides solution if any issue is identified. SA2 was not sure whether this issue also impacts RedCap, eRedCap and other features that are signalled within the UE Radio Capability for Paging Information. SA2 invites RAN2 and RAN3 to investigate this.

The issue was discussed on R19 LP-WUS session and the following agreement was achieved:

Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.

In this contribution, we would like to further discuss whether there are some problems for UE Radio Capability for Paging Information on a pre-R17 gNB and possible solutions to handle these problems.

- Discussion

  - Problems for UE Radio Capability for Paging Information on a pre-Rel-17 gNB

The following is the existing UE radio capability for Paging Information in TS 38.331 [2]. Since Rel-17, there are some new features introduced in UE radio capability for Paging Information which is highlighted in yellow, including:

- PEI related parameters, i.e. ue-RadioPagingInfo-r17, and LP-WUS related parameters are assumed to be extended in the container;

- inactiveStatePO-Determination;

- Redcap/eRedCap related parameters, i.e. numberOfRxRedCap-r17, halfDuplexFDD-TypeA-RedCap-r17 and numberOfRxERedCap-r18;

- 2RxXR, i.e. supportOf2RxXR-r18;

- NTN related parameters, i.e. dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN-r18 and dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN-r18.

#### – UERadioPagingInformation This message is used to transfer radio paging information, covering both upload to and download from the 5GC, and between gNBs. Direction: gNB to/ from 5GC and gNB to/from gNB UERadioPagingInformation message -- ASN1START -- TAG-UE-RADIO-PAGING-INFORMATION-START UERadioPagingInformation ::= SEQUENCE { criticalExtensions CHOICE { c1 CHOICE{ ueRadioPagingInformation UERadioPagingInformation-IEs, spare7 NULL, spare6 NULL, spare5 NULL, spare4 NULL, spare3 NULL, spare2 NULL, spare1 NULL }, criticalExtensionsFuture SEQUENCE {} } } UERadioPagingInformation-IEs ::= SEQUENCE { supportedBandListNRForPaging SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL, nonCriticalExtension UERadioPagingInformation-v15e0-IEs OPTIONAL } UERadioPagingInformation-v15e0-IEs ::= SEQUENCE { dl-SchedulingOffset-PDSCH-TypeA-FDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeA-TDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeA-TDD-FR2 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-FDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-TDD-FR1 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-TDD-FR2 ENUMERATED {supported} OPTIONAL, nonCriticalExtension UERadioPagingInformation-v1700-IEs OPTIONAL } UERadioPagingInformation-v1700-IEs ::= SEQUENCE { ue-RadioPagingInfo-r17 OCTET STRING (CONTAINING UE-RadioPagingInfo-r17) OPTIONAL, inactiveStatePO-Determination-r17 ENUMERATED {supported} OPTIONAL, numberOfRxRedCap-r17 ENUMERATED {one, two} OPTIONAL, halfDuplexFDD-TypeA-RedCap-r17 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL, nonCriticalExtension UERadioPagingInformation-v1800-IEs OPTIONAL } UERadioPagingInformation-v1800-IEs ::= SEQUENCE { numberOfRxERedCap-r18 ENUMERATED {one, two} OPTIONAL, supportOf2RxXR-r18 ENUMERATED {supported} OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } UERadioPagingInformation-v1840-IEs ::= SEQUENCE { dl-SchedulingOffset-PDSCH-TypeA-FDD-FR2-NTN-r18 ENUMERATED {supported} OPTIONAL, dl-SchedulingOffset-PDSCH-TypeB-FDD-FR2-NTN-r18 ENUMERATED {supported} OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } -- TAG-UE-RADIO-PAGING-INFORMATION-STOP -- ASN1STOP

According to TS 38.413 [3], UE Radio Capability for Paging which includes UERadioPagingInformation can be included in the following messages as shown in Table 1.

Table 1: Messages including UE Radio Capability for Paging which includes UERadioPagingInformation

| Message | Direction | Purpose |
| --- | --- | --- |
| UE RADIO CAPABILITY INFO INDICATION | gNB AMF | To provide to the AMF UE radio capability-related information |
| PAGING | AMF gNB | CN paging |
| INITIAL CONTEXT SETUP REQUEST | AMF gNB | To request the setup of a UE context |
| UE CONTEXT MODIFICATION REQUEST | AMF gNB | To provide UE Context information changes to gNB |
| HANDOVER REQUEST | AMF gNB | To request the preparation of resources |
| PATH SWITCH REQUEST ACKNOWLEDGE | AMF gNB | To inform the gNB that the path switch has been successfully completed in the 5GC |

The UE radio Capability can be firstly transferred to the AMF via UE RADIO CAPABILITY INFO INDICATION and stored at the AMF. And the AMF sends the UE Radio Capability for Paging to other gNBs when necessary, e.g. for HO or Paging. In addition, UERadioPagingInformation is included in Core Network Assistance Information for RRC INACTIVE when transmitted from AMF to gNB. The gNB can use the info in RAN paging. Hence, no matter it is a CN paging or a RAN paging, UE Radio Capability for Paging is originated from AMF.

PEI&LP-WUS

When Initial Registration is performed on a pre-R17 gNB, the whole IE UE-RadioPagingInfo is absent in the UERadioPagingInformation message when it is sent from the pre-R17 gNB to AMF via UE RADIO CAPABILITY INFO INDICATION message. Then PEI related aspects of UE Radio Capability for Paging Information for the UE are lost in AMF. When a CN paging or a RAN paging for the UE is triggered, UE Radio Capability for Paging is incomplete as it is originated from AMF and PEI related aspects of UE Radio Capability for Paging Information for the UE are lost. Then there is paging problem. E.g., Legacy paging for the UE is transmitted by a R17 gNB that supports PEI as PEI related info is lost. While the UE monitors PEI in the gNB. The UE cannot be waked up to monitor the associated PO as PEI is not transmitted for the UE. The UE loses the paging.

LP-WUS related parameters are assumed to be extended in the container (i.e. UE-RadioPagingInfo) which includes PEI related capabilities. Hence, LP-WUS related aspects of UE Radio Capability for Paging Information for the UE are lost if Initial Registration is performed on a pre-R17 gNB which is similar to the case of PEI. Thus, there is paging problem. For example, the UE comps on a Rel-19 gNB which broadcasts it supports LP-WUS but it is not aware of the UE capability that the UE supports LP-WUS since it is lost. Then, the gNB sends Paging without LP-WUS. However, the UE monitors LP-WUS when conditions of LP-WUS are satisfied. The UE cannot be waked up to monitor the associated PO as LP-WUS is not transmitted for the UE. In the end, the UE loses the paging.

Observation 1: PEI and LP-WUS related aspects of UE Radio Capability for Paging Information are lost when Initial Registration is performed on a pre-R17 gNB. Then there are paging problems, i.e., paging may be lost.

inactiveStatePO-Determination

Similarly, inactiveStatePO-Determination is lost when Initial Registration is performed on a pre-R17 gNB. Then there is paging problem, e.g. a R17 gNB that broadcasts ranPagingInIdlePO with value "true" determines the i_s based on parameters of RRC_INACTIVE state for the UE as inactiveStatePO-Determination is lost, while the RRC_INACTIVE UE supports inactiveStatePO-Determination in the gNB uses the same i_s as for RRC_IDLE state. Thus, the UE monitors the “wrong” PO which results in paging loss.

Observation 2: inactiveStatePO-Determination is lost when Initial Registration is performed on a pre-R17 gNB. Then there is paging problem, i.e., paging may be lost.

RedCap/eRedCap

Different from LP-WUS features, UEs support Redcap and/or eRedcap features are not allowed to access legacy gNBs, e.g, field intraFreqReselectionRedCap can be used to determine whether the cell is considered as barred for a RedCap UE. Therefore, for UEs supporting Redcap and/or eRedcap features, Initial Registration is always performed on R17 and R17 later gNB. (e)Redcap Related aspects of UE radio capability for Paging Information are not lost.

Observation 3: (e)Redcap Related aspects of UE radio capability for Paging Information are not lost as UEs support Redcap and/or eRedcap features are not allowed to access legacy gNBs. There is no paging problem.

2RxXR

supportOf2RxXR was introduced in R18 to indicate whether the UE is a 2Rx XR UE. UEs supporting 2RxXR are allowed to access legacy gNBs. Then 2RxXR related aspects of UE radio capability for Paging Information, i.e. supportOf2RxXR is lost for CN paging when Initial Registration is performed on a pre-R18 gNB. But supportOf2RxXR is used for the network to avoid sending a paging message to the cell which the UE cannot camp on. If the IE is lost, the gNB may transmit redundant paging message for the 2RxXR UE even if the UE cannot camp on. But there is no paging problem.

Observation 4: supportOf2RxXR is lost when Initial Registration is performed on a pre-R18 gNB. But there is no paging problem.

NTN

In R18, indicators to indicate whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A/B in FDD FR2-NTN are introduced within the UERadioPagingInformation message. These indicators are lost for CN paging when Initial Registration is performed on a pre-R18 gNB. Similar as indicators to indicate whether the UE supports DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A/B for Terrestrial Network, if these new capabilities for NTN are lost, the network can paging the UE with K0=0. There is no paging problem.

Observation 5: NTN related aspects of UE Radio Capability for Paging Information are lost when Initial Registration is performed on a pre-R18 gNB. But there is no paging problem.

  - Solutions to handle paging problem

According Observation 1-5, there are paging problems for PEI, LP-WUS and inactiveStatePO-Determination when Initial Registration is performed on a pre-R17 gNB. The possible options to handle the issue can be:

- Option 1: Up to network implementation to avoid lost issue.

In order to avoid lost issue for PEI, LP-WUS and inactiveStatePO-Determination, the network ensures that all gNBs can be updated to recognize R17 UE radio capability for paging. Then there is no paging problem as there is no lost issue for UE radio capability for paging in AMF.

- Option 2: An indicator is sent to UE to indicate R17 related paging info has been sent to CN if UE capability transfer procedure (gNB->AMF) is completed and UE Radio Capability for Paging in the gNB includes R17 related paging info.

With option 2, when UE Radio Capability for Paging for a UE which include R17 related paging info has been sent from gNB to AMF, e.g. when Initial Registration is performed on R17 gNB or UE Radio Capability for Paging info which includes R17 related paging info is updated from gNB to AMF, an indicator is sent to the UE to indicate R17 related paging info has been sent to CN. Then the UE can be aware if PEI, LP-WUS and inactiveStatePO-Determination related features can be enabled. The indicator will be valid until a new UE capability transfer procedure is triggered. Without the indicator, the UE is not allowed to use PEI, LP-WUS and inactiveStatePO-Determination related features.

Proposal 1: There are paging problems for PEI, LP-WUS and inactiveStatePO-Determination when Initial Registration is performed on a pre-R17 gNB.

Proposal 2: RAN2 discuss which option is adopted for the lost issue which leads paging problems.

- Option 1: Up to network implementation to avoid lost issue.

- Option 2: An indicator is sent to UE to indicate R17 related paging info has been sent to CN if UE capability transfer procedure (gNB->AMF) is completed and UE Radio Capability for Paging in the gNB includes R17 related paging info.

- Conclusion

According to the analysis in section 2, the following observations and proposals are provided:

Observation 1: PEI and LP-WUS related aspects of UE Radio Capability for Paging Information are lost when Initial Registration is performed on a pre-R17 gNB. Then there are paging problems, i.e., paging may be lost.

Observation 2: inactiveStatePO-Determination is lost when Initial Registration is performed on a pre-R17 gNB. Then there is paging problem, i.e., paging may be lost.

Observation 3: (e)Redcap Related aspects of UE radio capability for Paging Information are not lost as UEs support Redcap and/or eRedcap features are not allowed to access legacy gNBs. There is no paging problem.

Observation 4: supportOf2RxXR is lost when Initial Registration is performed on a pre-R18 gNB. But there is no paging problem.

Observation 5: NTN related aspects of UE Radio Capability for Paging Information are lost when Initial Registration is performed on a pre-R18 gNB. But there is no paging problem.

Proposal 1: There are paging problems for PEI, LP-WUS and inactiveStatePO-Determination when Initial Registration is performed on a pre-R17 gNB.

Proposal 2: RAN2 discuss which option is adopted for the lost issue which leads paging problems.

- Option 1: Up to network implementation to avoid lost issue.

- Option 2: An indicator is sent to UE to indicate R17 related paging info has been sent to CN if UE capability transfer procedure (gNB->AMF) is completed and UE Radio Capability for Paging in the gNB includes R17 related paging info.

- Reference

- R2-2500050/S2-2412876, LS Reply on LP-WUS subgrouping

- 3GPP TS 38.331 V18.5.1.

- 3GPP TS 38.413 V18.5.0.