3GPP TSG-RAN WG2 Meeting #129bis		R2-2502160

Wuhan, China, 7th – 11th April 2025

Source:	vivo

Title:		Discussion on LTM measurement event evaluation and reporting

Agenda Item:		8.6.3

Document for:	Discussion and Decision

- Introduction

In RAN2 #129 meeting, L1 event triggered measurement reporting was discussed and some agreements have been reached on the configuration structure, MAC CE format and other aspects.

In this contribution, we would like to discuss some remaining open issues, including:

- The MAC CE format for L1 event-trigger MR

- The MAC CE format for CSI-RS/CSI-IM Resource Set Activation/Deactivation on candidate cell

- mTRP impact on L1 MR

- Measurement resource configuration

- Padding L1 MR MAC CE

- Discussion

  - MAC CE format for L1 event-triggered MR

After the discussion during several meetings, now all the information we agreed to be included in the MAC CE and the format are as follows[1][2][3]:

RAN2 #127bis agreements on L1 event triggered MR

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

RAN2 #128 agreements on L1 event triggered MR

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

RAN2 #129 agreements on L1 event triggered MR

- Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

- Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

- R18 differential RSRP encoding is used as baseline.

Based on all the information, now we can summarize what we should include in the MAC CE and corresponding bits they take.

- SSBRI/CRI and RSRP:

According to TS 38.212[4], SSBRI/CRI should be 9 bits, and using differential RSRP would take 4 bits, rather than 7 bits for normal RSRP.

Table 6.3.1.1.2-6: CRI, SSBRI, RSRP, and CapabilityIndex

| Field | Bitwidth |
| --- | --- |
| CRI |  |
| SSBRI |  |
| RSRP | 7 |
| Differential RSRP | 4 |
| CapabilityIndex | 2 |

One question is that for serving cell beam measure result, whether we should use the differential RSRP or not. Considering it is better to use unified MAC CE format and we agreed event LTM2 where only serving beam measured results should be included, it is better to just use 7 bits of RSRP to represent the serving beam measured result for LTM2 to LTM5. Also, we understand the UE and the gNB are always aligned with the current beam the UE is using, so no RSRI of serving beam is needed in the MAC CE. Therefore, we propose:

Proposal 1: For current beam in serving cell, 7 bits of RSRP is included in the L1 MR MAC CE and no RSRI is needed.

- Report ID:

In our understanding, the report ID could be used to indicate which event is satisfied for the report, as events would be configured inside the report configuration and no need to introduce the measId-like IE. 6 bits as in legacy would be fine.

- Indication of whether entering event conditions have been met within TTT for the beam

As we agreed to use differential RSRP encoding, with making the RSRP 4 bits, additional 3 bits can be saved for other information for each beam. Therefore, we think it is simple to use 1 bit as a per-beam indication to indicate whether the beam meeting the entering event condition within TTT or not.

Proposal 2: Introduce per-beam indication of 1 bit to indicate whether a beam meets the event condition.

However, it should be noticed that, there is no extra bit for the first beam in the MAC CE as it takes 7 bits RSRP rather than differential RSRP. So, this per-beam indication for the first beam can be placed in the first octet.

Observation 1: the per-beam indication for the first beam in L1 MR MAC CE can be placed in the first octet.

- Indication of leaving event condition met

In our understanding, 1 bit in the header for leaving condition met would be enough. It seems no strong motivation for the gNB to know which exact beam is the triggered beam. Therefore, we propose:

Proposal 3: 1 bit indication is used to inform the leaving event condition is met in the MAC CE (i.e., not per-beam indication).

Based on the analysis above, the MAC CE format can be illustrated as follows:

Figure 1. Example of L1 event triggered measurement report and truncated L1 event triggered measurement report MAC CE

Where:

- Report ID: 6 bits, indicating the satisfied event information;

- S: 1 bit, indicating whether the first beam (RSRP of 7 bits) satisfies the event or not;

- T: 1 bit, indicating whether this report is triggered by entering condition or leaving condition;

- RSRI: 9 bits, indicating beam index;

- RSRP/DIFFRSRP: 7 bits for the first beam, 4 bits for the other beams, indicating the beam measured results;

- CM: 1 bit, indicating whether the beam satisfies the event or not;

- RSRPserving: 7 bits, the current beam measured result of serving cell;

- R: reserved bit, set to 0.

Proposal 4: RAN2 to agree the MAC CE format for L1 MR in Figure1. as the baseline, which includes:

- Report ID: 6 bits, indicating the satisfied event information;

- S: 1 bit, indicating whether the first beam (RSRP of 7 bits) satisfies the event or not;

- T: 1 bit, indicating whether this report is triggered by entering condition or leaving condition;

- RSRI: 9 bits, indicating beam index;

- RSRP/DIFFRSRP: 7 bits for the first beam, 4 bits for the other beams, indicating the beam measured results;

- CM: 1 bit, indicating whether the beam satisfies the event or not;

- RSRPserving: 7 bits, the current beam measured result of serving cell;

- R: reserved bit, set to 0.

  - MAC CE format for CSI-RS/CSI-IM Resource Set Activation/Deactivation on candidate cell

In RAN1 #120, an LS is sent to RAN2[5] which mentioned that semi-persistent CSI-RS resource is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting, and:

- MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2

Observation 2: RAN1 agreed that MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource for candidate cells.

The SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE for legacy mechanism is as follows[6]:

Figure 6.1.3.12-1: SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE

As the legacy CSI-IM resource is mainly used for measurement interference aspects, and RAN1 didn’t agree on any L1-SINR measurement report, we understand the CSI-IM resource related fields are not needed.

Observation 3: As RAN1 does not agree on L1-SINR measurement report, CSI-IM resource related field are not needed in semi-persistent CSI-RS resource Set Activation/Deactivation MAC CE for candidate cell.

And as this is for candidate cell, BWP ID is not needed either. Other fields can be easily reused. So the final MAC CE format can be as follows, which includes:

- A/D: 1 bit. This field indicates whether to activate or deactivate indicated SP CSI-RS resource set(s);

- Candidate Cell ID: 3 bits. This field indicates the identity of the candidate Cell for which the MAC CE applies. As we follow Rel-18 LTM to have 8 candidate cells in total, 3 bits would be OK;

- SP CSI-RS resource set ID: 6 bits. This field contains an index of NZP-CSI-RS-ResourceSet containing Semi Persistent NZP CSI-RS resources;

- TCI State IDi: This field contains TCI-StateId, as specified in TS 38.331 [5], of a TCI State, which is used as QCL source for the resource within the Semi Persistent NZP CSI-RS resource set indicated by SP CSI-RS resource set ID field;

- R: Reserved bit, set to 0.

Figure 2. Example of SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE for candidate cell

Proposal 5: RAN2 to agree the MAC CE format for SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for candidate cell in Figure 2. as the baseline, which includes:

- A/D: 1 bit. This field indicates whether to activate or deactivate indicated SP CSI-RS resource set(s);

- Candidate Cell ID: 3 bits. This field indicates the identity of the candidate Cell for which the MAC CE applies;

- SP CSI-RS resource set ID: 6 bits. This field contains an index of NZP-CSI-RS-ResourceSet containing Semi Persistent NZP CSI-RS resources;

- TCI State IDi: This field contains TCI-StateId, as specified in TS 38.331 [5], of a TCI State, which is used as QCL source for the resource within the Semi Persistent NZP CSI-RS resource set indicated by SP CSI-RS resource set ID field;

- R: Reserved bit, set to 0.

  - mTRP impact on L1 MR

In RAN2 #127bis meeting, the mTRP was discussed in RAN2 and it is agreed that we should wait for RAN1 progress[1]:

10. Consideration of mTRP (P6-1, P6-2 in R2-2409096, Samsung) - Option1: sTRP is prioritized in Rel-19 - Option2: mTRP is also considered in Rel-19 - We can wait for RAN1 decision.

Then in RAN1 #120 meeting[7], the following agreement has been reached:

Conclusion The following is up to RAN2: Coexistence of LTM event triggered reporting and mTRP operation at serving cells.

Observation 4: RAN1 agreed that coexistence of LTM event triggered reporting and mTRP operation at serving cells is up to RAN2.

In the previous discussions of L1 MR, we only focused on the single TRP scenarios. To expand LTM application scenarios, whether/how to consider event-triggered reporting in mTRP scenarios should be considered. In Rel-18 LTM, source cell configured with mTRP is naturally supported with no specification efforts, and target cell configured with mTRP under two TAGs is also supported. Except for TAG indication, no enhancement about TCI indication and beam report is introduced for the case of target cell configured with mTRP under two TAGs. As Rel-18 LTM is the baseline for the Rel-19 LTM, the above mechanism also applies to Rel-19 LTM, so mTRP scenarios should be considered.

Proposal 6: Same as Rel-18 LTM, the source cell and/or target cell configured with mTRP is supported in Rel-19 LTM.

When the source cell is configured with mTRP, the number of the indicated TCI states will be 2, regardless of sDCI-based mTRP and mDCI-based mTRP. Thus, how to determine the measurement RS of the serving cell beam based on the two indicated TCI states should be considered.

Observation 5: When the source cell is configured with mTRP, the number of indicated TCI states is 2.

The simplest approach is to select one of them as the serving cell beam.

- For inter cell multi-TRP, there is only one indicated TCI state associated to serving cell, and the other indicated TCI state is associated to neighbour cell, so we can just use the beam whose indicated TCI state is associated to serving cell as the current beam.

- For intra-cell multi-TRP, the two indicated TCI states are both associated to serving cell, and the simplest way is to use the indicated TCI state associated with CORESETPoolIndex#0 for mDCI-based mTRP, or the first indicated TCI state for sDCI-based mTRP.

In a word, we can just make clear/simple rule to select 1 TCI state as the current beam, and that is all. Therefore, we propose:

Proposal 7: For inter cell multi-TRP, the beam whose indicated TCI state is associated to serving cell is considered as the current beam.

Proposal 8: For intra cell multi-TRP, the beam whose indicated TCI state is associated with CORESETPoolIndex#0 (for mDCI) or is the first indicated TCI state (for sDCI), is considered as the current beam.

  - Measurement resource configuration

Another issue is about the measurement resource configuration[3].

Proposal5: Confirm on the previous agreement that "For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible". Comeback session: [ZTE]: Most of NW companies are ok with including candidate cell id into reporting configuration. However, we may need more time to check UE sides. [Samsung]: Including candidate cell id in reporting configuration would bring more complication to the UE since the UE needs to filter 2 steps (first based on LTM resource set and second based on candidate cell id among the ones that satisfied the first step). [LG]: Alternative is to enable NW to configure list of LTM resource set ids into reporting configuration, we can further think of until next meeting. - Will be revisited next meeting.

In our understanding, it is still not clear why candidate cell ID is needed in the reporting configuration. Basically, if the network really wants to have separate report from different candidate cells, the beams of different cell can be included in different resource set, based on NW implementation, and thus can be associated to different report configuration. Therefore, we don’t think candidate cell ID is needed in the reporting configuration and we can reuse the Rel-18 signalling.

Observation 6: It is up to network implementation to configure beams of different candidate cells in different resource sets.

Proposal 9: For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused, i.e., source cell can configure resource set that contains the beams of different candidate cell and associate it to a single report configuration.

Besides, in RAN2 #126 meeting, we agreed that:

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

However, there is no beam specific offset configuration now in the current RRC specification.

Observation 7: Beam specific offset is agreed to be applied but not supported in current signalling design.

In newest running CR[8], the measurement resource configuration is as follows:

#### LTM-CSI-ResourceConfig The IE LTM-CSI-ResourceConfig defines a group of one or more CSI resources for one or more LTM candidate configurations LTM-CSI-ResourceConfig information element -- ASN1START -- TAG-LTM-CSI-RESOURCECONFIG-START LTM-CSI-ResourceConfig-r18 ::= SEQUENCE { ltm-CSI-ResourceConfigId-r18 LTM-CSI-ResourceConfigId-r18, ltm-SSB-ResourceSet-r18 LTM-SSB-ResourceSet-r18, ... , [[ ltm-CSI-RS-ResourceSet-r19 LTM-CSI-RS-ResourceSet-r19 OPTIONAL --Need R ]] } LTM-SSB-ResourceSet-r18 ::= SEQUENCE { ltm-SSB-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF SSB-Index, ltm-CandidateIdList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18, ... } LTM-CSI-RS-ResourceSet-r19 ::= SEQUENCE { ltm-CSI-RS-ResourceList-r19 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r19)) OF NZP-CSI-RS-ResourceId, ltm-CandidateIdList-r19 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r19)) OF LTM-CandidateId-r18, ... } -- Editor's NOTE: FFS: how to associate between the measurement CSI-RS resources and candidate cells. explicit or implicit signaling of candidate cells per RAN1's agreement. Current change following the legacy SSB configuration serves as a placeholder with a explicit candidate cell's ID. -- TAG-LTM-CSI-RESOURCECONFIG-STOP -- ASN1STOP

To support the beam-level offset and to reuse the Rel-18 signalling as much as possible, we can just include a beam-offset list under the LTM-SSB-ResourceSet-r18, to associate the beam offset with corresponding beam index, e.g.:

LTM-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

LTM-BeamOffsetList-r19

}

Therefore, we propose:

Proposal 10: Introduce the beam offset list in LTM-SSB-ResourceSet-r18 to support Beam specific offset in event-triggered L1 MR.

  - Padding L1 measurement report MAC CE

Considering truncated measurement report MAC CE is already agreed to be supported, we could take one step further to consider whether padding measurement report MAC CE could also be supported.

In legacy BSR and SL-BSR discussion, the introduction of Padding BSR is driven by the following key considerations:

1/ Resource Utilization Optimization

When the uplink resources (UL Grant) allocated by the gNB are not fully utilized by the UE’s pending data, the remaining space would otherwise be filled with padding bits. Padding BSR enables the UE to utilize these padding bits to report buffer status, thereby minimizing resource waste and improving uplink efficiency.

2/ Reduced Signalling Overhead and Latency

Unlike Regular BSR, which requires explicit scheduling requests (SR) or dedicated grants, Padding BSR can be "piggybacked" on existing resource allocations. This eliminates additional signalling interactions, reducing latency and overhead, especially for scenarios with frequent buffer updates.

3/ Dynamic Buffer State Updates

While Regular and Periodic BSRs rely on specific triggers or timers, Padding BSR leverages padding space to dynamically report buffer status during any resource allocation. This allows the gNB to capture real-time data volume changes and optimize subsequent scheduling decisions.

4/ Support for Multi-Priority Logical Channels

When padding space is limited, Padding BSR employs Truncated BSR to prioritize reporting for the highest-priority LCG.

When it comes to L1 measurement reporting, similar reason could be considered, e.g., the L1 measurement results can update frequently, or the UL grant may not be able to accommodate all beam measurement results. Therefore, padding bits can be used to report beams such as:

- The updated beam measurement results that are already reported and do not satisfy the leaving condition

- The unsatisfied beam measurement results that has not been reported yet because of no triggering event is satisfied.

Therefore, we suggest that RAN2 to discuss whether padding L1 measurement report MAC CE should be supported. We prefer to support it considering the benefits it brings and the spec impact seems not much.

Proposal 11: Introduce padding L1 measurement report MAC CE.

- Conclusion

In this contribution, we discussed the MAC CE format for L1 event-triggered MR and CSI-RS/CSI-IM Resource Set Activation/Deactivation on candidate cell, and give the figure example of them. We also discussed mTRP and we prefer to support it in Rel-19. And we discuss and propose to add beam-level offset list in measurement resource configuration. At last, we suggest to introduce padding L1 MR MAC CE.

We have the following Observations and Proposals:

For L1 MR MAC CE format:

Observation 1: the per-beam indication for the first beam in L1 MR MAC CE can be placed in the first octet.

Proposal 1: For current beam in serving cell, 7 bits of RSRP is included in the L1 MR MAC CE and no RSRI is needed.

Proposal 2: Introduce per-beam indication of 1 bit to indicate whether a beam meets the event condition.

Proposal 3: 1 bit indication is used to inform the leaving event condition is met in the MAC CE (i.e., not per-beam indication).

Proposal 4: RAN2 to agree the MAC CE format for L1 MR in Figure1. as the baseline, which includes:

- Report ID: 6 bits, indicating the satisfied event information;

- S: 1 bit, indicating whether the first beam (RSRP of 7 bits) satisfies the event or not;

- T: 1 bit, indicating whether this report is triggered by entering condition or leaving condition;

- RSRI: 9 bits, indicating beam index;

- RSRP/DIFFRSRP: 7 bits for the first beam, 4 bits for the other beams, indicating the beam measured results;

- CM: 1 bit, indicating whether the beam satisfies the event or not;

- RSRPserving: 7 bits, the current beam measured result of serving cell;

- R: reserved bit, set to 0.

For CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE format:

Observation 2: RAN1 agreed that MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource for candidate cells.

Observation 3: As RAN1 does not agree on L1-SINR measurement report, CSI-IM resource related field are not needed in semi-persistent CSI-RS resource Set Activation/Deactivation MAC CE for candidate cell.

Proposal 5: RAN2 to agree the MAC CE format for SP CSI-RS/CSI-IM Resource Set Activation/Deactivation for candidate cell in Figure 2. as the baseline, which includes:

- A/D: 1 bit. This field indicates whether to activate or deactivate indicated SP CSI-RS resource set(s);

- Candidate Cell ID: 3 bits. This field indicates the identity of the candidate Cell for which the MAC CE applies;

- SP CSI-RS resource set ID: 6 bits. This field contains an index of NZP-CSI-RS-ResourceSet containing Semi Persistent NZP CSI-RS resources;

- TCI State IDi: This field contains TCI-StateId, as specified in TS 38.331 [5], of a TCI State, which is used as QCL source for the resource within the Semi Persistent NZP CSI-RS resource set indicated by SP CSI-RS resource set ID field;

- R: Reserved bit, set to 0.

For mTRP:

Observation 4: RAN1 agreed that coexistence of LTM event triggered reporting and mTRP operation at serving cells is up to RAN2.

Observation 5: When the source cell is configured with mTRP, the number of indicated TCI states is 2.

Proposal 6: Same as Rel-18 LTM, the source cell and/or target cell configured with mTRP is supported in Rel-19 LTM.

Proposal 7: For inter cell multi-TRP, the beam whose indicated TCI state is associated to serving cell is considered as the current beam.

Proposal 8: For intra cell multi-TRP, the beam whose indicated TCI state is associated with CORESETPoolIndex#0 (for mDCI) or is the first indicated TCI state (for sDCI), is considered as the current beam.

For Measurement resource configuration:

Observation 6: It is up to network implementation to configure beams of different candidate cells in different resource sets.

Observation 7: Beam specific offset is agreed to be applied but not supported in current signalling design.

Proposal 9: For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused, i.e., source cell can configure resource set that contains the beams of different candidate cell and associate it to a single report configuration.

Proposal 10: Introduce the beam offset list in LTM-SSB-ResourceSet-r18 to support Beam specific offset in event-triggered L1 MR.

For Padding L1 measurement report MAC CE:

Proposal 11: Introduce padding L1 measurement report MAC CE.

- References

- RAN2 #127bis meeting chair notes;

- RAN2 #128 meeting chair notes;

- RAN2 #129 meeting chair notes;

- 3GPP TS 38.212 V18.6.0 (2025-03);

- R1-2501500, LS on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells;

- 3GPP TS 38.321 V18.5.0 (2025-03);

- RAN1 #120 meeting chair notes;

- R2-2502096, Draft running CR for event-triggered meas report for RRC