3GPP TSG-RAN WG2 Meeting #129bis		R2-2502161

Wuhan, China, 7th – 11th April 2025

Source:	vivo

Title:		Discussion on conditional LTM

Agenda Item:		8.6.4

Document for:	Discussion and Decision

- Introduction

In RAN2 #129 meeting, CLTM was discussed and some agreements have been reached[1]:

Agreements on C-LTM:

- From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

- The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

- For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

- Do not support DG-based first UL transmission for RACH-less conditional LTM.

- For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

- For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

In this contribution, we would like to discuss some remaining open issues, including:

- RACH-less procedure when no CG at CLTM execution or CG transmission failure happens;

- TA handling for CLTM, including the target cell and other candidate cells;

- Content of the MAC CE for CLTM early TA;

- CLTM supervisor timer.

- Discussion

  - RACH-Less procedure with no CG at CLTM execution

In RAN2 #129 meeting, the following agreement clarifies the UE’s behaviour to use RACH-Less procedure for CLTM at execution[1]:

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

However, there was some discussions about what should be the UE’s behaviour when there is no CG occasion at the time of CLTM execution, considering now only CG-based RACH-Less is supported for CLTM.

Observation 1: UE’s behaviour is not clear when there is no CG occasion at the time of CLTM execution.

In our understanding, the right condition for the UE to perform RACH-less C-LTM is to judge whether the TA timer is still running in the first available CG occasion for initial CG transmission. i.e., even the UE has a valid TA at the time of CLTM execution, if the TA timer is going to expire at the first available CG occasion, the UE should not perform RACH-less CLTM.

Proposal 1: If the UE has a valid TA at the time of CLTM execution, but the TA timer is going to expire at the first available CG occasion, the UE should perform RACH-based CLTM.

Then, a similar question would be raised that, if the TA timer is running at the first available CG occasion, and the RACH-less CLTM begins, what the UE’s behaviour should be if the transmission on first available CG fails.

Observation 2: The UE’s behaviour is not clear when RACH-less CLTM begins but the transmission on CG occasion fails.

In that case, the UE should wait for the next available CG, and the UE should again to evaluate whether the TA timer is going to expire at the next available CG occasion. If that happens (i.e., TA timer is going to expire at the next available CG occasion), then the UE can:

- Option-1: to consider the RACH-less procedure failed and perform re-establishment;

- Option-2: to transfer to RACH-based CLTM.

In this case, as RACH-less procedure already started, we think option-1 should be easier. Therefore,

Proposal 2: If UE performs RACH-LESS CLTM but transmission on first CG fails, the UE continue to perform RACH-LESS procedure if the TAT is still running at the next available CG occasion, otherwise the ongoing RACH-LESS CLTM is considered failure.

  - TA handling for CLTM

In last meeting discussion[1], there was a left issue about TA timer operation for target cell, as follows:

 CLTM TAT for target cell is not stopped upon CLTM cell switch execution to the target cell. Rapporteur suggests to discuss how/whether to start the timeAlignmentTimer associated with PTAG upon CLTM cell switch execution to the target cell during CB session: - Option 1 (revert the offline agreement): start the timeAlignmentTimer associated with PTAG as in Rel-18; - Option 2 (keep the offline agreement): discuss the options below in the MAC running CR o Option 2.1: clarify the running TAT to act as the timeAlignmentTimer associated with PTAG by a note in the MAC running CR; o Option 2.2: define that UE can perform UL tx when the TAT for the target cell is running. Besides, we need to add a note that, if SCell and SpCell are in the different TAG, SCell may not work when this TAT for target cell is running. o Option 2.3: the UE stops the CLTM timer and starts the PTAG using the remaining time of the CLTM timer. - => Will be revisited next meeting.

In our understanding, it is better to align with the offline agreement that we should use the running TAT of candidate cell (named as ltm-Candidate-TimeAlignmentTimer in the running CRs) to maintain the TA, so it is better to adopt option-2.

As for the sub-options under option-2, in order to minimize the spec impact, it is better to not change the original text about TAT with PTAG much. Therefore, we think option 2.1 is better, because it has least spec impact after we clarify in the note that the running TAT acts as the timeAlignmentTimer associated with PTAG. Therefore,

Proposal 3: RAN2 agree to clarify the running TAT (i.e. ltm-Candidate-TimeAlignmentTimer) to act as the timeAlignmentTimer associated with PTAG by a note in the MAC running CR.

Besides the target cell, there are other candidate cells of CLTM and how to handle the TA of them is also not discussed. And editor’s note is also added as follows[2]:

Editor’s NOTE: FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

There are two options:

- Option-1: TAs of other candidate cells are considered not valid anymore upon CLTM execution;

- Option-2: TAs of other candidate cells are considered still valid upon CLTM execution.

Option-1 is easy, while the motivation for option-2 may be that in subsequent CLTM, it can save the early TA acquisition procedure for other candidate cells before their TA is not valid at the UE, and the UE may change to another CLTM candidate cell quickly.

Observation 3: It may be helpful in subsequent CLTM if TAs of other candidate cells are considered still valid upon CLTM execution.

Similar discussion also happens in Rel-18 in RAN3, and it was agreed in RAN3 that the target DU receives the TA values for subsequent LTM in Cell Switch Notification[3],which means the TA of other candidate cell acquired in the source cell will be maintained by the target cell. So we understand this can be similar situation here.

Observation 4: RAN3 agreed target DU receives the TA values for subsequent LTM in Rel-18.

Anyway, the new target cell can send updated valid TA to UE based on its implementation, so we understand it is ok to keep the valid TA of other candidate cells upon CLTM execution. Therefore, we propose:

Proposal 4: Valid TAs of other candidate cells are considered still valid upon CLTM execution until TAT expires or updated TA has been received from target cell.

  - Content of the MAC CE for CLTM early TA acquisition

In RAN2 #128 meeting[4], it was agreed that:

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

Then we will discuss the contents and format of the new MAC CE in this section. Firstly, the TA value corresponding to the early TA acquisition should be included in the MAC CE. We think the field TAC in Rel-18 intra-CU LTM Cell Switch MAC CE could be reused directly, i.e., the field with 12 bits length indicates the index value TA used to control the amount of timing adjustment that the MAC entity has to apply as specified in TS 38.213. Besides, the following issues should be considered for the TA value within the new MAC CE:

- Issue#1: Whether the TA value could be FFF similar to the TAC field in Rel-18 intra-CU LTM, which means that no valid timing adjustment is available.

- Issue#2: Whether two TAGs could be configured for the CLTM candidate cell.

Regarding the issue#1, since the new MAC CE is only sent to UE after UE performed early RACH, the valid TA could be acquired by the source gNB in this case, thus there is no need to consider the case that the TAC field of the new MAC CE is FFF.

Regarding the issue#2, since Rel-18 intra-CU LTM could be configured together with the MIMO 2 TA, the same principle should be reused for CLTM. Thus, the scenario in which the CLTM candidate cell is configured with two TAGs may occur, in this case, the new MAC CE should indicate the related TAG information in TAC field. In Rel-18 LTM, it is indicated by the TCI state ID within the LTM Cell Switch MAC CE and the tag-Id-ptr configured by RRC signalling for the TCI stated ID, however, there is no need to include the TCI state ID in the new MAC CE, and it is straightforward using 1 bit to indicate the TAG information in this case.

Proposal 5: TAC field is included in the MAC CE for CLTM early TA acquisition, and the value couldn’t be FFF.

Proposal 6: TAG information field is included in the MAC CE for CLTM early TA acquisition.

Furthermore, in Rel-18 LTM, the candidate cell may reserve the dedicated CG resource for a UE for the first UL data transmission during the RACH-less LTM. Since the LTM is subsequent, the CG resource will be reserved for a long time for the UE and couldn’t be used anymore. To reduce the resource wastage, we made the following agreements in Rel-18:

- Observation: In cases for which it is desired that CG used for LTM is not used further once the UE has made the cell its new serving cell, it is assumed that the network could release Type1 CG resource on LTM completion (existing functionality) - P4: RAN2 to define the UE behaviour on the R18 CG for RACH-less LTM, if it is not released by NW after LTM completion: Option 1: UE stops using those CG (FFS on the spec impact/wording details);

However, only when the candidate cell becomes current serving cell, its CG resources could be released or re-assigned to be used, and the CG resources in other candidate cells will always be reserved.

Observation 5: The CG handling method could only save the resources in the target cell which becomes serving cell.

It could be observed there is CG resource wastage issue in Rel-19 for CLTM. Considering the new MAC CE, i.e., the LTM Candidate Timing Advance Command MAC CE, will be introduced for early TA acquisition, we could introduce the CG resource information in the new MAC CE. In this case, the candidate cell will only assign the CG resource to UEs when it receives the preamble for early TA and release the CG resource after UE completes the LTM. In this way the candidate cell won’t reserve the dedicated CG resource for a UE forever and the resources could be saved.

Regarding the detail content of the CG resource related information in the MAC CE, we think the DCI format 0_0 which used for the scheduling of PUSCH could be used as baseline, more specifically, the following information could be considered to be included in the LTM Candidate Timing Advance Command MAC CE:

- Frequency domain resource assignment

- Time domain resource assignment

- Frequency hopping flag

- Modulation and coding scheme

- Redundancy version

- HARQ process number

- TPC command

Someone may argue that the size of the MAC CE will be large if included the above CG related information, to resolve this concern, we could consider to include the CG resource in LTM candidate configuration transmitted via RRC message as legacy while the CG resource won’t be reserved by candidate cells for this UE and using one indication in the MAC CE to notify whether the CG resource is available for this UE. In this way, only 1 bit is enough. Anyway, both methods could work well, we could further discuss it later on the CG resource related information.

Proposal 7: CG resources related information is included in the MAC CE for CLTM early TA acquisition, detail is FFS.

  - CLTM supervisor timer

For completion phase, there are other question to be answered, e.g.

- Whether/how to define the supervisor timer for conditional LTM?

Basically, we understand we should reuse the legacy LTM as much as possible. LTM supervisor timer was discussed in Rel-18 and T304 is finally extended to cover the LTM case, as follows[5]:

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T304 | <omitted...> Also, for the MCG and SCG upon an indication from lower layer that an LTM cell switch procedure is triggered and, for the MCG, upon performing an LTM cell switch procedure following cell selection performed while timer T311 is running. | Upon successful completion of random access on the corresponding SpCell. Upon receiving an indication from lower layers of successful completion of Rach-less handover, or upon receiving an indication from lower layers of successful completion of an LTM RACH-less cell switch. <omitted...> | For T304 of MCG, in case of the handover from NR or intra-NR handover, or path switch from a L2 U2N Relay UE to a NR cell, or an LTM cell switch procedure, initiate the RRC re-establishment procedure; In case of handover to NR, perform the actions defined in the specifications applicable for the source RAT. If any DAPS bearer is configured and if there is no RLF in source PCell, initiate the failure information procedure. <omitted...> |

In our understanding, T304 can be simply reused for conditional LTM, i.e. same UE behavior as R18 LTM.

For the stop condition of T304, which is how to determine the conditional LTM is successfully complete, it is agreed in RAN2 #127bis meeting that

The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

which means that the stop condition of legacy LTM can also be reused, as follows[6]:

8. The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell. If the UE has performed a RA procedure in step 7 the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data.

Generally speaking, almost all the UE behaviors for conditional LTM execution and completion are quite similar, because the main difference for conditional LTM and legacy LTM is on the configuration and evaluation phase before LTM cell switch is triggered.

Therefore, we propose:

Proposal 8: T304 is reused as the supervisor timer for conditional LTM, i.e. the timer starts upon conditional LTM execution, stops upon successful conditional LTM procedure completion, and expiry with RRC re-establishment procedure by the UE.

- Conclusion

In this contribution, we discussed RACH-less procedure when no CG at CLTM execution or CG transmission failure happens, TA handling for CLTM including the target cell and other candidate cells, content of MAC CE for CLTM early TA and the CLTM supervisor timer. We have the following Observations and Proposals:

For RACH-Less procedure with no CG at CLTM execution/CG transmission failure:

Observation 1: UE’s behaviour is not clear when there is no CG occasion at the time of CLTM execution.

Observation 2: The UE’s behaviour is not clear when RACH-less CLTM begins but the transmission on CG occasion fails.

Proposal 1: If the UE has a valid TA at the time of CLTM execution, but the TA timer is going to expire at the first available CG occasion, the UE should perform RACH-based CLTM.

Proposal 2: If UE performs RACH-LESS CLTM but transmission on first CG fails, the UE continue to perform RACH-LESS procedure if the TAT is still running at the next available CG occasion, otherwise the ongoing RACH-LESS CLTM is considered failure.

TA handling for CLTM:

Observation 3: It may be helpful in subsequent CLTM if TAs of other candidate cells are considered still valid upon CLTM execution.

Observation 4: RAN3 agreed target DU receives the TA values for subsequent LTM in Rel-18.

Proposal 3: RAN2 agree to clarify the running TAT (i.e. ltm-Candidate-TimeAlignmentTimer) to act as the timeAlignmentTimer associated with PTAG by a note in the MAC running CR.

Proposal 4: Valid TAs of other candidate cells are considered still valid upon CLTM execution until TAT expires or updated TA has been received from target cell.

MAC CE content for CLTM early TA

Observation 5: The CG handling method could only save the resources in the target cell which becomes serving cell.

Proposal 5: TAC field is included in the MAC CE for CLTM early TA acquisition, and the value couldn’t be FFF.

Proposal 6: TAG information field is included in the MAC CE for CLTM early TA acquisition.

Proposal 7: CG resources related information is included in the MAC CE for CLTM early TA acquisition, detail is FFS.

CLTM supervisor timer:

Proposal 8: T304 is reused as the supervisor timer for conditional LTM, i.e. the timer starts upon conditional LTM execution, stops upon successful conditional LTM procedure completion, and expiry with RRC re-establishment procedure by the UE.

- References

- RAN2 #129 meeting chair notes;

- R2-250xxx_Running MAC CR for Mob Ph4_v10_Rapp;

- RAN3 #123 meeting chair notes

- RAN2 #128meeting;

- 3GPP TS 38.331 V18.6.0 (2025-03);

- 3GPP TS 38.300 V18.6.0 (2025-03);