3GPP TSG-RAN WG2 Meeting #129bis	R2-2502163

Wuhan, China, 7th – 11th April 2025

Source:	vivo

Title:	Remaining issues on LCP enhancements for XR

Agenda Item:	8.7.4.1

Document for:	Discussion and Decision

- Introduction

In RAN2#129, there were the following conclusions regarding LCP enhancements [1]:

- Only one additional priority is configured to an LCH for LCP enhancement. - We keep an existing agreement (remaining time threshold is configured per LCH) - There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. - Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact - FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination - No additional PBR is needed for priority adjusted data - FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

According to the above agreements, there are the following open issues in relation to LCP enhancements:

- Whether/how the additional LCH priority impact the SR priority determination;

- Whether allow an LCH with LCH priority-adjusted data to be transmitted when Bj is negative.

This contribution will investigate the above open issues.

- Discussion

  - Impact on intra-UE prioritization

According to the conclusion from RAN2#129, it was agreed that the additional priority should be considered for UL grant prioritization. It is to be further discussed on whether/how to capture this in the MAC specification. As far as our understanding, whether the additional priority is applied for an LCH is determined during the LCP procedure according to the LCH priority-adjusted data existence for the LCH, whether the additional priority is applied for an LCH for the corresponding UL grant priority determination should also be based on the corresponding LCP procedure. We have noticed that this has already been well captured in the existing running CR of MAC:

- Whether additional priority of an LCH is applied for the corresponding UL grant in the intra-UE prioritization procedure is determined according to the corresponding LCP procedure of the LCH, which is already reflected in the current running CR.

According to the current RRC specification, only one SR-PUCCH resource can be configured per LCH. Under such condition, if a UE prioritizes the SR transmission according to additional priority in case of overlapping its SR-PUCCH and a UL grant for other data, the gNB cannot differentiate whether the SR is triggered due to LCH priority-adjusted data or not based on the SR reception. Another potential solution is to introduce an additional SR-PUCCH configuration for an LCH used for SR transmission triggered by LCH priority-adjusted data, so that the gNB can determine whether the SR is triggered by LCH priority-adjusted data or not. However, the feasibility of additional SR-PUCCH is questionable as most probably there are only very occasional SRs triggered by LCH priority-adjusted data for an LCH and the utility of the additional SR-PUCCH resource is very low. Considering this, the overhead for introducing an additional SR-PUCCH resource per LCH merely for LCH priority-adjusted data is not necessary.

- With single SR-PUCCH resource configured per LCH, the gNB cannot determine whether a received SR is triggered due to LCH priority-adjusted data or not.

- If an additional SR-PUCCH is configured for an LCH just because occasionally LCH priority-adjusted data transmission, the overhead is not necessary.

Based on the above discussion, it can be concluded that it is meaningless to apply the additional priority of an LCH for the corresponding SR priority determination:

- Additional LCH priority does not impact SR priority determination.

  - LCH data transmission with negative Bj

For a first LCH with LCH priority-adjusted data, the gNB may issue UL grants to speed up the LCH priority-adjusted data transmissions. After receiving the UL grants, the UE may prioritize the resource allocation for the first LCH according to the additional priority of this LCH. However, when the UE has a large amount of LCH priority-adjusted data for the first LCH, after the UE has allocated radio resource up to Bj for the first LCH, there may be still remaining LCH priority-adjusted data for the first LCH. According to the current LCP procedure, the UE will allocate the remaining resource for the remaining LCHs of lower priorities. After radio resource allocation for the remaining LCHs, there may be no or not enough remaining resources to transmit all the remaining LCH priority-adjusted data of the first LCH.

To speed up the LCH priority-adjusted data transmission for an LCH, the UE can be temporarily allowed to allocate radio resources for LCH priority-adjusted data when Bj is negative, this results in the following two impacts:

- In one aspect, when there is LCH priority-adjusted data for an LCH, the LCH priority-adjusted data of the LCH can be allocated with more radio resources in time by allocating the radio resource even when Bj is negative, which reduces the risk of packet discarding for XR data; in the meanwhile, less resources will be allocated for the remaining LCHs of lower priorities.

- In the other aspect, when there is no LCH priority-adjusted data for an LCH and its corresponding Bj is negative, the remaining LCHs of lower priority will be prioritized for resource allocation and those remaining LCHs of lower priorities but with positive Bj(s) will be allocated with more resources, i.e. these remaining LCHs of lower priorities will be compensated when the congestion for XR service is relieved.

- By temporarily allowing LCH priority-adjusted data transmission when Bj is negative, more radio resources can be timely allocated for LCH priority-adjusted data transmission.

- The LCHs of lower priority will be compensated with more radio resources when there is no LCH priority-adjusted data for LCHs with negative Bj.

Based on the above discussion, temporary allowance of LCH priority-adjusted data transmission for an LCH with negative Bj can help timely LCH priority-adjusted data transmission for the LCH without seriously negative impact on the radio resource allocation for the LCHs of lower priorities. We propose:

- Support to temporarily allow UE to transmit LCH priority-adjusted data for an LCH when its Bj is negative.

To avoid the situation that a huge amount of LCH priority-adjusted data for an LCH consumes all radio resources and no radio resource is available for LCHs of lower priorities in a long period, a per-LCH negative bound for Bj may be introduced for LCH priority-adjusted data transmission. When the negative Bj bound is configured for an LCH, the UE is only allowed to allocate radio resource for the LCH priority-adjusted data of the LCH only when the corresponding Bj is above the negative Bj bound. In such way, the starvation risk of LCHs with lower priorities can be avoided.

- Introduce a configurable negative bound for Bj per LCH for LCH priority-adjusted data transmission.

- If a negative Bj bound is configured for an LCH, the UE can transmit the LCH priority-adjusted data of the LCH when the corresponding Bj is above its negative Bj bound.

- Conclusion

In this contribution, we have discussed the remaining issues of the LCP enhancements and potential solutions. Based on the discussions, there are the following observations:

Observation 1	With single SR-PUCCH resource configured per LCH, the gNB cannot determine whether a received SR is triggered due to LCH priority-adjusted data or not.

Observation 2	If an additional SR-PUCCH is configured for an LCH just because occasionally LCH priority-adjusted data transmission, the overhead is not necessary.

Observation 3	By temporarily allowing LCH priority-adjusted data transmission when Bj is negative, more radio resources can be timely allocated for LCH priority-adjusted data transmission.

Observation 4	The LCHs of lower priority will be compensated with more radio resources when there is no LCH priority-adjusted data for LCHs with negative Bj.

Based on the above discussions and the observations, we have the following proposals regarding the remaining open issues of LCP enhancements:

Proposal 1	Whether additional priority of an LCH is applied for the corresponding UL grant in the intra-UE prioritization procedure is determined according to the corresponding LCP procedure of the LCH, which is already reflected in the current running CR.

Proposal 2	Additional LCH priority does not impact SR priority determination.

Proposal 3	Support to temporarily allow UE to transmit LCH priority-adjusted data for an LCH when its Bj is negative.

Proposal 4	Introduce a configurable negative bound for Bj per LCH for LCH priority-adjusted data transmission.

Proposal 5	If a negative Bj bound is configured for an LCH, the UE can transmit the LCH priority-adjusted data of the LCH when the corresponding Bj is above its negative Bj bound.

- References

- RAN2#129 Chair notes