3GPP TSG-RAN WG2 Meeting #129bis	R2-2502164

Wuhan, China, 7th – 11th April 2025

Source:	vivo

Title:	Remaining issues on DSR enhancements for XR

Agenda Item:	8.7.4.2

Document for:	Discussion and Decision

- Introduction

In RAN2#129 [1], there were following agreements regarding DSR enhancements:

- One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE. - FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. - We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size. - Different LCGs may be configured with different number of reporting thresholds. - If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold. - Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold). - Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

In this contribution, the open issues listed in the above agreements and the left remaining issues not summarized in the above agreements, are to be discussed, comprising:

- DSR capability;

- DSR format selection;

- Data volume calculation for PDCP/RLC control PDUs and retransmitted data.

- Discussion

  - DSR capability

In previous meetings, there is an open issue on whether a UE with ehanced DSR capability should also support legacy DSR, there are the following options:

- Option 1: A UE with enhanced DSR capability shall also support legacy DSR capability;

- Option 2: A UE with enhanced DSR capability does not have to support legacy DSR capability.

In RAN2#129, there are the following agreements regarding enhanced DSR:

- Different LCGs may be configured with different number of reporting thresholds. - If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold.

According to the above agreements, a UE with enhanced DSR capability can be configured with respective number of reporting thresholds for different LCGs. For instance, one LCG of a UE with enhanced DSR capability, can be configured with single reporting threshold while another LCG of the same UE can be configured with multiple reporting thresholds. This means that the legacy DSR of single pair of remaining time/buffer size per LCG can also be supported with enhanced DSR by configuring single configured reporting threshold for an LCG. In such sense, a UE with enhanced DSR capability has not to also support legacy DSR capability and whether a UE with enhanced DSR capability also support legacy DSR capability should be left to UE implementation.

- It should be left to UE implementation whether a UE supporting enhanced DSR also supports legacy DSR.

  - DSR format selection

By redefining the R bit of legacy DSR MAC CE to be extension bit in enhanced DSR format, the DSR with enhanced DSR format has the same size as the DSR with legacy DSR format for an LCG having just single pair of remaining time/buffer size to be reported. That is, there is no no additional benfit for a UE to conditionally select between legacy DSR format and enhanced DSR format. To avoid the additional complexity for DSR generation and transmission in UE side, we should keep the DSR format selection procedure as simple as possible. Hence, we propose:

- UE should always select enhanced DSR format when at least one LCG is configured with reporting threshold(s).

  - Remaining issues of data volume calculation

To simplify the description, the term “delay-reporting data”, which is used to refer to the data associated with certain DSR reporting threshold of an LCG in the running CR of PDCP, is used in the following discussions. In the chair notes of RAN2#129, there were the following issues on the data volume calculation with respect to the PDCP/RLC control PDUs and the retransmitted data:

R2-2501348 Report of [AT129][504][XR] Data volume calculation for DSR LG Electronics discussion Rel-19 NR_XR_Ph3-Core - Noted Proposal: Choose one of the followings. Option 1. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about the issue of LCH only having the control PDU and retransmitted data at the next meeting. Option 2. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest configured reporting threshold. Option 3. PDCP considers Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about how RLC considers Control PDU and retransmitted data at the next meeting.

As far as our understanding, there are two open issues in the following with respect to the above 3 options:

- Issue 1: Whether PDCP/RLC entity counts PDCP/RLC Control PDUs and retransmitted data into the delay-reporting data volume of the shortest reporting threshold which is triggered by LCH(s) of a LCG or that of the shorstest configured reporting threshold for the LCG.

- Issue 2: How to handle the delay-reporting data volume calculation for a LCH with only PDCP/RLC ontrol PDUs and the retransmitted data.

If the PDCP/RLC control PDUs and retransmitted data are counted into the delay-reporting data volume of the shortest reporting threshold which is triggered by the shorstest remaining time of the packets of LCHs of a LCG, based on the received delay-reporting data volume associated with the shortest remaining time the gNB can allocate sufficient radio resources for PDCP/RLC control PDUs, retransmitted data and the delay-reporting data associated with the shortest reporting threshold in the DSR for the LCG, without any negative impact on the UE experience.

If the PDCP/RLC control PDUs and retransmitted data are counted into the delay-reporting data volume of the shortest configured reporting threshold for a LCG, there are following two affects which needs to be further addressed:

- In one aspect, it can cause an additional pair of remaining time/buffer size in DSR, if no other delay-reporting data corresponding to the shortest configured reporting threshold for the LCG with pending DSR.

- In another aspect, there is an issue on how to determine the remaining time corresponding to the shortest configured reporting threshold if there is no other delay-reporting data associated the shortest configured reporting threshold for a LCG, since there is no remaining time for PDCP/RLC control PDUs and the remaining time(s) of retransmitted data may be larger than the shortest configured reporting threshold for the LCG.

Based on the above discussion, it can be derived that Option 1 is good enough and simpler than Option 2, while Option 3 is not concrete yet. Hence, we propose:

- RAN2 to adopt Option 1, i.e. both PDCP/RLC control PDUs and the retransmitted data should be counted into the delay-reporting data volume corresponding to the the shortest reporting threshold included in the DSR.

Regarding the LCH with only PDCP/RLC control PDUs and retransmitted data (i.e. without other delay-reporting data), it is preferred that these PDCP/RLC control PDUs and retransmitted data are treated in the same way as the PDCP/RLC control PDUs and retransmitted data of the LCH with delay-reporting data to ensure that these PDCP/RLC control PDUs and retransmitted data can also get sufficient resources in time.

- The PDCP/RLC control PDUs and retransmitted data of an LCH without other delay-reporting data should be counted into the data volume of the the shortest reporting threshold included in the DSR.

- Conclusion

Based on the above discussions and the observations, we have the following proposals:

Proposal 1	It should be left to UE implementation whether a UE supporting enhanced DSR also supports legacy DSR.

Proposal 2	UE should always select enhanced DSR format when at least one LCG is configured with reporting threshold(s).

Proposal 3	RAN2 to adopt Option 1, i.e. both PDCP/RLC control PDUs and the retransmitted data should be counted into the delay-reporting data volume corresponding to the the shortest reporting threshold included in the DSR.

Proposal 4	The PDCP/RLC control PDUs and retransmitted data of an LCH without other delay-reporting data should be counted into the data volume of the the shortest reporting threshold included in the DSR.

References

- RAN2-129, Chair notes