3GPP TSG-RAN WG2 Meeting #129bis													     R2-2502177

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.3.5

Source:	Apple

Title:	On LCM for AI/ML mobility

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we provide our views on LCM for AI/ML mobility.

# 2   	Discussion

Before diving into the depth of LCM discussion, we should remember that LCM which is currently being defined is a very generic and forward-looking framework. It should cater not only to the cases which where known at the time it is being defined, but also to future cases. As such, it is a rather inclusive framework, covering many functions – most of which are not necessarily relevant to each AI/ML functionality. This is true in general and definitely for AI/ML mobility in particular.

Observation 1: we should not assume that all LCM functionalities defined for the generic and forward-looking LCM framework are necessarily relevant to AI/ML mobility.

With this in mind, we should not assume that all the many LCM functionalities envisioned in the grand LCM framework are automatically relevant to AI/ML mobility. Each LCM aspect should be studied individually to determine whether it is relevant to AI/ML mobility.

This is what the SID [2] in fact mandates, as is evident if one reads the relevant study item objective:

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

That is, the SID emphasizes that the LCM part is only potential and only the LCM parts that have already been agreed in the AI/ML air interface WID shall be considered.

Proposal 1: For each and every LCM function, we should study whether it is relevant to AI/ML mobility; many LCM functions are likely to be irrelevant. Furthermore, only the LCM parts that have already been agreed in the AI/ML air interface WID shall be considered.

TR 38.843 [1] originally envisioned two LCM flavors: functionality-based and model-based. Since only the functionality-based LCM is being considered for AI/ML for air interface, it is obvious that the same should apply to AI/ML mobility. Obvious or not, this should be captured in the TR.

Proposal 2: only functionality-based LCM is considered for AI/ML mobility.

With this in mind, we should now inspect which LCM functions indeed have been already agreed in the context of AI/ML for air interface, in order to determine which subset of those functions may be relevant for AI/ML mobility.

The following functions are part of the AI/ML air interface WID [3]:

- Functionality selection, activation, deactivation, switching, fallback

- Model training and inference

- Performance monitoring

- Data collection for NW-sided models

- Signalling mechanism of applicable functionalities

Not everything from the list above have already been agreed, though. Here is our list of the LCM functionalities which have more or less have been agreed (at least on the high level) in the context of AI/ML for air interface and therefore can be studied in the current SI:

- Functionality activation/deactivation and fallback

- Applicability reporting

- Data collection for network-sided models

- UE-sided model training

Proposal 3: only the following LCM functions are considered: functionality activation/deactivation and fallback, applicability reporting, data collection for network-sided models, and UE-sided model training (for now, may be revised later depending on the progress in the AI/ML air interface).

A final high-level point, before we proceed to discuss the details. Many LCM functions depend on RAN4. One such example is monitoring – if RAN4 manage to define AI/ML requirements, for which there would be a test case that a UE can pass, then there would be no motivation to deploy monitoring for such features. That’s the nature of requirements and certifications – if a UE passes a test case, then the network can be assured about the UE behavior and no monitoring is needed (which would incur significant unnecessary effort and load – both for the network and for the UE).

Observation 2: many LCM functions, with monitoring being one example, depend on RAN4 progress and may or may not be required.

Proposal 4: monitoring discussion is put on hold pending RAN4 progress.

### 2.1 	LCM for UE-sided RRM prediction

I suppose we can define activation/deactivation and applicability signaling for UE-sided RRM prediction and I’d be surprised if people don’t propose this. That being said, I wonder whether such activation functionality would ever be used. The idea here would be, I guess, that the network would, in some cases, activate RRM prediction in a UE. This raises many questions, such as:

- Why would the network want to activate prediction?

- Even if it does, how the network would decide when and where to activate it?

The motivation for the network to even use this and the mechanisms to decide when and where are completely obscure at the moment. We are willing to hear arguments in favor, but such arguments should provide answers to the questions raised above.

Furthermore, this also depends on RAN4 – if, for instance, RAN4 come up with requirements for UE-sided RRM prediction which are equivalent to the “legacy” measurement requirements, then whether/when to use prediction becomes a UE internal implementation matter and the UE should be able to use prediction or real measurement based on its own considerations – as long as the requirements are met, the network shouldn’t be concerned with such internal implementation matters.

In the context of this discussion we should remember the conclusions from the model generalization evaluation, where we’ve determined that there are no generalization issues neither in frequency, nor in speed, not in cell configuration cases. See [4] and [5] for details.

Observation 3: we’ve determined that there are no generalization issues neither in frequency, nor in speed, not in cell configuration cases; this indications, among other things, that very little in terms of LCM is technically needed.

Proposal 5: no LCM functionality is needed for UE-sided RRM prediction. A UE may decide to use predictions or real measurements, as long as its implementation satisfies the RAN4 requirements. This may be revisited when RAN4 requirements become clear.

### 2.2	LCM for network-sided RRM prediction

The inference part of the network-sided RRM prediction probably requires no standardization impact. Even today the network may use all sorts of considerations (which may include AI/ML) when scheduling measurements and performing mobility decisions. We don’t see what changes.

Observation 4: inference (activation/deactivation, etc) of network-sided RRM prediction has no standards impact.

However, the data collection part is a completely different story. From a UE (and more importantly, from a user) perspective, this is substantial difference between providing measurements for mobility decisions (which is required for the whole system to operate) and providing measurements for the purpose of a network AI/ML model training. Here are a few important differences to highlight (there are more):

- While a UE should always provide measurements needed for mobility (which is in the interest of the UE, the network and the user), this is not necessarily the case with measurements for network sided model training. For example, when a UE battery is low, it should not be required (or indeed expected) to provide measurements for training.

- Similar consideration is valid for the case when a UE experiences high load – data transfer and measurements for mobility should obviously be prioritized over measurements for network model training.

- Last, but absolutely not least, the user should be aware that their UE is providing measurements which would not be used once (as is the case for mobility), but would be stored, processed and potentially kept for a significant time – either as “raw” measurements or as part of a trained model. We should also be mindful that this is more than just a standards issue – in some jurisdictions, e.g. Europe, this may fall under the GDPR regulation.

Observation 5: reporting measurements for mobility and for network model training are radically different use cases, which have different implications on UE implementation, user privacy and indeed regulations.

Therefore, at the very least, we should ensure that when the network is collecting measurements for model training, it should have obtained user consent for this.

Proposal 6: user consent is required for network-sided data collection.

While user consent should cover at least the regulatory concerns, there are still technical concerns we’ve mentioned above (UE battery status, load, etc). For this, the UE should be aware (through signalling) whether it has been configured to report measurements for mobility or for network model training.

Proposal 7: UE should be aware whether the measurements configured are for the purpose of mobility or training (of a network sided model).

### 2.3	LCM for measurement event prediction

The situation with measurement event prediction is different, here it is actually quite possible that at least activation/deactivation and applicability reporting will be needed.

However, currently it is hard to discuss these details, as the actual functionality to implement measurement event prediction configuration and reporting is not yet clear. This is being discussed in our companion contribution [6], where we cover all standards impacts aspects other than LCM. So we are generally sympathetic with this part of the objective, but it would be much more prudent to continue this discussion once the standards impacts of the measurement event prediction functionality are clear.

Observation 6: it would be much more prudent to continue the discussion on LCM for measurement event prediction, once the standards impacts of the measurement event prediction functionality are clear.

Proposal 8: functionality activation and deactivation, as well as applicability reporting, are needed for measurement event prediction.

# 3	Proposals

Observation 1: we should not assume that all LCM functionalities defined for the generic and forward-looking LCM framework are necessarily relevant to AI/ML mobility.

Proposal 1: For each and every LCM function, we should study whether it is relevant to AI/ML mobility; many LCM functions are likely to be irrelevant. Furthermore, only the LCM parts that have already been agreed in the AI/ML air interface WID shall be considered.

Proposal 2: only functionality-based LCM is considered for AI/ML mobility.

Proposal 3: only the following LCM functions are considered: functionality activation/deactivation and fallback, applicability reporting, data collection for network-sided models, and UE-sided model training (for now, may be revised later depending on the progress in the AI/ML air interface).

Observation 2: many LCM functions, with monitoring being one example, depend on RAN4 progress and may or may not be required.

Proposal 4: monitoring discussion is put on hold pending RAN4 progress.

Observation 3: we’ve determined that there are no generalization issues neither in frequency, nor in speed, not in cell configuration cases; this indications, among other things, that very little in terms of LCM is technically needed.

Proposal 5: no LCM functionality is needed for UE-sided RRM prediction. A UE may decide to use predictions or real measurements, as long as its implementation satisfies the RAN4 requirements. This may be revisited when RAN4 requirements become clear.

Observation 4: inference (activation/deactivation, etc) of network-sided RRM prediction has no standards impact.

Observation 5: reporting measurements for mobility and for network model training are radically different use cases, which have different implications on UE implementation, user privacy and indeed regulations.

Proposal 6: user consent is required for network-sided data collection.

Proposal 7: UE should be aware whether the measurements configured are for the purpose of mobility or training (of a network sided model).

Observation 6: it would be much more prudent to continue the discussion on LCM for measurement event prediction, once the standards impacts of the measurement event prediction functionality are clear.

Proposal 8: functionality activation and deactivation, as well as applicability reporting, are needed for measurement event prediction.

# 5	References

[1]				TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[2]				RP-240082 Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR

[3]				RP-243244 Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

[4]				RAN2#129 meeting minutes

[5]				R2-2502176 On generalization across cell configurations Apple

[6]				R2-2502179 On standards impacts other than LCM Apple