3GPP TSG-RAN WG2 Meeting #129bis	R2-2502180

Wuhan, China, 7 – 11 April 2025

Agenda item:	8.7.4.1

Source:	Apple

Title:	Discussions on Delay-based LCP Enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

For LCH priority adjustment based on packet remaining time, RAN2 #129 [1] has made the following agreements:

Agreements on LCP enhancements - Only one additional priority is configured to an LCH for LCP enhancement. - We keep an existing agreement (remaining time threshold is configured per LCH) - There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. - Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact - FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination - No additional PBR is needed for priority adjusted data - FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

Based on the highlighted parts above, this paper aims to address the following issues:

- Whether/how priority adjustment affects intra-UE prioritization in specifications?

- How to deal with the negative/small Bj of LCHs with upgraded priority ?

Furthermore, we would also like to point out some potential issues related to priority fall back, with considerations of PDU Set discarding.

# Discussions

## Specification Impacts to Intra-UE Prioritization

Intra-UE prioritization was introduced in Rel-16 IIoT work item, wherein multiple UL grants may have overlapping resources in time domain, and some mechanisms are needed for the UE to determine which of these overlapping grants can be prioritized for UL transmission. Furthermore, it has been extended to the cases where SR-PUCCH is also involved in resource conflict. According to TS 38.321, such determination is based on the “UL grant” priority, which reflects to the highest priority of the LCHs that can be (or already be) multiplexed into such a grant. Furthermore, whether a SR can be prioritized and transmitted is also based on the priority of LCH that triggers this SR.

TS 38.321, Clause 5.4.1: For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR. TS 38.321, Clause 5.4.4: 3> if the MAC entity is configured with lch-basedPrioritization, and the PUCCH resource for the SR transmission occasion does not overlap with the PUSCH duration of an uplink grant received in a Random Access Response or with the PUSCH duration of an uplink grant addressed to Temporary C-RNTI or with the PUSCH duration of a MSGA payload, and the PUCCH resource for the SR transmission occasion for the pending SR triggered as specified in clause 5.4.5 overlaps with any other UL-SCH resource(s), and the physical layer can signal the SR on one valid PUCCH resource for SR, and the priority of the logical channel that triggered SR is higher than the priority of the uplink grant(s) for any UL-SCH resource(s) where the uplink grant was not already de-prioritized and its simultaneous transmission with the SR is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCHgroups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, and the priority of the uplink grant is determined as specified in clause 5.4.1; or

Thus, for intra-UE prioritization, according to the current specification the determination of UL grant priority and SR priority are readily linked to the LCH priority. On the other hand, if delay-based LCP enhancement is configured, a logical channel can switch its priority between the default priority and the additional priority from time to time, based on the presence/absence of data with remaining time smaller than a threshold. The UE can switch the priority any time, even before the UL grant is received.

Thus, to apply the adjusted priority to intra-UE prioritization, the only confusion is that, whether “priority of logical channel” (highlighted in green above) in intra-UE prioritization procedures is only restricted to the default priority, or if it can be referred to the priority level that the logical channel is currently using (either the default priority or the additional priority).

Observation 1: To apply the adjusted LCH priority for intra-UE prioritization, the specification only needs to clarify that “priority of the logical channels” for UL grant and SR priority determination in intra-UE prioritization procedures is referring to the priority level that a logical channel is currently using, instead of the default logical channel priority.

To resolve the issue, we think a simple NOTE in TS 38.321 is sufficient, which clarifies that “priority of the logical channels” in the intra-UE prioritization procedure is referring to the prevailing priority of a logical channel, which could be either the default priority or the additional priority.

Proposal 1: To apply delay-based LCH priority adjustment in intra-UE prioritization, it is sufficient to add the following NOTE to TS 38.321:

- NOTE: The priority of the logical channels to be used in the lch-basedPriorization procedures is referred to as the prevailing logical channel priority.

## Issues on Bj of LCHs with Adjusted Priorities

According to the LCP procedure specified in TS 38.321, each LCH maintains a Bj value, and the LCHs with negative Bj values cannot be selected for resource allocation. Thus, even if the priority of a LCH can be adjusted based on buffer delay, we cannot guarantee that the data can be transmitted from this LCH due to Bj. In light of this, RAN2 has agreed to further study whether it is allowed to serve LCHs with negative Bj in the LCP procedure.

From our perspectives, we think the Bj value of a LCH may be in one of the following three situations:

- Case 1: Bj is negative

  - In this case, the LCH cannot be selected for resource allocation at all, even if the LCH priority is adjusted.

- Case 2: Bj is positive but quite small

  - In this case, the LCH can be selected for resource allocation, but it may not be able to allocate much of the urgent data in the buffer. With considerations of PDU Sets in XR, the amount of urgent data in the buffer could be high, and so a small Bj value may be insufficient to serve the purpose.

- Case 3: Bj is positive and sufficiently large

  - In this case, the LCH can be selected for resource allocation, and a fair amount of urgent data can be transmitted on the grant. No special handling is needed for LCHs in such situation at all.

With the above considerations, we think if RAN2 really tries to resolve the problems with Bj, it is desirable to have a more completed and unified solution that can handle both Case 1 and Case 2.  If the solution only focusses on Case 1, it may be quite unfair for the LCHs that are currently in Case 2.

Observation 2: If the issues with Bj is to be resolved by RAN2 in Rel-19, it is better to have a completed and unified solution that can handle both of the following cases:

- Bj is negative, and

- Bj is positive but small.

The solution can be very simple. Basically, we think the gNB can pre-configure a Temporary Bj Value (denoted as Bj_Temp) for the LCH via RRC. Then, before the resource allocation procedure is started for a received UL grant, the UE simply set the Bj value of a LCH with adjusted priority based on the following logic:

Bj = max(Bj, Bj_Temp)

That is, the Bj of the LCH is simply set by taking the larger value between the current Bj and the configured Temporary Bj. If the current Bj value is anyway quite high, the UE will continue to use this current Bj for LCP without altering the value at all. Conversely, if the current Bj value is either negative or quite low, the UE will change the Bj value to a configured Bj_Temp value that is configured to be high enough. Note that Bj_Temp is always positive and the value of which is up to network implementation, where the factors such as delay requirements, traffic characteristics, and fairness could be taken into account by the network.

This solution is quite straightforward and comprehensive, which simultaneously tackles both problematic cases with negative Bj and small Bj. At the same time, it clarifies the alternative Bj value that the UE could use for a LCH if needed (i.e. Bj_Temp).

Proposal 2: For a LCH with upgraded priority, the Bj of which is set based on Bj = max(Bj, Bj_Temp) before the resource allocation procedure, where Bj_Temp is pre-configured by the network.

## Considerations of PDU Sets for Priority Fallback

During RAN2 #128 [2], it has been agreed that in the second round of LCP procedure, the adjusted priority may fall back to the default priority if the UE is capable to do so:

Agreements on LCP prioritization - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

To reflect this agreement, in the current running CR of TS 38.321, it is assumed that whether the priority can fallback is determined solely based on the remaining value of PDCP discard timers associated to the data still in the buffer:

…… 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain: 3> if a logical channel has applied additionalPriority above; and 3> if none of its remaining data available for this new transmission has a running PDCP discardTimer whose remaining value, evaluated at the time of the first symbol of this new transmission, is below its priorityAdjustmentThreshold; and 3> the MAC entity supports lch-PriorityAdjustmentRound2: 4> apply priority of this logical channel; ……

However, we think there may be cases where the buffered packets are belonging to the same PDU Set but with different remaining times. This is evident from the discussions relevant to DSR. Thus, even if none of the remaining buffered data available has a remaining value of PDCP discard timer below priorityAdjustmentThreshold, if PDU Set discarding is configured and some remaining data in the buffer belong to a PDU Set that is already considered urgent, it is desirable to continue using the additional priority rather than fall back to the default priority. Otherwise, LCH priority adjustment may become useless from user experience perspective, as Application cannot make use of a PDU Set if only some of its packets are delivered on time. The situation is illustrated in Figure 1:

Figure 1 An illustration of a problematic case with LCH priority fallback, where PDU Set discarding is also configured.

Since all packets belonging to the same PDU Set should be considered as delay-critical in DSR with considerations of PDU Set Discarding, we think the same principle should be applied for LCH priority adjustment, such that the features of DSR and LCH priority adjustment can truly complement each other when they are configured together. Based on this, we think LCH priority fallback should also take PDU Set integrity into account.

Proposal 3: If PDU Set Discarding is configured, when there is still other data in the buffer that belong to the same PDU Set that is already considered urgent, the LCH does not fall back to the default priority even if the remaining time of these data is larger than priorityAdjustmentThreshold.

# Conclusions

This paper discusses some of our views on the remaining issues related to LCH priority adjustment. We have the following observations and proposals:

Observation 1: To apply the adjusted LCH priority for intra-UE prioritization, the specification only needs to clarify that “priority of the logical channels” for UL grant and SR priority determination in intra-UE prioritization procedures is referring to the priority level that a logical channel is currently using, instead of the default logical channel priority.

Observation 2: If the issues with Bj is to be resolved by RAN2 in Rel-19, it is better to have a completed and unified solution that can handle both of the following cases:

- Bj is negative, and

- Bj is positive but small.

Proposal 1: To apply delay-based LCH priority adjustment in intra-UE prioritization, it is sufficient to add the following NOTE to TS 38.321:

- NOTE: The priority of the logical channels to be used in the lch-basedPriorization procedures is referred to as the prevailing logical channel priority.

Proposal 2: For a LCH with upgraded priority, the Bj of which is set based on Bj = max(Bj, Bj_Temp) before the resource allocation procedure, where Bj_Temp is pre-configured by the network.

Proposal 3: If PDU Set Discarding is configured, when there is still other data in the buffer that belong to the same PDU Set that is already considered urgent, the LCH does not fall back to the default priority even if the remaining time of these data is larger than priorityAdjustmentThreshold.

# References

- 3GPP RAN2 #129 Chair’s Notes, Athens, Greece, February 2025.

- 3GPP RAN2 #128 Chair’s Notes, Orlando, USA, November 2024.