3GPP TSG-RAN WG2 Meeting #129bis	R2-2502181

Wuhan, China, 7 – 11 April 2025

Agenda item:	8.7.4.2

Source:	Apple, CATT, LG Electronics, OPPO, Samsung, Xiaomi

Title:	On Data Volume Calculations for Rel-19 DSR

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

During RAN2 #129, we had an offline discussion [AT129][504] to decide which DSR reporting threshold should be associated to the PDCP/RLC control PDUs and retransmission data. Based on the comments, the rapporteur has proposed the following options that we can further discuss [1]:

Option 1. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about the issue of LCH only having the control PDU and retransmitted data at the next meeting. Option 2. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest configured reporting threshold. Option 3. PDCP considers Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about how RLC considers Control PDU and retransmitted data at the next meeting.

This paper presents some of our views on this open issue.

# Discussions

## Reporting Threshold Association

With Rel-19 DSR, each LCG may be configured with multiple reporting thresholds, along with an independent triggering threshold. When the UE calculates data volume for a triggered DSR, it needs to associate each of PDCP SDUs in the buffer of the LCG (including both data and control information) to one of the configured reporting thresholds. It is straightforward for data PDUs that have never been transmitted before, as their association to reporting threshold is simply based on their remaining time till discard timer expiry. The dependency of PDU Set may also be considered if PDU Set discarding is configured.

However, for PDCP/RLC control PDUs and data that is pending for retransmission, since they are not associated to the discard timer, it is questionable which of the reporting thresholds should be associated to them for buffer size calculation. As mentioned, three options are listed in [1] to deal with this issue. We think Option 3 can be first precluded, as we do not see any reasonable justification to have different behaviors between PDCP and RLC for data volume calculation. Hence, RAN2 should only focus on Option 1 and Option 2.

The key difference between Option 1 and Option 2 is that, in Option 1 the association for PDCP/RLC control PDUs and retransmission further depend on the smallest remaining time among PDCP SDUs for initial transmission. Such dependency may cause some additional UE complexity as it forces the UE to measure the remaining time of the buffered PDCP SDUs first, before it can decide how to associate PDCP/RLC control PDUs and retransmission. Moreover, such association may have to be updated from time to time, as the delay status of PDCP SDUs is changing as time elapses. Additionally, in cases where one or more LCHs of a LCG only have PDCP/RLC control PDUs and retransmission data, the DSR data volume calculation for the LCG would require multiple PDCP entities to coordinate and determine which reporting threshold should be chosen for the PDCP/RLC control PDUs and retransmission data, which results in a lot of complications due to inter-PDCP dependency.

On the other hand, Option 2 allows more flexible and simpler UE implementations, as the UE can directly associate PDCP/RLC control PDUs and retransmission to the smallest configured reporting threshold, without considering the delay status of other PDCP SDUs. There is no need of update for such association regardless of how the delay status of other PDCP SDUs evolve.

We have to note that, the UE may only have a strict timeline to associate the reporting threshold and calculate the corresponding buffer size. The DSR MAC CE contents need to reflect the latest buffer delay status when the PUSCH starts, and so the UE may only be able to generate the up-to-date DSR MAC CE contents within the short time interval between the UL grant reception and the PUSCH. Thus, it is preferred to minimize the implementation complexity and processing burdens at the UE side.

Furthermore, typically it is desirable to transmit PDCP/RLC control PDUs and retransmission data in a more urgent fashion. For examples:

- The PDCP control PDU for SN gap reporting was introduced to minimize processing delay at the receiver side, so it should be transmitted as soon as possible.

- The RLC status report is needed to advance the TX/RX windows, which should be delivered quickly especially when packet delay budget is small.

- In RLC-AM, retransmission is always prioritized over initial transmission upon submission of AMD PDU to the lower layer, which is crucial as we are also specifying faster RLC retransmission in Rel-19 XR.

With the considerations above, we think it is reasonable to always associate PDCP/RLC control PDUs and retransmission to the smallest configured reporting threshold, which facilitates the network to schedule the uplink resource in a timely manner to accommodate them.

Proposal 1: For Rel-19 DSR data volume calculation, all PDCP/RLC control PDUs and retransmission data should always be associated to the smallest configured reporting threshold, which is the i-th dsr-ReportingThreshold with i=1 (i.e. Option 2).

## Remaining Time Field Setting in DSR MAC CE

When it comes to remaining time field setting for each reporting threshold in the DSR MAC CE, apparently it is set based on the smallest remaining time till discard timer expiry among PDCP SDUs associated to this threshold. Nevertheless, with Proposal 1, it is unclear how the remaining time field should be set in the DSR MAC CE for the smallest configured reporting threshold, if the corresponding buffer size only contains PDCP/RLC control PDUs and/or retransmission data, and so none of the data is associated to a discard timer.

We think this should be a very rare case in practice and so no optimization is needed. A simple solution would be sufficient. For instances,  the remaining time field could be set as zero or the i-th dsr-ReportingThreshold with i=1 in such cases. RAN2 can further discuss among these alternatives:

- Alt. 1: Set the remaining time field in DSR MAC CE as 0;

- Alt. 2: Set the remaining time field in DSR MAC CE as the i-th dsr-ReportingThreshold with i=1 (i.e. the smallest reporting threshold);

- Alt. 3: Leave it to UE implementation.

Proposal 2: For cases where the calculated buffer size for the smallest configured reporting threshold only contains PDCP/RLC Control PDUs and/or retransmission data, RAN2 selects one of the following alternatives to set the remaining time field for this reporting threshold in the DSR MAC CE:

- Alt. 1: Set the remaining time field in DSR MAC CE as 0;

- Alt. 2: Set the remaining time field in DSR MAC CE as the i-th dsr-ReportingThreshold with i=1 (i.e. the smallest reporting threshold);

- Alt. 3: Leave it to UE implementation.

# Conclusions

This paper presented our views on the reporting threshold association for data volume calculation, with considerations of PDCP/RLC Control PDUs and retransmissions. We have the following proposals:

Proposal 1: For Rel-19 DSR data volume calculation, all PDCP/RLC control PDUs and retransmission data should always be associated to the smallest configured reporting threshold, which is the i-th dsr-ReportingThreshold with i=1 (i.e. Option 2).

Proposal 2: For cases where the calculated buffer size for the smallest configured reporting threshold only contains PDCP/RLC Control PDUs and/or retransmission data, RAN2 selects one of the following alternatives to set the remaining time field for this reporting threshold in the DSR MAC CE:

- Alt. 1: Set the remaining time field in DSR MAC CE as 0;

- Alt. 2: Set the remaining time field in DSR MAC CE as the i-th dsr-ReportingThreshold with i=1 (i.e. the smallest reporting threshold);

- Alt. 3: Leave it to UE implementation.

# References

- RP-2501348, Report of [AT129][504][XR] Data volume calculation for DSR, LG Electronics, RAN2 #129, Feb. 2025, Athens, Greece.