3GPP TSG-RAN WG2 Meeting #129bis	R2-2502189

Wuhan, China, 7th – 11th April 2025

Agenda item:	8.13.3

Source:	Apple, FirstNet, Ericsson, AT&T, Kyocera

Title:	Fast & Parallel RRC Establishment/Configuration for Multi-hop U2N relaying

WID/SID:	NR_SL_relay_multihop – Release 19

Document for:	Discussion and Decision

# 1 Introduction

For the baseline Control Plane Approach (Approach 1), each intermediate relay UE(s) needs to enter RRC_CONNECTED state individually before the remote UE can establish the end-to-end RRC connection successfully. This creates a significant latency for RRC connection setup & configuration.

In this paper, we analyze this latency problem and propose a solution to allow parallel RRC connection setup so that the remote UE and relay UEs can obtain necessary configuration(s) to support end-to-end traffic with reduced latency.

# 2 Discussion

The current baseline procedure largely reuses the legacy Layer-2 UE-to-NW relay design. Basically, an intermediate relay UE, upon the reception of a child node’s connection setup request, would act as similar to Layer-2 U2N remote UE and enter RRC_CONNECTED itself first. As a result, the forwarding operation (in either UL or DL) will be on hold until this intermediate relay UE has been configured successfully via dedicated RRC signalling by gNB.

This “on-hold” requirement increases the end-to-end setup latency significantly because each UE in the multi-hop path needs to wait for its next-hop node to enter RRC_CONNECTED first. Therefore, each of the UEs only enter RRC_CONNECTED in a sequential manner in the order of “Last Relay UE”  “2nd Intermediate Relay UE” ”1st Intermediate Relay UE”  Remote UE, as shown in Figure 1.

In the example depicted by Figure 1, we assume each of the UEs is in RRC_IDLE state. When the Layer 2 Remote UE sends its first RRC message (RRCSetupRequest) via SL-RLC0, the receiving relay UE in RRC_IDLE state must buffer this RRC message and send its own RRCSetupRequest message instead. This sequence repeats for every relay UE in the path. Therefore, when the last relay UE enters RRC_CONNECTED, its SUI message will only report its adjacent “remote UE”, which is the 2nd intermediate relay UE to gNB, and gNB will allocate a local ID for this intermediate relay and the corresponding configuration for traffic forwarding towards this intermediate relay UE. Until the RRC Connection setup is complete for all intermediate UEs, the gNB to be aware of the terminal remote UE’s request and is thus unable to provide supporting configurations for any end-to-end Uu SRB signalling. This will only be possible when the RRC_CONNECTED 1st intermediate relay UE reports SUI message to gNB and solicits the dedicated configuration(s) to forward L2 U2N remote UE’s Uu SRB0 message.

In summary, because the relay UEs must enter RRC_CONNECTED sequentially, the relay UEs send their respective SUI message(s) to solicit relay-related configurations (SRAP mapping, local ID allocation, PC5 Relay RLC channel configurations) sequentially. Hence, all RRC signalling occurs sequentially,  incurring significant delay which raises scalability concerns. For example, the latency and signalling overhead issues have been analysed in R2-2409968 [1] and it has been shown that signalling overhead increases by O(n2) with the number of n extra hops.

Observation 1	In baseline approach, the requirement for UEs entering RRC_CONNECTED sequentially causes significant delay, which is not scalable as the number of hops increases.

Figure 1: End-to-End RRC Connection Setup & configuration in Baseline procedure

The root cause of the above issue is the lack of fast end-to-end delivery of Uu RRC messages. As a result, gNB is not aware that there are multiple UEs requesting transition to RRC_CONNECTED to enable end-to-end UP traffic. Otherwise, gNB should be able to handle RRC connection establishment procedures in parallel instead of one-by-one.

In regards of feasibility of fast end-to-end delivery, it can be analysed separately for Uu hop and PC5 hops:

- For the PC5 hops between remote UE and relay UE or between relay UEs, the delivery can be accelerated because the PC5 link(s) are already established. SI and Paging forwarding is already feasible before the end-to-end RRC connection establishment. The e2e Uu RRC message is not forwarded promptly over these hops as the current “baseline” design does not consider allowing the SL-RLC channel(s) to be used for Uu RRC message forwarding.

- For the Uu hop between last relay UE and gNB, there is a real physical reason for the delay because the RRC_IDLE/RRC_INACTIVE relay UEs must initiate the RACH procedure and RRC establishment procedure to setup the Uu link. This is required for the relay UE to forward e2e message to gNB.

The above analysis demonstrates that the last relay UE would still need to enter RRC_CONNECTED first. However, it is possible to expedite RRC messages along the PC5 hops. This reduces latency by performing parallel connection setups for intermediate relay UEs and remote UE, once the last relay UE is in RRC_CONNECTED.

Observation 2	Latency reduction can be achieved by enabling parallel RRC connection setups of Remote UE and Intermediate Relay UE(s).

Therefore, we envision enhancement to the baseline design as shown in Figure 2.

Figure 2: Fast & Parallel Connection Setup/configuration procedure

In the example depicted by Figure 2, besides generating its own RRCSetupRequest and sending it to the adjacent relay UE via SL-RLC0, each intermediate relay UE also forwards the received RRCSetupRequest in SL-RLC0 to its parent relay UE, with all RRCSetupRequest messages buffered in the last relay UE while the last relay is in RRC_IDLE state. Upon receiving the set of RRCSetupRequest messages, the last relay UE will first enter RRC_CONNECTED state and receive configuration from the gNB so that the set of RRCSetupRequest messages can be forwarded to the gNB via the configured Uu Relay RLC channel. The gNB can then speed up the process by performing parallel RRC procedures occurring towards the intermediate relay UE(s) and remote UE. By supporting the “parallel RRC setup” option, the DL RRC messages in SRB0/1 are forwarded to intermediate relay UEs and remote UE(s) via the SL-RLC channel(s), so that RRC establishment procedures for all UEs will occur simultaneously, instead of occurring sequentially. In this way, the overall latency can be largely reduced and the e2e UP traffic forwarding can start much earlier.

The key to the above solution is to allow Uu SRB messages to be forwarded by RRC_IDLE/RRC_INACTIVE intermediate relay UEs before they enter RRC_CONNECTED state. This relies on the use of SL-RLC0 and/or SL-RLC1 for forwarding Uu RRC messages. Forwarding may occur in both directions. For the UL direction, as each intermediate relay UE knows its parent relay node (i.e., has already established a PC5-RRC link for the model A discovery announcement), the forwarding of the UL RRC message is straight-forward. For the DL direction, the remote UE L2 ID is used to determine the egress PC5 link before a local ID is configured to the intermediate relay UE. This mode of L2 ID assignment is achieved by letting the remote UE share its L2 ID along the path with its first RRC message towards the gNB. L2 ID assignment for each intermediate relay UE is performed in the same manner. Moreover, to distinguish the forwarded SRB0 message and self-generated SRB0 message which are both transported in the same SL-RLC0 channel, the Source L2 ID information is also needed.

Observation 3	It is feasible to enable fast forwarding of Uu SRB message over PC5 hops via SL-RLC0 or SL-RLC1.

Note that it is convenient to leverage the legacy SL-RLC0 and SL-RLC1 channels to expedite the delivery of Uu SRB0/SRB1 message. But remote UE SRB2/DRB traffic  still depends on relay UEs in RRC_CONNECTED for forwarding. Thus, if a remote UE gets into RRC_CONNECTED earlier than an intermediate relay UE, which is rare but may occur, the SRB2 message (e.g., a NAS message) may still be buffered in the SRAP layer of the intermediate relay UE until a dedicated SRAP configuration is provided. This behaviour is similar to the buffering of first SRB0 message in the baseline approach but will encounter much less delay compared to the SRB0 message in the baseline approach.

Based on the above observations and analysis, it is vital and feasible to support this latency reduction proposal to enable parallel RRC connection setup, by allowing the Uu RRC messages to be forwarded via SL-RLC channel(s).

Proposal 1	To reduce latency in CP procedures for multi-hop U2N relay in the Baseline Approach, support forwarding of Uu RRC messages via SL-RLC0 and/or SL-RLC1 by intermediate relay UE(s) not yet in RRC_CONNECTED.

# 3 Conclusion

In this contribution, we discuss the latency issue on the CP design for Multi-hop SL relay, and have the following observations:

Observation 1	In baseline approach, the requirement for UEs entering RRC_CONNECTED sequentially causes significant delay, which is not scalable as the number of hops increases.

Observation 2	Latency reduction can be achieved by enabling parallel RRC connection setups of Remote UE and Intermediate Relay UE(s).

Observation 3	It is feasible to enable fast forwarding of Uu SRB message over PC5 hops via SL-RLC0 or SL-RLC1.

Based on the discussion, we propose:

Proposal 1	To reduce latency in CP procedures for multi-hop U2N relay in the Baseline Approach, support forwarding of Uu RRC messages via SL-RLC0 and/or SL-RLC1 by intermediate relay UE(s) not yet in RRC_CONNECTED.

# 4 References

[1] R2-2409968	, Discussion on End-to-End Connection Setup Approaches for Multi-hop UE-to-NW Relay, Apple