3GPP TSG-RAN WG2 Meeting #129bis	R2-2502190

Wuhan, China, 7th – 11th April 2025

Agenda item:	8.13.3

Source:	Apple

Title:	Text Proposal for TS 38.351 for fast SRB0/1 forwarding

WID/SID:	NR_SL_relay_multihop – Release 19

Document for:	Discussion and Decision

# 1 Introduction

It is feasible to enhance the baseline approach for multi-hop relay to reduce the latency for CP setup, as discussed in [1]. In this paper, we discuss the necessary changes in SRAP specification to enable this feature and provide the TP for reference.

# 2 Discussion

SRAP is used as a L2 U2N relaying mechanism to transport end-to-end radio bearers, with the specific SRAP header identifying end-to-end bearer(s) and remote UE identity (i.e., local ID). In legacy Rel-17/Rel-18 U2N design, SRAP header is used for every SRAP Data PDU after dedicated RRC configuration (e.g., local ID assignment) is provided by gNB. The only exception is that the first RRC message can be sent by remote UE w/o any SRAP header via SL-RLC0.

In multi-hop U2N scenarios, there is a need to forward end-to-end Uu RRC signalling before all the UEs in the path entering CONNECTED. This requires SRAP entity to identify the remote UE “end-to-end” traffic w/o local ID assigned.  This kind of forwarding can be enabled by tweaking the SRAP header design to contain a “Remote UE Layer-2 ID”.

An example enhanced SRAP header design is shown in Figure 1.

Figure 1. enhanced Rel-19 SRAP header option (used in absence of dedicated NW configuration)

Basically, the following usage of such an enhanced R19 SRAP header are introduced, as shown in the table below:

| Direction | BEARER ID | UE ID | Layer 2 ID |
| --- | --- | --- | --- |
| Uplink | SRB0/1 | Unused | L2 ID of Remote UE |
| Downlink | SRB0/1 | Local ID assigned by gNB | L2 ID of Remote UE |

:

For each relay UE (last relay UE or intermediate relay UE), it does not need to change or remove the enhanced SRAP header at all. It simply forwards the SRAP PDU to the next-hop based on default configuration (e.g., SL-RLC0 or SL-RLC1 over PC5) or dedicated configuration for Uu SRB0/1 (with Uu/PC5 Relay RLC channel). When gNB receives the SRAP PDU with enhanced SRAP header, as it knows the L2 ID to which it has assigned the local ID, it can associate the received Uu RRC message with the right remote UE context.

This enhanced SRAP header is only used at the control plane setup stage when remote UE is not yet in RRC_CONNECTED. The switch from enhanced SRAP header back to normal SRAP header by Remote UE in UL direction is triggered by the reception of dedicated NW configuration of SRAP mappings & assigned local ID. The switch from enhanced SRAP header to normal SRAP header in DL direction is determined by gNB and there is no need to specify NW behaviour.

The TP for implementing the above mechanisms in TS 38.351 is provided in Chapter 3.

Proposal 1	RAN2 adopts the attached TP for TS 38.351 to support fast SRB0/1 forwarding.

# 3 Text Proposal

Start of the change

## 5.2	DL Data transfer

### 5.2.1	Receiving operation of U2N Relay UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity on the Uu interface of U2N Relay UE shall:

-	Deliver the SRAP data packet to the transmitting part of the collocated SRAP entity on the PC5 interface.

### 5.2.2	Transmitting operation of U2N Relay UE

#### 5.2.2.0	General

The transmitting part of the SRAP entity on the PC5 interface of U2N Relay UE receives SRAP data packets from the receiving part of the SRAP entity on the Uu interface of the same U2N Relay UE, and construct SRAP Data PDUs as needed (see clause 4.2.2).

When the transmitting part of the SRAP entity on the PC5 interface has an SRAP Data PDU to transmit, the transmitting part of the SRAP entity on the PC5 interface shall:

-	Determine the egress link in accordance with clause 5.2.2.1;

-	Determine the egress RLC channel in accordance with clause 5.2.2.2;

-	If the SRAP Data PDU is for SRB0 (the BEARER ID field is 0, and the bearer is identified as SRB based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel from which the SRAP Data PDU is received) and if this SRAP entity belongs to a single hop U2N relay UE:

-	Remove the SRAP header from the SRAP Data PDU;

-	Submit this SRAP Data PDU to the determined egress RLC channel of the determined egress link.

#### 5.2.2.1	Egress link determination

For a SRAP Data PDU to be transmitted, SRAP entity shall:

-	If there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay matches the UE ID field in SRAP Data PDU:

-	Determine the egress link on PC5 interface corresponding to sl-L2IdentityRemote or sl-L2IdentityIntermediateRelay configured for the concerned sl-LocalIdentity as specified in TS 38.331 [3].

#### 5.2.2.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If the SRAP Data PDU is for SRB0 (the BEARER ID field is 0 and the bearer is identified as SRB based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel from which the SRAP Data PDU is received):

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3].

-	Else if there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay matches the UE ID field in SRAP Data PDU, which includes an sl-RemoteUE-RB-Identity that matches the SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel from which the SRAP Data PDU is received, and for DRB, the DRB identity is BEARER ID plus 1):

-	If the SRAP Data PDU is for SRB1 but the corresponding sl-EgressRLC-ChannelPC5 is absent in sl-SRAP-ConfigRelay:

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC1 as specified in TS 38.331 [3].

-	Else:

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to sl-EgressRLC-ChannelPC5 configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

### 5.2.3	Receiving operation of U2N Remote UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity shall:

-	If the SRAP Data PDU is not for SRB0 (not received from SL-RLC0 as specified in TS 38.331 [3]):

-	If the SRAP Data PDU is received from SL-RLC1 as specified in TS 38.331 [3]:

-	Remove the SRAP header of this SRAP Data PDU and deliver the SRAP SDU to PDCP entity of SRB1 by ignoring the UE ID field and BEARER ID field of this SRAP Data PDU;

-	Else:

-	Remove the SRAP header of this SRAP Data PDU and deliver the SRAP SDU to upper layer entity corresponding to the BEARER ID field of this SRAP Data PDU (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelPC5 which matches LCID of the PC5 Relay RLC Channel from which the SRAP Data PDU is received, and for DRB, the DRB identity is BEARER ID plus 1);

-	Else:

-	Deliver the SRAP SDU (i.e., same as SRAP PDU for SRB0) to upper layer, i.e., RRC layer entity (TS 38.331 [3]).

### 5.2.4	Forwarding operation of U2N Intermediate Relay UE

For each SRAP Data PDU is received from parent UE, the SRAP entity shall:

-	if UE ID is present but there is no entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay matches the UE ID field in SRAP Data PDU and BEARER ID field indicates SRB0 or SRB1:

-	Determine the egress link as the stored PC5 link associated with L2 ID field included in SRAP Data PDU;

-	Determine the egress RLC channel as SL-RLC0 if BEARER ID indicates SRB0 or SL-RLC1 if BEARER ID indicated as SRB1;

-	Store the UE ID information and associated it with the Remote UE Layer-2 ID included in L2 ID field if L2 ID field is present.

-	Else:

<Approach 1 (Baseline approach) text omitted>

-	Submit this SRAP Data PDU to the determined egress RLC channel of the determined egress link.

## 5.3	UL Data transfer

### 5.3.1	Transmitting operation of U2N Remote UE

The transmitting part of the SRAP entity on the PC5 interface of U2N Remote UE can receive SRAP SDU from upper layer and constructs SRAP Data PDU.

Upon receiving an SRAP SDU from upper layer, the transmitting part of the SRAP entity on the PC5 interface shall:

-	If this SRAP entity belongs to Multi-hop Remote UE and sl-LocalIdentity has already been configured for the U2N remote UE as specified in TS 38.331 [3] or:

-	If this SRAP entity belongs to single-hop Remote UE and if the SRAP SDU is not for SRB0:

-	Determine the UE ID field and BEARER ID field in accordance with clause 5.3.1.1;

-	Construct an SRAP Data PDU with SRAP header, where the UE ID field and BEARER ID field are set to the determined values, in accordance with clause 6.2.2;

-	Else if SRAP entity belongs to Multi-hop Remote UE and SRAP SDU is for SRB0 or SRB1:

-	Determine the BEARER ID field corresponding to SRB0 or SRB1;

-	Construct an SRAP Data PDU with enhanced SRAP header as specified in clause 6.2.2, where the L2 ID field is set as the Source Layer-2 ID of the U2N remote UE;

-	Else:

-	Construct an SRAP Data PDU without SRAP header in accordance with clause 6.2.2.

-	Determine the egress RLC channel in accordance with clause 5.3.1.2;

-	Submit this SRAP Data PDU to the determined egress RLC channel.

#### 5.3.1.1	UE ID field and BEARER ID field determination

For an SRAP SDU received from upper layer, the SRAP entity shall:

-	Determine the UE ID field corresponding to sl-LocalIdentity, configured as specified in TS 38.331 [3];

-	Determine the BEARER ID field corresponding to SRB identity for SRB (i.e., set the BEARER ID field to srb-Identity), or corresponding to DRB identity minus 1 for DRB (i.e., set the BEARER ID field to drb-Identity minus 1), from which the SRAP SDU is received, configured as specified in TS 38.331 [3].

#### 5.3.1.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If the SRAP Data PDU is for SRB0:

-	Determine the egress PC5 Relay RLC channel in the link with U2N Relay UE or the Intermediate Relay UE (as parent of remote UE as defined in 38.331 [3]) corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3].

-	Else if the SRAP Data PDU is for SRB1 and if there is not an entry in sl-MappingToAddModList, whose sl-RemoteUE-RB-Identity matches the SRB identity of the SRAP Data PDU, or if there is an entry in sl-MappingToAddModList without the corresponding sl-EgressRLC-ChannelPC5:

-	Determine the egress PC5 Relay RLC channel in the link with U2N Relay UE or the Intermediate Relay UE (as parent of remote UE as defined in 38.331 [3]) corresponding to logicalChannelIdentity for SL-RLC1 as specified in TS 38.331 [3].

-	Else if there is an entry in sl-MappingToAddModList, whose sl-RemoteUE-RB-Identity matches the SRB identity or DRB identity of the SRAP Data PDU:

-	Determine the egress PC5 Relay RLC channel of the link with U2N Relay UE or the Intermediate Relay UE (as parent of remote UE as defined in 38.331 [3]) corresponding to sl-EgressRLC-ChannelPC5 configured for the concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

### 5.3.2	Receiving operation of U2N Relay UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity on the PC5 interface shall:

-	Deliver the SRAP data packet to the transmitting part of the collocated SRAP entity on the Uu interface.

### 5.3.3	Transmitting operation of U2N Relay UE

The transmitting part of the SRAP entity on the Uu interface of U2N Relay UE can receive SRAP data packets from the receiving part of the SRAP entity on the PC5 interface of the same U2N Relay UE, and construct SRAP Data PDUs as needed (see clause 4.2.2).

When the transmitting part of the SRAP entity on the Uu interface has an SRAP Data PDU to transmit, the transmitting part of the SRAP entity on the Uu interface shall:

-	If the SRAP Data PDU is received from SL-RLC0 as specified in TS 38.331 [3] and if L2 ID field is not included in SRAP Data PDU:

-	Determine the UE ID field and BEARER ID field in accordance with clause 5.3.3.1;

-	Construct an SRAP Data PDU with SRAP header, where the UE ID field and BEARER ID field are set to the determined values, in accordance with clause 6.2.2;

-	Determine the egress RLC channel in accordance with clause 5.3.3.2;

-	Submit this SRAP Data PDU to the determined egress RLC channel.

#### 5.3.3.1	UE ID field and BEARER ID field determination

For an SRAP Data PDU received from SL-RLC0 as specified in TS 38.331 [3], the SRAP entity shall:

-	If there is an entry in sl-RemoteUE-ToAddModList, whose sl-L2IdentityRemote matches the Layer-2 ID of the remote UE from which the SRAP Data PDU is received:

-	Determine the UE ID field corresponding to sl-LocalIdentity configured for the concerned sl-L2IdentityRemote as specified in TS 38.331 [3];

-	Determine the BEARER ID field as 0 (i.e., set BEARER ID field as 0).

#### 5.3.3.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If there is an entry in sl-RemoteUE-ToAddModList, whose sl-L2IdentityRemote matches the L2ID field of the included in SRAP Data PDU received; or

-	If there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay matches the UE ID field in SRAP Data PDU:

-	If the SRAP Data PDU is for SRB0:

-	Determine the egress Uu Relay RLC channel corresponding to sl-EgressRLC-ChannelUu configured for SRB0 for the concerned sl-LocalIdentity as specified in TS 38.331 [3].

-	Else if the SRAP Data PDU is received from SL-RLC1 as specified in TS 38.331 [3]:

-	Determine the egress Uu Relay RLC channel corresponding to sl-EgressRLC-ChannelUu configured for SRB1 for the concerned sl-LocalIdentity as specified in TS 38.331 [3].

-	Else if there is an entry in sl-RemoteUE-ToAddModList which includes an sl-RemoteUE-RB-Identity matches SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelPC5 which matches LCID of the PC5 Relay RLC Channel from which the SRAP Data PDU is received, and for DRB, the DRB identity is BEARER ID plus 1):

-	Determine the egress Uu Relay RLC channel corresponding to sl-EgressRLC-ChannelUu configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

### 5.3.4	Forwarding operation of U2N Intermediate Relay UE

For each SRAP Data PDU is received from child UE, the SRAP entity shall:

-	If the SRAP Data PDU has an enhanced SRAP header (i.e., L2 ID field is present); or;

-	if UE ID is present but there is no entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay matches the UE ID field in SRAP Data PDU, and BEARER ID field indicates SRB0 or SRB1:

-	Determine the egress link as the PC5 link to its parent relay UE as specified in TS 38.331 [3];

-	Determine the egress RLC channel as SL-RLC0 if BEARER ID indicates SRB0 or SL-RLC1 if BEARER ID indicates SRB1;

-	Store the ingress PC5 link information and associated it with the Remote UE Layer-2 ID included in L2 ID field if L2 ID field is present.

-	Else:

<Approach 1 (Baseline approach) text omitted>

-	Submit this SRAP Data PDU to the determined egress RLC channel of the determined egress link.

Next change

### 6.2.2	Data PDU

Figure 6.2.2-1 shows the format of the U2N SRAP Data PDU with SRAP header being configured. This SRAP Data PDU format is applicable to U2N SRAP SDU except those for SRB0 delivered over PC5 interface.

Figure 6.2.2-1: U2N SRAP Data PDU format with SRAP header

Figure 6.2.2-2 shows the format of the U2N SRAP Data PDU consisting only of a data field without any SRAP header. This SRAP Data PDU format is applicable to U2N SRAP SDU for SRB0 delivered over PC5 interface.

Figure 6.2.2-2: U2N SRAP Data PDU format without SRAP header

Figure 6.2.2-3 shows the format of the U2U SRAP Data PDU. This SRAP Data PDU format is applicable to U2U SRAP SDU delivered over PC5 interface.

Figure 6.2.2-3: U2U SRAP Data PDU format with SRAP header

Figure 6.2.2-4 shows the format of the U2N SRAP Data PDU with enhanced SRAP header. This SRAP Data PDU format is applicable to multi-hop U2N relaying over both PC5 and Uu interface.

Figure 6.2.2-4: U2N SRAP Data PDU format with enhanced SRAP header

Next change

## 6.3	Parameters

.<text omitted>

### 6.3.x	F

Length: 1 bit

This field indicates SRAP PDU format (i.e., whether the corresponding SRAP header is of enhanced format or not). Bit set as "1" indicates the U2N SRAP Data PDU format with enhanced SRAP header as shown in Figure 6.2.2-4 is used. Bit set as "0" indicates U2N SRAP Data PDU format with SRAP header as shown in Figure 6.2.2.-1.

### 6.3.y	L2 ID

Length: 24 bits

This field indicates the Source Layer-2 ID used by the L2 Remote UE for L2 U2N Relay communication.

# 4 References

[1] R2-2502189 Fast & Parallel RRC Establishment/Configuration for Multi-hop U2N relaying, Apple, FirstNet, Ericsson, AT&T, Kyocera