3GPP TSG-RAN WG2 Meeting #129-bis		R2-2502205

Wuhan, China, April. 07th – 11th, 2025

Title: 	Data rate control for XR applications

Source: 	ZTE Corporation, Sanechips

Agenda item:	8.7.6

Document for:	Discussion and decision

# Introduction

In RAN#105 meeting[1], Revised WID on XR (eXtended Reality) for NR Phase 3 has been agreed and the following objective has been included:

- Specify uplink congestion signalling [RAN2]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

At RAN2#127-bis[2], it was agreed that further analysis is needed to decide between per-QoS flow vs per DRB indication of XR rate control. It was also agreed to define a new table for bit rate recommendation with better granularity for XR applications.

At RAN2#128, the following agreements have been made[3]:

RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

Further, at RAN2#129, RAN3 has sent an LS to RAN2 indicating that both solutions for XR rate control have RAN3 impact (see R2-2501508)

In this paper, we will discuss the remaining issues for this and propose a way forward for the XR rate control in Rel-19.

# Discussion

Based on the agreements at RAN2#128, we need to discuss further how the gNB can configure the XR rate control mechanism. The most flexible way to control the XR rate is to define a per-QoS flow level XR rate control using a MAC CE. However, in order to specify such an approach, work may also be needed in RAN3 to support the necessary signalling between the CU and DU nodes in the network side. If it is preferred in RAN2 to avoid such impact, then one option is to configure the QoS flow level applicability using RRC and then using the MAC CE to perform the dynamic rate control. However, in R2-2501508, RAN3 indicated that both solutions have RAN2 impact. It is a bit unclear what the impact to F1 interface is with the RRC based approach. In this approach, the gNB configures which QoS flows are subject to rate control and this signalling is RRC level signalling which is from the CU to the UE. DU has no visibility of this. The DU then generates rate control MAC CEs based on the monitoring of the traffic locally at the DU side (on a per DRB basis) and generates rate control commands. The UE applies the recommended rate to the QoS flow which is configured by RRC. Thus, in this case the monitoring of the rate is performed directly at DU and the rate control commands are issued from the DU too. So, the impact to F1 interface is not really clear.

Observation 1: In case of the RRC based solution, the monitoring of the XR traffic is done on a per DRB basis and is performed at the DU (which also generates the MAC CE to control the data rate)

It should however be noted that the RRC based solution makes it slightly less flexible for the gNB, in the sense that only one QoS flow per DRB can be rate controlled with this approach. However, the gNB can then also choose to map only one QoS flow that is subject to rate control to such a DRB.

Observation 2: The RRC based solution for XR rate control implicitly requires that only one QoS flow subject to rate control can be mapped to a given DRB. Such restriction is not there for the MAC based solution.

On the other hand, the MAC based solution where the QoS flow identifier is directly included in the MAC CE does have some impacts to F1 interface. Specifically, in this case the XR rate monitoring on a per QoS flow level needs to be performed in the CU (since the DU is may not be aware of the QoS flows mapped within the same DRB). Then the CU has to generate the per QoS flow level recommended rate and provide this to the DU. This will require F1 signalling. However, now that RAN plenary agreed the necessary TUs for RAN3, defining such signalling should be feasible. So, RAN2 should make the decision based on the technical merits and not based on the RAN3 work.

Observation 3: The MAC based solution to signal the XR rate control on a per QoS flow basis is the most flexible option, but will require changes to RAN3 specs (i.e. F1 interface) and this work is now feasible in RAN3 as there are TUs available for this work

Although both solutions seem to work, the MAC based solution seems to be the most flexible one. So, we propose that RAN2 goes ahead with this solution and instructs RAN3 to specify the necessary F1 signalling.

Proposal 1: If the understanding is that both solutions have RAN3 impact, then RAN2 should select the MAC based solution which seems to be the most flexible one (i.e. gNB to indicate the recommended bit rate on a per QoS level basis in the MAC CE)

Proposal 2: RAN2 should send an LS to RAN3 to inform them about the agreed solution and ask RAN3 to define any corresponding F1 signalling (if needed)

# Conclusion

The following observations and proposals are made in this contribution:

Observation 1: In case of the RRC based solution, the monitoring of the XR traffic is done on a per DRB basis and is performed at the DU (which also generates the MAC CE to control the data rate)

Observation 2: The RRC based solution for XR rate control implicitly requires that only one QoS flow subject to rate control can be mapped to a given DRB. Such restriction is not there for the MAC based solution.

Observation 3: The MAC based solution to signal the XR rate control on a per QoS flow basis is the most flexible option, but will require changes to RAN3 specs (i.e. F1 interface) and this work is now feasible in RAN3 as there are TUs available for this work

Proposal 1: If the understanding is that both solutions have RAN3 impact, then RAN2 should select the MAC based solution which seems to be the most flexible one (i.e. gNB to indicate the recommended bit rate on a per QoS level basis in the MAC CE)

Proposal 2: RAN2 should send an LS to RAN3 to inform them about the agreed solution and ask RAN3 to define any corresponding F1 signalling (if needed)

# References

- RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3	RAN#105

- Draft_RAN2_127b_Meeting_Report_v1

- Draft_RAN2_128_Meeting_Report_v1

- 3GPP TS 38.321, V18.2.0 (2024-06), NR; Medium Access Control (MAC) protocol specification