3GPP TSG-RAN WG2 Meeting #129bis                                                         R2-2502206

Wuhan, China, 7th – 11th April, 2025

Source: 		  ZTE Corporation, Sanechips

Title:                  Further consideration on Paging procedure for A-IoT

Agenda item:    8.2.2

Document for:  Discussion and Decision

# Introduction

During RAN2#125bis~RAN2#128, RAN2 has completed the working group study for supporting Ambient IoT in NR and achieved much progress on the aspects of protocol stack, functionalities, random access-like and paging-like procedures etc. The study result has been captured in the TR 38.769 [1].

In RAN#106, WID for A-IoT has been agreed and further revised in RAN#107 meeting in [2].

In RAN2#129 meeting, for several sub-topics in Paging topic, the following agreements have been reached:

Simultaneous Paging Procedures - Parallel service requests by the same reader is not supported. - The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed). Which entirty generates the transaction ID - The “transaction ID” can be generated by reader based on CN correlation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID Transaction ID Size - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size. Multiple Reader Scenario - RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it. Paging ID - 1. The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2. - 2. The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer Resource Information - the A-IoT paging message can include a number of msg1 resources Scheduling of MSG1 Transmissions - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field Service Type Indication - The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

Furthermore, some detailed issues have been discussed in post-meeting email discussion “[POST129][035][AIoT] Paging”.

In this paper, we will continue to discuss the key issues in the basic Paging procedure, as well as some issues during email discussion which have diverse views, and then give our proposals.

# Discussion

## 2.1 Sub-grouping of the paged devices by the reader

For the Paging procedure, the device ID information (e.g., Paging identifier) should be included in the Paging message. The device needs to compare its stored device ID information with the provided device ID information and determine whether it’s the target device and needs to respond the A-IoT operation.

When a group of devices are paged a common way for the CN/AF is to implicitly indicate a group of devices by providing information of structured device ID and the corresponding “mask/filter”.

In the case where a very large number of devices are to be inventoried (this can be a typical/special service in AIoT which is not in general IoT system), the reader may need to perform sub-grouping (mask only a subset) according to actual device density and radio resource usage. The efficiency of the slotted-Aloha algorithm suffers significantly when the system is operating in the non-linear region of the aloha procedure. In this region, the inventory times increase exponentially and hence it is absolutely essential that the reader avoids such situations. It should be noted that the actual conditions in which the algorithm operates in this region will depend on the device density and radio conditions which are not known to the CN/AF. Thus, even if the CN does some masking or sub-grouping, this may be not enough to achieve the desired results and if the only choice the reader has is to play with the group size provided by the CN, then the actual procedure over the air may suffer depending on the collision situation which may materialise during the inventory process. Moreover, the CN is responsible for the inventory control across different readers. So, it cannot be expected that the CN can always tailor the group size for each individual reader.

In Figure 1we show the advantages of splitting large group of paged devices into sub-groups by using a mask to choose subset of devices and compare this with the case when there is no group splitting. This problem is well known also in RFID systems and hence the approach to “select” as sub-set of devices before paging the group is adopted and is popular in RFID based systems.

Figure 1: Impact of group-splitting on the inventory times

The following observations can be made from the above analysis:

Observation 1: Operating the system without group splitting (i.e. masking the paging identifier) will result in highly negative performance when the device density is high where the slotted-Aloha system operates in non-linear region

Observation 2: At low device densities, the optimal strategy is to not split the groups into sub-groups

Observation 3: The crossover point for the optimal operation/splitting depends on device density, contention levels and other radio conditions such as coverage etc

Observation 4: The CN cannot control the contention based on the device density or the actual contention levels because these are not known at the CN. Furthermore, if a wrong group size/mask is selected by the CN, then the inventory procedure at the reader will suffer significantly and may never converge

Observation 5: Existing systems such as RFID employ reader based “select” procedure which enable sub-grouping of the contending devices on the radio side

Given the above, the reader should be able to control the contention during the random-access phase by masking the device ID and selecting a sub-group of the paged devices. Thus, we propose the following:

Proposal 1: The reader should be able to apply an AS layer “mask/filter” scheme on the Paging identifier information from a CN/AF service request to select a subset of devices.

## 2.2 Handling of multi-reader scenarios

### 2.2.1 Single service procedure triggered by one reader

For basic Paging procedure, RAN2 has agreed that parallel service requests by the same reader is not supported. Then, an example timing diagram of a reader executing a CN AIoT service request, e.g., to perform multiple Paging rounds on multiple devices (only one device is listed here as example), is shown below in Figure 1.

Figure 1

One of the key issues that needs to be addressed is the setting of the transaction ID.

RAN2 has had general agreement that the “transaction ID” can be generated by reader based on CN correlation ID. In above basic Paging procedure involving only one reader, as each CN AIoT service request may trigger multiple Paging rounds with same transaction ID, it’s easy to understand that the setting of transaction ID needs to ensure that Paging messages triggered by two consecutive different CN AIoT service requests to have distinct transaction IDs. Here different CN AIoT service requests may be identified by different correlation IDs, which is a concept discussed by other working groups and generally used to identify a single service request within network interfaces.

One straightforward way to assign the transaction ID is to directly associate it with different service requests and/or different readers. Moreover, we assume the value of correlation ID would be a continuously encoded value for the actual services initiated by the AF, and the range of the encoded values may be relatively large.

Meanwhile, the transaction ID will be sent in the AIoT air interface and cannot be too long, therefore, setting of “transaction ID” corresponds to a way of mapping correlation ID with a larger value range to transaction ID with smaller value range. The following alternatives for such mapping have been mentioned in email discussion “[035][AIoT] Paging”:

- Alt1: To use truncated correlation ID, e.g., LSB or MSB X bits of the correlation ID as transaction ID. However, Alt1 cannot always meet the requirement that Paging messages triggered by two consecutive different CN AIoT service requests (may be with different correlation IDs) have distinct transaction IDs. In some case, Alt1 might result in the same transaction ID for Paging messages triggered by different CN AIoT service requests. For example, let the transaction ID be LSB 2 bits (X=2) of the correlation ID. If the first CN AIoT service request has correlation ID = 5 (“101”) while the second one has correlation ID = 9 (“1001”), this could lead to conflict that both first triggered and second triggered Paging messages have same transaction ID of #1. Here, we assume that the CN continuously encodes the correlation ID for each AIoT service request. However, since different AIoT service requests might be sent to different readers, the correlation IDs in the CN AIoT service requests received by a certain reader might not be continuous.

- Alt2: To sequentially encode the transaction ID for the triggered Paging messages, e.g., the first Paging message triggered by the firstly received CN AIoT service request would have transaction ID #0, the second Paging message triggered by the secondly received CN AIoT service request (containing different correlation ID) would have transaction ID #1, and so forth. When the maximum value of transaction ID range is reached, the encoding restarts from #0. Alt2 is easy to meet the requirement that Paging messages triggered by two consecutive different CN AIoT service requests (may be with different correlation IDs) have distinct transaction IDs.

Although the setting of transaction ID can be the reader implementation which may not need to be specified, as it’s related to the discussion on the size of transaction ID and also the setting of transaction ID in other more complicated scenario, some discussion is still needed for this aspect. Given that the transaction ID should indicate both the service ID and the reader ID, the length should be sufficiently long to cover both multi-reader and multi-service cases. Thus, a transaction ID of at least 4 bits should be selected. Based on the above, the following observation/proposal are made

Observation 6: The reader can adopt the following options for setting of transaction ID in Paging message:

- Alt1: transaction ID = LSB or MSB X bits of the correlation ID

- Alt2: transaction ID is a sequentially encoded value according to the order in which CN AIoT service requests were received

Proposal 2: The length of the transaction ID should be at least 4 bits

### 2.2.1 Single service procedure triggered by multiple readers

Besides the basic Paging procedure (Case#1), during the email discussion, the following cases have been mentioned for the more complicated scenario of single service procedure triggered by multiple readers. For each sub-case, some alternatives have been mentioned on how to set the transaction ID to achieve the expected device behavior.

| Case#2: Same service request is received from different readers | Case#2-1: A CN AIoT service request (e.g., inventory) is sent to multiple AIoT readers, e.g., reader-A and B (due to that CN has no priori/existing knowledge on the association between the reader and the target devices). The reader-A sends a Paging to a group of devices. Meanwhile, an adjacent reader, e.g., reader-B also receives same CN AIoT service and then triggers another Paging procedure although the Paging procedure trigged by reader-A is still ongoing (reader-B may have no idea on what reader-A is doing). From the device perspective, if a device has previously responded a Paging message from reader-A and completed the corresponding RA/data transmission procedure successfully, when it receives another Paging message and it can determine the received Paging is from another reader, e.g., reader-B, and corresponds to the same AIoT service request, the device can ignore the latter Paging from reader-B as the inventory result of the device can be delivered to CN/AF by the reader-A. To achieve the above UE behaviour, the following alternatives can be considered: - Alt1: To make Paging messages from different readers but triggered by the same AIoT service contain the same transaction ID can facilitate the above mentioned device behaviour (e.g., to ignore the latter Paging) and also simplify device operations, meaning that the device does not need to distinguish whether Paging messages with the same transaction ID come from the same or different readers. |
| --- | --- |
|  | Case#2-2: A CN AIoT service request (e.g., inventory) is sent to multiple AIoT readers, e.g., reader-A and B for the purpose of coarse positioning of the device, e.g., the network intentionally have multiple readers inventory the same batch of devices and expect these devices to respond to the requests from each reader. To achieve the above UE behaviour, the following alternatives can be considered: - Alt1: to make Paging messages from different readers but triggered by the same AIoT service still contain the same transaction ID. In order to avoid same behavior as that in Case#2-1 (not respond to the Paging sent by reader-B), additional information about reader or other indication that redundant response is needed may be needed. - Alt2: to make Paging messages from different readers just contain different transaction IDs (although they are triggered by a same AIoT service). Since the latter Paging message contains different transaction ID from that in previous Paging, the device can simply assume the latter Paging to be associated with a new service request and respond to it. |

Based on the basic procedure in Figure 1, we give an example procedure in Figure 4 to explain the Case#2-1(using Alt1):

Figure 4

In this example procedure for Case#2-1:

- The nth AIoT service request is sent to both reader-A and reader-B from CN/AF. The reader-A triggers an initial Paging message including transaction ID (here assuming an example value of transaction ID=1). For a certain device-1, it determines itself is the target one and attempt to respond. It also successfully responds in this Paging round.

- As reader-B also receives the nth AIoT service request, it may also trigger an initial Paging message including same transaction ID (e.g., transaction ID=1). This Paging message may reach to device-1 shortly after the previous Paging message from the reader-A. As device-1 marks it has successfully responded a Paging with transaction ID=1, if no any other information, the device will think this is the same Paging/CN AIoT service request as the one it has previously responded to successfully, it will ignore this Paging message from reader-B. This is the desired behavior in Case#2-1 to avoid redundant responses for a same service request.

- Similar as that in basic Paging procedure in Figure 1, the device-1 will also ignore the Paging messages including same transaction ID in the following Paging round(s) triggered by reader-A.

- After reader-A considers the current process for nth CN AIoT service request can be finished, the reader-A may subsequently process the pending (n+1)th CN AIoT service request (if exists) with the same steps as that for nth CN AIoT service request. But the different transaction ID (e.g., transaction ID=2) would be included in the Paging messages. As the device-1 can determine this is a different Paging/CN AIoT service request from the one it has previously responded to successfully, the device-1 will attempt to respond and also successfully respond in one of the Paging rounds.

Furthermore, we give an example procedure in Figure 5 to explain the Case#2-2 (using Alt2):

Figure 5

In this example procedure for Case#2-2:

- The nth AIoT service request is sent to both reader-A and reader-B from CN/AF. The reader-A triggers an initial Paging message including transaction ID (here assuming an example value of transaction ID=1). For a certain device-1, it determines itself is the target one and attempt to respond. It also successfully responds in this Paging round.

- As reader-B also receives the nth AIoT service request, it may also trigger an initial Paging message but including a different transaction ID (e.g., transaction ID=2). This Paging message may reach to device-1 shortly after the previous Paging message from the reader-A. Although device-1 marks it has successfully responded a Paging, according to the info of transaction ID=2, the device determines this is a different Paging/CN AIoT service request from the one it has previously responded to successfully, the device-1 will attempt to respond this Paging message from reader-B and also successfully respond in a Paging round. This is also the desired behavior in Case#2-2 to let network know that the device is under the coverage of both the reader-A and reader-B.

- However, if device-1 records only the latest transaction ID it has addressed (e.g., transaction ID=2), when it receives another Paging sent by reader-A in later Paging round, as the transaction ID included in the Paging sent by reader-A (e.g., transaction ID=1) is different from the stored one, the device-1 will assume it needs to respond this “new” Paging. Such repeated responses to reader-A (the RA procedure in red circle in Figure 3) can be seen as a wrong process.

- Another issue is that, after reader-A considers the current process for nth CN AIoT service request can be finished, the reader-A may subsequently process the pending (n+1)th CN AIoT service request (if exists) with the same steps as that for nth CN AIoT service request. However, as it’s possible that transaction ID in the Paging triggered by (n+1)th CN AIoT service request is also set to 2, the device may wrongly consider it has already responded successfully to this Paging (but that response was sent to reader-B) and then ignore this Paging from reader-A corresponding to the (n+1)th CN AIoT service request. Such ignorance (an RA procedure is expected as that in purple circle in Figure 3) is also a wrong process.

In a summary, it can be seen that different cases have different requirements on UE behaviour and also different requirements on the setting of transaction ID.

Generally, we would expect a unified solution to address all the agreed cases. But based on all above analysis, we can see some issues of Alt2. So Alt1 may be preferred.

So, we suggest RAN2 to discuss the following issues:

Proposal 3: RAN2 should discuss which alternative in below can be a baseline assumption for addressing these cases:

- Alt1: to make Paging messages from different readers but triggered by the same CN AIoT service contain the same transaction ID

- Alt2: to make Paging messages from different readers but triggered by the same CN AIoT service contain different transaction ID

Then, from the specification perspective, we need to capture the device behaviour. The main question here is what happens if the device receives a new message with a new transaction ID when the device is already responding to a given paging request using a different transaction ID. Since we already agreed that the device only supports one service (i.e. no parallel services), the device shall simply ignore such message. As a consequence of this, it is also proposed that all R2D messages carry the transaction ID.

Proposal 4: If the device receives an R2D message with a transaction ID that doesn’t match the stored transaction ID (to which the device is currently responding to), then the device shall ignore such message.

Proposal 5: All R2D messages shall include a transaction ID

# Conclusion

The following observations and proposals are made in this contribution:

Observation 1: Operating the system without group splitting (i.e. masking the paging identifier) will result in highly negative performance when the device density is high where the slotted-Aloha system operates in non-linear region

Observation 2: At low device densities, the optimal strategy is to not split the groups into sub-groups

Observation 3: The crossover point for the optimal operation for group splitting depends on device density, contention levels and other radio conditions such as coverage etc

Observation 4: The CN cannot control the contention based on the device density or the actual contention levels because these are not known at the CN. Furthermore, if a wrong group size/mask is selected by the CN, then the inventory procedure at the reader will suffer significantly and may never converge

Observation 5: Existing systems such as RFID employ reader based “select” procedure which enable sub-grouping of the contending devices on the radio side

Observation 6: The reader can adopt the following options for setting of transaction ID in Paging message:

- Alt1: transaction ID = LSB or MSB X bits of the correlation ID

- Alt2: transaction ID is a sequentially encoded value according to the order in which CN AIoT service requests were received

Proposal 1: The reader should be able to apply an AS layer “mask/filter” scheme on the Paging identifier information from a CN/AF service request to select a subset of devices.

Proposal 2: The length of the transaction ID should be at least 4 bits

Proposal 3: RAN2 should discuss which alternative in below can be a baseline assumption for addressing these cases:

- Alt1: to make Paging messages from different readers but triggered by the same CN AIoT service contain the same transaction ID

- Alt2: to make Paging messages from different readers but triggered by the same CN AIoT service contain different transaction ID

Proposal 4: If the device receives an R2D message with a transaction ID that doesn’t match the stored transaction ID (to which the device is currently responding to), then the device shall ignore such message.

Proposal 5: All R2D messages shall include a transaction ID

- References

- 3GPP TR 38.769 V2.0.1, Study on solutions for ambient IoT (Internet of Things) (Release 19), December, 2024

- 3GPP RP-250796, Revised WID: Solutions for Ambient IoT (Internet of Things) in NR, RAN Plenary#107, March, 2025

- TR 23.700-13 v2.0.0 (2025-02), Study on Architecture support of Ambient power-enabled Internet of Things (Release 19), January. 2025