3GPP TSG-RAN WG2 Meeting #129-bis						        R2-2502207

Wuhan, China, 07 – 11 April, 2025

Source: 			ZTE Corporation, Sanechips

Title: 	Open issues for A-IoT random access

Agenda item:	8.2.3

Document for: Discussion and Decision

# Introduction

In this contribution, we discuss the remaining open issues for the A-IoT random access procedure and propose a way forward for these.

# Procedure for MSG1 transmission

The random-access procedure for A-IoT uses slotted Aloha. The slotted Aloha procedure needs the following:

- A common timing to be established between the reader and the paged devices.

- A mechanism for the devices to select one access occasion (to transmit MSG1) which is to be chosen in a random way from the access occasions that are provided by the reader. This requires the UE to know:

  - how the access occasions are distributed in time domain

  - how the access occasions are distributed in the frequency domain

The overall procedure to achieve 1) and 2) above is depicted in Figure 1.

Figure 1: Overall procedure for determining MSG1 resources (showing all possible options)

First, the paging message needs to establish the total length of a paging frame, in time domain. This is determined by the device based on the total number of access occasions indicated in the paging message.

Proposal 1: The device determines the length of the paging frame based on the number of access occasions indicated in the paging message

Then, for 1) (i.e. to establish and maintain the timing), the reader needs to provide a stable clock for the devices. This is because the simple devices that are targeted for A-IoT may not be capable of maintaining a stable clock. Thus, for establishing and maintaining the timing between the reader and the paged devices, the reader has to send an R2D message. This message is referred to as the R2D Trigger message – see Figure 1. Each of these R2D trigger messages indicate the start of a set of MSG1 resources that are distributed in Time/Frequency domain.

Proposal 2: In order to establish and maintain a common timing between the reader and the devices that are paged, the reader sends R2D trigger message(s) after the paging message

Proposal 3: Each of these R2D trigger messages trigger the start of a set of MSG1 resources that are distributed in Time/Frequency domain

As can be seen in the Figure 1, there are many options with regards to the distribution of the time/frequency MSG1 resources triggered by the R2D Trigger message. A given R2D message triggers X time domain and Y frequency domain and then, there are the following options. The first question is whether the number of frequency domain resources (Y) is constant within a given paging frame or not. If the number of frequency domain resources for a given slot are not constant within the paging frame, then this number needs to be indicated explicitly in every R2D trigger. This adds a large overhead to the R2D trigger and also adds complexity for the device. So, it is proposed that the number of frequency domain resources are always the same within a paging frame. Having this understanding is also important for the random selection mechanism that the device uses (see proposal 7 below).

Proposal 4: The number of frequency domain resource (Y) triggered by a given R2D trigger message is the same for every R2D trigger message within a given paging frame

Then, we need to discuss the number of time domain resources. As depicted in Figure 1, the time domain resources may also vary across different R2D triggers. Again, with the same logic as in the frequency domain. If the time domain resources vary from one R2D trigger to the other, then this also increases the overhead in the R2D trigger message and adds complexity at the device. So, we propose that this is also constant across a given paging frame.

Proposal 5: The number of time domain resources (X) triggered by a given R2D trigger message is the same for every R2D trigger message within a given paging frame

Then, it was already agreed that FDMA of MSG1 resources is possible. So, Y can be 1 or more than 1 and is indicated in the paging message. However, we need to understand if X > 1 can be supported. There are a number of issues with allowing X > 1:

- If X>1, the devices may not be able to maintain timing (because there is no R2D trigger to establish synchronization between the messages)

- If the reader detects no response in a given resource, it is possible to terminate the triggered slot and move to the next one if X = 1. But if X > 1 this is not feasible. This results in inferior system performance [1].

- Having multiple options for X will in general increase the signalling overhead and make the device implementation complex.

Given the above, we propose to fix X to be 1.

Proposal 6: The number of time domain resources (X) triggered by a given R2D trigger message is 1.

Based on Proposals 1 – 6, the simplified overall procedure for RACH is as shown in Figure 2.

Figure 2: Proposed simplified over-all procedure for MSG1 transmission

With the above procedure in mind, we then need to agree how the devices will select a given access occasion (), within a paging frame. The paging message will indicate the total number of Access Occasions within the paging frame. There are two options from the device perspective:

Option 1: The device selects a random number between 1 and N*Y, then the device determines the access occasion based on the selected random number

Option 2: The device selects a random (time) slot number (between 1 and N) and then within this time slot, the device then determines a random frequency location (between 1 and Y)

Firstly, with P4 agreed, option 1 and option 2 will result in the same collision probability for a given access occasion. Then, the difference between these two schemes may be with regards to how the MSG1 resources are staggered in time domain. Option 1 may result in the MSG1 transmissions being more concentrated in some slots whilst some time slots having fewer or no MSG1 resources. Whilst option 2 may result in a more uniform spread of MSG1s in time domain. Thus, it seems that Option 2 may be slightly better for reader processing/scheduling. So, we propose this option.

Proposal 7: The device shall determine the MSG1 access occasion by selecting:

- a uniformly distributed random number between 1, N where N is the total number of MSG1 slots within a paging frame (as indicated in the paging message) and

- a uniformly distributed random number between 1, Y where Y is the total number of MSG1 frequency domain resources within a MSG1 slot (as indicated in the paging message)

Once the device determines the access occasion, it shall wait for the R2D trigger to count down the time slots and once the MSG1 slot matching the chosen random value for N arrives, the device shall transmit on the chosen frequency resource on this slot.

Proposal 8: After the device (randomly) selects a MSG1 slot number, the device shall count down the MSG1 slots based on the R2D trigger message and when the selected MSG1 slot arrives, the device shall transmit on the (randomly) chosen frequency location on this slot.

# Procedure for CFRA

At RAN2#129, it was agreed that CFRA is only applicable when a single device is paged. There are three main options for CFRA in this case:

Option 1: The device includes only RN16 in MSG1 and the subsequent procedure for CFRA is same as CBRA

Option 2: The device includes RN16 + upper layer device ID in MSG1 and the subsequent procedure is same as for CBRA

Option 3: The device includes only the upper layer device ID in MSG1 and further discussion is needed on how an ASID is assigned to the device for command use case

In our view, option 1 and option 2 both minimize the device complexity since the device always generates and includes RN16 in MSG1. Thus, from device perspective there will not much difference in CBRA and CFRA. So, RAN2 should select between option 1 and option 2. Inclusion of RN16 is then common between these two options (and with CBRA). So, the following proposals are made:

Proposal 9: RN16 is included in MSG1 for CFRA

Proposal 10: RAN2 should discuss whether device ID can also be included in MSG1 for CFRA or not

Proposal 11: Subsequent steps in CFRA (after MSG1 transmission) are same as CBRA

# Conclusion

The following proposals are made:

Determination of MSG1 resources

Proposal 1: The device determines the length of the paging frame based on the number of access occasions indicated in the paging message

Proposal 2: In order to establish and maintain a common timing between the reader and the devices that are paged, the reader sends R2D trigger message(s) after the paging message

Proposal 3: Each of these R2D trigger messages trigger the start of a set of MSG1 resources that are distributed in Time/Frequency domain

Proposal 4: The number of frequency domain resources (Y) triggered by a given R2D trigger message is the same for every R2D trigger message within a given paging frame

Proposal 5: The number of time domain resources (X) triggered by a given R2D trigger message is the same for every R2D trigger message within a given paging frame

Proposal 6: The number of time domain resources (X) triggered by a given R2D trigger message is 1.

Proposal 7: The device shall determine the MSG1 access occasion by selecting:

- a uniformly distributed random number between 1, N where N is the total number of time domain MSG1 slots within a paging frame (N is signalled in the paging message) and

- a uniformly distributed random number between 1, Y where Y is the total number of MSG1 frequency domain resources within a MSG1 slot (Y is indicated in the paging message)

Proposal 8: After the device (randomly) selects a MSG1 slot number, the device shall count down the MSG1 slots based on the R2D trigger message and when the selected MSG1 slot arrives, the device shall transmit on the (randomly) chosen frequency location on this slot.

ASID for CFRA

Proposal 9: RN16 is included in MSG1 for CFRA

Proposal 10: RAN2 should discuss whether device ID can also be included in MSG1 for CFRA or not

Proposal 11: Subsequent steps in CFRA (after MSG1 transmission) are same as CBRA

# References

- R1-2500127, Discussion on Ambient IoT multiple access and timing, ZTE Corporation, Sanechips