3GPP TSG-RAN WG2#129-bis	R2-2502225

Wuhan, China, April. 7th – 11th, 2025

Source: 	Fujitsu

Agenda item:	8.5.2

Title: 	Remaining issues on on-demand SSB SCell operation

Document for:	Discussion and decision

# Introduction

In RAN2#129 meeting, RAN2 has discussed the on-demand SSB SCell operation, and the following agreement was made [1]:

Agreements on OD-SSB 1. RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

In this contribution, we would like to discuss the remaining issues on on-demand SSB SCell operation.

# Discussion

## On-demand SSB SCell

### L3 measurements

Framework of L3 measurements

In RAN2#127bis meeting, the following two options were discussed for L3 measurements framework but RAN2 have not agreed because it is not clear for RAN4 impact.

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

On the other hand, RAN4 agreed the fast mode window for L3 measurements at on-demand SSB transmission. During the fast mode window, the measurement requirement is defined based on on-demand SSB periodicity.

For option 1, it is less impact from RAN4 perspective, because currently L3 measurement requirement in TS 38.133 is defined based on the SMTC configuration, not for the SSB configuration. From RAN2 perspective, although on-demand SSB has different (typically short) periodicity from that of always-on SSB, the same measurement mechanism can be applied by adaptive SMTC window for on-demand SSB periodicity in the fast mode window. Since RAN2 already supports multiple SMTC lists (e.g., smtc3list, smtc4list), it would be small impacts on the specifications in RAN2.

For option2, it may be beneficial if on-demand SSB pattern needs full flexibility. For example, at least applicable value for on-demand SSB periodicity can be indicated by a new MAC CE for on-demand SSB transmission activation. As the SSB periodicity is set by one of {ms5, ms10, ms20, ms40, ms80, ms160}, then in case of option 1, at worst the NES UE should be pre-configured with 6 SMTCs for on-demand SSB measurements. However, even if on-demand SSB needs some flexibility for quick DL synchronisations and/or L1/L3 measurements, in practical, at most 2 measurement configurations would be sufficient (i.e., for scenarios 2/2A).

Observation 1:	In practical, it is enough to support at most 2 measurement configurations for on-demand SSB (i.e., for scenarios 2/2A).

From signalling perspective, option 2 may be simpler than option 1 because option 2 can reuse the configured on-demand SSB patterns for L3 measurements whereas option 1 needs an additional configuration for adaptive SMTC. However, in RAN2#129 meeting, the similar issue was discussed on SSB adaptation and RAN2 finally made the following agreement.

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

Observation 2:	For SSB adaptation, RAN2 agreed to keep SMTC based L3 RRM framework.

Because it is a similar concept of the option1, it means that signalling reduction/optimization is not a critical issue for L3 measurement.

Considering the above, there is no clear motivation to support such a flexibility for L3 measurements, and option 1 is less impact on other WGs with reasonable RAN2 impact. So, we prefer to support option 1 for L3 measurements framework of on-demand SSB. The limited number of SMTC window for on-demand SSB can be pre-configured by RRC at the SCell addition. The UE can apply the SMTC window for on-demand SSB indicated by MAC CE or RRC when the on-demand SSB transmission is activated. It is also beneficial for RAN2 to adopt a unified L3 measurements framework between on-demand SSB and SSB adaptation.

Proposal 1:	RAN2 to adopt “Option1: Based on different measurement configuration when OD-SSB is transmitted” as on-demand SSB L3 measurements framework.

Proposal 2:	The UE should apply the SMTC window configured for on-demand SSB when the corresponding on-demand SSB transmission is activated.

L3 measurements for neighbour cell:

In RAN4#112bis meeting, RAN4 also agreed the legacy measurement framework is applied for neighbour cell measurement [2].

Issue 1-1-2: OD-SSB impact on neighbour cell measurement Agreement: - For the RRM measurement on neigboring cell, the legacy meaurement framework is applied. - Note: The discussion of SMTC configuration is a separate issue. This agreement can be revisited if it is not aligned with further agreement in other WGs.

From RAN2 perspective, as the UE does not know the SSB transmission status (on-demand or broadcasted) of neighbour cells, it is difficult to measure the on-demand SSB per neighbour cell. Hence, it is reasonable to follow the above RAN4 agreement as is.

Proposal 3:	RAN2 to confirm the legacy measurement framework is applied for neighbour cell measurements.

### On-demand SSB activation/deactivation MAC CE

RAN1 agreements related to the new MAC CE format design are highlighted yellow:

RAN1#118 Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

RAN1#119 Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used

Considering the above RAN1 agreements, a new MAC CE should indicate an explicit activation or deactivation for on-demand SSB for the multiple SCells. It can be simply indicated by the bitmap field Ci corresponding to the SCellIndex i as in the legacy SCell Activation/Deactivation MAC CE. In addition, an index value of SSB periodicity or on-demand SSB configuration will be also included. The candidate values with index could be configured in SCell configuration by RRC. It depends on RAN1 conclusion how many bits will be needed for the index field per on-demand SSB SCell. This field can be optionally present. If the NW does not configure the multiple candidate values for all SCells, the NW does not include this field. To identify the presence of the optional field, a 1-bit in the first octet will be used to indicate whether it is followed by the OD-SSB configuration index field.

Proposal 4:	A new MAC CE can include a bitmap indication field to indicate multiple on-demand SSB activation and/or deactivation simultaneously.

Proposal 5:	A new MAC CE can optionally include an index field to indicate one of candidate value to applied for the on-demand SSB SCell.

Proposal 6:	A new MAC CE can include a 1-bit field in the first octet to indicate whether it is followed by the index field.

In summary, Figure.1 (with up to 7 SCells and 2 OD-SSB configurations) and Figure.2 (with up to 31 SCells and 2 OD-SSB configurations) are the examples of new MAC CE format design.

Note: the size of index field depends on the maximum number of on-demand SSB configurations provided by RRC.

Figure 1: Example of OD-SSB activation/deactivation MAC CE (up to 7 SCells)

Figure 2: Example of OD-SSB activation/deactivation MAC CE (up to 31 SCells)

# Conclusion

In this contribution, we have some discussions about on-demand SSB SCell operations, and the following proposals are made:

Framework of L3 measurements

Proposal 1:	RAN2 to adopt “Option1: Based on different measurement configuration when OD-SSB is transmitted” as on-demand SSB L3 measurements framework.

Proposal 2:	The UE should apply the SMTC window configured for on-demand SSB when the corresponding on-demand SSB transmission is activated.

Proposal 3:	RAN2 to confirm the legacy measurement framework is applied for neighbour cell measurements.

On-demand SSB activation/deactivation MAC CE

Proposal 4:	A new MAC CE can include a bitmap indication field to indicate multiple on-demand SSB activation and/or deactivation simultaneously.

Proposal 5:	A new MAC CE can optionally include an index field to indicate one of candidate value to applied for the on-demand SSB SCell.

Proposal 6:	A new MAC CE can include a 1-bit field in the first octet to indicate whether it is followed by the index field.

# References

- R2-2501331, Report from session on V2X/SL, R19 NES and MOB

- R4-2416916, WF on RRM requirements for Netw_Energy_NR_enh_Part1