3GPP TSG-RAN WG2 Meeting #129bis	R2-2502236

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.6.3

Source: 	China Telecom

Title:	Discussion on L1 event triggered measurement reporting

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In previous RAN2 meetings, the Rel-19 WI of Mobility enhancement was widely discussed and much progress has been made on measurement enhancements for LTM. However, there are still open issues on the detailed design for MR MAC CE. In this contribution, we would like to share our opinions on some of the remaining issues, including multiplexing of MR MAC CE and support of padding MR MAC CE.

- Discussion

Multiplexing of LTM MR MAC CE

In RAN2#129, the following agreements were reached on how to generate and multiplex the LTM MR MAC CE.

Agreements in RAN2#129

1. LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

2. The Measurement Report MAC CE is transmitted with a higher LCP than 'data from any Logical Channel, except data from UL-CCCH'.

3. The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR.

4. There is no restriction on which serving cell(s) L1 Measurement Report MAC CE can be transmitted on, i.e. not limited to SpCell.

It was agreed to include only one triggered measurement event (i.e. ltm-CSI-ReportConfigId) in one LTM MR MAC CE, which can be multiplexed in any available UL grant. However, it is possible to have multiple available L1 measurement results with different ltm-CSI-ReportConfigId at the time to transmit the MR MAC CE. In that case, the multiplexing of LTM MR MAC CE may need further discussion. The potential cases are listed as follows.

- Case 1: In one UL grant, there could be more than one normal MR MAC CEs and/or more than one truncated MR MAC CE to be multiplexed, if available.

- Case 2: In one UL grant, there could be at most one normal MR MAC CE and at most one truncated MR MAC CE to be multiplexed, if available.

- Case 3: In one UL grant, there could be at most one normal MR MAC CE or at most one truncated MR MAC CE to be multiplexed, if available.

For Case 1, if there is enough UL grant, multiple MR MAC CE could be multiplexed in one UL grant and each MR MAC CE includes only one triggered measurement event. Then, comparing to include multiple triggered measurement events in one MR MAC CE, it need to add more MAC subheaders, which consumes more bits. For Case 2, if there are more than one triggered measurement events to be reported, one more truncated MR MAC CE could be multiplexed in addition to one normal MR MAC CE, if available. In that case, the extra truncated MR MAC CE can help to indicate or request the gNB to allocate more UL grants for the pending measurement results. For case 3, it is restricted that at most one MR MAC CE or at most one MR MAC CE to be multiplexed in one UL grant. In that case, multiple triggered measurement events have to be reported in different MR MAC CEs in multiple UL grants, which may lead to the delay of the measurement reports.

To allow the above case 1 or case 2 can help reduce the overhead and provide more flexibility for L1 event triggered measurement reporting, we suggest RAN2 clarify whether to support the above cases.

Proposal 1: RAN2 clarifies whether to restrict at most one normal MR MAC CE or at most one truncated MR MAC CE to be multiplexed in one UL grant.

Proposal 2: RAN2 considers to support more than one normal or truncated MR MAC CEs to be multiplexed in one UL grant, if available.

No matter whether to restrict at most one MR MAC CE or at most one MR MAC CE to be multiplexed in one UL grant, there might be another open issue on how to select the reported measurement event among multiple triggered measurement events by the UE. For example, the measurement event that hasn’t been reported yet could be prioritised or the measurement event that has the most beams met the trigger conditions could be prioritised. Or the UE can select the event in the MR_list one by one. It is better to further clarify the UE behaviour on selecting the reported measurement event when there are multiple triggered events to be reported.

Proposal 3: RAN2 clarifies the UE behaviour on how to select the reported measurement event when there are multiple triggered events to be reported.

Whether to support padding MR MAC CE

To better utilize the UL resource, padding BSR is introduced to provide the UL data volume to the gNB when there are padding bits left. With similar motivation, it is also useful to have padding MR MAC CE so that more available L1 measurement results could be provided to the gNB on time.

Proposal 4: RAN2 considers to support padding MR MAC CE.

To support padding MR MAC CE, we need to consider the content of padding MR MAC CE. If it is restricted that at most one normal MR MAC CE or at most one truncated MR MAC CE to be multiplexed in one UL grant, it is meaningful to allow one extra padding MR MAC CE to be multiplexed in one UL grant when available. In that case, the padding MR MAC CE could include one event config ID and at least one triggered beam id (SSBRI or CRI) plus the corresponding L1 RSRP. If more than one normal or truncated MR MAC CEs are supported to be multiplexed in one UL grant, it is also meaningful to allow one extra padding MR MAC CE to be multiplexed in one UL grant when available, which can provide additional L1 measurement results to the gNB. Moreover, if there is no triggered event to be reported, the padding MR MAC CE could include any available L1 measurement results.

Proposal 5: The padding MR MAC CE could include one event config ID and at least one triggered beam id (SSBRI or CRI) plus the corresponding L1 RSRP, when there are triggered events to be reported.

Proposal 6: The padding MR MAC CE could include one event config ID and at least one any beam id (SSBRI or CRI) with available L1 RSRP, when there is no triggered event to be reported.

In last RAN2 meeting, the following agreements on Logical Channel Prioritization of MR MAC CE were reached.

Agreements in RAN2#129

1.	The Measurement Report MAC CE is transmitted with a higher LCP than 'data from any Logical Channel, except data from UL-CCCH'.

2.	The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR.

If padding MR MAC CE is introduced, we need further discuss the LCP of padding MR MAC CE. For the padding MR MAC CE including triggered event, it is more important than padding BSR. While for the padding MR MAC CE including any available beam which doesn’t meet the trigger condition, it is less important than padding BSR.

Proposal 7: The padding MR MAC CE with a triggered event is transmitted with a higher LCH priority than padding BSR MAC CE.

Proposal 8: The padding MR MAC CE with a non-triggered event is transmitted with a lower LCH priority than padding SL-BSR MAC CE.

- Conclusion

In this contribution, we discuss the remaining issues on L1 event triggered measurement reporting for Rel-19 LTM. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: RAN2 clarifies whether to restrict at most one normal MR MAC CE or at most one truncated MR MAC CE to be multiplexed in one UL grant.

Proposal 2: RAN2 considers to support more than one normal or truncated MR MAC CEs to be multiplexed in one UL grant, if available.

Proposal 3: RAN2 clarifies the UE behaviour on how to select the reported measurement event when there are multiple triggered events to be reported.

Proposal 4: RAN2 considers to support padding MR MAC CE.

Proposal 5: The padding MR MAC CE could include one event config ID and at least one triggered beam id (SSBRI or CRI) plus the corresponding L1 RSRP, when there are triggered events to be reported.

Proposal 6: The padding MR MAC CE could include one event config ID and at least one any beam id (SSBRI or CRI) with available L1 RSRP, when there is no triggered event to be reported.

Proposal 7: The padding MR MAC CE with a triggered event is transmitted with a higher LCH priority than padding BSR MAC CE.

Proposal 8: The padding MR MAC CE with a non-triggered event is transmitted with a lower LCH priority than padding SL-BSR MAC CE.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

RP-24xxxxx	draft Report of 3GPP TSG RAN WG1 meeting #116bis

3GPP TS 38.331 18.0.0: "NR; Radio Resource Control (RRC) protocol specification".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB

R2-24xxxx	Draft report of 3GPP TSG RAN WG2 meeting #127