3GPP TSG-RAN WG2 Meeting #129bis	           R2-2502239

Wuhan, China, April 7th – April 11th, 2025

Agenda item:	8.1.2.2

Source:	China Telecom

Title:	Further consideration on LCM for UE-sided model for BM use case

Document for:	Discussion

- Introduction

In this paper, we would like to further discuss LCM for UE-sided model for BM use case from the following aspects: applicability reporting, inference results reporting, functionality management in various mobility cases, and performance monitoring.

- Discussion

2.1 Applicability reporting

In last meeting, RAN2 agreed to support the reporting of applicability/inapplicability in initial report and subsequent reporting it only applicability it changed. And whether to report cause explicitly remains FFS. Regarding this issue, it is necessary to consider the possible reasons for changes of applicability/inapplicability and what benefits that explicit cause reporting can bring. For the change of applicability/inapplicability, it involves two directions: 1) applicable functionality becomes inapplicable; 2) inapplicable functionality becomes applicable. For the first direction, for example, when the UE reports that its available computing resources cannot support AI/ML functionality for BM use case, NW can issue a simpler Set B and Set A configuration. It seems meaningful to at least report the reason why the applicable functionality becomes inapplicable, so that NW may make the functionality status change to be applicable by issuing new configurations or modifying NW-sided parameters. The signalling overhead also needs to be taken into consideration. Whether to report or not maybe a trade-off between the benefits gained and the increased signalling overhead. Therefore, it is recommended to split this issue into two directions for discussion and give priority to discuss whether to report the reason why the applicable functionality becomes inapplicable.

Observation 1: The change of applicability/inapplicability functionality involves two directions:

1) applicable functionality becomes inapplicable;

2) inapplicable functionality becomes applicable.

Proposal 1: Take the benefits gained and the increased signalling overhead into consideration, it is recommended that RAN2 prioritize discussions on whether to explicitly report the reason why applicable functionality becomes inapplicable. Whether to report the reason why inapplicable functionality becomes applicable is FFS.

The applicability report is based on one or more of CSI-ReportConfig and/or sets of inference related parameters, and the above design may contain a large number of IEs. To reduce the signaling overheads, RAN2 has agreed that in the subsequent reporting, UE only reports only applicability it changed, which is incremental reporting. It is recommended that ToAddModList and ToReleaseList are used to indicate the added applicable and inapplicable functionality, respectively.

Proposal 2: It is recommended that ToAddModList and ToReleaseList are used to indicate the added applicable and inapplicable functionality.

When reporting applicable functionality, a prohibit timer should be set by NW to avoid frequent reports. In practical scenarios, there are some cases in which the state of the UE's applicable functionality changes rapidly; for example, when an application is started, the CPU occupancy and memory occupancy may increase significantly and then return to the average level when the loading of this application is completed. In those cases where the computational and storage resources of the UE may be suddenly high and suddenly low, if the prohibition timer is not activated, the UE may report the applicable functionalities frequently, increasing signaling overheads.

Proposal 3: When reporting applicable functionality, a prohibit timer can be set by NW to avoid frequent reports.

2.2 Inference results reporting

When applicable functionality is activated, the UE needs to send an inference result report based on NW configuration. At the RAN1#120 meeting, RAN1 has agreed to the following agreements regarding the inference results report.

Agreement For report content of inference results for UE-sided model for BM-Case 1, the RSRP of predicted beam(s)in the report of inference results, is the predicted RSRP, where the predicted RSRP is based on AI/ML output. Note: how to capture it in the spec is a separate discussion. Agreement For UE-side AI/ML model inference and BM-Case2, for the quantization of a RSRP value of inference results in a report over multiple future time instances, - the largest RSRP value based on prediction of all time instances is the reference RSRP, and differential RSRPs in the report are computed relative to the reference RSRP. - The time instance information of the beam with the largest RSRP are additionally indicated in the report.

In the inference results report for the UE-sided AI/ML model and BM-Case 1/2, the predicted RSRP is considered to be contained and its quantitative format has been specified.

Observation 2: RAN1 has agreed that the inference results report contains the predicted RSRP for the UE-sided AI/ML model and BM-Case 1/2.

However, for BM-Case 1, there are some AI/ML models that only output probabilities without outputting RSRP. For these models, the top-K beam ID with the highest probability can be carried in the inference results report. For BM-Case 2, the model is used to predict the future performance of the designated beam and it will output the predicted RSRP in future instances.

Proposal 4: For BM-Case 1, the top-K beam ID with the highest probability can be carried in the inference results report.

As RAN1 has started to discuss the content of the inference results report, we recommend that RAN2 start to consider the process as well as the signaling design of the inference results report, and follow the progress of RAN1.

Proposal 5: It is recommended that RAN2 consider the procedure as well as the signaling design of the inference results report, and follow the progress of RAN1.

2.3 Functionality management during mobility

At the RAN2#128 meeting, RAN2 has the following agreements regarding applicability reporting.

Agreements: 4. Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

Regarding functionality management during mobility, RAN2 has reached a consensus during RAN2#128 meeting that, as a baseline, the applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell. Nevertheless, it is imperative to note that diverse handover scenarios exist in practical deployments. For example, CHO allows NW to configure the handover conditions and resources of multiple target cells for UE in advance. In some complex mobility scenarios, such as high-speed mobility scenarios or areas with unstable signal coverage, the UE handover between neighboring cells may occur very rapidly and CHO can deal with the problem.

In addition, LTM, which mainly utilizes the information from the physical layer and the link layer to trigger handover, is also noticed. The physical layer and the link layer can detect signal changes more quickly, such as rapid fluctuations in signal strength or instantaneous deterioration of link quality. Compared with the handover based on higher layers (such as the RRC layer), LTM can respond more rapidly. Whether the agreed baseline can be applied to other handover scenarios needs to be discussed in order to prevent some abnormal situations that require special consideration.

Proposal 6: It is recommended that RAN2 discuss the applicable functionality reporting for other handover types, such as CHO, LTM, etc.

If Proposal 6 is supported, handover will serve as the baseline. Assuming we are discussing the CHO scenario, the question arises as to whether the applicable functionality reporting in CHO is still supported by the same RRC procedure. For multiple potential target gNBs, consideration must be given to how to issue the network-side additional conditions and/or the inference configuration. Moreover, when the UE evaluates the execution conditions for CHO, it also needs to consider whether additional attention needs to be paid to the network-side additional conditions and/or inference configurations related to the target gNB to successfully complete the handover process.

Proposal 7: Taking handover as the baseline, discuss whether the applicable functionality reporting in CHO is still supported by the same RRC procedure.

2.4 Performance monitoring

RAN1 has discussed performance monitoring for several meetings and the following are related agreements:

RAN1#117: Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring RAN1#118: Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window) For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4. RAN1#118bis: Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), consider the following options with potential down selection for the configuration for monitoring: - Option 1: The resource set(s) for monitoring and report configuration for monitoring are configured (when applicable) within CSI report configuration used for inference - FFS: the resource set(s) for monitoring - UE measures the resource set(s) for monitoring. - FFS how/when to report the monitoring results. - Option 2: Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The dedicated report configuration used for monitoring links to an inference report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - UE measures the resource set(s) for monitoring. - FFS when to report the monitoring results. RAN1#119: Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

According to RAN1 agreements, two types of performance monitoring are supported, i.e. NW-side performance monitoring and UE-assisted performance monitoring. The NW-side performance monitoring is that the UE is configured or triggered by the network to report measurement results of the resource set for monitoring, and then the network calculate the performance metric based on reported measurement results. The UE-assisted performance monitoring is that the UEs calculate the performance metric based on measurement and report the calculated performance metric to the network. Furthermore, RAN1 support reusing CSI framework to configure monitoring result reporting via L1 signaling.

From our point of view, RAN2 can discuss the network control of performance monitoring from the following aspects:

- When to report monitoring result

- Who determine whether the monitoring performance meets the requirement or not

- Performance feedback

When to report monitoring result

For NW-side performance monitoring, since UEs do not calculate the performance metric, and the content of monitoring result is L1-RSRP and/or RS index, the network control of monitoring result report can be supported with existing CSI report types. Because the network is aware of the calculated performance metric, the network can react when the performance metric is low enough.

For UE-assisted performance monitoring, UEs are aware of the calculated performance metric. It is possible that the network is only interested in the situation where the performance metric calculated by UEs is low. It is beneficial that UEs report monitoring result when the calculated performance metric is lower than threshold, even though the network has not release or deactivate the CSI report configuration dedicated for monitoring. Therefore, the network is able to configure UEs to stop monitoring in advance, which is good for UE power consumption and the overhead of monitoring result reporting. If UEs are not triggered to report the calculated performance metric in advance, the network control of monitoring result report can be supported with existing CSI report, similar to NW-side performance monitoring.

Proposal 8: For UE-assisted performance monitoring, UE report monitoring result when trigger conditions are fulfilled, such as performance metric calculated by UE is lower than a configured threshold.

Who determine whether the monitoring performance meets the requirement or not

In our understanding, the performance requirement is determined by the network, e.g. prediction accuracy. For NW-side performance monitoring, it is natural that the network determine whether the calculated performance metric meets the requirement or not. However, for UE-assisted performance monitoring, it is possible that UE or the network determines whether the calculated performance metric meets the requirement or not, given that the network can configure the required performance metric to UE in advance.

Proposal 9: For NW-side performance monitoring, the network determines whether the monitoring performance metric meets the requirement or not.

Proposal 10: For UE-assisted performance monitoring, UE or the network can determine whether the monitoring performance metric meets the requirement or not.

Performance feedback

In previous discussion related to applicable functionality for UE-sided model, it is assumed that UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. In our understanding, UE need to be aware of whether the monitoring performance metric meet the requirement or not, in order to confirm whether the model be monitored is still available or not. Furthermore, UE can confirm if there is other available model and determine whether the corresponding functionality is still applicable or not. Based on the above reason, UE need to be notified when the network determines that the monitoring performance metric does not meet the requirement based on monitoring result report.

Proposal 11: UE need to be notified when the network determines that the monitoring performance metric does not meet the requirement.

3	Conclusions

Based on the discussion above, we have the following proposals.

Observation 1: The change of applicability/inapplicability functionality involves two directions:

1) applicable functionality becomes inapplicable;

2) inapplicable functionality becomes applicable.

Proposal 1: Take the benefits gained and the increased signalling overhead into consideration, it is recommended that RAN2 prioritize discussions on whether to explicitly report the reason why applicable functionality becomes inapplicable. Whether to report the reason why inapplicable functionality becomes applicable is FFS.

Proposal 2: It is recommended that ToAddModList and ToReleaseList are used to indicate the added applicable and inapplicable functionality.

Proposal 3: When reporting applicable functionality, a prohibit timer can be set by NW to avoid frequent reports.

Observation 2: RAN1 has agreed that the inference results report contains the predicted RSRP for the UE-sided AI/ML model and BM-Case 1/2.

Proposal 4: For BM-Case 1, the top-K beam ID with the highest probability can be carried in the inference results report.

Proposal 5: It is recommended that RAN2 consider the procedure as well as the signaling design of the inference results report, and follow the progress of RAN1.

Proposal 6: It is recommended that RAN2 discuss the applicable functionality reporting for other handover types, such as CHO, LTM, etc.

Proposal 7: Taking handover as the baseline, discuss whether the applicable functionality reporting in CHO is still supported by the same RRC procedure.

Proposal 8: For UE-assisted performance monitoring, UE report monitoring result when trigger conditions are fulfilled, such as performance metric calculated by UE is lower than a configured threshold.

Proposal 9: For NW-side performance monitoring, the network determines whether the monitoring performance metric meets the requirement or not.

Proposal 10: For UE-assisted performance monitoring, UE or the network can determine whether the monitoring performance metric meets the requirement or not.

Proposal 11: UE need to be notified when the network determines that the monitoring performance metric does not meet the requirement.