3GPP TSG RAN WG2 #129bis		R2-2502263

Wuhan, CN, Apr 7 – 11, 2025

Title:	LCM for UE-sided Model for BM

Agenda item:	8.1.2.2

Source:	Google

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#129 meeting, there were some agreements and FFSs on the LCM for UE-sided model for BM case.

Agreements

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

- Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5). The provided periodic CSI configuration should be consistent with reported UE capabilities

In RAN2#129 meeting, there were some agreements and FFSs for UE-side data collection.

- Agreements - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

In this paper, we further discuss the remaining FFSs and give related considerations.

# 2 Discussion

## Applicability Reporting

As agreed in RAN2#129 meeting, the NW provides periodic CSI configurations in step 3 for opt A. If multiple periodic CSI report configurations are applicable, the UE can activate all the applicable functionalities upon reporting them via RRCReconfigurationComplete message in step 4. It is essential for the NW to make sure that the configured periodic CSI configurations are consistent with reported UE capabilities. Regarding the reported UE capabilities relevant to periodic CSI report configurations, the details are currently unclear. RAN2 should clarify which UE capabilities the NW can consider during the provision of periodic CSI report configurations.

Currently, there are CSI report framework related capabilities, which define UE capabilities for simultaneous CSI report setting and processing capabilities. With the knowledge of them, the NW can ensure provided CSI configurations to be aligned with these UE capabilities. Similar principles should be considered for the NW to configure periodic CSI configurations for prediction.

Proposal 1: RAN2 should study what UE capabilities (e.g., CSI setting/processing capabilities) the NW needs to consider for ensuring consistent periodic CSI configurations.

For both option A and B, the NW needs to provide a full inference configuration to activate an applicable functionality. As an example in option A, we reuses parameters in the field CSI-reportConfig for full inference configuration. When providing CSI report configurations for prediction, it leads to substantial signalling overhead. To improve the efficiency of applicability reporting, it would be beneficial for the UE to report its supported or preferred inference configurations or parameters to the NW.

The preference for inference configurations/parameters can be initiated early and used for reference when the NW decides NW configured inference configurations/parameters. Alternatively, the preference for inference configurations/parameters can be initiated if none of functionality is applicable based on received NW configured inference configurations/parameters. The NW can update inference configurations/parameters correspondingly.

Proposal 2: The UE can send its preference for inference configurations/parameters.

Based on the agreements in RAN2#128 meeting, the updated report is triggered if a functionality becomes from non-applicable to applicable.

When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.

On the contrary, if the conditions to determining the applicability determination change over time, an initially applicable functionality may become non-applicable. This information is crucial for the NW to avoid incorrect activation configurations for these functionalities. Thus, it is necessary that the UAI is triggered when an applicable functionality becomes non-applicable.

Proposal 3: Upon applicability changes from applicable to non-applicable, the UE sends the applicability report via UAI.

From NW perspective, it expects the latest applicability information for further functionality control. The reasons for the changes is not necessary.

Proposal 4: When the applicability information changes (applicability to inapplicability, or inapplicability to applicability), no explicit cause is reported.

In RAN2#128 meeting, RAN2 discussed UE behaviours when the UE detects an activated applicable functionality becoming non-applicable. The UE doesn’t autonomously deactivate the non-applicable functionality which is activated currently. Instead, the NW is expected to deactivate the non-applicable functionality and the UE waits for the deactivation command from the NW.

When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable

As previously mentioned, when an activated applicable functionality becomes non-applicable, the UE reports this to the NW. The NW is expected to deactivate the active functionality. However, this process introduces latency for the UE to deactivate the non-applicable functionality. During this delay, the UE continues operating with the (now non-applicable) activated functionality for inference. This might lead to degraded inference accuracy compared to traditional, non-AI measurements.

To mitigate the latency and improve the system performance, the NW can pre-emptively send a deactivation indication before receiving the non-applicable report. If the deactivation indication is present, UE can deactivate the activated functionality when it becomes non-applicable. Subsequently, the UE reports the non-applicable information or the deactivation information of the activated functionality to the NW. If the deactivation indication is absent, the UE reports the non-applicable functionality to the NW and waits for the NW to deactivate the functionality via a subsequent response message.

Proposal 5: The NW can send a deactivation indication to the UE prior to receiving a non-applicability report.  The UE's behaviour depends:

- Deactivation Indication Present: The UE deactivates the previously activated functionality when it becomes non-applicable and informs the NW of the deactivation.

- Deactivation Indication Absent: The UE sends a non-applicability report and expects the NW to deactivate the functionality in a subsequent response.

To enable the flexible request and reporting for the applicable functionality, the NW can configure the request in the RRC Resume and Reestablishment messages. Correspondingly, the UE can indicate the applicable functionality information in the RRC Resume/Reestablishment Complete messages respectively.

Proposal 6: RRC Resume and Reestablishment procedures can be used for the configuration of inference configurations/parameters and applicability reporting.

## Data Collection for UE-sided Model

### Data Collection Configuration Procedure

Data Collection Configuration with UE Request

In this approach, the NW firstly configures that a UE is allowed to initiate request for data collection. Then, the UE initiates a data collection request. Based on UE capability, the UE may support model training for specific use case(s), features or feature groups. In a simple method, the UE provides its data collection capabilities and the NW provides data collection configurations aligned with these capabilities upon receiving a request indicator for data collection.

However, the UE might only want to perform data collection for some specific model(s) training. With the above method, it is possible that the data collection configurations from the NW is for the models(s) which is not the preferred one but within UE’s capability. To improve the efficiency for UE-side data collection, it is straightforward to for the UE to provide its suggested data collection configuration/associated IDs (if specified and known by the UE)/parameters.

Proposal 7: For data collection configuration with UE request, the UE provides suggested data collection configurations/associated IDs (if specified and known by the UE)/parameters in the data collection request.

Upon receiving the data collection request, the NW can configure the relevant data collection configuration/associated IDs (if needed) in the data collection configuration (case a shown in figure 1). Furthermore, to guarantee that the suggested data collection configuration/associated IDs from the UE can be supported by the NW, one option is for the NW to send supported data collection configuration information to the UE. This information specifies which data collection types/data collection configuration/associated IDs can be supported in the current NW. When initiating a data collection request, the UE must consider this supported data collection configuration information. This also helps to avoid potential inconsistencies between the UE's preferences and the configurations supported by the NW.

Proposal 8: For data collection configuration with UE request, the NW configures supported data collection configuration information to indicate the supported data collection configuration/associated IDs.

In another case b (as shown in figure 1), when receiving the data collection request, the NW may decide not to send related training data collection configuration to the UE. To avoid frequent data collection request from the UE, the prohibit timer similar as onDemandSIB-RequestProhibitTimer should be considered. The prohibit timer can be provided to the UE in system information or a dedicated RRC message.

Proposal 9: For data collection configuration with UE request, a prohibit timer is introduced to prevent the UE from immediately re-initiating a data collection request.

Based on the above discussion, we summarize a possible illustration for the above scheme in the following figure 1.

Figure 1 Data collection configuration with UE request

Data Collection Configuration without UE Request

In the scheme, the UE will not initiate a request for data collection or provide suggested data collection configurations/associated ID. To ensure the NW configures the data collection configurations supported by the UE, it is an intuitive method for the UE to report its data collection associated capabilities to the NW.  If the NW supports elated data collection configurations based on these capabilities and has sufficient resources, it can proactively provide the related data collection configuration to the UE. This improves configuration efficiency and avoids potential UE malfunction.

Proposal 10: For data collection configuration without UE request, the UE provides data collection related capability to the NW.

Figure 2 Data collection configuration without UE request

### Failure Case for UE-side Data Collection

In RAN2#128 meeting, there was an FFS whether to indicate the NW if the UE cannot perform data collection.

FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

Based on the previous solutions, it is noticed that when the UE receives the data collection configuration from NW for UE-side data collection, the NW concurrently provides the CSI measurement resources. When the UE is unable to perform data collection based on received configuration, it is essential for the UE to notify the NW. The notification allows the NW to stop providing CSI measurement resources and ultimately conserves network resources.

Proposal 11: The UE sends an indication to the NW if unable to perform the configured data collection.

Moreover, the UE can provide additional information, e.g., specifying the reason for its inability to perform data collection. The potential reasons can include abnormal UE states (e.g., low power, low memory, overheating, or inconsistency of UE/NW-side associated ID).

Proposal 12: The UE sends a failure cause to the NW (e.g., low power, low memory, overheating, or inconsistency of UE/NW-side associated ID).

# 3 Conclusion

In this contribution, we discussed the potential solutions on the remaining FFSs and gave more considerations on the LCM for UE-sided model for BM. The related proposals are summarized below:

Applicability Reporting

Proposal 1: RAN2 should study what UE capabilities (e.g., CSI setting/processing capabilities) the NW needs to consider for ensuring consistent periodic CSI configurations.

Proposal 2: The UE can send its preference for inference configurations/parameters.

Proposal 3: Upon applicability changes from applicable to non-applicable, the UE sends the applicability report via UAI.

Proposal 4: When the applicability information changes (applicability to inapplicability, or inapplicability to applicability), no explicit cause is reported.

Proposal 5: The NW can send a deactivation indication to the UE prior to receiving a non-applicability report.  The UE's behaviour depends:

- Deactivation Indication Present: The UE deactivates the previously activated functionality when it becomes non-applicable and informs the NW of the deactivation.

- Deactivation Indication Absent: The UE sends a non-applicability report and expects the NW to deactivate the functionality in a subsequent response.

Proposal 6: RRC Resume and Reestablishment procedures can be used for the configuration of inference configurations/parameters and applicability reporting.

UE-sided Data Collection

Proposal 7: For data collection configuration with UE request, the UE provides suggested data collection configurations/associated IDs (if specified and known by the UE)/parameters in the data collection request.

Proposal 8: For data collection configuration with UE request, the NW configures supported data collection configuration information to indicate the supported data collection configuration/associated IDs.

Proposal 9: For data collection configuration with UE request, a prohibit timer is introduced to prevent the UE from immediately re-initiating a data collection request.

Proposal 10: For data collection configuration without UE request, the UE provides data collection related capability to the NW.

Proposal 11: The UE sends an indication to the NW if unable to perform the configured data collection.

Proposal 12: The UE sends a failure cause to the NW (e.g., low power, low memory or overheating, inconsistency of UE/NW-side associated ID).