3GPP TSG-RAN WG2 Meeting #129bis	R2-2502266

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.7.4.2

Source:	Nokia, Nokia Shanghai Bell

Title:	DSR enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#129:

Agreements on DSR enhancements - One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE. - FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. - We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size. - Different LCGs may be configured with different number of reporting thresholds. - If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold. - Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold). - Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

# 2	Discussion

## 2.1	Reporting control and retransmission PDUs

Current running CR has not covered the behavior for reporting control and retransmission PDUs. Clause 5.15 of TS 38.323 indicates the transmitting PDCP entity shall consider the following as delay-critical PDCP data volume for Rel-18 DSR:

- the delay-critical PDCP SDUs for which no PDCP Data PDUs have been constructed; - the PDCP Data PDUs that contain the delay-critical PDCP SDUs and have not been submitted to lower layers; - the PDCP Control PDUs; - for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13; - for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

Then, both PDCP Control PDUs and AM DRBs containing PDCP Data PDUs for retransmissions are considered as delay-critical. An important clarification is needed to determine within which reporting thresholds each of these PDCP PDUs should be reported.

Since the PDCP control PDUs carry control plane data, in our views, it is required their data volume to be included in the first reporting threshold (i.e., smallest reporting threshold). It will guarantee their transmission together with the data having the smallest remaining time, as the scheduler is being aware of the data urgency and take the proper actions to at least transmit them, in case the number of resources is limited.

Proposal 1: Control PDUs data volume are included in the first reporting threshold.

Another open issue is whether only Control PDU can be included as in DSR if there is no other delay critical data for the LCG. We understand it is not included for Rel-18 since there is no remaining time for control PDU. Same could be applied for Rel-19.

Proposal 2: If an LCG is with only Control PDU, they are not reported in DSR.

On the other hand, the PDCP Data PDUs for retransmissions are directly linked to their respective PDCP discard timers. In Rel-18, those PDCP Data PDUs are included in delay-critical data even if their remaining time is above the reporting threshold. However, for the Rel-19 enhanced DSR, the inclusion of multiple reporting thresholds gives more manurable margin to include the data volume of these PDUs in the corresponding reporting thresholds. As the PDCP discard timers reduce, they will be changing the reporting threshold. Therefore, we consider they should be treated as any other data PDUs, and their data volume should be included in the respective reporting threshold to allow finer granularity for retransmission data as well which is the original purpose of introducing Rel-19 DSR. However, we see no big issue to report them all in the first reporting threshold either.

Proposal 3: Retransmission PDU data volume is included in the reporting threshold corresponding to their respective remaining time value.

## 2.3	DSR MAC CE details

The following agreement about only to include entries with BS size > 0 is currently missing in the running CR.

For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - • Buffer size of data volume in each portion - • Shortest remaining time among PDCP SDUs buffered in each portion.

Some discussions during the running CR review without conclusion as zero size BS is also allowed for Rel-18, but it should only be the case when the UE is not able to change DSR during LCP. However, we believe it should be made clear that it is not intended behaviour that the UE always reports fixed size DSR with number of configured thresholds even though some of the entries are with BS zero. The UE shall not include zero size BS if it is already so based on the buffer status before LCP, better UE implementation can determine not to include zero BS based on buffer status after LCP.

Proposal 4: confirm Rel-19 DSR is flexible size that only reports entries with non-zero BS (at least based on the buffer status before LCP, better UE implementation can determine not to include zero BS based on buffer status after LCP).

## 2.4	Non-delay critical data ahead in the queue

Regarding non-delay critical data ahead of delay critical data in the queue, it was agreed in RAN2#128 that the UE may support (UE capability) including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

However, the Rel-19 DSR may also report non-delay critical data (also called e.g. delay-reporting data) since the triggering threshold, which is also used for determining if the data is delay critical, may be lower than at least one of the reporting thresholds.

Therefore, the RAN#128 agreement on non-delay critical data ahead of delay critical data should be generalized to non-delay critical data ahead of non-delay critical data, e.g. delay reporting data. E.g., the buffer size calculation for a given remaining time range should also include data that is ahead of any PDU whose associated remaining time is within the reported remaining time range.

During the PDCP running CR discussions, there were different understandings on how it works with PDU set. To better illustrate this, let’s consider an example where data belonging to different PDU sets (PDU set#0, #1, and #2 in Figure2.2-1) arrive in the UE buffer interlaced. We assume that PDU set based discard (pdu-SetDiscard-r18) is configured.

Figure 2.2-1: DSR with low-importance and high-importance data

In the example, the UE is configured with 3 reporting thresholds: RTT0, RTT1 and RTT2. The triggering threshold is assumed to be equal to RTT0. When the DSR is triggered, there is data of PDU set #1 (i.e. PDU #4), whose associated remaining time RT2 is in the range [RTT0, RTT1), which is ahead of data of PDU set#0, whose associated remaining time 
RT0 is in the range [0, RTT0). If the data ahead of PDU #4 with remaining time larger than the remaining time of PDU #4 (i.e. PDUs #2 and #3) is not included in the BS calculation of the range [RTT0, RTT1), then the DSR with multiple reporting thresholds would look like the one depicted in Table 2.2-1.

Table 2.2-1: Buffer size in multiple reporting thresholds

| Remaining Time range | Reported Remaining Time | Reported Buffer Size |
| --- | --- | --- |
| [0,RTT0) | RT0 | S0+S1+S4 |
| [RTT0,RTT1) | RT2 | S2+S3+S5+S6 |

Then, the gNB may think that allocating a transport block size of S0+S1+S4 would be enough for the UE to transmit data with associated remaining time RT0. However, this is not the case. Therefore, a better option for the DSR with multiple reporting thresholds would be the one shown in Table 2.2-2.

Table 2.2-2: Buffer size in multiple reporting thresholds

| Remaining Time range | Reported Remaining Time | Reported Buffer Size |
| --- | --- | --- |
| [0,RTT0) | RT0 | S0+S1+S2+S3+S4 |
| [RTT0,RTT1) | RT2 | S5+S6 |

Proposal 5: Include in the BS calculation for DSR corresponding to a remaining time range all PDUs until the last PDU belonging to any PDU set whose associated remaining time is within the remaining time range.

# 3	Conclusion

Remaining issues on DSR enhancements are discussed in this contribution with the following observations/proposals proposed:

Proposal 1: Control PDUs data volume are included in the first reporting threshold.

Proposal 2: If an LCG is with only Control PDU, they are not reported in DSR.

Proposal 3: Retransmission PDU data volume is included in the reporting threshold corresponding to their respective remaining time value.

Proposal 4: confirm Rel-19 DSR is flexible size that only reports entries with non-zero BS (at least based on the buffer status before LCP, better UE implementation can determine not to include zero BS based on buffer status after LCP).

Proposal 5: Include in the BS calculation for DSR corresponding to a remaining time range all PDUs until the last PDU belonging to any PDU set whose associated remaining time is within the remaining time range.