3GPP TSG-RAN WG2 #129bis meeting	R2-2502295

# Wuhan, China, 7th Apr – 11th Apr, 2025

Source:		NEC

Agenda item:		8.7.4.1	LCP enhancements

Title:		Considerations on LCP enhancements for XR

Document for:		Discussion and decision

# 1. Introduction

In the RAN2#129 meeting, agreements [1] on LCP enhancements for XR were made as below:

Only one additional priority is configured to an LCH for LCP enhancement. We keep an existing agreement (remaining time threshold is configured per LCH) There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination No additional PBR is needed for priority adjusted data FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

In this contribution, we will provide our views on the remaining issues for LCP procedure.

# 2. Discussion

2.1 LCP enhancements for priority-adjusted data.

In the RAN2#129 meeting, it was discussed whether an LCH with an additional priority is allowed to be transmitted even if Bj < 0.

FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

From our understanding, delay-sensitive LCP is introduced to ensure the transmission of priority-adjusted data, thereby preventing the data from becoming outdated. From this perspective, it is reasonable to allow an LCH with an additional priority to be transmitted even if Bj < 0. However, to avoid starvation of other LCHs, only data with remaining time below the configured threshold can be transmitted when Bj < 0. In other words, we only guarantee the transmission of priority-adjusted data under such conditions.

Proposal 1: For an LCH, only data with remaining time below the configured threshold can be transmitted when Bj < 0.

The concept of PDU Set has been introduced in Rel-18, and all PDUs belonging to a PDU Set may be needed for the decoding of the corresponding information unit. In this situation, all PDUs belonging to a PDU Set shall be discarded as a whole if at least one PDU is discarded. Similarly, as all PDUs belonging to a PDU Set share the same PSDB, to avoid only part of a PDU Set being multiplexed in a MAC PDU but all PDUs are needed for decoding, which still brings extra delay for a PDU Set, RAN2 should consider maximizing the transmission of a PDU Set during delay-awareness LCP procedure.

Proposal 2: Maximize the transmission of a PDU Set during delay-awareness LCP procedure when all PDUs are needed for a PDU Set.

The MAC entity shall, when a new transmission is performed: 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. NOTE 1: The value of Bj can be negative.

In the legacy LCP process, logical channels with equal priority should be served equally. However, with the enhanced delay-awareness LCP, there's a need to discuss whether an LCH with priority-adjusted data should be served before one without such data when both have equal priority. To ensure the timely transmission of delay-critical data, we prefer to serve the LCH with priority-adjusted data first in this scenario, and a note can be appended for this.

Proposal 3: RAN2 to discuss whether an LCH with priority-adjusted data should be served before one without such data during LCP when both have equal priority.

Regarding the configured UL grant originally for XR traffic, the delay-awareness LCP mechanism can be applied to multiplex the priority-adjusted data with a higher priority. However, there exist other types of periodic traffic (e.g., voice or URLLC service) that are configured with configured UL grant as well. For these services, if delay-awareness LCP is applied to the configured UL grants, priority-adjusted data of XR traffic may occupy the resources originally reserved for the periodic voice or URLLC traffic, which shall bring extra delay for these services and cause a bad user experience. For this, we propose that different configured UL grant configurations can apply different strategies of whether to apply the delay-awareness LCP mechanism.

Proposal 4: Different configured UL grant configurations can apply different strategies of whether to apply delay-awareness LCP mechanism.

In the RAN2#125bis meeting, it was agreed to consider the potential impact of delay-awareness LCP on traffic from SRBs. From our understanding, we should discuss SRB 0 and SRBs other than SRB 0 separately.

RAN2 should consider potential impact on traffic from SRBs.

For SRB0 (i.e., data from UL-CCCH), it always has a higher priority than delay-critical data (i.e., data from any Logical Channel, except data from UL-CCCH), which has already been specified by the priority order of logical channels in current specification [2]. There is no need to break the rules.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first): - MAC CE for C-RNTI, or data from UL-CCCH; - MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation; … - MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment; - data from any Logical Channel, except data from UL-CCCH; … NOTE 2: Prioritization among MAC CEs of same priority is up to UE implementation.

For SRBs other than SRB 0, a priority value is associated with the corresponding logical channel, either by applying the default configuration or an explicit configuration. From our understanding, by configuring a proper priority value for the logical channel with delay-critical data, the network can control the impact and the priority order between logical channels with delay-critical data and logical channels for SRBs other than SRB 0. So it can left to the network’s implementation and there should be no specification impact.

Proposal 5: It can be left to the network’s implementation to configure the priorities of LCHs with delay-critical data and LCHs for SRBs other than SRB 0.

2.2 Intra-UE prioritization

As shown in the cited texts from the MAC specification, in intra-UE prioritization procedure, LCH priority is used to determine the priority of a UL grant when there is overlapping between UL grants or between SR and UL grants.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

Additionally, for intra CG Prioritization, LCH priority is used to determine the priority of HARQ process.

For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize the HARQ Process ID with the highest priority, where the priority of HARQ process is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection among initial transmission and retransmission with equal priority, the UE shall prioritize retransmissions before initial transmissions.

Thus, for the specification impact, perhaps we only need to specify which priority value shall be used for an LCH. Specially, in intra-UE/CG prioritization, if an LCH has priority-adjusted data, its priority is determined by the additional priority value; Otherwise, its priority is determined by the legacy priority value (e.g., LogicalChannelConfig -> priority).

Proposal 6: In intra-UE/CG prioritization, if an LCH has priority-adjusted data, its priority is determined by the additional priority value; Otherwise, its priority is determined by the legacy priority value.

Whether intra-UE prioritization should also use the additional LCH priority for SR priority determination is still FFS. Firstly, we would like to clarify that there is no concept of SR priority. According to the MAC specification, the priority of a UL grant is compared with the priority of the LCH triggering the SR. Therefore, our concern should be the priority of the LCH triggering the SR, rather than the SR priority.

Observation 1: There is no concept of SR priority. Our concern should be the priority of the LCH triggering the SR.

Furthermore, we advocate that the additional priority value should also be applied in the SR case. This allows SR associated with LCH with priority-adjusted data to be prioritized in case of overlapping UL grants/SRs. Specially, in intra-UE prioritization, if an LCH triggering an SR has priority-adjusted data, its priority is determined by the additional priority value; Otherwise, its priority is determined by the legacy priority value (e.g., LogicalChannelConfig -> priority).

Proposal 7: In intra-UE prioritization, if an LCH triggering an SR has priority-adjusted data, its priority is determined by the additional priority value; Otherwise, its priority is determined by the legacy priority value.

# Summary

This contribution provides our analysis on the enhancements of LCP procedure, and has the following proposals:

Proposal 1: For an LCH, only data with remaining time below the configured threshold can be transmitted when Bj < 0.

Proposal 2: Maximize the transmission of a PDU Set during delay-awareness LCP procedure when all PDUs are needed for a PDU Set.

Proposal 3: RAN2 to discuss whether an LCH with priority-adjusted data should be served before one without such data during LCP when both have equal priority.

Proposal 4: Different configured UL grant configurations can apply different strategies of whether to apply delay-awareness LCP mechanism.

Proposal 5: It can be left to the network’s implementation to configure the priorities of LCHs with delay-critical data and LCHs for SRBs other than SRB 0.

Proposal 6: In intra-UE/CG prioritization, if an LCH has priority-adjusted data, its priority is determined by the additional priority value; Otherwise, its priority is determined by the legacy priority value.

Observation 1: There is no concept of SR priority. Our concern should be the priority of the LCH triggering the SR.

Proposal 7: In intra-UE prioritization, if an LCH triggering an SR has priority-adjusted data, its priority is determined by the additional priority value; Otherwise, its priority is determined by the legacy priority value.

# References

[1] Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece, 17th Feb - 21st Feb, 2025

[2] TS 38.321, Medium Access Control (MAC) protocol specification, v18.3.0