3GPP TSG-RAN WG2 #129bis meeting	R2-2502296

# Wuhan, China, 7th Apr – 11th Apr, 2025

Source:		NEC

Agenda item:		8.7.4.2	DSR enhancements

Title:		Considerations on DSR enhancements for XR

Document for:		Discussion and decision

# 1. Introduction

In the RAN2#129 meeting, agreements [1] on DSR enhancements for XR were made as below:

One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE. FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size. Different LCGs may be configured with different number of reporting thresholds. If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold. Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold). Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

In this contribution, we will further discuss the remaining issues related to the enhanced DSR.

# 2. Discussion

Enhanced DSR

It was agreed in the RAN2#129 meeting that truncated DSR and fallback to legacy DSR is not supported in case of limited PUSCH grant size. However, we still need to discuss how to report enhanced DSR if the UL-SCH resources cannot accommodate an enhanced DSR MAC CE plus its subheader, and the following options can be taken into consideration:

- Option 1: Trigger an SR. This behaves similarly to legacy DSR.

- Option 2: Merge pairs of delay information until available UL-SCH resources can accommodate the enhanced DSR MAC CE and its subheader.

- Option 3: Report at most one pair of delay information for an LCG, which may include all the delay information.

Proposal 1: RAN2 to consider the following options if the UL-SCH resources cannot accommodate an enhanced DSR MAC CE plus its subheader:

- Option 1: Trigger an SR.

- Option 2: Merge pairs of delay information.

- Option 3: Report at most one pair of delay information for an LCG.

In Rel-18 DSR, the definition of a PDCP SDU associated with a DSR (e.g., legacy DSR) has been introduced, which is used when determining whether a pending DSR shall be cancelled.

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR. A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

Similarly, a PDCP SDU associated with an enhanced DSR should be defined as well. To ensure that all delay-critical data's delay information is accurately reported without redundancy, and to distinguish the operations (e.g., cancellation) of legacy DSR and enhanced DSR, we must prevent PDCP SDUs from being associated with both DSR types. Therefore, it is essential to differentiate PDCP SDUs associated with enhanced DSR from those with legacy DSR. For example, the following definition can be considered as a starting point.

A PDCP SDU is considered to be associated with an enhanced DSR if it has not been transmitted in any MAC PDU and is a PDCP SDU with remaining value of the running PDCP discardTimer below the largest configured reporting threshold and associated with the logical channel which triggered the DSR.

Proposal 2: RAN2 agrees to introduce the definition of PDCP SDUs associated with an enhanced DSR, distinct from those associated with legacy DSR.

Enhanced DSR and legacy DSR

RAN2 has agreed that triggering and reporting thresholds for enhanced DSR can be configured without constraints. In other words, the largest configured reporting threshold for enhanced DSR may be greater than, equal to, or lower than the remainingTimeThreshold for the legacy DSR.

Figure 1: issues when legacy and enhanced DSR coexist

If both DSR types coexist, the following issues should be considered:

- If the largest configured reporting threshold is greater than or equal to remainingTimeThreshold, the reported enhanced DSR shall cover all the delay information of legacy DSR. In this case, the legacy DSR can be cancelled if enhanced DSR is triggered.

- If the largest configured reporting threshold (e.g., T2) is lower than the remainingTimeThreshold (e.g., T3), as illustrated in Figure 1,

  - If legacy DSR is cancelled or not triggered upon triggering the enhanced DSR, then the delay information of unreported residual delay-critical data with remaining time range [T2, T3) will be missed.

  - If legacy DSR is not cancelled upon triggering the enhanced DSR, and there is unreported residual delay-critical data with remaining time range [T2, T3) after reporting of enhanced DSR, then legacy DSR shall also be reported. In this case, legacy DSR should avoid reporting redundent delay information with the remaining time range [0, T2) that has been reported by enhanced DSR.

Observation 1: The coexistence of legacy and enhanced DSR may result in unreported residual delay-critical data and the reporting of redundant delay information.

As described above, the coexistence of legacy and enhanced DSR will pose further issues and increase complexity. Consequently, to simplify the implementation and avoid intertwining the procedures related to enhanced DSR and legacy DSR, we propose that only one DSR type be used at any given time. Specifically, only enhanced DSR MAC CE shall be used when at least one LCG is configured with multiple reporting thresholds.

Proposal 3: Only enhanced DSR MAC CE shall be used when at least one LCG is configured with multiple reporting thresholds.

Delay/non-delay critical data volume

It was agreed in RAN2#128 meeting that non-delay critical data ahead of delay critical data shall be counted in the buffer size calculation for the enhanced DSR.

The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

Given that buffer size of both non-delay critical and delay critical data may be reported by enhanced DSR, there exists a scenario where the “delay-critical buffer size plus non-delay-critical buffer size” equals “total buffer size”. To avoid redundant buffer size reporting, for an LCH or LCG, triggered BSR should be cancelled when the buffer size reported in enhanced DSR equals the total buffer size.

Proposal 4: For an LCH or LCG, triggered BSR should be cancelled when the buffer size reported in enhanced DSR equals the total buffer size (i.e., “delay-critical buffer size + non-delay-critical buffer size” equals “total buffer size”).

To calculate the buffer size of non-delay critical data ahead of delay critical data, we need to explicitly identify which types of data can be categorized as non-delay critical data. For instance, the non-delay critical data should encompass not only PDCP SDUs or PDU Sets whose remaining time exceeds a configured threshold, but also low importance PDU Sets.

Proposal 5: Regarding the non-delay critical data volume calculation, it should encompass not only PDCP SDUs or PDU Sets whose remaining time exceeds a configured threshold, but also low importance PDU Sets.

Furthermore, since all PDUs in a PDU Set share the same PSDB, if all PDUs belonging to a PDU Set are needed (e.g., if pdu-SetDiscard is configured), and at least one SDU belonging to the PDU Set becomes delay-critical, then all SDUs belonging to the PDU Set should be counted in the delay-critical data volume.

Proposal 6: If all PDUs belonging to a PDU Set are needed and at least one SDU belonging to the PDU Set becomes delay-critical, then all SDUs belonging to the PDU Set should be counted in the delay-critical data volume.

# Summary

This contribution provides our analysis on the enhancement of DSR, and has the following proposals:

Proposal 1: RAN2 to consider the following options if the UL-SCH resources cannot accommodate an enhanced DSR MAC CE plus its subheader:

- Option 1: Trigger an SR.

- Option 2: Merge pairs of delay information.

- Option 3: Report at most one pair of delay information for an LCG.

Proposal 2: RAN2 agrees to introduce the definition of PDCP SDUs associated with an enhanced DSR, distinct from those associated with legacy DSR.

Observation 1: The coexistence of legacy and enhanced DSR may result in unreported residual delay-critical data and the reporting of redundant delay information.

Proposal 3: Only enhanced DSR MAC CE shall be used when at least one LCG is configured with multiple reporting thresholds.

Proposal 4: For an LCH or LCG, triggered BSR should be cancelled when the buffer size reported in enhanced DSR equals the total buffer size (i.e., “delay-critical buffer size + non-delay-critical buffer size” equals “total buffer size”).

Proposal 5: Regarding the non-delay critical data volume calculation, it should encompass not only PDCP SDUs or PDU Sets whose remaining time exceeds a configured threshold, but also low importance PDU Sets.

Proposal 6: If all PDUs belonging to a PDU Set are needed and at least one SDU belonging to the PDU Set becomes delay-critical, then all SDUs belonging to the PDU Set should be counted in the delay-critical data volume.

# References

[1] Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece, 17th Feb - 21st Feb, 2025

[2] TS 38.321, Medium Access Control (MAC) protocol specification, v18.4.0