3GPP TSG-RAN WG2 #129bis meeting	R2-2502298

# Wuhan, China, 7th Apr – 11th Apr, 2025

Source:		NEC

Agenda item:		8.7.6	XR rate control

Title:		Uplink rate control for XR

Document for:		Discussion and decision

# 1. Introduction

In the RAN2#129 meeting, agreements [1] on XR rate control were made as below:

- RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. - We may revisit UAI option based on SA2/RAN3 reply - Specify a new table for XR rate control. FFS distribution (exponential, linear), codepoints etc. - We will try to design a table first and check whether it is possible to meet the required range/granularity. Afterwards, we can check whether multipliers are needed - Working assumption: - Support rate query MAC CE with the target to use same design that we will agree for rate indication MAC CE. - The rate query MAC CE is configurable by the network, i.e. the network may turn it off completely (same as legacy). - Companies to check with their SA4 colleagues whether there are any issues with this

In this contribution, we will further discuss the remaining issues of XR rate control.

# 2. Discussion

2.1 Recommended bit rate.

Regarding whether indicating the recommended bit rate by RRC signalling or by MAC CE, we believe MAC CE-based indication should be adopted. According to the latest WID for XR, the goal of rate control enhancement is to enable faster source rate adaptation to uplink congestion. In contrast to MAC CE-based rate indication, which is efficient, RRC signalling-based rate indication is less timely and may lead to increased delay in rate adaptation. Furthermore, since the legacy MAC CE-based solution for rate indication has worked well, it is reasonable to retain this approach with improvements to support per-QoS flow rate indication.

Proposal 1: The recommended bit rate is indicated by MAC CE rather than RRC signalling.

Based on the LS reply from RAN3 [3], both per-QoS flow and per-DRB bit rate indication impact the F1 interface. Even with per-DRB bit rate indication, the RAN still needs to estimate bit rates for each QoS flow mapped to the DRB to derive the indicated bit rate for the DRB. Additionally, the ratio for allocating rates among QoS flows may need to be specified. Consequently, compared to per-QoS flow indication, per-DRB indication is more complex and offers no additional benefit. Therefore, we propose that RAN2 supports the option of per-QoS flow bit rate indication using a MAC CE.

Proposal 2: RAN2 supports per-QoS flow bit rate indication rather than per-DRB indication.

Different QoS flows may have different preferences for bit rate recommendation granularity. For example, one might favor bit rate recommendation on a per-LCH basis, while another might prefer bit rate recommendation on a per-QoS flow basis. Consequently, UE should inform the network of its preference of bit rate recommendation granularity for a given QoS flow. Based on this, the network can indicate the recommended bit rate in the UE's preferred granularity.

Proposal 3: For a QoS flow, UE informs the network of its preference for bit rate recommendation granularity, either on a per-LCH basis or on a per-QoS flow basis.

When determining the recommended bit rate, only QoS flows that support bit rate adaptation should be considered. Regarding the source of this information for RAN, the following agreements were reached in RAN2#128&129 meetings.

- Agreements @ RAN2#128 - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN. - Agreements @ RAN2#129 - RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2. - We may revisit UAI option based on SA2/RAN3 reply

If the indication of QoS flows supporting bit rate adaptation from CN to RAN is available, RAN should also configure this information to UE. UE may consider only the indicated QoS flows when calculating the desired recommended bit rate in the recommended bit rate query.

Proposal 4: The network configures the QoS flows supporting bit rate adaptation to UE if this information from CN to RAN is available.

If the indication of QoS flows supporting bit rate adaptation from CN to RAN is not available, the UE, which has the knowledge of whether a QoS flow supports source bit rate adaptation for uplink traffic, can report this information to the network, for example, via UAI. This allows the network to properly configure or determine the recommended bit rate for uplink traffic.

Proposal 5: For uplink traffic, UE reports the QoS flows supporting bit rate adaptation via UAI if this information from CN to RAN is not available.

2.2 recommended bit rate during CA/DC.

The data of a QoS flow may be transmitted over different carriers (or LCHs) belonging to the same or different MAC entities when performing PDCP duplication or split transmission. Due to varying congestion situations in different cells, UE may receive different recommended bit rate values for the same QoS flow from different cells or cell groups (e.g., during CA/DC). In this case, RAN2 should discuss how to apply the indicated recommended bit rates.

Figure 1: recommended bit rates from different cells/CGs

For example, UE may be provided with recommended bit rate 1 of a QoS flow from cell 1 and recommended bit rate 2 of the same QoS flow from cell 2.

- For PDCP duplication, the UE may not be expected to go beyond the recommended bit rate of min (recommended bit rate 1, recommended bit rate 2) for the QoS flow.

- For split transmission, the recommended bit rate for a QoS flow may be applied to the cell or cell groups where the MAC CE is received. UE should guarantee that the bit rate of the QoS flow in each cell or cell group will not go beyond the recommended bit rate of the corresponding cell or cell groups.

Proposal 6: RAN2 to discuss how to apply the indicated recommended bit rate for a QoS flow when more than one indication is received from different cells or cell groups (e.g., during CA/DC).

# Summary

This contribution provides our analysis on the uplink rate control for XR, and has the following proposals:

Proposal 1: The recommended bit rate is indicated by MAC CE rather than RRC signalling.

Proposal 2: RAN2 supports per-QoS flow bit rate indication rather than per-DRB indication.

Proposal 3: For a QoS flow, UE informs the network of its preference for bit rate recommendation granularity, either on a per-LCH basis or on a per-QoS flow basis.

Proposal 4: The network configures the QoS flows supporting bit rate adaptation to UE if this information from CN to RAN is available.

Proposal 5: For uplink traffic, UE reports the QoS flows supporting bit rate adaptation via UAI if this information from CN to RAN is not available.

Proposal 6: RAN2 to discuss how to apply the indicated recommended bit rate for a QoS flow when more than one indication is received from different cells or cell groups (e.g., during CA/DC).

# References

[1] Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece, 17th Feb - 21st Feb, 2025

[2] TS 38.321, Medium Access Control (MAC) protocol specification, v18.4.0

[3] R2-2501508, Reply LS on XR UL Rate Control,	RAN3, To:RAN2	Cc:SA4