3GPP TSG-RAN WG2 #129bis	R2-2502320

Wuhan, China, 7 – 11 April 2025

Agenda item:		8.5.2	On-demand SSB SCell operation

Title:		Further discussion on On-demand SSB for SCell

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#127 and #127bis, RAN2 reached some agreements on the On-demand SSB operator for SCell [1][2].

Agreements on OD-SSB @ #127

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Agreements on OD-SSB SCell @ #127bis

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

Additionally, in the last RAN#129 it was agreed to rely on RAN4 for measurement targets in Case 2 [3].

Agreements on OD-SSB

- RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

In this contribution, we discuss some details of network signalling and provide our views.

# 2. Discussion

2.1	RRC configuration and new MAC CE contents

We discuss details of new MAC CE activating the OD-SSB. In RAN1#119, there were many agreements [4] and some of them are closely related to RAN2 discussions, for example:

Agreement - For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2. - For Case #1 (i.e., No always-on SSB on the cell), - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0 - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0. - If half frame index parameter is NOT configured, UE assumes half frame index set to 0. - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced. - The value range of half frame index is 0 or 1. - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives. - Alt A: Same as for Case #1 - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset) Agreement New periodicity value for on-demand SSB other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms) is NOT introduced in Rel-19.

Observation 1: RAN1 agreed to configure time domain location of OD-SSB per OD-SSB periodicity by RRC for both Case #1 and Case #2.

Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

Observation 2: RAN1 agreed to support configuration or indication of the number N of OD-SSB bursts to be transmitted after OD-SSB is indicated.

In RAN1#120, some more progress was made for OD-SSB [5]:

Agreement - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2. - For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2. - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) - FFS: Whether this parameter can be absent for Case #2 - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) - FFS: Whether this parameter can be absent for Case #2 - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity) - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing) - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB. - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId) - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB. - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex) - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower) - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB. - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx) - FFS: Which of the above parameters are a list

Observation 3: RAN1 agreed to introduce some new RRC parameters, while those details are still FFS.

Agreement For RRC based OD-SSB indication, UE expects on-demand SSB is transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission.

Observation 4: RAN1 agreed that for RRC-based OD-SSB indication (activation case), the UE expects the OD-SSB transmission from first OD-SSB burst after receiving the RRC configuration.

Considering the agreements in RAN2 and RAN1 so far, the following can be understood:

- RRC can configure the OD-SSB with (initial and reconfigured) state as either activated or deactivated.

  - For deactivated case, the new MAC CE activating OD-SSB is to be transmitted later.

- RRC can configure the time domain location of OD-SSB per OD-SSB periodicity.

- RRC can configure the number “N” of OD-SSB bursts to be transmitted after OD-SSB is indicated.

- New MAC CE can activate/deactivate OD-SSB for multiple SCells simultaneously.

It seems that a missing explicit agreement is to support multiple OD-SSB patterns, i.e. combinations of OD-SSB periodicity and time domain location of the corresponding OD-SSB. To make the function more useful, the network should be able to indicate one of the preconfigured OD-SSB patterns via the new MAC CE.

Proposal 1: RAN2 to agree that the RRC can configure multiple OD-SSB patterns (i.e. combination of OD-SSB periodicity and time domain location of the corresponding OD-SSB) and the new MAC CE (de)activating OD-SSB can indicate one of the preconfigured OD-SSB patterns.

Proposal 2: The new MAC CE (de)activating OD-SSB includes at least:

- Information to indicate the target SCell(s)

- Information to indicate one of the preconfigured OD-SSB pattern per SCell.

# 3. Conclusion

In this contribution we discussed some details of network signalling for OD-SSB transmission and made the following observations and proposals.

Some observations on RAN1 status:

Observation 1: RAN1 agreed to configure time domain location of OD-SSB per OD-SSB periodicity by RRC for both Case #1 and Case #2.

Observation 2: RAN1 agreed to support configuration or indication of the number N of OD-SSB bursts to be transmitted after OD-SSB is indicated.

Observation 3: RAN1 agreed to introduce some new RRC parameters, while those details are still FFS.

Observation 4: RAN1 agreed that for RRC-based OD-SSB indication (activation case), the UE expects the OD-SSB transmission from first OD-SSB burst after receiving the RRC configuration.

Proposal 1: RAN2 to agree that the RRC can configure multiple OD-SSB patterns (i.e. combination of OD-SSB periodicity and time domain location of the corresponding OD-SSB) and the new MAC CE (de)activating OD-SSB can indicate one of the preconfigured OD-SSB patterns.

Proposal 2: The new MAC CE (de)activating OD-SSB includes at least:

- Information to indicate the target SCell(s)

- Information to indicate one of the preconfigured OD-SSB pattern per SCell.

# References

[1] R2-2407571, Report from session on V2X/SL, R19 NES and MOB, Vice Chairman (Samsung)

[2] R2-2409211, Report from session on V2X/SL, R18/19 MOB, and R19 NES, Vice Chairman (Samsung)

[3] R2-2501331, Report from session on R17/18 V2X/SL, R18/19 MOB and R19 NES, Vice Chairman (Samsung)

[4] R1-2500003, Final Report of 3GPP TSG RAN WG1 #119 v1.0.0

[5] Draft_Minutes_report_RAN1#120_v030