3GPP TSG-RAN WG2 Meeting #129bis	R2-2502337

Wuhan, China, April 7th – 11th, 2025

Agenda item:	6.1.3.1

Title:	On eDRX operation with emergency services

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In RAN2#128 meeting, RAN2 sent an LS [1] to SA2, asking about the handling of eDRX when UE is in RRC_INACTIVE state with emergency service. This meeting, RAN2 receive the reply LS [2] from SA2, in the reply LS, SA2 provide the following feedback.

SA2 analysed the identified issue above and would like to provide the following feedback: - Existing specifications have specified that each PLMN/operator reserves a specific ARP value for emergency PDU session and provide to RAN node. See related text in TS 23.501 clause 5.16.4.3, 5.16.4.7, 5.16.4.9 and TS 23.502 clause 6.1.3.10. Therefore the RAN node can be aware of an emergency PDU session based on the reserved ARP value when the PDU session context exist in the RAN node. - In addition there is inactivity monitoring for the presence of user plane resources for the emergency PDU session in SMF as defined in TS 23.502 clause 4.4.2.2 and TS 23.503 clause 6.1.3.10. Based on the above, some companies in SA2 believe that the RAN node is capable through configuration to not release the emergency PDU session user plane resources based on existing implementation, and to not release the UE to RRC_INACTIVE state/RRC_IDLE state when the UE has an emergency PDU session. Therefore the issue highlighted in RAN2 LS may not exist. However there is no consensus in SA2 whether the UP of emergency PDU session can be deactivated, or whether to introduce new signalling to NG-RAN. SA2 would like to ask RAN2 to provide feedback.

This contribution provides further consideration based on the feedback from SA2.

Discussion

First of all, SA2 mention that in existing specifications it is already specified that, each PLMN/operator can reserve a specific ARP value for emergency PDU session, and the ARP value will be sent to RAN when the PDU session is established.

#### TS 23.501 #### 5.16.4.3 Mobility Restrictions and Access Restrictions for Emergency Services When Emergency Services are supported and local regulation requires IMS Emergency Sessions to be provided regardless of Mobility Restrictions or Access Restrictions, the Mobility Restrictions or Access Restrictions (see clause 5.3.4.1) should not be applied to UEs receiving Emergency Services. Additionally, due to Mobility Restrictions or Access Restrictions (e.g. CAG restrictions) for normally registered UEs, that have established both non-emergency PDU Sessions and emergency PDU Session, the AMF indicates to the SMF to perform a local release of all non-emergency PDU Sessions via PDU Session Release procedure as specified in clause 4.3.4 of TS 23.502 [3]. The UE locally releases non-emergency PDU Sessions. The AMF and the UE behave as if the UE is emergency registered as described in TS 24.501 [47]. When the (R)AN resources for Emergency Services are established, the ARP value for Emergency Services indicates the usage for Emergency Services to the 5G-AN. //skipped part

#### TS 23.503 #### 6.1.3.10 IMS emergency session support //skipped part The PCC Rule Authorization function selects QoS parameters that allow prioritization of IMS Emergency sessions. If an IMS Emergency session is prioritized the QoS parameters in the PCC Rule shall contain an ARP value that is reserved for intra-operator use of IMS Emergency services. The PCF does not perform subscription check; instead it utilizes the locally configured operator policies to make authorization and policy decisions. NOTE 1: Reserved value range for intra-operator use is defined in TS 23.501 [2]. //skipped part

Based on above, the existing SA2 specifications specify that each PLMN/operator reserves a specific ARP value for emergency PDU session, and the ARP value will be provided to RAN node when emergency PDU session is established, based on this, the RAN node can be aware of an emergency PDU session if the received ARP value is the same as the ARP value reserved for emergency IMS service.

Observation 1: According to reply LS, the SA2 specifications already specify that each PLMN/operator reserves a specific ARP value for emergency PDU session, so, based on the received ARP value, RAN node can be aware of the presence of emergency PDU session.

In SA2 reply LS, it also indicates that there is no consensus on whether to introduce new signalling to NG-RAN. In our understanding, there is no need to introduce new signaling to indicate the emergency PDU session since reserve ARP value for emergency PDU session is specified and is signaled to RAN.

Proposal 1: RAN2 to confirm that no new signaling from CN to RAN is needed to indicate the presence of emergency PDU session.

Furthermore, some companies in SA2 thought that UE configured with emergency PDU session should be kept in RRC_CONNECTED. The intention is to speed up the MT callback that follows an emergency call. This is also the reason why eDRX configuration for CN paging is not applied for RRC_IDLE UE as captured in TS 23.501.

Based on the specific ARP value reserved for emergency PDU session, the RAN node is capable to not release the UE to RRC_INACTIVE state or not to release the UE to RRC_INACTIVE with eDRX for RAN paging.

Observation 2: It is helpful to speed up the MT callback following the emergency call by not releasing the UE to RRC_INACTIVE or not releasing the UE to RRC_INACTIVE with eDRX for RAN paging.

If the UE configured with emergency PDU session is maintained in RRC_CONNCTED, or it is not released to RRC_INACTIVE with eDRX configuration for RAN paging, the issue that identified in previous RAN2 meeting no longer exists.

As to whether the UE configured with emergency PDU session can be release to RRC_IDLE with/without eDRX for CN paging is already covered by existing specification, i.e. the UE in RRC_IDLE does not apply eDRX for CN paging.

Observation 3: If the UE is not released to RRC_INACTIVE or is not released to RRC_INACTIVE with eDRX for RAN paging, the issue identified in previous RAN2 meeting no longer exists.

Based on above observation and discussions, there are two options to address the issue:

Option 1: To capture in stage 2 specification that when UE is configured with emergency PDU session, the UE is not expected to be released to RRC_INACTIVE with eDRX configuration for RAN paging.

Option 2: To capture in stage 2 specification that when UE is configured with emergency PDU session, the UE is not expected to be released to RRC_INACTIVE.

Between the two options, we think Option 2 is too strict, as long as the UE is released to RRC_INACTIVE state without RAN eDRX, there is no impact on paging delay. So, we suggest:

Proposal 2: RAN2 confirm that when UE is configured with emergency PDU session, the UE is not expected to be released to RRC_INACTIVE with eDRX configuration for RAN paging (to be captured in stage 2 spec).

According to the SA2 reply LS, there is no consensus in SA2 whether the UP of emergency PDU session can be deactivated, or whether to introduce new signalling to NG-RAN. Considering the decision in RAN2 may have impact on SA2 specification, a reply LS to SA2 is needed to inform the decision made in RAN2. And as also indicated in SA2’s reply LS, they expect RAN2 to provide feedback.

Proposal 3: Send reply LS to SA2 about the decisions made in RAN2.

Conclusion

Observation 1: According to reply LS, the SA2 specifications already specify that each PLMN/operator reserves a specific ARP value for emergency PDU session, so, based on the received ARP value, RAN node can be aware of the presence of emergency PDU session.

Observation 2: It is helpful to speed up the MT callback following the emergency call by not releasing the UE to RRC_INACTIVE or not releasing the UE to RRC_INACTIVE with eDRX for RAN paging.

Observation 3: If the UE is not released to RRC_INACTIVE or is not released to RRC_INACTIVE with eDRX for RAN paging, the issue identified in previous RAN2 meeting no longer exists.

Proposal 1: RAN2 to confirm that no new signaling from CN to RAN is needed to indicate the presence of emergency PDU session.

Proposal 2: RAN2 confirm that when UE is configured with emergency PDU session, the UE is not expected to be released to RRC_INACTIVE with eDRX configuration for RAN paging (to be captured in stage 2 spec).

Proposal 3: Send reply LS to SA2 about the decisions made in RAN2.

References

- R2-2411041 LS on emergency services and eDRX    from RAN2

- R2-2501752/S2-2502427 Reply LS on emergency call back and paging    from SA2

Annex-----Reply LS to SA2

3GPP TSG-RAN WG2 Meeting #129bis	R2-25xxxxx

Wuhan, China, April 7th – 11th, 2025

Title:	[DRAFT]Reply LS on emergency call back and paging

Response to:	S2-2502427 / R2-2501752

Release:	Rel-17

Work Item:	NR_newRAT-Core, NR_redcap-Core

Source:	RAN2

To:	SA2,

Cc:	RAN3, CT1

Contact person:	Jing Liu

liu.jing30@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN2 thanks SA2 for the reply LS on emergency service call back and paging.

RAN2 further discussed this issue and would like to provide the following feedbacks:

- RAN2 confirm the feasibility of the solution mentioned in reply LS, that RAN can be aware of emergency PDU session based on the reserved ARP value. So, there is no need to introduce new Ng-C signaling from CN to NG-RAN to indicate whether presence of emergency PDU session.

- RAN2 agrees to capture in RAN2 specification that the UE configured with emergency PDU session is not expected to be released to RRC_INACTIVE state with eDRX for RAN paging. The corresponding CR is attached for reference.

2. Actions:

To SA2 group:

ACTION:  RAN2 kindly requests SA2 to take the above information into account.

3. Date of Next RAN2 Meetings:

RAN2#130					                       19th – 23rd May 2025			                      Malta, MT

RAN2#131	   	    	                 25th – 29th Aug 2025			                      India, IN