3GPP TSG-RAN WG2 Meeting#129bis	R2-2502348

Wuhan, CN, April 7–11, 2025

Agenda item:	 8.10.2.2

Source:		Lenovo

Title:		Discussion on MRO enhancements for CHO with candidate SCGs

Document for:		Discussion and Decision

1	Introduction

In this paper, we would further discuss MRO enhancements for CHO with candidate SCG(s).

2	Discussion

RAN2#128 meeting agreed that:

- RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements.

- Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

Additionally, RAN2#129 meeting agreed that:

- Enhance RLF report for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any.

- Enhance RLF report for CHO with candidate SCGs to include the associations between CHO and CPAC.

- Enhance RLF report for CHO with candidate SCGs to include at least the following information:

- Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered;

- Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered;

- The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

- Enhance SCGFailureInformation for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any.

- We should avoid specifying the procedure in a way that the UE sends redundant information

It seems the discussion for MRO for failure case in CHO with candidate SCGs is stable, we should focus on near failure successful case then.

For legacy SHR or SPR, T310/T312/T304 related trigger threshold is configured by network to trigger SHR or SPR. It seems that there is no issue to reuse legacy SHR or SPR trigger threshold for the CHO with candidate SCG(s) procedure.

When CHO with candidate SCG(s) is performed, both CHO execution condition of the target PCell and corresponding CPAC execution condition of the associated target PSCell are fulfilled, there is a possibility that the time duration between the fulfilment of CHO execution condition and the fulfilment of CPAC execution condition is higher than a threshold, which may cause a near-failure successful CHO with candidate SCG(s). Therefore, besides legacy T310/T312/T304 related trigger threshold, RAN2 can consider new trigger condition(s) for SHR or SPR for the CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell. When the time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell is higher than the threshold, SHR and SPR may be generated by the UE.

Proposal 1: Introduce new trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell.

No matter what is the trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), for sub-optimal successful CHO execution, to help network know how to modify the configuration of the CHO with candidate SCG(s) procedure, R17 SHR can be enhanced, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) can be stored and reported by the UE in SHR. Also, to follow the agreement that “the time information (as agreed for RLF) are included in SHR”, the time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell) can be stored and reported by the UE in SHR.

Proposal 2: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell), can be stored and reported by the UE in SHR.

Similarly, for any sub-optimal successful CPAC, R18 SPR can be enhanced, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) and the time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell) can be stored and reported by the UE in SPR.

Proposal 3: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell), can be stored and reported by the UE in SPR.

3	Conclusion

In this paper, MRO enhancements for CHO with candidate SCG(s) are discussed. We have the following proposals:

Proposal 1: Introduce new trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell.

Proposal 2: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell), can be stored and reported by the UE in SHR.

Proposal 3: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell), can be stored and reported by the UE in SPR.