3GPP TSG-RAN WG2 Meeting #129bis	R2-2502358

Wuhan, China, Apr. 7th – 11th, 2025                           Revision of R2-2500621

Agenda item:		8.17

Source:		Lenovo

Title:		Discussion on TDD support in IoT NTN

Document for:		Discussion

- Introduction

In RAN#106 a new work item “introduction of IoT-NTN TDD mode” was approved with the following objectives:

Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4].

And RAN2#129 made the following agreements:

Agreements: 1. RAN2 will continue studying paging aspects based, on RAN1 progress 2. RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed. 3. RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network. 4. RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated. 5. RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system. 6. RAN2 can continue the discussion also on RAR window 7. Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

In this contribution we discuss some potential issues to support TDD mode in IoT-NTN from RAN2 perspective.

- Discussion

This work item on TDD support for IoT-NTN aims to design a periodic pattern that restricts usable DL and UL resources within a period. Although the details of the pattern including the parameters and value ranges have not been decided by RAN1, it is common sense that some essential parameters including the periodicity, DL and UL resources and guard period (for DL/UL switch) are needed. In R1-2409440 an example of the TDD pattern is provided as in Figure 1, including:

- DL-UL-TransmissionPeriodicity (referred to as N in the WID) which indicates the periodicity of the DL-UL pattern. It can be provided in number of radio frames. At least N equal to 9 Radio frames should be supported.

- nrOfDownlinkSubframes corresponding to the number of consecutive/ contiguous DL subframes at the beginning of each DL-UL pattern.

- nrOfUplinkSubframes which provides the number of consecutive/ contiguous UL subframes.

- DownlinkToUplinkGuardPeriod which indicates the Downlink to uplink Guard Period for TDD operation.

- An offset to indicate the start of the TDD pattern period.

Figure 1. An example of the TDD pattern for NTN

It is observed that with the TDD pattern configured, the DL and UL resources that can be used in a period is restricted and the timing of the resource at eNB and UE are different due to large propagation delay in NTN. The DL subframes are to be offseted late with half of UE-eNB RTT, and UL subframes are offseted early with half of UE-eNB RTT. From network perspective the TDD pattern configured shall guarantee workable and efficient DL/UL resource scheduling and from UE perspective the impact of non-D(ownlink) and non-U(plink) subframes on UE reception and transmission shall be considered.

To avoid possible collision or too long inactive time (as the actual gap at UE in Figure 1) at UE, the guard period shall cover the UE-eNB RTT with limited margin. More specifically, if the guard period is less than UE-eNB RTT, there will be DL/UL resource collision at UE; if the guard period is much larger than UE-eNB RTT, there will be too long inactive time at UE that decreases resource efficiency.

Observation 1: The guard period configured in the TDD pattern is expected to be aligned with the latest UE-eNB RTT.

Therefore, the eNB may need information that can derive UE-eNB RTT to determine the guard period, otherwise the eNB may have to configure a worst value corresponding to the maximum propagation delay in the NTN cell, which could decrease efficiency of resource usage. This could be implemented directly by UE reporting of the UE-eNB RTT currently applied e.g., for HARQ RTT timers, or indirectly by other UE reporting including the TA currently compensated or the UE-specific TA derived.

Proposal 1: RAN2 to discuss how to ensure that eNB has the latest UE-eNB RTT (e.g. by UE reporting or assistance) for TDD pattern configuration.

Meanwhile, consider a worse case that there is collision or too long inactive time at UE (meaning too short or too long guard period configuration), the eNB may benefit from UE’s feedback to update/optimize the configuration.

Proposal 2: RAN2 to discuss the necessity of UE reporting in case of UL/DL collision or too long inactive time due to TDD configuration from eNB, e.g. to help eNB update/optimize the TDD pattern configuration.

Due to the scheduling of configured TDD pattern, a downlink/uplink operation may overlap with non-D(ownlink)/non-U(plink) subframes, and RAN1#119 made the following agreements:

RAN1 to further discuss how to handle the overlap of NPUSCH with non-U NB-IoT subframes, the overlap of NPRACH with non-U NB-IoT subframes, and the overlap of NPDCCH/NPDSCH (other than the one carrying SIB1-NB) (including e.g. starting point, windows for SI or RAR and other window sizes for DL channels/signals, PO, etc.) with non-D NB-IoT subframes.

From RAN2 perspective, due to the scheduling of configured TDD pattern, an uplink reporting could be delayed if triggered in UL inactive time or non-U subframes. The reporting could be time-sensitive including:

- Timing Advance Report (TAR) that includes the timing advance applied at UE and can be used by eNB for UL scheduling. The value varies over time.

- Propagation Delay Difference (PDD) reporting that includes the propagation delay difference(s) between serving cell and neighbour cell(s) and can be used by eNB for SMTC window or measurement gap configuration. The value varies over time.

- GNSS validity duration reporting that includes the remaining validity duration of the latest GNSS position fix result and can be used by eNB for GNSS measurement gap configuration. The value varies over time but can be easily derived.

Therefore, it may be necessary to discuss how to handle these reports triggered at a non-U subframe.

Proposal 3: RAN2 to discuss how to handle an UL reporting triggered at a non-U subframe based on TDD resource configuration.

- Conclusion

In this contribution we discuss issues and potential solutions in cell barring, cell reselection and operation information exchange for S&F operations in IoT NTN. The following proposal are provided:

Proposal 1: RAN2 to discuss how to ensure that eNB has the latest UE-eNB RTT (e.g. by UE reporting or assistance) for TDD pattern configuration.

Proposal 2: RAN2 to discuss the necessity of UE reporting in case of UL/DL collision or too long inactive time due to TDD configuration from eNB, e.g. to help eNB update/optimize the TDD pattern configuration.

Proposal 3: RAN2 to discuss how to handle an UL reporting triggered at a non-U subframe based on TDD resource configuration.