3GPP TSG-RAN WG2 Meeting #129bis	R2-2502359

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:		8.6.2

Source:		Lenovo

Title:		Remaining issues for Inter-CU LTM

Document for:		Discussion and Decision

- Introduction

This contribution aims to discuss the further potential issues for Inter-CU LTM.

- Discussion

Rel-19 ID transferred to target gNB

RAN2 agreed that Rel-19 ID is configured for a candidate configuration. If the Rel-19 ID is different for the source cell (i.e. ltm-ServingCellNoSecurityChangeID) and the target cell (i.e. ltm-NoSecurityChangeID), the UE performs PDCP re-establishment, including security key update. For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18. Source cell is responsible for Rel-19 ID assignment for the candidate cells. Once the candidate cell becomes the new serving cell because of cell switching, the new serving cell will be responsible for Rel-19 ID assignment and maintenance when/ if adding or releasing a candidate cell. Therefore, the new serving cell should be aware of the assigned Rel-19 ID list.

Besides, Inter-CU MCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3 in inter-CU SCG LTM.

Proposal 1: The assigned Rel-19 ID list for all the serving cell and candidate cells should be transferred to target gNB (target MN for Inter-CU MCG LTM or target SN for Inter-CU SCG LTM) upon successful handover. If so, an LS is useful to be sent to RAN3.

Coexistence between Inter-CU MCG LTM and Subsequent CPAC

In Rel-18 LTM, simultaneous configuration for both LTM and CPAC is allowed. After Inter-MN PCell change, the configuration for subsequent CPAC should be released by network via explicit indication. We can follow the same behavior that the configuration for subsequent CPAC should be released by network via explicit indication after Inter-CU MCG LTM.

- RAN2#121 In SCG selective activation, the CPC/CPA configurations of the UE should be released after Pcell change, at least for inter MN (by explicit indication from network, FFS other case).

Proposal 2: Subsequent CPAC can be configured while Inter-CU MCG LTM or SCG LTM is configured.

Proposal 3: The configuration for subsequent CPAC should be released by network via explicit indication after Inter-CU MCG LTM as legacy.

Inter-CU SCG LTM and MCG failure recovery

In Rel-18 Intra-CU SCG LTM, Intra-CU SCG LTM switch is allowed while MCG failure recovery procedure is ongoing. More specifically, UE will trigger fast MCG link recovery upon RLF on MCG. After UE transmits MCGFailureInformation message to MCG via SCG, UE starts T316 and waits for the response e.g handover command. Intra-CU SCG LTM could be triggered while T316 is running. That means the response will be pending in SCG until UE completes SCG change triggered LTM.

Regarding Inter-CU SCG LTM, if Inter-CU SCG LTM cell switch is allowed while MCG failure recovery procedure is ongoing. The response from MCG to UE should be transmitted to target SN. Therefore, once source SN activates Inter-CU LTM switching, source SN indicates it to MN. Then, MN transmits the response e.g reconfiguration message for PCell change to target SN. Specifically, the SN-DU inform the SN-CU about the initiation of the cell switch command to the UE via DU-CU Cell Switch Notification. Then, SN-CU informs it to MN-CU.

Proposal 4: Inter-CU SCG LTM switch is allowed to be triggered while MCG failure recovery procedure is ongoing as legacy Intra-CU SCG LTM.

Proposal 5: Once source SN activates Inter-CU LTM switching while MCG failure recovery procedure is ongoing, source SN indicates it to MN. Then, MN transmits the response of MCGFailureInforamtion to target SN.

Security in Inter-CU mobility

RAN2 has RAN2 has just received a reply from SA3 in R2-2501756_S3-251124:

Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover.

In addition to SA3 response, it is important for UE to distinguish if a key derivation is applied (horizontal or vertical), or not. If a new NH, NCC pair would become available at any time (i.e., due to a new NAS AKA run), then the network should transmit keySetChangeIndicator to the UE.

Proposal 6: Regarding inter-CU security change, MAC signalling should enable UE to distinguish if key derivation is to applied (same as previous NCC value means horizontal derivation and a different NCC means vertical derivation), or not (in which case there will be no security update).

A NAS AKA may have been run and therefore an [NH, NCC] pair can be available – but this is not very frequent case and therefore use of RRC signalling (instead of MAC) could be acceptable to transfer the keySetChangeIndicator to the UE.

Proposal 7: If a NAS AKA is run (and NH, NCC pair becomes available), then a fresh RRC signalling including keySetChangeIndicator is sent to the UE.

Finally, we also asked SA3 question on security Algorithm change. Since this should be relatively rare, implementing the key change using RRC should be acceptable.

Proposal 8: When the security Algorithm change is required, it is performed using layer 3 handover.

Conclusion

In this contribution, the following proposals are given based on the discussion:

Rel-19 ID transferred to target gNB

Proposal 1: The assigned Rel-19 ID list for all the serving cell and candidate cells should be transferred to target gNB (target MN for Inter-CU MCG LTM or target SN for Inter-CU SCG LTM) upon successful handover. If so, an LS is useful to be sent to RAN3.

Coexistence between Inter-CU MCG LTM and Subsequent CPAC

Proposal 2: Subsequent CPAC can be configured while Inter-CU MCG LTM or SCG LTM is configured.

Proposal 3: The configuration for subsequent CPAC should be released by network via explicit indication after Inter-CU MCG LTM as legacy.

Inter-CU SCG LTM and MCG failure recovery

Proposal 4: Inter-CU SCG LTM switch is allowed to be triggered while MCG failure recovery procedure is ongoing as legacy Intra-CU SCG LTM.

Proposal 5: Once source SN activates Inter-CU LTM switching while MCG failure recovery procedure is ongoing, source SN indicates it to MN. Then, MN transmits the response of MCGFailureInforamtion to target SN.

Security in Inter-CU mobility

Proposal 6: Regarding inter-CU security change, MAC signalling should enable UE to distinguish if key derivation is to applied (same as previous NCC value means horizontal derivation and a different NCC means vertical derivation), or not (in which case there will be no security update).

Proposal 7: If a NAS AKA is run (and NH, NCC pair becomes available), then a fresh RRC signalling including keySetChangeIndicator is sent to the UE.

Proposal 8: When the security Algorithm change is required, it is performed using layer 3 handover.

References

[1] Chair notes for RAN2#129