3GPP TSG-RAN WG2 Meeting #129bis	R2-2502376

Wuhan, China, April 7th – 11th, 2025

Agenda item	:		8.8.4

Source	: 	Sharp

Title	: 	Discussion on UE behaviours within intended service area

Document for	:	Discussion and Decision

Introduction

In the last meeting, RAN2 reached some agreements/decisions as follows [1];

Agreements: - In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list - It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH) - Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates”

In this paper, we’d like to discuss UE behaviours for MBS related procedures.

Discussion

MCCH re-acquisition

Based on the agreement of the RAN2#129 meeting [1], the new SIB contains a list of ISAs and a list of NW-indicated IDs associated with each ISA entry in the list. The following configuration is considered as an example according to ASN.1 format. In this case, we assume the signalling structure that associates an ISA data with an ISA ID.

SIBxx-r19 ::=    SEQUENCE {

isa-InfoList ::=   SEQUENCE (SIZE (1..maxIsaInfo-r19)) OF Isa-Info,

...

}

Isa-Info ::=    SEQUENCE {

isa-areaId-r19         Isa-areaId-r19,

intendedServiceArea-r19      IntendedServiceArea-r19,

The agreement suggested that network-indicated area ID included in new SIB is not represented by a pointer aggregating multiple intended service areas, but is an ID associated with an ISA, and that each ID is also included in the MCCH message in such a way that it is associated with the session. If there is a change in the contents of the new SIB for some reason, it is likely that a change in ID assignment is occurring at the same time. For example, the contents of the list of ISAs may change, or the mapping between ISAs and IDs may change, and so on. In this case, the UE, which knows the association between the session and ISAs by MCCH messages, needs to re-acquire MCCH messages to update them with the new information.

Observation 1: As a result of ISA information change in the New SIB, MCCH content may also be changed. In this case, UE needs to re-acquire MCCH.

The UE can follow the legacy modification procedure in order to know that there has been a change in the new SIB. However, the timing of MCCH update, and the timing of UE MCCH acquisition are not synchronized. Therefore, if the MCCH is reacquired immediately after the new SIB is received, the UE may receive an old MCCH which has not been updated yet.

Observation 2: UE cannot get the latest MCCH information in some cases if the UE re-acquire MCCH immediately after new SIB is changed.

In the current specification, when the network changes (some of) the MCCH information, it notifies the UEs about the change starting from the beginning of the MCCH modification period via PDCCH in that modification period. Based on this notification, the UE initiates the MCCH re-acquisition procedure. If information is added to the new SIB before the MCCH modification period, and/or if information is removed from the new SIB after the MCCH modification period, the UE can follow the legacy MCCH reacquisition procedure without causing any discrepancy between the MCCH and new SIB.

Observation 3: Based on the current mechanism, network can control the timing of the new SIB change and MCCH modification without causing any discrepancy between the MCCH and new SIB.

Proposal 1 ： No change is needed in the procedure for re-acquisition of MCCH for the introducing the new SIB.

MBS interest indication transmission by UE within ISA

In RAN2#127 meeting, RAN2 agreed UE behaviours for MRB establishment/release [2].

Agreements: - The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). - RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) - For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

There are some other remaining MBS related procedures (e.g. MII (MBS interest indication), cell reselection priorities handling). If these procedures are performed by UEs which are not in intended service area, there are negative impacts for NW operation, especially if UL resources are used by the procedure. For example, if UE is allowed to send MII from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the service. To prevent such negative impacts, the specification should also limit UE behaviours for these MBS related procedures. Therefore, if the MBS service is associated to the intended service area(s), MII should not be sent by UE located outside of the intended service area.

Observation 4: If UE is allowed to send MBSInterestIndication message from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the MBS service.

Proposal 2: MBSInterestIndication message should not be sent by UE located outside of the intended service area.

Conclusion

In this paper, we made the following observations and proposals;

[MCCH re-acquisition]

Observation 1: As a result of ISA information change in the New SIB, MCCH content may also be changed. In this case, UE needs to re-acquire MCCH.

Observation 2: UE cannot get the latest MCCH information in some cases if the UE re-acquire MCCH immediately after new SIB is changed.

Observation 3: Based on the current mechanism, network can control the timing of the new SIB change and MCCH modification without causing any discrepancy between the MCCH and new SIB.

Proposal 1： No change is needed in the procedure for re-acquisition of MCCH for the introducing the new SIB.

[MBS interest indication transmission by UE within ISA]

Observation 4: If UE is allowed to send MBSInterestIndication message from outside of the intended service area, UL resources are wasted by UEs which are not intended to receive the MBS service.

Proposal 2: MBSInterestIndication message should not be sent by UE located outside of the intended service area.

Reference

- Draft_RAN2_129_Meeting_Report_v3.zip

- R2-2409210 Report of 3GPP TSG RAN WG2 meeting #127 Maastricht, Netherlands

- RP-243300 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3