3GPP TSG-RAN-WG2 Meeting #129bis	R2-2502379

Wuhan, China, 7th– 11th Apr, 2025

Agenda item	:		8.13.3

Source	: 	Sharp

Title	: 	discussion on C-plane procedure for multi-hop relay

Document for	:	Discussion and Decision

Introduction

In RAN2#129 meeting, RAN2 reached the following agreements [1]:

Agreements: RAN2 assumes that discovery and PC5 connection establishment can be performed in each intermediate UE prior to processing the received RRCSetupRequest by the remote UE. The related FFS can be removed from the stage 2 description. For the baseline solution, the last relay sending SUI on behalf of other relay UEs is not supported. The related FFS can be removed from the stage 2 description. For the baseline solution, Rel17 SUI message and format is re-used. No further clarification in stage 2 description is needed to clarify that a relay UE can establish its RLC channel for relaying of SRB1 from its immediate child node during its own connection establishment and details will be clarified in stage 3 (i.e., follow legacy U2N procedure from Rel-17 to set up the RLC channel for SRB1). The related FFS can be removed from the stage 2 description. For system information acquisition in multi-hop, the remote UE: o When RRC_CONNECTED, uses end-to-end RRC signaling to obtain its system information directly from its connected cell. o When RRC_IDLE/RRC_INACTIVE, can request SI using PC5-RRC signaling (e.g., RemoteUEInformationSidelink message) o When in RRC_IDLE/RRC_INACTIVE, receives the required SI from PC5-RRC signaling (e.g., UuMessageTransferSidelink) For system information acquisition by the remote UE in multi-hop, the same triggers as Rel17 are supported for sending the PC5-RRC message (e.g., RemoteUEInformationSidelink) namely: o when there is a change in the required SI while in RRC_IDLE/RRC_INACTIVE, or when entering RRC_IDLE/RRC_INACTIVE o when it entering RRC_CONNECTED, a PC5-RRC message (e.g., RemoteUEInformationSidelink) is sent to cancel a previously sent required SI The last relay UE in multihop can forward SI (e.g., in a UuMessageTransferSidelink) to an intermediate Relay upon: • acquisition of the SIB(s) requested (in a hop by hop manner) by a connected child node (intermediate node and/or remote UE, but the last relay UE is not required to determine which node originated the request) • reception of updates of any SIBs requested by a remote UE or another a child relay UE (in a hop-by-hop manner), including SIB1 • deciding to perform unsolicited SIB1 forwarding An intermediate relay UE that is RRC_CONNECTED may use end-to-end RRC signaling to obtain its system information directly from its connected cell (i.e., behaving as a remote UE). The intermediate UE, when it receives SI from a parent relay, forwards the SI message only to the child UEs which requested the SI (i.e., the intermediate UE keeps track of the required SI for each child node). FFS on the need for including a “remote UE ID” in the request. The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list. FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI. Re-use RemoteUEInformationSidelink as the PC5-RRC message transmitted by the remote UE or by the intermediate relay UE to the parent node (intermediate relay or last relay) to request the required SI. The PC5-RRC message transmitted by the last relay UE or by the intermediate relay UE that provides the SI to a child UE contains at least containers with SIB1 and other system information requested by the child UE. Re-use UuMessageTransferSidelink as the PC5-RRC message transmitted by the Last relay or by the intermediate relay UE that provides SI to the child UE. The remote UE in multi-hop: o When RRC_IDLE/RRC_INACTIVE, can request to receive paging by sending its paging information using PC5-RRC signaling (e.g., RemoteUEInformationSidelink message) o When in RRC_IDLE/RRC_INACTIVE, can receive paging record from PC5-RRC signaling (e.g., UuMessageTransferSidelink)? For a remote UE in multi-hop, the same triggers as Rel17 are supported for sending the PC5-RRC message (e.g., RemoteUEInformationSidelink) namely: o when there is a change in the paging information while in IDLE/INACTIVE, or when entering RRC_IDLE/RRC_INACTIVE o when it entering RRC_CONNECTED, a PC5-RRC message (e.g., RemoteUEInformationSidelink) is sent to release the paging information The last relay UE in multihop can forward paging to an intermediate Relay upon receiving paging message related to a multihop remote UE (including the case of an intermediate relay UE functioning as a remote UE). When the intermediate UE receives a paging message from a parent relay on PC5, it forwards the paging message only to the remote UE being paged or the intermediate relay UE serving a remote UE being paged. The PC5-RRC message containing the paging information that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least paging UE ID and paging cycle of the remote UE and any intermediate relay UEs serving the remote UE. Re-use RemoteUEInformationSidelink as the PC5-RRC message transmitted by the remote UE or by the intermediate relay UE to the parent node (intermediate relay or last relay) to provide the paging information. The PC5-RRC message transmitted by the last relay UE or by the intermediate relay UE contains at least one or multiple paging record(s) associated with intermediate relay UE(s) and/or remote UE(s). Re-use UuMessageTransferSidelink as the PC5-RRC message transmitted by the Last relay or by the intermediate relay UE that provides paging record to the child UE(s). When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain the SI required by it or requested by the remote UE by requesting SI from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink). FFS if it can also receive it directly from SIB broadcast by the last relay UE’s serving cell on Uu (when the intermediate relay UE is in coverage of the same cell that serves the last relay UE). The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay), following legacy remote UE behaviour: • when there is a change in the SI required by the intermediate UE for its own use (as a remote UE) • when the intermediate UE enters RRC_IDLE/RRC_INACTIVE • when the intermediate UE enters RRC_CONNECTED (to cancel a previously sent SI request; if a child node requested the SI, the intermediate UE acquires it by dedicated signalling) The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay): • upon reception of new/changed required SI received from a remote UE/child relay UE (where the concerned SI was not previously requested) • FFS when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay. The intermediate relay UE can send SI (e.g., in UuMessageTransferSidelink) to a child node: a) Upon reception of SI received from intermediate relay or last relay containing SI requested by a child node (intermediate relay or remote UE) b) Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE) c) Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE) d) Upon reception of SIB1 received from a parent relay (i.e., this case may correspond to SIB1 update detected by the last relay, or unsolicited SIB1 forwarding by the last relay) e) Upon receiving updated SIB1 from the network (as in Rel17) f) Upon unsolicited SIB1 forwarding to a connected child node (intermediate relay UE or remote UE) FFS if b), c), and e) are limited to RRC_CONNECTED relay UE or apply in all states. FFS whether to consolidate some of the conditions in stage 3 specification text. If multiple remote UEs are connected to a single intermediate relay UE, the multiple remote UEs must connect through the same upstream path (same parent relay UE(s)). An intermediate relay UE in RRC_CONNECTED releases its paging-related information in the parent UE at least for itself. FFS whether it also releases it for the child UEs and how the child UEs receive the paging when the intermediate relay UE is in RRC_CONNECTED. When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain paging from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink). FFS if it can also receive it directly on Uu (when the intermediate relay UE is in coverage). What triggers the intermediate relay UE to request paging monitoring by the parent relay (intermediate relay or last relay) in PC5-RRC (e.g., in RemoteUEInformationSidelink)? • FFS: when there is a change in the paging information of the intermediate UE or child UE • when the intermediate UE enters RRC_IDLE/RRC_INACTIVE • when the intermediate UE enters RRC_CONNECTED (to cancel paging monitoring request) • upon reception of paging monitoring request from a remote UE/child relay UE • FFS: upon change in the ability of the intermediate UE to monitor paging on Uu (e.g., moving in/out of coverage) to initiate/cancel paging monitoring by the parent relay). The intermediate relay UE sends paging message (e.g., in UuMessageTransferSidelink) to a child node • Upon reception of paging message received from a parent node (intermediate relay or last relay) that is intended for a child node (intermediate relay or remote UE) • FFS upon acquisition (from the network) of paging message that is for a child node (intermediate relay or remote UE) In multi-hop L2 U2N relay, besides the agreed remote UE ID and BEARER ID, in addition, at least include D/C field in the SRAP PDU header. In multi-hop L2 U2N relay, for SRAP header, at least support the legacy single-hop U2N SRAP header, i.e., the agreed UE ID field is used for the remote UE local ID (including the case of an intermediate relay UE acting as a remote UE). FFS on whether to have one additional format to include remote UE L2 ID and what use case it is applied for. Each Layer-2 Intermediate Relay UE has a single PC5 SRAP entity.

In this paper, we discuss how to introduce multi-hop relay.

Discussion

  - Configuration for multi-hop relaying

RRC connection maintenance

RAN2 needs to discuss RRC connections between UEs and NW, and PC5 RRC connections between UEs. In the last meeting, RAN2 reached following agreements;

Agreements:

The following connections are assumed as a baseline to be needed:

-	From last Relay UE perspective:

A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and

A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED.

-	From intermediate relay UE perspective (including first relay UE):

A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and

An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED.

FFS what RRC states are supported for the intermediate relay UE.

-	From U2N Remote UE perspective:

A direct (non-relayed) PC5 connection with Intermediate Relay UE, and

An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED.

According to the agreements, we can illustrate the connections as shown in figure 1.

Figure1. RRC connections between UEs and between UEs and gNB for two-hop U2N relay

Next, RAN2 needs to discuss whether the legacy U2N relay mechanism can work on this architecture.

In this phase, we should discuss whether the current mechanism can be reused. We discuss following contents in this paper:

- Paging delivery

- SIB delivery

- RRC connection establishment

- SRAP configuration for C-plane and U-plane

- QoS handling

- Failure handling

Firstly, current SIB delivery is supported by following mechanism [2]:

- The remote UE in RRC_IDLE or RRC_INACTIVE can inform the interested SIB(s) to the connected relay UE and receive the SIB(s) from the relay UE via PC5-RRC message.

- The remote UE in RRC_CONNECTED receives SIB1 and other SIBs by dedicated RRC message and performs on-demand SI request.

- For SIB1 forwarding, for L2 U2N Remote UE, both request-based delivery and unsolicited forwarding are supported by L2 U2N Relay UE. If SIB1 changes, for L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE, the L2 U2N Relay UE always forwards SIB1.

And RAN2 made some agreement on SIB delivery in multi-hop relay, according to the agreements:

- Remote UE in CONNECTED ca n receive SI directly via E2E RRC message.

- Remote UE in IDLE/ INACTIVE can request SI using PC5-RRC signalling.

- Remote UE in IDLE/ INACTIVE can receive the required SI from PC5-RRC signalling.

- etc.

and there were some FFS as below.

- FFS on the need for including a “remote UE ID” in the request.

- FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.

- FFS if it can also receive it directly from SIB broadcast by the last relay UE’s serving cell on Uu (when the intermediate relay UE is in coverage of the same cell that serves the last relay UE).

- FFS when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay.

Here, we discussed on above FFS.

- Whether the intermediate relay UE can directly receive required SIB by the cell on Uu

In Rel-17, we think the behaviour of remote UE receiving SIB directly on the Uu interface is not specified. On the other hand, there is no statement restricting this behaviour. And we assume the intermediate relay UE also acts as a remote UE. Therefore, Rel-17 behaviour of remote UE should be applied. So, we propose that direct reception of SIBs does not need to be restricted or specified.

Proposal 1. Direct reception of SIBs on Uu by the intermediate relay UE does not need to be restricted or specified as with Rel-17 remote UE.

- Whether the intermediate relay UE send an indication to a parent UE when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu to initiate/cancel SI forwarding by the parent relay

As mentioned above, we assume that direct reception of SIBs on Uu by the intermediate relay UE is not specified. Furthermore, the intermediate relay UE should be located near the cell edge. If it is trigger of the request, the intermediate relay UE may transmit a request frequently since the UE frequently enter and exit the cell. Therefore, it should not be trigger of transmission of the request.

Observation 1. In cell edge scenario, intermediate relay UE may transmit a request frequently if the change of availability of the SI is the trigger of the request.

Proposal 2. The intermediate relay UE doesn’t send SI request in PC5-RRC to the parent relay when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu.

- Whether the UE ID should be included in the SIB request

The parent UE needs to correctly understand which SIBs are requested by multiple child UEs, and performs forwarding processing based on that request. Some companies propose that UE ID is useful for this purpose. However, we think there is no need to explicitly indicate UE ID to the parent UE. We will describe how to request and forward required SIBs by using figure 2.

Step 0. UE#1, UE#2 and UE#3 have their own wish lists.

Step 1. UE#1 sends a request to UE#2. UE#2 stores the information related to the request, i.e., makes internal routing table based on who transmitted the request.

Step 2. UE#2 sends a request including information related to its own wish list and UE#1’s request to UE#3. UE#3 stores the information related to the request, i.e., makes internal routing table based on who transmitted the request.

Step 3. UE#3 sends a request including information related to its own wish list and UE#2’s request (that also includesUE#1’s request) to UE#4. UE#4 stores the information related to the request, i.e., makes internal routing table based on who transmitted the request.

Step 4. UE#4/3/2/1 receive requested SIBs and forward the SIBs based on the internal routing table.

Figure 2. an example of SIB delivery mechanism in multi-hop relaying

Therefore, we think UE ID is not needed in SIB request.

Observation 2. Relay UEs can forward SIBs based on the internal routing table as shown in figure 2.

Proposal 3. UE ID is not required in SIB request message and SIB delivery message.

- Whether the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI.

According to the current RRC spec, L2 U2N relay UE sends SI via UuMessageTransferSidelink upon acquisition of the SIB or upon receiving the updated SIBs from NW, regardless of whether the L2 U2N relay UE has the latest SI requested by the L2 U2N remote UE. It seems waste if the L2 U2N relay UE has the latest SI. In multi-hop case, compared to the single-hop case, we can evaluate the need for additional features based on the following criteria:

- Latency: The latency to get the latest SIB is only related to the number of hops. (No significant impact)

- Signalling overhead: If the first relay UE has the latest SI requested by the child UE, there is only one PC5-RRC message to send the SI.

From a signalling overhead perspective, the intermediate relay UE should respond directly if it has the requested SI. However, there is a large UE impact. Since UE applies SI to UE configuration upon receiving it, UE doesn’t store the latest SI as it is. If an intermediate relay UE needs to forward it to a child UE, the intermediate relay UE applies it and forward the SI container. And then, after processing the SI, UE will remove the SI. It means that the intermediate relay UE needs to store or create the latest SI for (potential) SI requests to respond SI directly to a child UE. Therefore, we suggest that Responding requested SI directly by an intermediate relay UE is not supported.

Observation 4. From a signalling overhead perspective, responding SI directly if it has the requested SI is reasonable. However, an intermediate relay UE needs to store or create the latest SI to respond directly.

Proposal 4. Responding requested SI directly by an intermediate relay UE is not supported.

Secondly, current paging delivery is supported by following mechanism [2]:

- Method 1: The L2 U2N Relay UE monitors POs of its connected L2 U2N Remote UE(s) if the active DL BWP of the L2 U2N Relay UE is configured with common search space including paging search space.

- Method 2: The delivery of the L2 U2N Remote UE's paging can be performed through a dedicated RRC message from the gNB to the L2 U2N Relay UE. The dedicated RRC message for delivering L2 U2N Remote UE paging to the RRC_CONNECTED L2 U2N Relay UE may contain one or more Remote UE IDs (5G-S-TMSI or I-RNTI).

As with SIB delivery, some agreements were made and some FFS remained.

- FFS whether the intermediate relay UE monitors PO for child UEs by the cell on Uu.

- FFS whether the intermediate relay UE sends an indication to a parent UE when there is a change in the ability of the intermediate UE to monitor POs on Uu to initiate/cancel POs monitoring by the parent relay.

As with SIB delivery, monitoring POs on Uu performed by the intermediate relay UE does not need to be restricted or specified. And change of availability of monitoring POs on Uu should not trigger the PO monitoring request/cancel.

Proposal 5. Monitoring POs on Uu performed by the intermediate relay UE does not need to be restricted or specified as with Rel-17 remote UE.

Proposal 6. Change of availability of monitoring POs on Uu should not trigger the PO monitoring request/cancel.

- FFS whether the intermediate relay UE also releases paging information for the child UEs in a parent UE and how the child UEs receive the paging when the intermediate relay UE is in RRC_CONNECTED.

For the paging information of the remote UE, the intermediate relay UE can receive paging information for the remote UE via dedicated RRC message. And an RRC message is forwarded at SRAP layer of a last relay UE (without reaching the RRC layer). Therefore, we think it is better to send paging record via dedicated RRC signaling. Thus, if there is no concern about forcing the network to send paging records for remote UE by dedicated RRC signaling, the intermediate relay UE that is RRC_CONNECTED releases paging related information for the remote UE in a parent UE.

Observation 3. Paging delivery by using hop-by-hop PC5-RRC message should have a large delay.

Proposal 7. if there is no concern about forcing the network to send paging records for remote UE by dedicated RRC signaling, the intermediate relay UE that is RRC_CONNECTED releases paging related information for the remote UE in a parent UE.

- Whether the reception of paging records from NW via RRC signalling for child UEs should be supported

There is no reason to restrict receiving SIB via dedicated RRC signalling as with Rel-17.

Proposal 8. The intermediate relay UE in RRC_CONNECTED can receive paging record(s) by dedicated RRC signalling for child UE(s).

And the intermediate relay UE should send paging message upon receiving the paging message for the child UE if it can receive the paging message by dedicated RRC signalling.

Proposal 9. The intermediate relay UE should send paging message upon receiving the paging message for the child UE if it can receive the paging message by dedicated RRC signalling.

SRAP configuration

In RAN2#127, RAN2 agreed with following protocol stack for control plane as below.

Figure 3. Agreed protocol stack for C-plane/U-plane in multi-hop relay.

In the last meeting, RAN2 agreed that an intermediate relay UE has single SRAP entity. In U2U relay, U2U relay UE also has single PC5-SRAP entity. Therefore, RAN2 should update the agreement and the protocol stack as below:

Figure 4. updated protocol stack for C-plane/U-plane in multi-hop relay

Proposal 10. Update the agreement and the protocol stack for C-plane and U-plane as shown in Figure 4.

SRAP layer supports the routing function, i.e., determination of egress RLC channel/link, determination of UE/BEARER ID field and data transfer. For routing function, gNB configures relay UE(s) and remote UE with local UE ID and mapping rule between E2E RB and RLC channel. Then, the relay UE(s) and remote UE determines the route of the packet based on the SRAP header of the packet. There are two types of SRAP header, one is for U2N SRAP data PDU and another one is for U2U SRAP data PDU. RAN2 already agreed that single local UE ID (L2 U2N relay) format is used. And the value of BEARER ID refers appropriate bearer identity. It means that, in case of U2N relay, the value refers srb-Identity or drb-Identity (minus 1). In case of U2U relay, the value refers SL-SRB identity or slrb-PC5-ConfigIndex, the dedicated values should be used for SL-SRB 0/1/2/3 and other values are used for SL-DRBs. The reason why dedicated values are used for SL-SRBs in U2U relay is that both-side hop is PC5-RLC channel. In the multi-hop relay case, the situation at an intermediate relay UE is same with U2U relay. Therefore, the value of BEARER ID 0,1,2 should be used for E2E SRB 0,1,2.

Proposal 11. The value of BEARER ID 0,1,2 should be used for E2E SRB 0,1,2 as with U2U relay.

It is FFS whether the additional L2 ID format should be supported. In Rel-17/18 U2N relay, multiple RRC establishment procedures are not processed simultaneously. If only one remote UE performs RRC establishment procedure for the path, it should work without L2 ID format. If an instantaneous mesh network is an important use case, the L2 ID format may be useful because traditional methods have high connection latency (multiple paths need to be processed sequentially to connect). However, such use cases are not specified, and it is important to support robust and (relatively) static on-tree networks. Therefore, SRAP header including L2 ID is not introduced in Rel-19.

Proposal 12. SRAP header including L2 ID is not introduced in Rel-19.

And, as with legacy, to avoid the local ID collisions, gNB should allocate local ID for each remote UE for managing sources of UL data and destinations of DL data.

Proposal 13. gNB should allocate local ID for remote UE(s) for SRAP configuration.

QoS handling

The latency requirement is important when we consider the QoS handling. Especially, some use cases, e.g., public safety, wearable and V2X, needs very low E2E latency. Therefore, optimization should be done to achieve low latency.

Proposal 14. QoS related optimization should be considered since very low E2E latency is required for some use cases.

When we consider QoS related optimization, it is important to know achievable QoS at each hop. Here, gNB can guarantee at least one hop (i.e., last relay hop) QoS if gNB allocate sidelink and/or uplink grant for the last relay UE using resource allocation mode 1. Thus, we propose that the principles of resource allocation for U2N relay operation follow the legacy mechanism.

Proposal 15. The principles of resource allocation follow the legacy mechanism, i.e., last relay can be configured with mode 1 resource allocation, and remote UE and other relay UEs are configured with mode 2 resource allocation.

Observation 4. gNB can guarantee at least one hop (i.e., last relay hop) QoS using mode 1 resource allocation or UL grant allocation.

In U2U relay, U2U relay UE split the E2E QoS for each hop, and then each UE configures each hop configuration to guarantee the split QoS. Because U2U relay UE knows quality of both hops and can configure appropriate values. In multi-hop relay, there is no special UE like a U2U relay UE. However, if gNB knows quality of each hop, gNB can configure appropriate QoS for each hop and also select appropriate indirect path with good quality. Thus, we propose that the gNB sets the appropriate (split) QoS to be met for each hop and UE reports hop-by-hop link quality to gNB for path selection and split QoS configuration.

Observation 5. If gNB knows quality of each hop, gNB can configure appropriate QoS for each hop and also select appropriate indirect path with good quality.

Proposal 16. UE reports hop-by-hop link quality to gNB for path selection and split QoS configuration.

- 2.2	Failure handling

We consider how to set the contents of NotificationMessageSidelink when the UE received the notification. One option is setting the same indicationType with the received notification. However, the UE may perform relay reselection due to PC5-RLF indication. In this case, UE receiving the NotificationMessageSidelink including the same indicationType cannot understand whether the event indicated by the indicationType is occurring in the UE sent the notification, or the UE simply reflecting it. If UE doesn’t have to perform RRC re-establishment, such information is important to determine UE behavior. Then, another option is setting specific indicationType (e.g., receiving_Notification, perform_reestablish) for multi-hop.

Proposal 17. RAN2 discusses whether the UE received NotificationMessageSidelink sets the same indicationType with the received notification or multi-hop specific indicationType.

- 2.3	Others

In Rel-17, remote/relay UE sends SUI to gNB for resource acquisition including UE-type information. And in Rel-18, U2U specific UE-type is introduced. In multi-hop case, the last relay UE should use the ue-Type as relayUE, and the remote UE should set the ue-Type as remoteUE. For the intermediate relay UE, RAN2 should discuss whether the new UE type is specified, or legacy UE type (remoteUE or relayUE) is reused. In our understanding, a CONNECTED intermediate relay UE is always a remote UE for receiving/sending RRC messages from/to gNB. But the intermediate relay UE may or may not have its own UP data. Therefore, new UE type should be introduced for an intermediate relay since it has two roles.

Proposal 18. New UE type in SUI is introduced for an intermediate relay UE.

In Rel-18 multi-path scenario, RAN2 agreed to assume that a multi-path relay UE is not in IDLE/INACTIVE state. Since the multi-path remote UE is always in CONNECTED, the MP relay UE should be in CONNECTED. In case of multi-hop relay, a last relay UE and intermediate relay UEs should not be in IDLE/INACTIVE while a remote UE is in CONNECTED. In MAC specification, there is a mechanism for autonomous entering to IDLE state i.e., dataInactivityTimer. The dataInactivityTimer is not applicable STCH but there is no restriction to configure it with sidelink UE. Here, we suggest discussing whether RAN2 assumes or specifies NW ensures that an intermediate relay UE is not configured with dataInactivityTimer.

Proposal 19. Discuss whether RAN2 assumes or specifies NW ensures that an intermediate relay UE is not configured with dataInactivityTimer.

Conclusion

In this paper, we made following observations and proposals:

Proposal 1. Direct reception of SIBs on Uu by the intermediate relay UE does not need to be restricted or specified as with Rel-17 remote UE.

Observation 1. In cell edge scenario, intermediate relay UE may transmit a request frequently if the change of availability of the SI is the trigger of the request.

Proposal 2. The intermediate relay UE doesn’t send SI request in PC5-RRC to the parent relay when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu.

Observation 2. Relay UEs can forward SIBs based on the internal routing table as shown in figure 2.

Proposal 3. UE ID is not required in SIB request message and SIB delivery message.

Observation 4. From a signalling overhead perspective, responding SI directly if it has the requested SI is reasonable. However, an intermediate relay UE needs to store or create the latest SI to respond directly.

Proposal 4. Responding requested SI directly by an intermediate relay UE is not supported.

Proposal 5. Monitoring POs on Uu performed by the intermediate relay UE does not need to be restricted or specified as with Rel-17 remote UE.

Proposal 6. Change of availability of monitoring POs on Uu should not trigger the PO monitoring request/cancel.

Observation 3. Paging delivery by using hop-by-hop PC5-RRC message should have a large delay.

Proposal 7. if there is no concern about forcing the network to send paging records for remote UE by dedicated RRC signaling, the intermediate relay UE that is RRC_CONNECTED releases paging related information for the remote UE in a parent UE.

Proposal 8. The intermediate relay UE in RRC_CONNECTED can receive paging record(s) by dedicated RRC signalling for child UE(s).

Proposal 9. The intermediate relay UE should send paging message upon receiving the paging message for the child UE if it can receive the paging message by dedicated RRC signalling.

Proposal 10. Update the agreement and the protocol stack for C-plane and U-plane as shown in Figure 4.

Proposal 11. The value of BEARER ID 0,1,2 should be used for E2E SRB 0,1,2 as with U2U relay.

Proposal 12. SRAP header including L2 ID is not introduced in Rel-19.

Proposal 13. gNB should allocate local ID for remote UE(s) for SRAP configuration.

Proposal 14. QoS related optimization should be considered since very low E2E latency is required for some use cases.

Proposal 15. The principles of resource allocation follow the legacy mechanism, i.e., last relay can be configured with mode 1 resource allocation, and remote UE and other relay UEs are configured with mode 2 resource allocation.

Observation 4. gNB can guarantee at least one hop (i.e., last relay hop) QoS using mode 1 resource allocation or UL grant allocation.

Observation 5. If gNB knows quality of each hop, gNB can configure appropriate QoS for each hop and also select appropriate indirect path with good quality.

Proposal 16. UE reports hop-by-hop link quality to gNB for path selection and split QoS configuration.

Proposal 17. RAN2 discusses whether the UE received NotificationMessageSidelink sets the same indicationType with the received notification or multi-hop specific indicationType.

Proposal 18. New UE type in SUI is introduced for an intermediate relay UE.

Proposal 19. Discuss whether RAN2 assumes or specifies NW ensures that an intermediate relay UE is not configured with dataInactivityTimer.

Reference

- R2-2501334, Report from session on positioning and sidelink relay

- TS 38.300