3GPP TSG-RAN-WG2 Meeting #129bis	R2-2502382

Wuhan, China,7th– 11th Apr, 2025

Agenda item	:		8.6.3

Source	: 	Sharp

Title	: 	Discussion on issues for supporting L1 event triggered measurement reporting

Document for	:	Discussion and Decision

Introduction

In this paper, we discuss the issue related to L1 event triggered measurement reporting.

Discussion

MR triggered by entering condition

In RAN2#127bis, RAN2 agreed that MR MAC CE includes basic information i.e. (1) Beam information (i.e. SSBRI or CRI), (2)Beam quantity and (3)Triggered event information. In RAN2#128, RAN2 agreed that CRI and SSBRI are used to present the candidate beam ID. In RAN2#129, RAN2 agreed that R18 differential RSRP encoding is used as baseline. Based on these agreements, we can consider the MR MAC CE format as below:

| Report Config ID |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| RSRI(1) |  |  |  |  |  | Information about beam#1 |
|  | Absolute RSRP(1) |  |  |  |  |  |
| RSRI(2) |  |  |  |  |  | Information about beam#2 |
|  | Diff RSRP(2) |  |  |  |  |  |
| ・・・ |  |  |  |  |  |  |
| RSRI(n) |  |  |  |  |  | Information about beam#n |
|  | Diff RSRP(n) |  |  |  |  |  |
| Figure 1. MR MAC CE format |  |  |  |  |  |  |

In the RAN2#129, RAN2 made following agreement:

10. Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

According to the above agreement, MR MAC CE needs to include the explicit indication to inform which beam(s) meets the event condition with TTT, but detailed signalling is FFS and should be discussed. In our understanding, we can consider two types of explicit indication:

Alternative 1. Using flag which indicate that this beam meets the event condition with TTT or not.

Alternative 2. MR MAC CE includes the beam meets the event condition with TTT on top of the MR MAC CE and indicates the number of such beams.

Observation 1. RAN2 consider the two alternatives for explicit indication:

- Alternative 1. Using flag which indicate that this beam meets the event condition with TTT or not.

- Alternative 2. MR MAC CE includes the beam meets the event condition with TTT on top of the MR MAC CE and indicates the number of such beams.

For alt1, considering the available space for each information about beam, it is easy to introduce the 1 bit flag. For alt2, if max number of beams included in MR MAC CE is 16 as MR UCI, total 4 bits is needed and given the remaining space, it may be necessary to add an additional octet. In addition, for alt 2, if a beam which meets the event condition with TTT has lower RSRP than a beam which does not meet the event condition with TTT, differential RSRP can be negative number. This is undesirable because it complicates the system. Based on these reasons, we prefer the alt 2.

Proposal 1. MR MAC CE includes the 1 bit flag field indicating whether the beam meets the event condition with TTT or not for each beam should be introduced in MR MAC CE.

Conclusion

Observation 1. RAN2 consider the two alternatives for explicit indication:

- Alternative 1. Using flag which indicate that this beam meets the event condition with TTT or not.

- Alternative 2. MR MAC CE includes the beam meets the event condition with TTT on top of the MR MAC CE and indicates the number of such beams.

Proposal 1. MR MAC CE includes the 1 bit flag field indicating whether the beam meets the event condition with TTT or not for each beam should be introduced in MR MAC CE.

Reference