3GPP TSG RAN WG2 #129bis	R2-2502397

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.8.2

Source: 	ITRI

Title: 	Discussion on supporting location/time-based SMTC selection

Document for:	Discussion

1	Introduction

RAN2 assumes different SSB periodicities per frequency may be configured among neighbouring cells to cope with extended SSB periodicity (up to 160ms) [1].

In this contribution we discussed the related SMTC impacts for UE in RRC_IDLE/RRC_INACTIVE and RRC_CONNECTED states. Location-based and/or time-based SMTC selection for UE to perform measurements for neighbour cells are investigated and proposals are provided accordingly.

2	Discussion

RAN2 discussed the impacts of extended SSB periodicity (up to 160 ms) for NTN DL coverage enhancements, and achieved the following agreements in RAN2 #129 meeting [1]:

Agreements (RAN2 #129): 1. RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer 2. RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame 3. RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell) 4. The number of SMTC/gaps a UE needs to consider at any time will not be increased further

And the following common understands are captured in the meeting report [1]:

- From RAN2 perspective, we support option a (clustered cells illuminated) and option b (scattered cells illuminated) for further discussion on SMTC. No LS is sent to RAN1.

- RAN2 observes that if the cells active simultaneously are in clusters, existing SMTC mechanism (a maximum of 4 SMTCs per frequency) may be sufficient (FFS if any solution identified for option b applies to option a as well).

- RAN2 observes that if the cells active simultaneously are scattered, (for the case of intra-frequency neighbouring cells), the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough.

2.1 	SMTC impacts

According to RAN2 agreements, neighbour cells of the same frequency may have different SSB periodicities. Therefore, it is feasible that the surrounding neighbour cells could be deployed on the same frequency and be served by different satellites with different SSB periodicities.

Observation 1: Neighbour cells could be served by different satellites and be deployed with different SSB periodicities.

Take Figure 1 as an example, NGSO satellites may serve a location in sequence. Each satellite may serve multiple NTN cells and the cells may be quasi-earth fixed or earth-moving deployed. The SSB of the NTN cells could be deployed on the same frequency location. NTN cells served by the same satellite have the same SSB periodicity. NTN cells served by different satellites may have different SSB periodicities.

Figure 1. Neighbour cells with different SSB periodicities

In Figure 2, circles in different color indicates cells of different SSB periodicities. Assuming a serving cell S has 6 intra-frequency neighbour cells and the 6 neighbour cells possess 3 different SSB periodicities.

Figure 2. The surrounding neighbour cells of a NTN cell

SMTC provided by System Information would be applied by UE in RRC_IDLE/RRC_INACTIVE state for neighbour cell measurements. The propagation delay between a UE and the serving cell may be different from that between the UE and a neighbour cell, the UE may adjust SMTC offset value associated with the neighbour cell before SMTC setup [2] (e.g., according to UE location and the ephemerisInfo-r17 associated with the PCI of the neighbour cell in SIB19).

Observation 2: NTN UE may need adjusting SMTC offset values for the PCI included in the SMTC elements.

Considering the SSB periodicity could be up to 160 ms, the offset value difference of two cells served by the same satellite may be larger than 5 ms that the two cells could not be detected in the same SMTC duration.

Observation 3: The difference between the offset values of two cells served by the same satellite may be larger than 5 ms that could not be detected by a UE in the same SMTC duration.

The SMTC provided by System Information may take account of the cells that would not be detectable by a UE (e.g., the cell may be far from the UE location, or the cell may already move away or not yet move to serve the location of the UE).

For example, in Figure 3 the SSB in the same colour are transmitted by the cells served by the same satellite and have the same SSB periodicity. Cell 1 and cell 2 would stop serving by T1, and neighbour cell 6 would not serve the UE location until T2. Cell 6 would not be detectable by the UE until T2, and cell 1 and cell2 would not be detectable by the UE after T1.

Figure 3. Intra-frequency SSB transmitted by different cells

Observation 4:  Network may configure multiple SMTC per frequency for RRC_IDLE/RRC_INACTIVE UE to perform neighbour cell measurements even the neighbour cell is not detectable by the UE (e.g., the neighbour cell is not starting serving the area, or the neighbour cell is far from the UE location).

NTN UE may support setting up 2 or 4 SMTC per frequency regarding UE capabilities. Though it is possible for neighbour cell measurements that the network may configure multiple SMTC elements per frequency, or a UE could adjust SMTC offset value of each of the neighbour cell autonomously, the actual SMTC could be set up is limited to UE capability. It is benefit for UE to select SMTC for neighbour cell measurements.

Observation 5: UE may support setting up 2 or 4 SMTC per frequency regarding UE capabilities. It is benefit for UE to select SMTC for neighbour cell measurements.

Proposal 1: It is proposed to support SMTC selection for UE in RRC_IDLE/RRC_INACTIVE state.

2.2 	SMTC setup for RRC_IDLE/RRC_INACTIVE UE

The coverage of a NTN cell could be large. A UE may only need to perform measurements of a portion of configured neighbour cells, and the neighbour cells a UE in RRC_IDLE/RRC_INACTIVE need to measure may change with time.

Figure 4. The detectable neighbour cells may change with time

For example in Figure 4, the neighbour cells to be measured are cell 1 and cell 2 at T1, and the neighbour cells to be measured are cell 5 and cell 6 at T2 when the UE moves in quasi-earth fixed case or when the satellite moves in earth-moving deployment. It is benefit for UE in RRC_IDLE/RRC_INACTIVE state to select SMTC for neighbour cell measurements with taking account of UE location.

Observation 6: Location-based SMTC selection could be benefit for UE in RRC_IDLE/RRC_INACTIVE state to perform neighbour cell measurements.

In another case, a UE may select to set up SMTC associated with upcoming cells rather than SMTC associated with leaving cells, or may select to set up SMTC associated with cells starting to serve the area to cope with UE capability of supported SMTC number per frequency.

Observation 7: Time-based SMTC selection could be benefit for UE in RRC_IDLE/RRC_INACTIVE state to perform neighbour cell measurements.

Location-based and/or time-based SMTC selection could benefit RRC_IDLE/RRC_INACTIVE UE in selecting SMTC to set up for neighbour cell measurements. Therefore, it is proposed to support RRC_IDLE/RRC_INACTIVE UE to perform location-based and/or time-based SMTC selection for neighbour cell measurements.

Proposal 2: it is proposed to support RRC_IDLE/RRC_INACTIVE UE to perform location-based and/or time-based SMTC selection for neighbour cell measurements.

2.3 	SMTC setup for RRC_CONNECTED UE

Network could obtain the location and the capabilities of a RRC_CONNECTED state UE. It is possible for a serving cell to configure measurement objects associated with neighbour cells that are detectable to UE. Network could configure smtc1, smtc2 or smtc4List to RRC_CONNECTED UE for both intra-frequency and inter-frequency neighbour cell measurements. The existing SMTC mechanism is sufficient for measurement configuration to RRC_CONNECTED UE. Therefore, no SMTC enhancement is required for RRC_CONNECTED state UE.

Observation 8: The existing SMTC mechanism is sufficient for neighbour cell measurement configuration of RRC_CONNECTED state UE. No SMTC enhancement is required for RRC_CONNECTED state UE to perform neighbour cell measurements.

Proposal 3: No SMTC enhancement is needed for RRC_CONNECTED UE.

3	Conclusion

In this contribution we provide discussions of SMTC impacts regarding extended SSB periodicity with the following observations:

Observation 1: Neighbour cells could be served by different satellites and be deployed with different SSB periodicities.

Observation 2: NTN UE may need to adjust SMTC offset values for the PCI included in the SMTC elements.

Observation 3: The difference between the offset values of two cells served by the same satellite may be larger than 5 ms that could not be detected by a UE in the same SMTC duration.

Observation 4:  Network may configure multiple SMTC per frequency for RRC_IDLE/RRC_INACTIVE UE to perform neighbour cell measurements even the neighbour cell is not detectable by the UE (e.g., the neighbour cell is not starting serving the area, or the neighbour cell is far from the UE location).

Observation 5: UE may support setting up 2 or 4 SMTC per frequency regarding UE capabilities. It is benefit for UE to select SMTC for neighbour cell measurements.

Observation 6: Location-based SMTC selection could be benefit for UE in RRC_IDLE/RRC_INACTIVE state to perform neighbour cell measurements.

Observation 7: Time-based SMTC selection could be benefit for UE in RRC_IDLE/RRC_INACTIVE state to perform neighbour cell measurements.

Observation 8: The existing SMTC mechanism is sufficient for neighbour cell measurement configuration of RRC_CONNECTED state UE. No SMTC enhancement is required for RRC_CONNECTED state UE to perform neighbour cell measurements.

And the proposals are provided accordingly:

Proposal 1: It is proposed to support SMTC selection for UE in RRC_IDLE/RRC_INACTIVE state.

Proposal 2: it is proposed to support RRC_IDLE/RRC_INACTIVE UE to perform location-based and/or time-based SMTC selection for neighbour cell measurements.

Proposal 3: No SMTC enhancement is needed for RRC_CONNECTED UE.

4	References

[1] R2-250xxxx	Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece.

[2] 3GPP TS 38.331 V18.4.0: "NR; Radio Resource Control (RRC); Protocol specification".