3GPP TSG-RAN WG2 Meeting #129bis	R2-2502398

Wuhan, China, 7th – 11st April 2025

Agenda Item:	8.7.4.1

Source:	ITRI

Title:	Discussion on enhanced LCP for XR

Document for:	Discussion and Decision

1 Introduction

In RAN2 #129, the following agreements of enhanced LCP had been reached.

- Only one additional priority is configured to an LCH for LCP enhancement.

- We keep an existing agreement (remaining time threshold is configured per LCH)

- There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority.

- Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact

- FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination

- No additional PBR is needed for priority adjusted data

- FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold..

In this document, we discuss the two FFSs, impact on Schedulling Request (SR) priority determination during intra-UE prioritization, and impact on Bj. Since RAN2 had agreed multiple reporting ports for Rel-19 Delay Status Reporting (DSR), and UE may transmit a Rel-19 DSR in the following UL grant for limited PUSCH resource case, we think that an optimization on the SR priority determination caused by adjusted priority is not needed during intra-UE prioritization. As to the Bj relaxation, because we care about the fairness of Logical Channel Prioritization (LCP) more than the unsured capacity gain,  the Bj relaxation for logical channel (LCH) with priority-adjusted data is suggested not to be considerd in Rel-19 LCP.

2 Discussion

2.1 Impact on SR priority determination during intra-UE prioritization

We wonder how much gain SR priority determination will do during intra-UE prioritization. We understand that some companies thought that SR priority determination needs to be specified during intra-UE prioritization as this allows the network to know which priority is applied by the UE in different situations. However, the SR is used for requesting UL-SCH resources for new transmission, and for a LCH, at most one PUCCH resource for SR is configured per BWP. If the optimization of SR priority determination during intra-UE prioritization is really necessary, for example, there is an SR collision during intra-UE prioritization, maybe we can think about based on simplicity that a specific SR configuration for DSR is configured.

However, we should note that when an SR is triggered, there is actually no LCP performed simultaneously. On the other hand, RAN2 had agreed multiple reporting ports for Rel-19 DSR, and UE may transmit Rel-19 DSR in the following UL grant for limited PUSCH resource case. From our point of view, an optimization on the SR priority determination caused by adjusted priority is not needed during intra-UE prioritization.

Proposal 1: Not to consider SR priority determination caused by adjusted priority during intra-UE prioritization.

2.2 Impact on Bj

We worried about the fairness issue of LCP with Bj relaxation. We understand that if an LCH with an upgraded priority is allowed to be transmitted even if Bj is negative, while the remaining time is less the configured threshold, these delay-critical data may be multiplexed into a MAC PDU as soon as possible and transmitted in time.

However, if the data volume of these delay-critical data is very large, the Bj relaxation may cause a severe penalty to LCHs which priorities are higher than the default priority of the LCH with delay-critical data but lower than the corresponding upgraded priority. Some companies may argue that the adjusted priorities are configured by network individually so that it had reduced the possible negative impacts to the LCP fairness. But until now, there are only scenario analyses without performance analyses of capacity gain and LCP fairness. In addition, for LCHs with lower default priorities, it can also cause starvation if Bj relaxation is allowed for an LCH with priority-adjusted data. From our point of view, it is enough for XR traffic that each LCH is configured by network with an appropriate PBR, a default priority and an adjusted priority in Rel-19 LCP.

Proposal 2: Not to consider the Bj relaxation for LCH with priority-adjusted data in Rel-19 LCP.

3 Conclusion

Based on the discussion in the previous section we propose the following:

Proposal 1: Not to consider SR priority determination caused by adjusted priority during intra-UE prioritization.

Proposal 2: Not to consider the Bj relaxation for LCH with priority-adjusted data in Rel-19 LCP.

4 References

- R2_129 R18 MBS, R18 QoE and R19 XR session notes (Dawid)_EOM.docx